3GPP TSG-RAN5 Meeting # 107	R5-251945r1

# St.Julian, Malta,  19th May 2025 – 23th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3675 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR NTN: Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is an error in a conversion of a t value into NtnUeSpecificTA for the NGSO scenario. This should be corrected in order to achieve the UE synchronisation with simulated NTN cell. - There is a lack of specification of NTN-specific timing handling in TTCN and SS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the values for NGSO scenario and added a Note with a clarification to the Table 7.5.2.2-1. - Added a clarification how to handle UL transmission delay in TTCN. Added clarification for configuration t-PollRetransmit value in SS SRB entities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - A conformant UE may not synchronise with the simulated NTN cell. - Lack of specification of NTN-specific timing handling in TTCN and SS will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.2, 7.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: change 2 added. |  |  |  |  |  |  |  |  |

#### 7.5.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 38.211 [19] clause 4.3.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Tc: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common = NTA,adjcommon * Tc: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific = NTA,adjUE * Tc: the TA corresponding to the service link delay.

The NTN test environment in TS 38.508-1 [5] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore, the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7.5.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 38.213 [21] clause 4.2) |
| TTA,legacy |  | InitialValue integer (0..3846) | NTA = InitialValue * 16∙642μ |
| TTA,common | NTA,adjcommon | NtnCommonTA integer (0..66485757) | See below. |
| TTA,UE-specific | NTA,adjUE | NtnUeSpecificTA integer (0..66485757) | NTA,adjUE= 8* NtnUeSpecificTA See below. |
| NOTE 1: The time unit TC is defined in TS 38.211 [19] clause 4.1. NOTE 2: NtnUeSpecificTA is expressed with the granularity of NtnCommonTA, that is 4.072 * 10^(-3) μs, as specified in TS 38.331 [16]. |  |  |  |

From the TA-Info-r17 specified in TS 38.331 [16], only the value of ta-Common-r17 is defined in TS 38.508-1 [5] SIB19 for the GSO and NGSO scenarios. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of NTA,adjcommon according to the formula given in TS 38.213 [21] clause 4.2.

NOTE:	It is assumed that the SS uses the calculation: NTA,adjcommon = 8 * NtnCommonTA.

NTA,adjUE is calculated using the NTN serving satellite static position specified in TS 38.508-1 [5] SIB19's ephemeris data and the UE static position specified in TS 38.508-1 [5] clause 4.5C1. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the NTN serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

d=∆x+∆y+∆z

with

∆x=(xsat-xUE)2

∆y= (ysat-yUE)2

∆z=(zsat-zUE)2

and

xsat, ysat, zsat being serving satellite coordinates, in m

xUE, yUE, zUE being UE coordinates, in m

To use above formulas the positioning data provided in TS 38.508-1 [3] is first converted to the coordinates accordingly.

Example of the calculation of ∆x:

given UE latitude 25.08439333 and positionX-r17 of -16976014:

xsat= -16976014* 1.3 m = -22068818.2 m

xUE= -3025296.94 m

∆x=(-22068818.2 m-(-3025296.94 m))2=(-19043521.26 m)2= 362655701980071.94 m2

Following the above calculations, the final value t for a GSO scenario results in 0.24338972709427653 sec that yields NtnUeSpecificTA to have a value of 59771543.981895022, that is rounded to 59771544.

For an NGSO scenario, the calculated final value of t results in 0.004062662259880647 sec and a corresponding NtnUeSpecificTA has a value of 997706.8418174478. The last one is rounded to 997707.

#### 7.5.2.3	NTN scheduling offsets

The NTN timing parameters listed in Table 7.5.2.3-1 come into play when scheduling data in an NTN cell.

Table 7.5.2.3-1: NTN timing parameters in an NTN cell

| NTN timing parameter | Definition | Reference |
| --- | --- | --- |
| UE-gNB RTT | TTA + Kmac | TS 38.321 [13] clause 3.1 |
| Kmac | k-Mac or 0 | TS 38.213 [21] clause 8.2 |
| Koffset | cellSpecificKoffset - Differential Koffset | TS 38.213 [21] clause 4.2 |

TTCN configures the NTN offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL.

From the TTCN implementation, Koffset is considered for every UL message, including lower layer message e.g. Scheduling Request, HARQ feedback. This shall be accounted for when calculating wait times. For example, if the UE needs to send a message but has no UL grant available, then the UE will send a Scheduling Request and after having been assigned the requested UL grant it will send the message. Therefore, Koffset is to be considered twice.

TTCN configures SS SRB RLC entities with t-PollRetransmit set to ms2000 such that it takes Koffset into account.

Note: 	t-PollRetransmit is incremented by 2 Koffset, for Scheduling Request and Status PDU transmissions, and set to the next allowed value.