3GPP TSG-RAN5 Meeting #107	R5-252164r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3462 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Table 4.7.2-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element : MS -> NW: 0017H (3GPP PS data off UE status); When the container identifier indicates 3GPP PS data off UE status, the container identifier contents field contains information of the status of 3GPP PS data off in the UE for a PDN connection where "01H" indicates ’deactivated’ and "02H" indicates 'activated’. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. From above it is clear that this IE can have either '01'O, or '02'O and the condition DATA_OFF should not be associated with whether the UE has activated or deactivated this feature. This needs ot be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Contianer ID contents. Please see below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcases based on current Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-252164r1 : Updated the clauses affected in coversheet. |  |  |  |  |  |  |  |  |

### 4.7.2	Contents of 5GSM messages

#### –	PDU session establishment request

Table 4.7.2-1: PDU SESSION ESTABLISHMENT REQUEST

| Derivation Path: 24.501 clause 8.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Any value according to TS 24.501 [25] subclause 9.4 |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION ESTABLISHMENT REQUEST message identity |  | ‘1100 0001’B |  |  |
| Integrity protection maximum data rate |  | Present but contents not checked |  |  |
| PDU session type |  | Any value between '001'B, '010'B, '011'B and ‘101’B | The allowed values are respectively IPv4, IPv6, IPv4v6 and Ethernet (EtherType as defined in IEEE 802.3) |  |
| SSC mode |  | Not checked |  |  |
| 5GSM capability |  | Not checked |  |  |
| Maximum number of supported packet filters |  | Not checked |  |  |
| Always-on PDU session requested |  | Not checked |  |  |
| Always-on PDU session requested |  |  |  | SST_URLLC |
| APSR |  | ‘1’B | Always-on PDU session requested |  |
| SM PDU DN request container |  | Not checked |  |  |
| Extended protocol configuration options |  | Not checked | The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of PDU SESSION ESTABLISHMENT ACCEPT. |  |
| Extended protocol configuration options |  | Present including at least the following container |  | DATA_OFF |
| Container ID n |  | ‘0017’H | 3GPP PS data off UE status |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | Any value between ‘01’H and ‘02’H |  |  |
| IP header compression configuration |  | Not checked |  |  |
| DS-TT Ethernet port MAC address |  | Not checked |  |  |
| DS-TT Ethernet port MAC address |  | Any value according to TS 24.501 [25] subclause 9.11.4.25 |  | Ethernet |
| UE-DS-TT residence time |  | Not checked |  |  |
| Port management information container |  | Not checked |  |  |
| Ethernet header compression configuration |  | Not checked |  |  |
| Ethernet header compression configuration |  | Any value according to TS 24.501 [25] subclause 9.11.4.28 |  | Ethernet |
| Suggested interface identifier |  | Not checked |  |  |
| Service-level-AA container |  | Not checked |  |  |
| Requested MBS container |  | Not checked |  |  |
| PDU session pair ID |  | Not checked |  |  |
| RSN |  | Not checked |  |  |
| URSP rule enforcement reports |  | Not checked |  |  |

| Condition | Explanation |
| --- | --- |
| DATA_OFF | If the UE supports 3GPP PS data off as specified in TS 38.508-2 [10], Table A.4.3.7-1/xx |
| Ethernet | If the PDU session type = '101'B. |