3GPP TSG-RAN5 Meeting #107	R5-252186r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4960 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new ATG TC 6.8.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TC 6.8.1.2 of ATG idle mode operations / Cell selection / Serving cell selection (cellBarredATG) needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 6.8.1.2 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0580 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: Update the common test environment configured in TS 38.508-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.8.1.2	ATG idle mode operations / Cell selection / Serving cell selection (cellBarredATG)

6.8.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell for ATG access and cellBarredATG in SIB1 is not present }

then { UE considers the cell as barred and no camping on the cell can take place }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell for ATG access and cellBarredATG in SIB1 is “barred” }

then { UE considers the cell as barred and no camping on the cell can take place }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell for ATG access and cellBarredATG in SIB1 is “notBarred” }

then { UE considers the cell suitable and camps on it }

}

6.8.1.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331 clause 5.2.2.4.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331 clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

...

1>	if the access is for ATG:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running; and

2>	if the cellBarredATG in the acquired SIB1 is set to barred or the cellBarredATG is not included in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

6.8.1.2.3	Test description

6.8.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

-	System information combination NR-33 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used for NR cell.

UE:

- The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5D.

Preamble:

-	The UE is in Switched OFF (State 0N-B) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.8.1.2.3.2	Test procedure sequence

Table 6.8.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | RRCSetupRequest | 1 | F |
| 3 | Power Off the UE. | - | - | - | - |
| 4 | The SS updates the SIB1 for NR Cell 1 with cellBarredATG set to "barred" | - | - | - | - |
| 5 | Power on the UE. | - | - | - | - |
| 6 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | RRCSetupRequest | 2 | F |
| 7 | Power Off the UE. | - | - | - | - |
| 8 | The SS updates the SIB1 for NR Cell 1 with cellBarredATG set to "notBarred" | - | - | - | - |
| 9 | Power on the UE. | - | - | - | - |
| 10 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | RRCSetupRequest | 3 | P |
| 11-23 | Steps 3 to 15 of the procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| 24 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |

6.8.1.2.3.3	Specific message contents

Table 6.8.1.2.3.3-1: SIB1 (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 with condition ATG |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| cellBarredATG-r18 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.8.1.2.3.3-2: SIB1 (Step 4, Table 6.8.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 with condition ATG |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| cellBarredATG-r18 |  | barred |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.8.1.2.3.3-3: SIB1 (Step 8, Table 6.8.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 with condition ATG |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| cellBarredATG-r18 |  | notBarred |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |