3GPP TSG-RAN5 Meeting #107	R5-252217r2

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1598 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eCall test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At RAN5#106 new eCall test cases 21.22, 21.23 and 21.24 have been introduced with "eCall capable" instead of "eCall over IMS" for clarification in the title. The same needs to be applied for test cases 21.1 .. 21.6. - Test case 21.13: Test procedure says "1A-1H) IMS registration according to C.2 is executed" but steps are missing in the Expected sequence. - In test cases 21.13 .. 21.18 the UE needs to acknowledge the 4XX/6XX response with a SIP ACK. - In test cases 21.13 .. 21.21 the SIP ACK needs condition A4 (ACK for non-2xx response). - The 486 Busy Here in 21.19, 600 Busy Everywhere in 21.20 and 603 Decline in 21.21 need condition A1 (Response sent by SS for INVITE for eCall over IMS). - Condition A9 (BYE sent by the SS for emergency call or eCall) is not applicable for the BYE of a test eCall. - There is no condition A5 in A.2.19 anymore and condition A3 is used per default. - The terms "eCall capable" and "eCall only" are not well defined for the eCall test cases. - Inconsistent initial conditions. - Editorial issues. - Missing clarification for eCall-only test cases that normal and emergency registration may happen in parallel. - Inconsistent references to 36.508 Tables 4.5A.27.3-1 and 4.5A.7.3-1. - Missing references to USIM/ISIM configuration in initial conditions. - For the eCall test cases, the working assumption has been made that upon eCall initiation, a UE configured for eCall only mode performs an IMS normal registration, occuring in parallel to the IMS emergency registration. However this working assumption is not documented anywhere. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Title changed for test cases 21.1 .. 21.6. - Steps 1A-1H added to test case 21.13 (similar to e.g. 21.14). - SIP ACK added to test procedure and message sequence of test cases 21.13 .. 21.18. - Specific message contents added for SIP ACK in test cases 21.13 .. 21.21. - Specific message contents added for 486 Busy Here in 21.19, 600 Busy Everywhere in 21.20 and 603 Decline in 21.21. - Specific message content for BYE removed from test case 21.26. - Conditions removed from specific message content for SIP INFO at steps 15 and 37 of test case 21.26. - Clause 21.0 added for definitions. - Initial conditions aligned. - Editorial corrections. - Clarification added to eCall-only test cases that normal and emergency registration may happen in parallel. - References to 36.508 table 4.5A.27.3-1 and 4.5A.7.3-1 done in the same way as e.g. to 36.508 table 4.5A.26.3-1. - References to USIM/ISIM configuration added to initial conditions. - A new NOTE, capturing the working assumption, has been added in the new clause 21.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent and incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Changes 11, 12, 13, added. Revision2: Change 14 added. |  |  |  |  |  |  |  |  |

<Start of modification>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[6]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[7]	ISO/IEC 9646-1: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[8]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[9]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[10]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[11]	Void.

[12]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[13]	Void.

[14]	3GPP TS 33.203: "Access security for IP based services".

[15]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[16]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[17]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[18]	Void.

[19]	IETF RFC 3608: "Session Initiation Protocol (SIP) Extension Header Field for Service Route Discovery During Registration".

[20]	IETF RFC 3327: "Session Initiation Protocol Extension Header Field for Registering Non-Adjacent Contacts".

[21]	IETF RFC 3329: "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[22]	IETF RFC 3680: "A Session Initiation Protocol (SIP) Event Package for Registrations".

[23]	IETF RFC 3315: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[24]	IETF RFC 3320: “Signalling Compression (SigComp)”.

[25]	IETF RFC 3485: “The Session Initiation Protocol (SIP) and Session Description Protocol (SDP) Static Dictionary for Signalling Compression (SigComp)”.

[26]	IETF RFC 3486: “Compressing the Session Initiation Protocol (SIP)”.

[27]	IETF RFC 4566: "SDP: Session Description Protocol".

[28]	Void.

[29]	Void.

[30]	IETF RFC 3264: "An Offer/Answer Model with the Session Description Protocol (SDP)".

[31]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[32]	3GPP TS 23.003: "Numbering, addressing and identification".

[33]	IETF RFC 3262: "Registration of provisional responses in Session Initiation Protocol (SIP)".

[34]	Void.

[35]	3GPP TR 23.981 “Universal Mobile Telecommunications System (UMTS); Interworking aspects and migration scenarios for IPv4-based IP Multimedia Subsystem (IMS) implementations”.

[36]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language”.

[37]	ETSI ES 201 873-2: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 2: TTCN-3 Tabular Presentation Format (TFT)".

[38]	ETSI TR 201 873-3: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 3: TTCN-3 Graphical Presentation Format (GFT)".

[39]	3GPP TS 22.101: "Service aspects; Service principles".

[40]	3GPP TS 34.108: "Common test environments for User Equipment (UE); Conformance testing".

[41]	Void.

[42]	Void.

[43]	Void.

[44]	Void.

[45]	Void.

[46]	Void.

[47]	Void.

[48]	IETF RFC 3646: "DNS Configuration options for Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[49]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[50]	IETF RFC 3263: "Session Initiation Protocol (SIP): Locating SIP Servers".

[51]	IETF RFC 3319: "Dynamic Host Configuration Protocol (DHCPv6) Options for Session Initiation Protocol (SIP) Servers".

[52]	IETF RFC 1035: "Domain Names - Implementation And Specification".

[53]	Void.

[54]	Void.

[55]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[56]	IETF RFC 2782: "A DNS RR for specifying the location of services (DNS SRV)".

[57]	IETF RFC 3361: "Dynamic Host Configuration Protocol (DHCP-for-IPv4) Option for Session Initiation Protocol (SIP) Servers".

[58]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[59]	3GPP TR 33.978: "Security aspects of early IP Multimedia Subsystem (IMS)".

[60]	IETF RFC 3903: "Session Initiation Protocol (SIP) Extension for EventState Publication".

[61]	IETF RFC 5627: "Obtaining and Using Globally Routable User Agent (UA) URIs (GRUU) in the Session Initiation Protocol (SIP)".

[62]	IETF RFC 5628: "Reg Event Package Extension for GRUUs".

[63]	IETF RFC 3840: "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[64]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[65]	3GPP TS 24.173: "IMS Multimedia Telephony Communication Service and supplementary services; stage 3".

[66]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[67]	IETF RFC 4867: "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs".

[68]	IETF RFC 6050: "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[69]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[70]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[71]	Void.

[72]	IETF RFC 3515: "The Session Initiation Protocol (SIP) Refer Method".

[73]	Void.

[74]	Void.

[75]	Void.

[76]	Void.

[77]	Void.

[78]	Void.

[79]	Void.

[80]	Void.

[81]	Void.

[82]	Void.

[83]	IETF RFC 7044: "An Extension to the Session Initiation Protocol (SIP) for Request History Information".

[84]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[85]	Void".

[86]	IETF RFC 4575: "A Session Initiation Protocol (SIP) Event Package for Conference State".

[87]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[88]	IETF RFC 3842: "A Message Summary and Message Waiting Indication Event Package for the Session Initiation Protocol (SIP)".

[89]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[90]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[91]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[92]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[93]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[94]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Common Test Environments for User Equipment (UE) Conformance Testing".

[95]	3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem".

[96]	IETF RFC 3581: "An Extension to the Session Initiation Protocol (SIP) for Symmetric Response Routing".

[97]	IETF RFC 5031: "A Uniform Resource Name (URN) for Emergency and Other Well-Known Services".

[98]	IETF RFC 6442: "Location Conveyance for the Session Initiation Protocol".

[99]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[100]	Void.

[101]	3GPP TS 24.611: "Anonymous Communication Rejection (ACR)
and Communication Barring (CB) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[102]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[103]	3GPP TS 24.608: "Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[104]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[105]	3GPP TS 24.623: "Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services ".

[106]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[107]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem: Protocol specification".

[108]	3GPP TS 24.610: " Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[109]	IETF RFC 5626: "Managing Client-Initiated Connections in the Session Initiation Protocol (SIP)".

[110]	3GPP TS 24.237: “IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity”.

[111]	Void.

[112]	3GPP2 C.S0005-E: “Upper Layer (Layer 3) Signalling Standard for cdma2000 Spread Spectrum Systems”.

[113]	3GPP TS 31.121: "UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification".

[114]	Void.

[115]	Void.

[116]	Void.

[117]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[118]	3GPP TS 36.509: “Special conformance testing functions for User Equipment (UE)".

[119]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[120]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[121]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[122]	IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)".

[123]	3GPP TS 27.007: " AT command set for User Equipment (UE)".

[124]	IETF RFC 4835: "Cryptographic Algorithm Implementation Requirements for Encapsulating Security Payload (ESP) and Authentication Header (AH)".

[125]	IETF RFC 6809: "Mechanism to Indicate Support of Features and Capabilities in the Session Initiation Protocol (SIP)".

[126]	IETF RFC 4488: "Suppression of Session Initiation Protocol (SIP) REFER Method Implicit Subscription".

[127]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT)".

[128]	3GPP TS 24.628: “Common Basic Communication procedures using IP Multimedia (IM) Core Network (CN) subsystem”.

[129]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[130]	IETF RFC 6432: "Carrying Q.850 Codes in Reason Header Fields in SIP (Session Initiation Protocol) Responses".

[131]	IETF RFC 7462: "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP)".

[132]	IETF RFC 7315: "Private Header (P-Header) Extensions to the Session Initiation Protocol (SIP) for the 3GPP".

[133]	GSMA PRD IR.92: "IMS Profile for Voice and SMS".

[134]	GSMA PRD IR.94: "IMS Profile for Conversational Video Service".

[135]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[136]	Void.

[137]	IETF RFC 3311: "The Session Initiation Protocol (SIP) UPDATE Method".

[138]	IETF RFC 5009: "Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media".

[139]	IETF RFC 6086: “Session Initiation Protocol (SIP) INFO Method and Package Framework”.

[140]	IETF RFC 6665: “SIP-Specific Event Notification”.

[141]	3GPP TS 23.167: " IP Multimedia Subsystem (IMS) emergency sessions".

[142]	3GPP TS 24.238: "Session Initiation Protocol (SIP) based user configuration; Stage3".

[143]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".[144]	GSMA PRD NG.102: "IMS Profile for Converged IP Communications".

[145]	GSMA PRD RCC.07: "Rich Communication Suite 7.0 – Advanced Communications Services and Client Specification".

[146]	IETF RFC 4028 (April 2005): "Session Timers in the Session Initiation Protocol (SIP)".

[147]	3GPP TS 36.523-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS)proforma specification".

[148]	GSMA PRD IR.51: “IMS Profile for Voice, Video and SMS over untrusted Wi-Fi access”.

[149]	IETF RFC 8147 (May 2017): "Next-Generation Pan-European eCall".

[150]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[151]	GSMA PRD NG.108: "IMS Profile for Voice and SMS for UE category M1"

[152]	3GPP TS 24.390: " Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem IMS; Stage 3"

[153]	IETF RFC 5646: " Tags for Identifying Languages"

[154]	IETF RFC 7315: "P-Access-Network-Info ABNF Update"

[155]	3GPP TS 23.237: " IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[156]	3GPP TS 34.229-5: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)".

[157]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS"

[158]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[159]	3GPP TS 24.186: "IMS Data Channel applications; Protocol specification".

[160]	IETF RFC 5688: "A Session Initiation Protocol (SIP) Media Feature Tag for MIME Application Subtypes".

[161]	IETF RFC 7090: "Public Safety Answering Point (PSAP) Callback".

[xyz]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

<End of modification>

<Start of modification>

# 21	eCall over IMS

## 21.0	General

The following terminology is used for eCall test cases in clause 21:

eCall capable: A UE which is able to do an eCall is considered as "eCall capable". After switch-on such a UE does normal IMS registration.

eCall only mode: A UE configured for eCall only mode as defined in clause 3.1 of TS 23.401 [xyz].

NOTE:	In clause 21 eCall test cases, assumption is made that upon eCall initiation, a UE configured for eCall only mode performs an IMS normal registration, occuring in parallel to the IMS emergency registration.

## 21.1	eCall capable / Manual initiation / Emergency registration / Success / 200 OK with ACK

### 21.1.1	Definition

Test to verify that the UE can correctly perform eCall over IMS when initiated manually and that the SS releases the call after 200 OK with ACK is received. The process consists of emergency registration and MSD included in INVITE during call setup.

### 21.1.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.1.3	Test purpose

1)	To verify that the UE can correctly register to IMS emergency services and initiate an eCall over IMS in manual mode when UE is registered to IMS non-emergency services, according to TS 24.229 [10] clause 5.1.6.11.1;

2)	To verify that the UE sends a correctly composed initial INVITE request for eCall over IMS and will complete the eCall session setup after receiving 200 OK with ACK, according to 3GPP TS 24.229 [10] clause 5.1.6.11.2.

### 21.1.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends 200 OK with ACK.

5)	Void

6)-9)	MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate a manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9-12 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

BYE (Step 9, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.1.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for manual eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

## 21.2	eCall capable / Automatic initiation / Emergency registration / Success / 200 OK with ACK

### 21.2.1	Definition

Test to verify that the UE can correctly perform eCall over IMS when initiated automatically and that the SS releases the call after 200 OK with ACK is received. The process consists of emergency registration and MSD included in INVITE during call setup.

### 21.2.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.2.3	Test purpose

1)	To verify that the UE can correctly register to IMS emergency services and initiate an eCall over IMS in automatic mode when UE is registered to IMS non-emergency services, according to TS 24.229 [10] clause 5.1.6.11.1;

2)	To verify that the UE sends a correctly composed initial INVITE request for eCall over IMS and will complete the eCall session setup after receiving 200 OK with ACK, according to 3GPP TS 24.229 [10] clause 5.1.6.11.2.

### 21.2.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends 200 OK with ACK.

5)	Void

6)-9)	MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate an automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9-12 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

BYE (Step 9, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.2.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for automatic eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

## 21.3	eCall capable / Manual initiation / MSD transfer Failure / UE performs eCall in CS domain after Timer expiry / UTRAN or GERAN

### 21.3.1	Definition

Test to verify that the UE performs eCall in CS domain after MSD transfer failed during manual eCall Initiation. This process is described in 3GPP TS 24.229 [10].

### 21.3.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.3]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C];

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6.11.2.

### 21.3.3	Test purpose

1)	To verify that when the UE transmits eCall MSD with SIP INVITE, it waits for timer expiry to receive 200OK from network according to 3GPP TS 24.229 [10] 5.1.6.11.1 and 5.1.6.11.2

2)	To verify that the UE make eCall over CS domain after timer expiry

### 21.3.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

The SS is configured:

-	with 2 cells: as in TS 36.508

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

Note: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall initiated at UE.

2-5)	Emergency registration according to C.20 is executed

6)	SS waits for UE to send an INVITE request.

7)	SS waits until expiry of emerg-request timer (15 seconds) so that transfer of MSD transfer fails

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established. CS eCall is maintained for at least 5 seconds and then the call is cleared by SS.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | Make the UE attempt manual eCall Call |  |
| 2-5 | - |  | Step 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | - |  |  | SS waits until expiry of emerg-request timer (15 seconds) so that transfer of MSD fails |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and within 2 seconds after step 7 the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs CS fallback or cell reselection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM/GMM registration | NOTE 3: RAU procedure can take place in parallel to emergency CS call. |
| 9a2 | - |  | eCall is maintained for at least 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a3 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | -- |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs CS fallback or cell reselection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | eCall is maintained for at least 5 seconds |  |
| 9b3 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 2 as specified in annex C.47, which is referring to A.2.1 default message content of INVITE with condition A20.

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.3.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for manual eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 6 = 1 and all other bits are set to 0.

## 21.4	eCall capable / Manual initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / Success

### 21.4.1	Definition

Test to verify that the UE retry MSD transfer after MSD transfer failed during manual eCall Initiation. This process is described in 3GPP TS 24.229 [10].

### 21.4.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6.11.2.

### 21.4.3	Test purpose

1) 	To verify that the UE transmits eCall MSD with SIP INVITE for eCall over IMS in manual mode and receives corresponding 200 OK with ACK.

2) 	To verify that PSAP uses a SIP INFO with correct FROM-header to request an updated MSD and UE ACK the SIP INFO with a 200 OK.

3) 	UE sends updated MSD in a SIP INFO with correct Request-URI and To-header. PSAP ACK the updated MSD with 200 OK.

### 21.4.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall initiated at UE.

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends with 200 OK.

5)	SS waits for ACK.

6)	SS sends INFO request to transfer MSD.

7)	SS expects and receives 200 OK.

8)	SS expects and receives INFO request from the UE with MSD contents.

9)	SS responds with 200 OK for INFO request from UE.

10)	SS sends BYE to the UE.

11)	SS expects and receives 200 OK for BYE from the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9-12 |  |  | Steps 4-7 defined in C.47 | MSD transfer |
| 13-16 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

BYE (Step 13, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.4.5	Test requirements

The UE shall send requests and responses as described in clause 21.4.4.

## 21.5	eCall capable / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / Success

### 21.5.1	Definition

Test to verify that the UE updates MSD successfully after automatic eCall established using SIP INFO procedure. This process is described in 3GPP TS 24.229 [10].

### 21.5.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6.11.2

### 21.5.3	Test Purpose

1)	To verify that the UE transmits eCall MSD with SIP INVITE for eCall over IMS in automatic mode and receives 200 OK with ACK.

2)	To verify that PSAP uses a SIP INFO with correct FROM-Header to request an updated MSD and UE ACK the SIP INFO with a 200 OK.

3)	UE sends updated MSD in a SIP INFO with correct Request-URI and To-header. PSAP ACK the updated MSD with 200 OK.

### 21.5.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2-5)	Emergency registration according to C.20 is executed

6-12)	eCall is established successfully and MSD transferred successfully using INFO procedure as defined in C.47

13)	SS sends BYE to the UE

14)	SS expects and receives 200 OK for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE to initiate automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall Setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9-12 |  |  | Steps 4-7 defined in C.47 | MSD transfer |
| 13-16 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

BYE (Step 13, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.5.5	Test requirements

UE shall update MSD as defined in steps 11-12.

## 21.6	eCall capable / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for unsupported MSD / UE indicates unsuccessful in SIP INFO

### 21.6.1	Definition

Test to verify that the UE sends SIP INFO with correct contents as a response to SIP INFO with unsupported MSD transfer upon automatic eCall Initiation. This process is described in 3GPP TS 24.229 [10].

### 21.6.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6.11.2

### 21.6.3	Test purpose

1)	To verify that the UE transmits eCall MSD with SIP INVITE for eCall over IMS in automatic mode and receives 200 OK with ACK.

2)	To verify that PSAP uses a SIP INFO with correct FROM-Header to request an unsupported MSD and UE ACK the SIP INFO with a 200 OK.

3)	UE responds with SIP INFO with correct contents with indication of unsuccessful transfer. PSAP returns 200 OK.

### 21.6.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1) 	Automatic eCall over IMS initiated at UE

2-5)	Emergency registration according to C.20 is executed

6-7)	eCall is established successfully

8-11)	Execute MSD transfer procedure as defined in Annex C.47 from steps 4-7

12)	SS sends BYE to the UE

13)	SS expects and receives 200 OK for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt Automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-7A |  |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 8-11 |  |  | Steps 4-7 defined in C.47 | MSD transfer |
| 12-15 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

NOTE:	The default messages contents in annex A are used with condition "IMS security" or "GIBA" when applicable

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

INFO (Step 8, step 4 of C.47)

Use the default message “MT INFO” in annex A.2.20 with the datatype attribute set to ‘eCall.invalidMSD’ in the message body.

INFO (Step 10, step 6 of C.47)

Use the default message “MO INFO” in annex A.2.19 with condition A4 instead of A3.

BYE (Step 12, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.6.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for automatic eCall over IMS.

In step 10, UE shall transmit MO INFO with ‘success’ attribute set to “false” in message body.

## 21.7 to 21.12	Void

## 21.13	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN

### 21.13.1	Definition

Test to verify that on reception of reject cause 486 Busy here for a manually initiated INVITE for eCall over IMS, UE initiates the eCall in supported CS domain over UTRAN or GERAN.

### 21.13.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.13.3	Test purpose

1)	To verify that the UE is able to handle 486 (Busy here) SIP error message for a manually initiated INVITE for eCall over IMS; and

2)	To verify that the UE is able to establish legacy eCall in CS domain in UTRAN or GERAN system after receiving 486 (Busy here) SIP error message.

### 21.13.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	IMS Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request.

7)	SS sends 486 Busy here.

7A)	UE sends ACK.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| - | - |  | EXCEPTION; Steps 1A-1H and steps 2-5 may happen in parallel. | - |
| 1A-1H | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 6 | - |  | Step 1 defined in C.47 | eCall setup is initiated. |
| 7 | <- |  | 486 Busy here | The SS sends 486 Busy here |
| 7A | -> |  | ACK | UE sends ACK. |
|  | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and within 2 seconds of step 7 the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 |  |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | - |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

INVITE (Step 6)

As specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A20

486 Busy Here (Step 7)

As specified in Annex A.2.21 with condition A1

ACK (Step 7A)

As specified in Annex A.2.7 with condition A4

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.13.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for manual eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 6 = 1 and all other bits are set to 0.

## 21.14	eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN

### 21.14.1	Definition

Test to verify that on reception of reject cause 486 Busy here for automatically initiated INVITE for eCall over IMS, UE initiates the eCall in supported CS domain over UTRAN or GERAN.

### 21.14.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.14.3	Test purpose

1)	To verify that the UE is able to handle 486 (Busy here) SIP error message for an Automatically initiated INVITE for eCall over IMS; and

2)	To verify that the UE is able to establish legacy eCall in CS domain in UTRAN or GERAN system after receiving 486 (Busy here) SIP error message.

### 21.14.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 486 Busy here.

7A)	UE sends ACK.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall |  |
| - | - |  | EXCEPTION; Steps 1A-1H and steps 2-5 may happen in parallel. | - |
| 1A-1H | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 486 Busy here | The SS sends 486 Busy here |
| 7A | -> |  | ACK | UE sends ACK. |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

INVITE (Step 6)

As specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21

486 Busy Here (Step 7)

As in Annex A.2.21 with condition A1.

ACK (Step 7A)

As specified in Annex A.2.7 with condition A4

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.14.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for Automatic eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 7= 1 and all other bits are set to 0.

## 21.15	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN

### 21.15.1	Definition

Test to verify that on reception of reject cause 600 Busy everywhere for a manually initiated INVITE for eCall over IMS, UE initiates the eCall in supported CS domain over UTRAN or GERAN.

### 21.15.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.15.3	Test purpose

1)	To verify that the UE is able to handle 600 (Busy Everywhere) SIP error message for a manually initiated INVITE for eCall over IMS; and

2)	To verify that the UE is able to establish legacy eCall in CS domain in UTRAN or GERAN system after receiving 600 (Busy Everywhere) SIP error message.

### 21.15.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 600 Busy everywhere.

7A)	UE sends ACK.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| - | - |  | EXCEPTION; Steps 1A-1H and steps 2-5 may happen in parallel. | - |
| 1A-1H | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 600 Busy everywhere | The SS sends 600 Busy everywhere |
| 7A | -> |  | ACK | UE sends ACK. |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

INVITE (Step 6)

As specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A20

600 Busy Everywhere (Step 7)

As in Annex A.2.22 with condition A1.

ACK (Step 7A)

As specified in Annex A.2.7 with condition A4

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.15.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for manual eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 6 = 1 and all other bits are set to 0.

## 21.16	eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN

### 21.16.1	Definition

Test to verify that on reception of reject cause 600 Busy everywhere for an automatically initiated INVITE for eCall over IMS, UE initiates the eCall in supported CS domain over UTRAN or GERAN.

### 21.16.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.16.3	Test purpose

1)	To verify that the UE is able to handle 600 (Busy Everywhere) SIP error message for an automatically initiated INVITE for eCall over IMS; and

2)	To verify that the UE is able to establish legacy eCall in CS domain in UTRAN or GERAN system after receiving 600 (Busy Everywhere) SIP error message.

### 21.16.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 600 Busy everywhere.

7A)	UE sends ACK.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall |  |
| - | - |  | EXCEPTION; Steps 1A-1H and steps 2-5 may happen in parallel. | - |
| 1A-1H | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 600 Busy everywhere | The SS sends 600 Busy everywhere |
| 7A | -> |  | ACK | UE sends ACK. |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

INVITE (Step 6)

As specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21.

600 Busy Everywhere (Step 7)

As in Annex A.2.22 with condition A1.

ACK (Step 7A)

As specified in Annex A.2.7 with condition A4

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.16.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for automatic eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 7 = 1 and all other bits are set to 0.

## 21.17	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN

### 21.17.1	Definition

Test to verify that on reception of reject cause 603 Decline for a manually initiated INVITE for eCall over IMS, UE initiates the eCall in supported CS domain over UTRAN or GERAN.

### 21.17.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.17.3	Test purpose

1)	To verify that the UE is able to handle 603 (Decline) SIP error message for an automatically initiated INVITE for eCall over IMS; and

2)	To verify that the UE is able to establish legacy eCall in CS domain in UTRAN or GERAN system after receiving 603 (Decline) SIP error message.

### 21.17.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2.

The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	IMS Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 603 Decline.

7A)	UE sends ACK.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| - | - |  | EXCEPTION; Steps 1A-1H and steps 2-5 may happen in parallel. | - |
| 1A-1H | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 603 Decline | The SS sends 603 Decline |
| 7A | -> |  | ACK | UE sends ACK. |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

INVITE (Step 6)

As specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21

603 Decline (Step 7)

As in Annex A.2.23 with condition A1.

ACK (Step 7A)

As specified in Annex A.2.7 with condition A4

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.17.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for automatic eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 6 = 1 and all other bits are set to 0.

## 21.18	eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN

### 21.18.1	Definition

Test to verify that on reception of reject cause 603 Decline for an automatically initiated INVITE for eCall over IMS, UE initiates the eCall in supported CS domain over UTRAN or GERAN.

### 21.18.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.18.3	Test purpose

1)	To verify that the UE is able to handle 603 (Decline) SIP error message for an automatically initiated INVITE for eCall over IMS; and

2)	To verify that the UE is able to establish legacy eCall in CS domain in UTRAN or GERAN system after receiving 603 (Decline) SIP error message.

### 21.18.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2.

The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 603 Decline.

7A)	UE sends ACK.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall |  |
| - | - |  | EXCEPTION; Steps 1A-1H and steps 2-5 may happen in parallel. | - |
| 1A-1H | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 603 Decline | The SS sends 603 Decline |
| 7A | -> |  | ACK | UE sends ACK. |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

INVITE (Step 6)

As specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21

603 Decline (Step 7)

As in Annex A.2.23 with condition A1.

ACK (Step 7A)

As specified in Annex A.2.7 with condition A4

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 21.18.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for automatic eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 9a1 or 9b1, UE shall send an EMERGENCY SETUP message with the Service Category IE bit 7 = 1 and all other bits are set to 0.

## 21.19	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) with positive ACK / Successful MSD

### 21.19.1	Definition

Test to verify that on reception of reject cause 486 Busy here with positive acknowledgement for a manually initiated INVITE for eCall over IMS, UE considers MSD transmission as successful.

### 21.19.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.19.3	Test purpose

1)	To verify that the UE considers MSD transmission as successful after receiving 486 (Busy here) SIP error message with positive acknowledgement for a manually initiated INVITE for eCall over IMS.

### 21.19.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

2-12)	IMS registration according to C.2 is executed.

13-16)	 IMS Emergency registration according to C.20 is executed.

17)	 SS waits for UE to send an INVITE request.

18)	 SS sends 486 Busy here.

19) UE sends ACK.

20) SS releases the RRC connection.

21)	 UE stays on E-UTRAN cell A and does not perform domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported for 10 seconds.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| 2-4 | - |  | Void | - |
| - | - |  | EXCEPTION; Steps 5-12 and steps 13-16 may happen in parallel. | - |
| 5-12 | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 13-16 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 17 | -> |  | Step 1 defined in C.47 | Manual eCall is initiated. |
| 18 | <- |  | 486 Busy here | The SS sends 486 Busy here. |
| 19 | -> |  | ACK | UE sends ACK. |
| 20 | - |  | SS releases the RRC connection |  |
| 21 |  |  |  | The SS waits for a duration of ten seconds and checks that the UE does not initiate signalling on any of the cells. Generic test procedure in 36.508 [94] subclause 6.4.2.2 indicate that the UE is camped on E-UTRAN cell A. |

Specific Message Contents

INVITE (Step 17, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

486 Busy Here (Step 18)

As specified in Annex A.2.21 with condition A1

ACK (Step 19)

As specified in Annex A.2.7 with condition A4

### 21.19.5	Test requirements

In step 17, UE shall transmit INVITE with all applicable headers and including MSD for manual eCall over IMS.

In step 21, UE shall not initiate additional signalling on any of the cells, remain camped on E-UTRAN cell A and responds to incoming messages.

## 21.20	eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) with positive ACK / Successful MSD

### 21.20.1	Definition

Test to verify that on reception of reject cause 600 Busy Everywhere with positive acknowledgement for automatically initiated INVITE for eCall over IMS, UE considers MSD transmission as successful.

### 21.20.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.20.3	Test purpose

1)	To verify that the UE considers MSD transmission as successful after receiving 600 (Busy Everywhere) SIP error message with positive acknowledgement for automatically initiated INVITE for eCall over IMS.

### 21.20.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2-12)	IMS registration according to C.2 is executed.

13-16) IMS Emergency registration according to C.20 is executed.

17)	SS waits for UE to send an INVITE request.

18)	SS sends 600 Busy Everywhere.

19) UE sends ACK.

20) SS releases the RRC connection.

21) UE stays on E-UTRAN cell A and does not perform domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported for 10 seconds.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall |  |
| 2-4 | - |  | Void | - |
| - | - |  | EXCEPTION; Steps 5-12 and steps 13-16 may happen in parallel. | - |
| 5-12 | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 13-16 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 17 | -> |  | Step 1 defined in C.47 | Automatic eCall is initiated. |
| 18 | <- |  | 600 Busy Everywhere | The SS sends 600 Busy Everywhere |
| 19 | -> |  | ACK | UE sends ACK. |
| 20 | - |  | SS releases the RRC connection |  |
| 21 |  |  |  | The SS waits for a duration of ten seconds and checks that the UE does not initiate signaling on any of the cells. Generic test procedure in 36.508 [94] subclause 6.4.2.2 indicate that the UE is camped on E-UTRAN cell A. |

Specific Message Contents

INVITE (Step 17, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

600 Busy Everywhere (Step 18)

As specified in Annex A.2.22 with condition A1

ACK (Step 19)

As specified in Annex A.2.7 with condition A4

### 21.20.5	Test requirements

In step 17, UE shall transmit INVITE with all applicable headers and including MSD for automatic eCall over IMS.

In step 21, UE shall not initiate additional signaling on any of the cells, remain camped on E-UTRAN cell A and responds to incoming messages.

## 21.21	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) with positive ACK / Successful MSD

### 21.21.1	Definition

Test to verify that on reception of reject cause 603 Decline with positive acknowledgement for a manually initiated INVITE for eCall over IMS, UE considers MSD transmission as successful.

### 21.21.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.21.3	Test purpose

1)	To verify that the UE considers MSD transmission as successful after receiving 603 (Decline) SIP error message with positive acknowledgement for a manually initiated INVITE for eCall over IMS.

### 21.21.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

In addition, the SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

2-12)	IMS registration according to C.2 is executed.

13-16) IMS Emergency registration according to C.20 is executed.

17) SS waits for UE to send an INVITE request.

18) SS sends 603 Decline.

19) UE sends ACK.

20) SS releases the RRC connection.

21) UE stays on E-UTRAN cell A and does not perform domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported for 10 seconds.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| 2-4 | - |  | Void | - |
| - | - |  | EXCEPTION; Steps 5-12 and steps 13-16 may happen in parallel. | - |
| 5-12 | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 13-16 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 17 | -> |  | Step 1 defined in C.47 | Manual eCall is initiated. |
| 18 | <- |  | 603 Decline | The SS sends 603 Decline |
| 19 | -> |  | ACK | UE sends ACK. |
| 20 | - |  | SS releases the RRC connection |  |
| 21 |  |  |  | The SS waits for a duration of ten seconds and checks that the UE does not initiate signaling on any of the cells. Generic test procedure in TS 36.508 [94] subclause 6.4.2.2 indicate that the UE is camped on E-UTRAN cell A. |

Specific Message Contents

INVITE (Step 17, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

603 Decline (Step 18)

As specified in Annex A.2.23 with condition A1

ACK (Step 19)

As specified in Annex A.2.7 with condition A4

### 21.21.5	Test requirements

In step 17, UE shall transmit INVITE with all applicable headers and including MSD for manual eCall over IMS.

In step 21, UE shall not initiate additional signalling on any of the cells, remain camped on E-UTRAN cell A and responds to incoming messages.

## 21.22	eCall capable / Manual initiation / Emergency registration / 200 OK with NACK

### 21.22.1	Definition

Test to verify that the UE can correctly perform eCall over IMS when initiated manually and continues with call establishment after 200 OK with NACK is received from SS. The process consists of emergency registration and MSD included in INVITE during call setup.

### 21.22.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2a)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.22.3	Test purpose

1)	To verify that the UE can correctly register to IMS emergency services and initiate an eCall over IMS in manual mode when UE is registered to IMS non-emergency services, according to TS 24.229 [10] clause 5.1.6.11.1;

2)	To verify that the UE sends a correctly composed initial INVITE request for eCall over IMS and will continue with call establishment after receiving 200 OK with NACK, according to 3GPP TS 24.229 [10] clause 5.1.6.11.2.

### 21.22.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1)	Manual eCall over IMS initiated at UE.

2-5) Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request.

7)	SS sends 200 OK with <ack received="false">.

8)	UE responds with ACK for 200 OK.

9)	SS waits for 5 seconds.

10-13) MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate a manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9 |  |  |  | SS waits for 5 seconds |
| 10-13 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

200 OK for INVITE (Step 7, step 2 of C.47)

Use 200 OK as specified for step 2 in annex C.47 with condition A14 instead of A13.

BYE (Step 10, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.22.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for manual eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 8, UE shall transmit ACK for 200 OK and continues with call establishment.

## 21.23	eCall capable / Automatic initiation / Emergency registration / 200 OK with NACK

### 21.23.1	Definition

Test to verify that the UE can correctly perform eCall over IMS when initiated automatically and continues with call establishment after 200 OK with NACK is received. The process consists of emergency registration and MSD included in INVITE during call setup.

### 21.23.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2a)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.23.3	Test purpose

1)	To verify that the UE can correctly register to IMS emergency services and initiate an eCall over IMS in automatic mode when UE is registered to IMS non-emergency services, according to TS 24.229 [10] clause 5.1.6.11.1;

2)	To verify that the UE sends a correctly composed initial INVITE request for eCall over IMS and will continue with call establishment after receiving 200 OK with NACK, according to 3GPP TS 24.229 [10] clause 5.1.6.11.2.

### 21.23.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1)	Automatic eCall over IMS initiated at UE.

2-5) Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request.

7)	SS sends 200 OK with ACK as 'false'.

8)	UE responds with ACK for 200 OK.

9)	SS waits for 5 seconds.

10-13) MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate a automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9 |  |  |  | SS waits for 5 seconds |
| 10-13 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

200 OK for INVITE (Step 7, step 2 of C.47)

Use 200 OK as specified for step 2 in annex C.47 with condition A14 instead of A13.

BYE (Step 10, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.23.5	Test requirements

In step 6, UE shall transmit INVITE with all applicable headers for automatic eCall over IMS.

In step 6, UE shall transmit MSD in the INVITE.

In step 8, UE shall transmit ACK for 200 OK and continues with call establishment.

## 21.24	eCall capable / eCall to URI for test service / 200 OK with NACK

### 21.24.1	Definition

Test to verify that the UE can correctly perform test eCall over IMS when initiated and continues with call establishment after 200 OK with NACK is received. The process consists of normal registration and MSD included in INVITE during call setup.

### 21.24.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6A]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, theUE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	 a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

After a test eCall has been established and released, successfully or unsuccessfully, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD during an incoming IMS call when the following conditions are fulfilled:

1)	the incoming IMS call happens within an implementation dependent time period according to the national requirements within the region of eCall service (i.e. for EU region the timer value is 1 hour as defined by the T9 timer in CEN EN 17184:2024 [300]);

2)	the INVITE for the incoming IMS call contains a P-Asserted-Identity header field containing the same non-emergency service URN as included in the Request-URI for the test eCall or as included in a P-Asserted-Identity header field received in a response to the INVITE for the test eCall;

3)	the INVITE for the incoming IMS call contains an Accept header field that includes the value "application/EmergencyCallData.Control+xml"; and

4)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

When conditions 1) to 4) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 4:	As defined in CEN EN 17184:2024 [300], the recipient of a test eCall can instigate a callback to the UE to verify UE callback capability and ability to transfer updated MSD.

NOTE 5:	The recipient of a test eCall is encouraged to call back the UE after a shorter period in order to complete the test eCall without a long wait by a test eCall user.

When conditions 1) and 2) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 6:	A test eCall recipient can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the recipient, the serving PLMN, the HPLMN or by some other intermediate network.

NOTE 7:	For a trustable and secure identification of an eCall callback and the related MSD transfer between UE and the PSAP (or test center), the PSAP (or test center) is part of the trust domain of the IMS home operator network where the UE under test belongs to.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6A.

### 21.24.3	Test purpose

1)	To verify that the UE can correctly register to IMS emergency services and initiate an test eCall over IMS according to TS 24.229 [10] clause 5.1.6A;

2)	To verify that the UE sends a correctly composed initial INVITE request for test eCall over IMS and will continue with call establishment after receiving 200 OK with NACK, according to 3GPP TS 24.229 [10] clause 5.1.6.A.

### 21.24.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1) Test eCall over IMS initiated at UE.

2) SS waits for UE to send an INVITE request.

3) SS sends 200 OK with <ack received="false">.

4) UE responds with ACK for 200 OK.

5) SS waits for 5 seconds.

6-9) MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate test eCall |  |
| 2-4 |  |  | Steps 1-3 defined in C.47 | Test eCall setup |
| 5 |  |  |  | SS waits for 5 seconds |
| 6-9 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 2, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A25 instead of A20/A21 and without condition A7.

200 OK for INVITE (Step 3, step 2 of C.47)

Use 200 OK as specified for step 2 in annex C.47 with condition A14 instead of A13 and without condition A6.

### 21.24.5	Test requirements

In step 2, UE shall transmit INVITE with all applicable headers for test eCall over IMS and transmits MSD.

In step 4, UE shall transmit ACK for 200 OK and continues with call establishment.

## 21.25	eCall only mode / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / PSAP Callback / Success

### 21.25.1	Definition

Test to verify that the UE sends correctly composed SIP INFO for MSD transfer during automatically initiated eCall over IMS and PSAP callback after successful completion of eCall over IMS in ecall only mode. This process is described in 3GPP TS 24.229 [10].

### 21.25.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

…

[Rel-19, TS 24.229, clause 5.1.6B]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. The procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD shall be supported by a UE during an incoming IMS call when the following conditions are fulfilled:

1)	the UE previously established or attempted to establish an emergency call either over IMS or in the CS domain of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service;

2)	the incoming IMS call occurred within an implementation dependent time period of at least one hour;

3)	the INVITE for the incoming IMS call contains a Priority header field with header field value "psap-callback";

4)	the INVITE for the incoming IMS call contains an Accept header field that includes the value “application/EmergencyCallData.Control+xml”; and

5)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

NOTE 1:	When there are multiple attempts to establish the emergency call, the emergency call in condition 2) refers to the last attempt.

When conditions 1) to 5) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

When conditions 1) and 2) and, as an implementation option, condition 3) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 2:	A PSAP can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the PSAP, the serving PLMN, the HPLMN or by some other intermediate network.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.3 and 5.1.6B.

### 21.25.3	Test purpose

1) 	To verify that the UE checks the header fields in incoming IMS call for PSAP callback and responds with 200 OK including correctly composed Recv-Info and Accept header fields.

2) 	To verify that the UE sends updated MSD in a correctly composed SIP INFO during PSAP callback after automatic eCall over IMS is successfully released.

### 21.25.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall is initiated at UE.

2)	IMS registration according to C.2 is executed.

3)	Emergency registration according to C.20 is executed.

4)	SS waits for UE to send a properly composed INVITE request.

5)	SS sends with 200 OK.

6)	SS waits for ACK.

7)	SS sends INFO request to transfer MSD.

8)	SS expects and receives 200 OK.

9)	SS expects and receives INFO request from the UE with MSD contents.

10)	SS responds with 200 OK for INFO request from UE.

11) SS sends BYE to the UE.

12)	SS expects and receives 200 OK for BYE from the UE.

13) SS sends SIP INVITE with correctly composed Accept, Priority and Recv-Info header.

14) SS expects call setup with correctly composed 200 OK with Accept and Recv-Info headers and sends ACK.

15) SS sends INFO request to transfer MSD.

16) SS expects and receives 200 OK.

17) SS expects and receives INFO request from the UE with MSD contents.

18) SS responds with 200 OK for INFO request from UE.

19) SS sends BYE to the UE.

20) SS expects and receives 200 OK for BYE from the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | Make the UE attempt automatic eCall |  |
| - | - |  | EXCEPTION; Steps 2-9 and steps 10-13 may happen in parallel. | - |
| 2-9 | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW, establishes the default PDN, and performs IMS registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 10-13 | - |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.27.3-1. |
| 14-16 | - |  | Steps 1-3 defined in C.47 | eCall setup Referred from 36.508 [94] table 4.5A.27.3-1. |
| 17-20 |  |  | Steps 4-7 defined in C.47 | MSD transfer |
| 21-24 | - |  | Steps 1-4 defined in C.33 | SS releases the eCall. |
| 25-38 | - |  | Steps 1-13 defined in C.11 | MT call establishment. Referred from 36.508 [94] table 4.5A.7.3-1. |
| 39-42 | - |  | Steps 4-7 defined in C.47 | UE responds to incoming SIP INFO with 200 OK and sends updated MSD. |
| 43-46 | - |  | Steps 1-4 defined in C.33 | SS releases the call. |

Specific Message Contents:

INVITE (Step 14, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

INVITE (Step 25, step 1 of C.11)

Use INVITE as specified for step 1 in annex C.11 with condition A12.

200 OK for INVITE (Step 37, step 12 of C.11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A24.

BYE (Step 21 and Step 43)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.25.5	Test requirements

The UE shall send requests and responses as described in clause 21.25.4.

## 21.26	eCall only mode / eCall to URI for test service / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / PSAP Callback / Success

### 21.26.1	Definition

Test to verify that the UE sends correctly composed SIP INFO for MSD transfer during test ecall and during PSAP callback after successful completion of test eCall over IMS in ecall only mode. This process is described in 3GPP TS 24.229 [10].

### 21.26.2	Conformance requirement

[Rel-19, TS 24.229, clause 5.1.6A]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, the UE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	 a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to either "true" or "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C];

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

3)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

4)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall report the response and successful MSD transfer to upper layers; and

5)	in all other cases, the UE may perform a normal voice call redial to reattempt the test eCall as described above

While a test eCall remains established a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD and the procedures specified in 3GPP TS 26.267 [9C] for transfer of an updated MSD using the audio media stream.

NOTE 3:	For the updated MSD the test bit is set in the MSD according to CEN EN 15722:2020 [245].

After a test eCall has been established and released, successfully or unsuccessfully, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD during an incoming IMS call when the following conditions are fulfilled:

1)	the incoming IMS call happens within an implementation dependent time period according to the national requirements within the region of eCall service (i.e. for EU region the timer value is 1 hour as defined by the T9 timer in CEN EN 17184:2024 [300]);

2)	the INVITE for the incoming IMS call contains a P-Asserted-Identity header field containing the same non-emergency service URN as included in the Request-URI for the test eCall or as included in a P-Asserted-Identity header field received in a response to the INVITE for the test eCall;

3)	the INVITE for the incoming IMS call contains an Accept header field that includes the value "application/EmergencyCallData.Control+xml"; and

4)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

When conditions 1) to 4) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 4:	As defined in CEN EN 17184:2024 [300], the recipient of a test eCall can instigate a callback to the UE to verify UE callback capability and ability to transfer updated MSD.

NOTE 5:	The recipient of a test eCall is encouraged to call back the UE after a shorter period in order to complete the test eCall without a long wait by a test eCall user.

When conditions 1) and 2) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 6:	A test eCall recipient can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the recipient, the serving PLMN, the HPLMN or by some other intermediate network.

NOTE 7:	For a trustable and secure identification of an eCall callback and the related MSD transfer between UE and the PSAP (or test center), the PSAP (or test center) is part of the trust domain of the IMS home operator network where the UE under test belongs to.

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

…

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.3 and 5.1.6A.

### 21.26.3	Test purpose

1) 	To verify that the UE checks the header fields in incoming INVITE message for PSAP callback and responds with 200 OK including correctly composed Recv-Info and Accept header fields.

2) 	To verify that the UE sends updated MSD in a correctly composed SIP INFO during incoming IMS call after test eCall has been successfully released.

### 21.26.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 36.508 [94] table 4.9.3.5-2 and test eCall number configured as per TS 36.508 [94] Table 4.9.3.5-2. The UE is switched on but not registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1)	Test eCall is initiated at UE.

2)	IMS registration according to C.2 is executed.

4)	SS waits for UE to send a correctly composed INVITE request containing MSD.

5)	SS responds with 200 OK indicating successful MSD transmission.

6)	SS waits for ACK.

7)	SS sends INFO request to transfer MSD.

8)	SS expects and receives 200 OK.

9)	SS expects and receives INFO request from the UE with MSD contents.

10)	SS responds with 200 OK for INFO request from UE.

11) SS sends BYE to the UE.

12)	SS expects and receives 200 OK for BYE from the UE.

13) SS sends SIP INVITE with correctly composed Accept, P-Asserted Identity and Recv-Info header.

14) SS expects call setup with correctly composed 200 OK including Accept and Recv-Info headers and sends ACK.

15) SS sends INFO request to transfer MSD.

16) SS expects and receives 200 OK.

17) SS expects and receives INFO request from the UE with MSD contents.

18) SS responds with 200 OK for INFO request from UE.

19) SS sends BYE to the UE.

20) SS expects and receives 200 OK for BYE from the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate test eCall |  |
| - | - |  | EXCEPTION: Steps 6-9 may happen in parallel to Steps 10-12. | - |
| 2-9 | - |  | Steps 4-11 defined in C.2 | UE attaches to the NW and performs IMS registration. |
| 10-12 | - |  | Steps 1-3 defined in C.47 | Test eCall setup. |
| 13-16 | - |  | Steps 4-7 defined in C.47 | UE sends MSD in SIP INFO. |
| 17-20 | - |  | Steps 1-4 defined in C.33 | SS releases the test eCall. |
| - | - |  | - | SS waits for 10 seconds |
| 21-34 | - |  | Steps 1-13 defined in C.11 | MT call establishment. Referred from 36.508 [94] table 4.5A.7.3-1. |
| 35-38 | - |  | Steps 4-7 defined in C.47 | UE responds to incoming SIP INFO with 200 OK and sends updated MSD in SIP INFO. |
| 39-42 | - |  | Steps 1-4 defined in C.33 | SS releases the call. |

Specific Message Contents:

INVITE (Step 10, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A25 instead of A20/A21 and without condition A7.

200 OK for INVITE (Step 11, step 2 of C.47)

Use 200 OK as specified for step 2 in annex C.47 with conditions A12 and A13 and without condition A6.

INVITE (Step 21, step 1 of C.11)

Use INVITE as specified for step 1 in annex C.11 with condition A13.

200 OK for INVITE (Step 33, step 12 of C.11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A24.

INFO (Step 13 and Step 35, step 4 of C.47)

Use MT INFO as specified for step 4 in annex C.47 with condition A1.

INFO (Step 15 and Step 37, step 6 of C.47)

Use MO INFO as specified for step 6 in annex C.47.

### 21.26.5	Test requirements

The UE shall send requests and responses as described in clause 21.26.4.

<End of modification>