3GPP TSG-RAN5 Meeting #107	R5-252346r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1158 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating In-band emissions for CA (2UL CA) with PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updating 6.4A.2.3.1 minimum requirement and test requirement for PC5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating 6.4A.2.3.1.4.2 and 6.4A.2.3.1.5 with PC5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_FR2_FWA_Bn257_Bn258-UEConTest work item cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2.3.1.4.2, 6.4A.2.3.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revisions (TT added in test requirement table) |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

##### 6.4A.2.3.1	In-band emissions for CA (2UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The test procedure is incomplete due to that power window for CA is TBD

- Measurement Uncertainty and Test Tolerance are FFS.

- Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS.

- TP analysis is FFS

6.4A.2.3.1.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of in-band emissions.

6.4A.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.4A.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.3.0.

6.4A.2.3.1.4	Test description

6.4A.2.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.4A.2.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.3.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | - |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.4A.2.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.4A.2.3.1.4.3

6.4A.2.3.1.4.2	Test procedure

1.	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

2.	Configure SCC according to Annex C.0, C.1 and C.3.0 for all downlink physical channels.

3.	The SS shall configure SCC as per TS 38.508-1 [10] clause 5.5.1. Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in clause 6.4A.2.3.1.4.3.

4.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [28], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4A.2.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is Preq + PW ± PW, where Preq is the power level specified in Table 6.4A.2.3.1.4.2-1 according to the power class with power ID = 1. PW is the power window according to Table 6.4A.2.3.1.4.2-2 for the carrier frequency f and the channel bandwidth BW. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

8.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

9.	Measure In-band emission IEθ, IEφ on PCC using Global In-Channel Tx-Test (Annex E) for the θ- and φ-polarizations, respectively. Measure power spectral density on the SCC. For TDD, only slots consisting of only UL symbols are under test. Calculate IE = IEθ + IEφ, where the calculation is based on linear power ratios.

10.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

11. Repeat steps 6 through 10 until In-band emissions have been measured for all power IDs in Table 6.4A.2.3.1.4.2-1.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in Table 6.4A.2.3.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

Table 6.4A.2.3.1.4.2-1: Parameters for In-band emissions FFS

| Power ID | Unit | Level for power class 1 | Level for power class 2 | Level for power class 3 | Level for power class 4 |
| --- | --- | --- | --- | --- | --- |
| 1 | dBm | 27 | 16 | 10 | 21 |
| 2 | dBm | 17 | 6 | 0 | 11 |

Table 6.4A.2.3.1.4.2-2: Power Window (dB) for In-band emissions

FFS

6.4A.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.4A.2.3.1.5	Test requirement

For power ID1 and ID2, the averaged in-band emissions result, derived in Annex E.4.3 shall not exceed the corresponding values for IQ Image and Carrier Leakage in Table 6.4A.2.3.1.5-1 for power class 1 UEs.

Table 6.4A.2.3.1.5-1: Test Requirements for in-band emissions for power class 1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB+TT |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25+TT | Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20+TT | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25+TT | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20+TT | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.1-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

For power ID1 and ID2, the averaged in-band emissions result, derived in Annex E.4.3 shall not exceed the corresponding values for IQ Image and Carrier Leakage in Table 6.4A.2.3.1.5-2 for power class 2 UEs.

Table 6.4A.2.3.1.5-2: Test Requirements for in-band emissions for power class 2

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB+TT |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25+TT | Output power > 16 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20+TT | Output power ≤ 16 dBm |  |
| Carrier leakage | dBc | -25+TT | Output power > 6 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20+TT | -13 dBm ≤ Output power ≤ 6 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.1-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

For power ID1 and ID2, the averaged in-band emissions result, derived in Annex E.4.3 shall not exceed the corresponding values for IQ Image and Carrier Leakage in Table 6.4A.2.3.1.5-3 for power class 3 UEs.

Table 6.4A.2.3.1.5-3: Test Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB+TT |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25+TT | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20+TT | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25+TT | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20+TT | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.1-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

For power ID1 and ID2, the averaged in-band emissions result, derived in Annex E.4.3 shall not exceed the corresponding values for IQ Image and Carrier Leakage in Table 6.4A.2.3.1.5-4 for power class 4 UEs.

Table 6.4A.2.3.1.5-4: Test Requirements for in-band emissions for power class 4

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB+TT |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25+TT | Output power > 21 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20+TT | Output power ≤ 21 dBm |  |
| Carrier leakage | dBc | -25+TT | Output power > 11 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20+TT | -13 dBm ≤ Output power ≤ 11 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.1-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

For intra-band contiguous and non-contiguous carrier aggregation, the average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.6-1 for power class 6 UEs.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the in-band emissions requirements are specified in clause 6.4.2.3.6 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

Table 6.4A.2.3.1.5-5: Requirements for in-band emissions for power class 5

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 +TT | Output power > 17 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 +TT | Output power ≤ 17 dBm |  |
| Carrier leakage | dBc | -25 +TT | Output power > 7 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 +TT | -6 dBm ≤ Output power ≤ 7 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

##### 6.4A.2.3.2	In-band emissions for CA (3UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

<< Unchanged tables omitted >>

<<< STOP CHANGES >>>