3GPP TSG-RAN WG5 Meeting #107	R5-252408r2

# St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1524 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of signalling test frequencies and conditions for variable Tx-Rx spacing for NB-IoT for NTN band 256 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 specified variable Tx-Rx spacing for NTN bands and added a requirement to validate those different Tx-Rx spacing values for reference sensitivity test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds test frequencies/signaling updates for Band 256 to support conformance of variable Tx-Rx spacing for NB-IoT NTN UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of variable Tx-Rx spacing for NTN UEs cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.1.256, 8.1.4.3.3, 8.3.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revisions: Updates per request from TF 160 expert: i) Update to condition mnemonic and description; ii) Update carrierFreq IE to carrierFreq-ve90 since band > 64; fixed typos and clarified the table number for Band 256 in the notes; updated the test frequency tables to account by adding a Mid Range that is different from Low Range to ensure the frequencies do not overlap for protocol tests; ii) removed updates related to clauses associated with eMTC; iii) replaced “variable” with “flexible” for consistency with core specs as well as added NOTES as requested by TF160 expert; iv) minor update to the title since changes related to eMTC test coverage were removed. |  |  |  |  |  |  |  |  |

<<< NEXT CHANGE >>>

8.1.3.1.1.256	NB-IoT FDD reference test frequencies for operating band 256

Table 8.1.3.1.1.256-1: NB-IoT standalone Test frequencies for operating band 256

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261845 | 0 | 1980.1 | 229077 | 0 | 2170.1 |
| Mid Range | 261994 | 0 | 1995.0 | 229226 | 0 | 2185.0 |
| High Range | 262143 | 0 | 2009.9 | 229375 | 0 | 2199.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.256-2: NB-IoT standalone Test frequencies for operating band 256 with flexible Tx-Rx frequency separation

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low, Med Range1,2 | 262143 | 0 | 2009.9 | 229077 | 0 | 2170.1 |
| Mid Range1,3 | 262014 | 0 | 1997.0 | 229226 | 0 | 2185.0 |
| High Range1,2 | 261845 | 0 | 1980.1 | 229375 | 0 | 2199.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Low Range is based on the smallest Tx-Rx separation while High Range is based on the largest Tx-Rx separation. NOTE 3: The Mid Range is based on Tx-Rx separation of 188 MHz and has been specified for signalling tests. |  |  |  |  |  |  |

<<< NEXT CHANGE >>>

##### 8.1.4.3.3	NB-IoT Common contents of system information blocks

###### -	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

Table 8.1.4.3.3-1: SystemInformationBlockType2-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 SEQUENCE {} | RadioResourceConfigCommonSIB-NB-DEFAULT | See subclause 8.1.6 |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t300-r13 | ms10000 |  |  |
| t301-r13 | ms10000 |  |  |
| t310-r13 | ms1000 |  |  |
| n310-r13 | n2 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| freqInfo-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Not present | Default UL EARFCN applies | Standalone |
|  | See subclause 8.1.3.1 |  | Inband_Same, Inband_Different, Guardband, NTN_Flexible_TxRx_Separation |
| additionalSpectrumEmission-r13 | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4F -1. |  |
|  | 4 (NS_04) |  | NS_04 |
| } |  |  |  |
| timeAlignmentTimerCommon-r13 | infinity |  |  |
| multiBandInfoList-r13 | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  | FDD |
| freqInfo-v1530 SEQUENCE { |  |  | TDD |
| tdd-UL-DL-AlignmentOffset-r15 | khz0 |  |  |
| } |  |  |  |
| } |  |  |  |
| cp-Reestablishment-r14 | Not present |  |  |
| servingCellMeasInfo-r14 | Not present |  |  |
| cqi-Reporting-r14 | Not present |  |  |
| enhancedPHR-r15 | Not present |  |  |
| freqInfo-v1530 | Not present |  |  |
| cp-EDT-r15 | Not present |  |  |
| up-EDT-r15 | Not present |  |  |
| earlySecurityReactivation-r16 | Not present |  |  |
| cp-EDT-5GC-r16 | Not present |  |  |
| up-EDT-5GC-r16 | Not present |  |  |
| cp-PUR-EPC-r16 | Not present |  |  |
| up-PUR-EPC-r16 | Not present |  |  |
| cp-PUR-5GC-r16 | Not present |  |  |
| up-PUR-5GC-r16 | Not present |  |  |
| rai-ActivationEnh-r16 | Not present |  |  |
| gnss-PositionFixDurationReporting-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| NS_04 | When using the NS_04 frequency table variants in subclause 8.1.3.1 |
| NTN_Flexible_TxRx_Separation | Applicable to NTN cell configured with flexible Tx-Rx frequency separation |

<<< NEXT CHANGE >>>

##### 8.3.2.3.1	NB-IoT FDD Mode Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.1-1 for FDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.1.

Table 8.3.2.3.1-1: Test frequencies for NB-IoT FDD (200 kHz)

| NB-IoT Operating Band |  |  | Bandwidth [MHz] |  |  | f1, f5 |  |  |  |  |  | f2 |  |  |  |  |  | f3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NUL |  |  | NDL |  |  | NUL |  |  | NDL |  |  | NUL |  |  | NDL |  |  |
| 1 |  |  | 60 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 2 |  |  | 60 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 3 |  |  | 75 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 4 |  |  | 45 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 5 |  |  | 25 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  | 70 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 8 |  |  | 35 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  | 20 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 12 |  |  | 17 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 13 |  |  | 10 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 14 |  |  | 10 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  | 12 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 18 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 19 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 20 |  |  | 30 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 21 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  | 65 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 26 |  |  | 35 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  | 45 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  | 90 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 66 |  |  | 70+901 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 70 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 71 |  |  | 35 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 85 |  |  | 18 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 87 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 88 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 103 |  |  | 1 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 253 |  |  | 7 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 254 |  |  | 16.5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 255 |  |  | 34 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 256 |  |  | 30 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| NOTE 1: Asymmetric operating band (UL + DL). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.3.2.3.1-2: Test frequencies for NB-IoT FDD (200 kHz) for flexible Tx-Rx frequency separation

| NB-IoT Operating Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 256 | 30 | Mid1 | Mid1 | High1 | High1 | 261994Low1 | 229226Low1 |
| NOTE 1: Test frequencies are specified in Table 8.1.3.1.1.256-2 |  |  |  |  |  |  |  |

END OF CHANGES >>