3GPP TSG-RAN5 Meeting #107	R5-252466r2

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0669 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR IMS data channel test case 7.41 - MT Voice call without precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: SS send SDP offer with IMS DC and MT UE reply SDP answer with IMS DC. The UE can establish voice call without precondition + IMS DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NR IMS data channel test case 7.41 IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.41(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0368 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Test steps 22-24 refer to a wrong generic procdure.It should be corrected to Table 4.9.49.2.2-1. 2nd revision: 1 editorial update |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.41	IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE

7.41.1	Test Purpose (TP)

(1)

with { UE upporting IMS data channel and the network supports the IMS data channel }

ensure that {

when { UE receive INVITE message }

then { UE shall include the media feature tags for supported streaming media type with +sip.app-subtype="webrtc-datachannel" in the Contact header field of SIP 18x and 2xx responses to the SIP INVITE request }

}

(2)

with { UE upporting IMS data channel and the network supports the IMS data channel and UE is configured to use IMS data channel }

ensure that {

when { UE receives the INVITE request with an SDP offer, which includes the bootstrap data channel media descriptions }

then { UE generate the SDP answer and shall return a 18x or 2xx response to the INVITE request with the SDP answer }

}

7.41.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-18 requirements.

[TS 24.186 clause 9.3.3.1.2]

If the terminating UE determines that the UE and the network supports the IMS data channel, on the reception of SIP initial INVITE request, the terminating UE shall include the media feature tags defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field of SIP 18x and 2xx responses to the SIP INVITE request.

If the terminating UE receives the initial INVITE request with an SDP offer which includes the data channel media descriptions, i.e. the "m=" line containing the media set to "application", the UDP port number, the proto value set to "UDP/DTLS/SCTP" and the fmt value set to "webrtc-datachannel" and with associated "dcmap" attribute lines containing a subprotocol parameter set to "http" and any "stream-id" parameter set to values 0, 10, 100 or 110, and the terminating UE:

…

2)	is configured with IMS_DC_configuration node as specified in 3GPP TS 24.275 [11] and the DC_allowed leaf indicates that IMS data channel:

a)	is allowed and if the terminating UE accepts the offered bootstrap data channel(s), it shall generate the SDP answer based on the 3GPP TS 26.114 [4] and IETF RFC 8864 [14]; or

…

and the terminating UE shall return a 18x or 2xx response to the INVITE request with the above generated the SDP answer.

7.41.3	Test description

7.41.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	The UE is configured to not use preconditions.

-	The UE is configured to use IMS data channel.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.41.3.2	Test procedure sequence

Table 7.41.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 9 | SS sends INVITE with SDP offer containing the media lines for both voice and IMS data channel. (Step 1 of Annex A.5.2a) | <-- | INVITE | - | - |
| 10 | UE may send a 100 Trying provisional response (Step 2 of Annex A.5.2a) | --> | 100 Trying | - | - |
| 11 | UE sends SDP answer in 183 Session Progress. (Step 3 of Annex A.5.2a) | --> | 183 Session Progress | 1,2 | P |
| 12 | SS acks reception of 183 Session Progress (Step 4 of Annex A.5.2a) | <-- | PRACK | - | - |
| 13 | UE responds to PRACK (Step 5 of Annex A.5.2a) | --> | 200 OK | - | - |
| 14-16 | Steps 10-12 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 17 | UE sends 180 Ringing (Step 6 of Annex A.5.2a) | --> | 180 Ringing | - | - |
| 18 | SS sends PRACK (Step 7 of Annex A.5.2a) | <-- | PRACK | - | - |
| 19 | UE sends 200 OK for PRACK (Step 8 of Annex A.5.2a) | --> | 200 OK | - | - |
| 20 | Make the UE accept the voice call. (Note 1) | - | - | - | - |
| 21 | UE accepts the voice call (Step 9 of Annex A.5.2a) | --> | 200 OK | 1,2 | P |
| 22-24 | Steps 1-3 of generic procedure specified in Table 4.9.49.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 25 | SS acknowledges (Step 10 of Annex A.5.2a) | <-- | ACK | - | - |
| 26-27 | SS releases the call. (Annex A.8) | - | - | - | - |
| Note 1: This is achieved by MMI or AT command. Note 2: SS ignore the DRB including ClientHello for DTLS handshake sent by UE. |  |  |  |  |  |

7.41.3.3	Specific message contents

Table 7.41.3.3-1: INVITE (step 9, table 7.41.3.2-1)

| Derivation Path: TS 34.229-5, Step 1 of A.5.2a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 1 of A.5.2a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 3 of A.5.2a except adding one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=fingerprint:SHA-256 (fingerprint) [Note 3] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap:0 subprotocol="http” Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: Fingerprint is calculated by SS. Note 4: The tls-id-value is random number generated by SS. |  | TS 26.114 [33] RFC 8841 [bb] RFC 8841 [bb] RFC 8122 [cc] RFC 4145 [ff] RFC 8842 [dd] RFC 8864 [ee] |

Table 7.41.3.3-2: 183 Session Progress (step 11, table 7.41.3.2-1)

| Derivation Path: TS 34.229-5, Step 3 of A.5.2a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 3 of A.5.2a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 3 of A.5.2a except it may add one Media description as follow: Media description for IMS data channel: [Note 4] m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: [Note 4] a=max-message-size: (max-message-size-value) [Note 1] a=sctp-port:(sctp-port-value) [Note 2] a=setup: active a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note4: The media description and attributes for media for IMS data channel shall present in 183 Session Progress or 200 OK for INVITE. |  | TS 26.114 [33] |

Table 7.41.3.3-3: 200 OK for INVITE (step 21, table 7.41.3.2-1)

| Derivation Path: TS 34.229-5, Step 11 of A.5.2a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 11 of A.5.2a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body | A2 | SDP body is same as TS 34.229-5, Step 3 of A.5.2a except it add one Media description as follow: Media description for IMS data channel: [Note 4] m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: [Note 4] a=max-message-size: (max-message-size-value) [Note 1] a=sctp-port:(sctp-port-value) [Note 2] a=setup: active a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note 3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note4: The media description and attributes for media for IMS data channel shall present in 183 Session Progress or 200 OK for INVITE. |  | TS 26.114 [33] |
|  | A1 | SDP body is empty. |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines both for voice and IMS data channel in Step11. |
| A2 | 183 Session Progress includes SDP offer containing media lines only for voice in Step11. |

<End of modified section 1>