3GPP TSG-RAN5 Meeting #107	R5-252686r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1613 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Emergency call with Real Time text test case 19.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case 19.1.7 to be added for Emergency call with RTT activated scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 19.1.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for RTT according to EU directive 2019/882 will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0373 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-252686r1: 1. Table 4.5A.4.3-1 referred in expected sequence. 2. Specific message contents for INVITE (step 6) and 200 OK for INVITE (step 9) updated. |  |  |  |  |  |  |  |  |

<Start of modified section>

## 19.1.7	Emergency call / RTT activated

### 19.1.7.1	Definition

Test to verify that the UE performs emergency registration and initiates setup of emergency call with real-time text activated by sending a correctly composed SIP INVITE. This process is described in 3GPP TS 26.114 [56].

### 19.1.7.2	Conformance requirement

[TS 24.229, clause 5.1.6.2]:

When a UE performs an initial emergency registration the UE shall perform the actions as specified in subclause 5.1.1.2 with the following additions and modifications:

a)	the UE shall include a "sos" SIP URI parameter in the Contact header field as described in subclause 7.2A.13, indicating that this is an emergency registration and that the associated contact address is allowed only for emergency service; and

b)	the UE shall populate the From and To header fields of the REGISTER request with:

-	the first entry in the list of public user identities provisioned in the UE;

-	the default public user identity obtained during the normal registration, if the UE is not provisioned with a list of public user identites, but the UE is currently registered to the IM CN subsystem; and

-	the derived temporary public user identity, in all other cases.

[TS 24.229, clause 5.1.6.8.3]:

After a successful initial emergency registration, the UE shall apply the procedures as specified in subclause 5.1.2A and 5.1.3 with the following additions:

1)	the UE shall insert in the INVITE request, a From header field that includes the public user identity registered via emergency registration or the tel URI associated with the public user identity registered via emergency registration, as described in subclause 4.2;

2)	the UE shall include a service URN in the Request-URI of the INVITE request in accordance with subclause 5.1.6.8.1;

3)	the UE shall insert in the INVITE request, a To header field withthe same emergency service URN as in the Request-URI;

4)	if available to the UE, and if defined for the access type as specified in subclause 7.2A.4, the P-Access-Network-Info header field shall contain a location identifier such as the cell id, line id or the identity of the WLAN access node, which is relevant for routeing the IMS emergency call;

NOTE 1:	The IMS emergency specification in 3GPP TS 23.167 [4B] describes several methods how the UE can get its location information from the access network or from a server. Such methods are not in the scope of this specification.

5)	the UE shall insert in the INVITE request, one or two P-Preferred-Identity header field(s) that include the public user identity registered via emergency registration or the tel URI associated with the public user identity registered via emergency registration as described in subclause 4.2;

NOTE 2:	Providing two P-Preferred-Identity header fields is usually supported by UE acting as enterprise network.

6)	void;

7)	if the UE has its location information available, or a URI that points to the location information, then the UE shall include a Geolocation header field in the INVITE request in the following way:

-	if the UE is aware of the URI that points to where the UE's location is stored, include the URI as the Geolocation header field value, as described in RFC 6442 [89]; or

-	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described RFC 6442 [89];

8)	if the UE includes a Geolocation header field, the UE shall also include a Geolocation-Routing header field with a "yes" header field value, which indicates that the location of the UE can be used by other entities to make routing decisions, as described in RFC 6442 [89]; and

NOTE 3:	It is suggested that UE's only use the option of providing a URI when the domain part belongs to the current P-CSCF or S-CSCF provider. This is an issue on which the network operator needs to provide guidance to the end user. A URI that is only resolvable to the UE which is making the emergency call is not desirable.

9)	if the UE has neither geographical location information available, nor a URI that points to the location information, the UE shall not insert a Geolocation header field in the INVITE request.

[TS 26.114, clause 5.2.3]:

MTSI clients in terminals offering real time text conversation shall support:

-	ITU-T Recommendation T.140 [26] and [27].

T.140 specifies coding and presentation features of real-time text usage. Text characters are coded according to the UTF-8 transform of ISO 10646-1 (Unicode).

A minimal subset of the Unicode character set, corresponding to the Latin-1 part shall be supported, while the languages in the regions where the MTSI client in terminal is intended to be used should be supported.

Presentation control functions from ISO 6429 are allowed in the T.140 media stream. A mechanism for extending control functions is included in ITU-T Recommendation T.140 [26] and [27]. Any received non-implemented control code must not influence presentation.

A MTSI client in terminal shall store the conversation in a presentation buffer during a call for possible scrolling, saving, display re-arranging, erasure, etc. At least 800 characters shall be kept in the presentation buffer during a call.

Note that erasure (backspace) of characters is included in the T.140 editing control functions. It shall be possible to erase all characters in the presentation buffer. The display of the characters in the buffer shall also be impacted by the erasure.

[TS 26.114, clause 6.2.1]:

An MTSI client shall offer at least one RTP profile for each RTP media stream. Multiple RTP profiles may be offered using SDPCapNeg as described in Clause 6.2.1a. For voice and real-time text, the first SDP offer shall include at least the AVP profile.

[TS 26.114, clause 7.4.4]:

The following RTP payload format shall be used:

-	T.140 text conversation RTP payload format according to RFC 4103 [31].

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

Media type signalling for usage in SDP is specified in section 10 of RFC 4103 [31] and section 3 of RFC 4102 [49].

[TS 26.114, clause 7.5.2.3]:

Real-time text is intended for human conversation applications. Text shall not be transferred with higher rate than 30 characters per second (as defined for cps in section 6 of RFC 4103 [31]). A text-capable MTSI client shall be able to receive text with cps set up to 30.

[TS 26.114, clause 9.4]:

Redundant transmission provided by the RTP payload format as described in RFC 4103 [31] shall be supported. The transmitting application may use up to 200 % redundancy, i.e. a T140block transported in one RTP packet may be repeated once or twice in subsequent RTP packets. 200 % redundancy shall be used when the conditions along the call path are not known to be free of loss. However, the result of media negotiation shall be followed, and transmission without redundancy used if one of the parties does not show capability for redundancy.

The sampling time shall be 300 ms as a minimum (in order to keep the bandwidth down) and should not be longer than 500 ms. New text after an idle period shall be sent as soon as possible. The first packet after an idle-period shall have the M-bit set.

The procedure described in section 5 of RFC 4103 [31], or a procedure with equivalent or better performance, shall be used for packet-loss handling in the receiving MTSI client in terminal.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.2, 5.1.6.8.3 and 3GPP TS 26.114[56], clauses 5.2.3, 6.2.1, 7.4.4, 7.5.2.3, 9.4.(Release-12)

### 19.1.7.3	Test purpose

1) 	To verify that the UE capable of real-time text correctly performs emergency registration and places an emergency call with real-time text activated.

### 19.1.7.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is configured to enable RTT and is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Emergency call with real-time text activated is initiated at UE.

2-5) Emergency registration according to C.20 is executed.

6) UE sends correctly composed INVITE with SDP contents for audio and real-time text.

7) SS sends 100 Trying followed by 180 Ringing.

8) SS sends 200 OK for SIP INVITE.

9) UE sends ACK.

10)	UE sends BYE to end the call.

11) SS sends 200 OK for BYE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | User initiates an emergency call |  |
| 2-5 | - |  | Step 1-4 defined in Annex C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 6-10 | - |  | Steps 1-5 as defined in Annex C.22 | Emergency call setup with RTT. Resource reservation takes place for audio and text media according to <FFS> |
| 11-15 | - |  | Steps 1-5 as defined in Annex C.32 | UE releases the call followed by EPS bearer deactivation for audio and RTT. |

Specific Message Contents:

REGISTER (Step 2, Step 1 of C.20)

Use REGISTER as specified in Step 1 of annex C.20 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| feature-param | text |

INVITE (Step 6, step 1 of C.22)

Use INVITE as specified for Step 1 in annex C.22 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| feature-param | text |
| Message-body | Same SDP message as specified in Step 1 of C.22 with the following SDP types and values in addition: Media description for real time text: - m=text (transport port) RTP/AVP (fmt) - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for real time text media: - a=rtpmap: (payload type) t140/1000 - a=rtpmap: (payload type) red/1000 - a=fmtp: (format) (format specific parameters) |

200 OK for INVITE (Step 9, step 4 of C.22)

Use 200 OK for INVITE as specified for Step 4 in annex C.22 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| feature-param | text |
| Message-body | Same SDP message as specified in Step 4 of C.22 with the following SDP types and values in addition: Session description: - b=AS:39 Media description for real time text: - m=text (transport port) RTP/AVP (fmt) [Note 1] - b=AS: 2 - b=RS: 0 - b=RR: 0 Attributes for real time text media: - a=rtpmap: (payload type) t140/1000 [Note 1] - a=fmtp: (format) cps=30 [Note 1] - a=rtpmap: (payload type) red/1000 [Note 1] - a=fmtp: (format) (format specific parameters) [Note 1] Note 1: The values for fmt, payload type, bandwidth-value, format specific parameters and format are copied from INVITE. |

### 19.1.7.5	Test requirements

The UE shall send requests and responses as described in clause 19.1.7.4.

<End of modified section>