3GPP TSG-RAN5 Meeting #107	R5-252876r1

# Malta, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-6 | CR | 0008 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 5.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Element, Ericsson, FirstNet, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case, 5.8 Configuration / Rules based affiliation status change, included in the Work Plan is not implemented |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of test case, 5.8 Configuration / Rules based affiliation status change, accoding to the Work Plan |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be a test case for 5.8 Configuration / Rules based affiliation status change |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS37.579-1 CR0011 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 Fixed 3GU inconsistency on cover page |  |  |  |  |  |  |  |  |

5.4.3.3	Specific message contents

Same specific message contents as for MCPTT test case 5.5 (TS 37.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCVideo"

-	Condition MCVIDEO is used for all messages.

## 5.8	Configuration / Rules based affiliation status change

5.8.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service }

ensure that {

when { UE enters into or exits from a location area that triggers a group de-affiliation }

then { UE (MCVideo Client) sends a SIP PUBLISH message to de-affiliate from a group }

}

(2)

with { UE (MCVideo Client) registered and authorised for MCVideo Service }

ensure that {

when { UE enters into or exits from a location area that triggers a group affiliation }

then { UE (MCVideo Client) sends a SIP PUBLISH message to affiliate to a group }

}

5.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.281 clause 8.2.1.7.2, 8.2.1.2. Unless otherwise stated these are Rel-17 requirements.

[TS 24.281, clause 8.2.1.7.2]

User profile based affiliation rules are controlled by the elements <RulesForAffiliation> or <RulesForDeaffiliation> of the MCVideo user profile document identified by the MCVideo ID of the MCVideo user (see the MCVideo user profile document specified in 3GPP TS 24.484 [25]). The rules can be composed of functional alias based criteria. A rule is fulfilled if  any of the functional alias based criteria are met. These rules are evaluated whenever a functional alias is activated or deactivated. If, any defined rule is fulfilled, the MCVideo client shall initiate the affiliation status change procedure as specified in clause 8.2.1.2.

[TS 24.281, clause 8.2.1.2]

In order:

-	to indicate that an MCVideo user is interested in one or more MCVideo group(s) at an MCVideo client;

-	to indicate that the MCVideo user is no longer interested in one or more MCVideo group(s) at the MCVideo client;

-	to refresh indication of an MCVideo user interest in one or more MCVideo group(s) at an MCVideo client due to near expiration of the expiration time of an MCVideo group with the affiliation status set to the "affiliated" state received in a SIP NOTIFY request in clause 8.2.1.3;

-	to send an affiliation status change request in mandatory mode to another MCVideo user;

-	to indicate that an MCVideo user is interested in one or more MCVideo group(s) at an MCVideo client triggered by functional alias activation criteria;

-	to indicate that the MCVideo user is no longer interested in one or more MCVideo group(s) at the MCVideo client client triggered functional alias deactivation criteria; or

-	any combination of the above;

the MCVideo client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [11], IETF RFC 3903 [12], and IETF RFC 3856 [12].

When the MCVideo user indicates that he is no longer interested in one or more MCVideo group(s) at the MCVideo client, the MCVideo client shall first check value of the <manual-deaffiliation-not-allowed-if-affiliation-rules-are-met> element if present within the MCVideo user profile document (see the MCVideo user profile document specified in 3GPP TS 24.484 [25]). If the affiliation to the group has been activated due to a rule being fulfilled and the <manual-deaffiliation-not-allowed-if-affiliation-rules-are-met> element is present and is set to a value of "true", the MCVideo client shall suppress the MCVideo user’s request.

NOTE 1:	If the request is suppressed, a notification message can be displayed to the user

In the SIP PUBLISH request, the MCVideo client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCVideo function serving the MCVideo user;

2)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body. In the application/vnd.3gpp.mcvideo-info+xml MIME body, the MCVideo client shall include the <mcvideo-request-uri> element set to the MCVideo ID of the MCVideo user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14];

4)	if the targeted MCVideo user is interested in at least one MCVideo group at the targeted MCVideo client, shall set the Expires header field according to IETF RFC 3903 [12], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [15].

5)	if the targeted MCVideo user is no longer interested in any MCVideo group at the targeted MCVideo client, shall set the Expires header field according to IETF RFC 3903 [12], to zero; and

6)	shall include an application/pidf+xml MIME body indicating per-user affiliation information according to clause 8.3.1. In the MIME body, the MCVideo client:

a)	shall include all MCVideo groups where the targeted MCVideo user indicates its interest at the targeted MCVideo client;

b)	shall include the MCVideo client ID of the targeted MCVideo client;

c)	shall not include the "status" attribute and the "expires" attribute in the <affiliation> element; and

d)	shall set the <p-id> child element of the <presence> root element to a globally unique value.

The MCVideo client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [11].

5.8.3	Test description

5.8.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

-	GNSS simulator to simulate a location.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 to provide a location, as defined in TS 36.508 [24] Table 4.11.2-2, step 1 of scenario #4.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCVideo.

5.8.3.2	Test procedure sequence

Table 5.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 2 | Trigger the GNSS simulator to start step 2 of Scenario #4 to simulate the UE moving to location #7 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 3 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #7. | - | - | - | - |
| 4-6 | Check: Check: Does the UE (MCVideo Client) correctly perform steps 1a1-3 of procedure 'MCX Group Affiliation status change' as described in TS 37.579-1 [2] Table 5.3.34.3-1 to de-affiliate with group A? NOTE: The UE (MCVideo Client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message. | - | - | 1 | P |
| 7 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 8 | Trigger the GNSS simulator to start step 4 of Scenario #4 to simulate the UE moving to location #8 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 9 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #8. | - | - | - | - |
| 10-12 | Check: Check: Does the UE (MCVideo Client) correctly perform steps 1a1-3 of procedure 'MCX Group Affiliation status change' as described in TS 37.579-1 [2] Table 5.3.34.3-1 to affiliate with group A? NOTE: The UE (MCVideo Client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message. | - | - | 2 | P |
| 13 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.8.3.3	Specific message contents

Table 5.8.3.3-1: SIP PUBLISH from the UE (step 5, Table 5.8.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.34.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo -Info |  |  |
| MIME-part-body | MCVideo -Info as described in Table 5.8.3.3-2 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.8.3.3-3 |  |  |  |

Table 5.8.3.3-2: MCVideo-Info in SIP PUBLISH (Table 5.8.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideo-info |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-request-uri | px_MCVideo_User_A_ID |  |  |  |

Table 5.8.3.3-3: PIDF in SIP PUBLISH (Table 5.8.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.5.1-2, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status | not present |  |  |  |
| ..p-id | any allowed value |  | TS 24.281 [86] clause 8.3.1 |  |

Table 5.8.3.3-4: SIP PUBLISH from the UE (step 11, Table 5.8.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.34.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 5.8.3.3-2 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.8.3.3-5 |  |  |  |

Table 5.8.3.3-5: PIDF in SIP PUBLISH (Table 5.8.3.3-4)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.5.1-2, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| ..p-id | any allowed value |  | TS 24.281 [86] clause 8.3.1 |  |