| 3GPP TS 38.533 V18.7.0 (2025-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	61

1	Scope	62

2	References	62

3	Definitions, symbols and abbreviations	63

3.1	Definitions	63

3.2	Symbols	64

3.3	Abbreviations	65

3A	Requirements for the support of RRM	67

3A.1	General	67

3A.1.0	Overview of RRM requirements	67

3A.1.1	Test coverage across 5G NR connectivity options	68

3A.2	Requirements Classification for Statistical Testing	68

3A.3	Antenna Configuration	69

3A.4	NR band groups	69

3A.4.0	General	69

3A.4.1	NR operating bands in FR1	69

3A.4.1A	NR operating bands for satellite access in FR1	71

3A.4.2	NR operating bands in FR2	72

3A.5	NR operating band configuration	73

3A.6	UE with Multiband Capability	73

4	EN-DC with all NR cells in FR1	73

4.0	General	73

4.1	Void	74

4.2	Void	74

4.3	RRC_CONNECTED state mobility	74

4.3.1	Void	74

4.3.2	RRC connection mobility control	74

4.3.2.1	Void	74

4.3.2.2	Random access	74

4.3.2.2.1	EN-DC FR1 contention based random access	74

4.3.2.2.2	EN-DC FR1 non-contention based random access	80

4.3.2.2.3	EN-DC FR1 2-step contention based random access	87

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	93

4.3.2.3	Void	99

4.4	Timing	99

4.4.1	UE transmit timing	99

4.4.1.0	Minimum conformance requirements	99

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	99

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	101

4.4.2	UE timer accuracy	107

4.4.3	Timing advance	107

4.4.3.0	Minimum conformance requirements	107

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	107

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	108

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	108

4.5	Signaling characteristics	113

4.5.1	Radio link monitoring	113

4.5.1.00	General	113

4.5.1.0	Minimum conformance requirements	113

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	113

4.5.1.0.2	Void	114

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	114

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	115

4.5.1.0.5	Requirements for UE configured with Relaxed Measurement Criteria	117

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	118

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	123

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	128

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	133

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	138

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	143

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	148

4.5.1.8	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	153

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	159

4.5.2	Interruption	164

4.5.2.0	Minimum conformance requirements	164

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	164

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	165

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	166

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	166

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	168

4.5.2.0.6	Minimum conformance requirements for interruptions due to RRM and RLM/BFD measurements on deactivated NR SCG	169

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	169

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	173

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	177

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	183

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	189

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	195

4.5.2.7	199

4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	200

4.5.2.9	E-UTRAN - NR FR1 interruptions at E-UTRA SRS carrier based switching	205

4.5.2.10	EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	211

4.5.3	SCell activation and deactivation delay	215

4.5.3.0	Minimum conformance requirements	215

4.5.3.0.1	SCell activation and deactivation delay	215

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	219

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	230

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	232

4.5.3.4	SCell activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	234

4.5.3.5	EN-DC FR1 direct SCell activation at SCell addition of known SCell	241

4.5.3.6	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	251

4.5.3.7	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	259

4.5.3.8 to 4.5.3.10	262

4.5.3.11	EN-DC FR1 TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band in non-DRX	262

4.5.3.12	EN-DC FR1 Inter-band SSB-less SCell activation using A-TRS	272

4.5.4	UE UL carrier RRC reconfiguration delay	279

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	279

4.5.5	Link recovery procedures	286

4.5.5.0	Minimum conformance requirements	286

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	289

4.5.5.0.4	Requirements for Beam Failure Recovery in SCell	289

4.5.5.0.5	TRP specific Minimum conformance requirements	289

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	292

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	299

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	305

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	312

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	319

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	325

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	331

4.5.5.8	EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	339

4.5.6	Active BWP switch delay	347

4.5.6.1	DCI-based and time-based active BWP switch	347

4.5.6.1.0	Minimum conformance requirements	347

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	350

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	357

4.5.6.2	RRC-based active BWP switch	367

4.5.6.2.0	Minimum conformance requirements	367

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	367

4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	373

4.5.6.3.0	Minimum conformance requirements	373

4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	375

4.5.6.4	Simultaneous RRC-based active BWP switch on multiple CCs	384

4.5.6.4.0	Minimum conformance requirements	384

4.5.6.4.0.1	Minimum conformance requirements for simultaneous RRC-based BWP switch delay on multiple CCs	384

4.5.6.4.0.2	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	385

4.5.6.4.0.3	Minimum conformance requirements for interruption for NR victim cell	386

4.5.6.4.0.3.1	Interruptions due to SCell dormancy switch	387

4.5.6.4.0.3.2	Interruptions due to CQI measurements during SCell dormancy	387

4.5.6.4.0.3.3	Interruptions due to RRM measurements during SCell dormancy	387

4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	388

4.5.6.5	SCell dormancy switch	396

4.5.7	PSCell addition and release delay	396

4.5.7.0	Minimum conformance requirements	396

4.5.7.0.1	NR PSCell Addition Delay Requirement	396

4.5.7.0.2	NR PSCell Release Delay Requirement	397

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	397

4.5.8	UL switching	402

4.5.8.0	Minimum conformance requirements	402

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	403

4.5.9	UE specific CBW change	409

4.5.9.0	Minimum conformance requirements	409

4.5.9.0.1	Minimum conformance requirements for UE specific CBW change delay	409

4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC	410

4.5.10	PSCell activation and deactivation delay	414

4.5.10.0	Minimum conformance requirements	414

4.5.10.1	EN-DC FR1 PSCell activation and deactivation delay	415

4.5.11	PSCell addition and release delay	420

4.5.11.0	Minimum conformance requirements	420

4.5.11.0.1	Measurement time	421

4.5.11.1	EN-DC FR1 Addition and Release Delay of PSCell	421

4.6	Measurement procedures	427

4.6.1	Intra-frequency measurements	427

4.6.1.0	Minimum conformance requirements	427

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	428

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	432

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	434

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	439

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	442

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	448

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	453

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	456

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	461

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	465

4.6.2	Inter-frequency measurements	471

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	471

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	474

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	479

4.6.2.3	Void	484

4.6.2.4	Void	484

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	484

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	489

4.6.2.7	Void	494

4.6.2.8	Void	494

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	494

4.6.3	Void	498

4.6.4	L1-RSRP measurement for beam reporting	498

4.6.4.0	Minimum conformance requirements	498

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	498

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	502

4.6.4.1.5	Test requirement	506

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	506

4.6.4.2.3	Minimum conformance requirements	507

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	510

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	514

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	517

4.6.5	CLI measurements	522

4.6.5.0	Minimum conformance requirements	522

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	523

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	524

4.6.5.2	EN-DC FR1 CLI-RSSI measurement in non-DRX	529

4.6.6	Measurements with autonomous gaps	532

4.6.6.0	Minimum conformance requirements	532

4.6.6.0.1	Minimum conformance requirements for measurements with autonomous gaps	532

4.6.6.0.1.1	Minimum conformance requirements for E-UTRA FDD - NR measurements with autonomous gaps	532

4.6.6.0.1.2	Minimum conformance requirements for E-UTRA TDD - NR measurements with autonomous gaps	533

4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	533

4.6.7	L1-SINR measurement for beam reporting	538

4.6.7.0	Minimum conformance requirements	538

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	544

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	549

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	554

4.6.8	CSI-RS based intra-frequency measurement	558

4.6.8.0	Minimum conformance requirements	558

4.6.8.0.1	Minimum conformance requirements for EN-DC event triggered reporting tests without gap under DRX	558

4.6.9	CSI-RS based inter-frequency measurement	560

4.6.9.0	Minimum conformance requirements	560

4.6.9.0.1	Minimum conformance requirements for EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	560

4.7	Measurement performance requirements	562

4.7.1	SS-RSRP	562

4.7.1.0	Minimum conformance requirements	562

4.7.1.1	Intra-frequency measurements	567

4.7.1.1.1	EN-DC FR1 SS-RSRP absolute measurement accuracy	567

4.7.1.1.2	EN-DC FR1 SS-RSRP relative measurement accuracy	572

4.7.1.2	Inter-frequency measurements	575

4.7.1.2.1	EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy	575

4.7.1.2.2	EN-DC FR1-FR1 SS-RSRP relative measurement accuracy	580

4.7.2	SS-RSRQ	582

4.7.2.0	Minimum conformance requirements	582

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	584

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	589

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	589

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	593

4.7.3	SS-SINR	595

4.7.3.0	Minimum conformance requirements	595

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	595

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	596

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	597

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	597

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	601

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	601

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	605

4.7.4	L1-RSRP	607

4.7.4.0	Minimum conformance requirements	607

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	607

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	610

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	611

4.7.4.1	SSB based L1-RSRP measurements	612

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	612

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	617

4.7.4.2	CSI-RS based L1-RSRP measurements	619

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	619

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	623

4.7.5	SFTD	625

4.7.5.0	Minimum conformance requirements	625

4.7.5.0.1	SFTD Accuracy Requirement	625

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	626

4.7.6	CLI measurements	630

4.7.6.0	Minimum conformance requirements	630

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	630

4.7.6.0.1.1	SRS-RSRP report mapping	631

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	632

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	632

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	641

4.7.7	L1-SINR measurement for beam reporting	644

4.7.7.0	Minimum conformance requirements	644

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	644

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	647

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	649

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	653

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	653

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	658

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	660

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	665

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	665

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	671

4.7.8	CSI-RSRP	673

4.7.8.0	Minimum conformance requirements	673

4.7.8.0.1	Intra-frequency absolute CSI-RSRP measurement accuracy	673

4.7.8.0.2	Intra-frequency relative CSI-RSRP measurement accuracy requirements	676

4.7.8.0.3	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	677

4.7.8.0.4	Inter-frequency relative CSI-RSRP measurement accuracy requirements	678

4.7.8.1	Intra-frequency CSI-RSRP measurement accuracy	679

4.7.8.1.1	EN-DC FR1 CSI-RSRP absolute measurement accuracy	679

4.7.8.1.2	EN-DC FR1 CSI-RSRP relative measurement accuracy	686

4.7.8.2	Inter-frequency measurement accuracy	688

4.7.8.2.1	EN-DC FR1 CSI-RSRP absolute measurement accuracy	688

4.7.8.2.2	EN-DC FR1 CSI-RSRP relative measurement accuracy	694

4.7.9	CSI-RSRQ	696

4.7.9.0	Minimum conformance requirements	696

4.7.9.0.1	Intra-frequency CSI-RSRQ accuracy requirements	696

4.7.9.0.2	Inter-frequency CSI-RSRQ accuracy requirements	697

4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	700

4.7.9.1.1	Test purpose	700

4.7.9.1.2	Test applicability	700

4.7.9.1.3	Minimum conformance requirements	700

4.7.9.1.4	Test description	700

4.7.9.1.5	Test Requirements	702

4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	706

4.7.9.2.1	Test purpose	706

4.7.9.2.2	  Test applicability	706

4.7.9.2.3	Minimum conformance requirements	706

4.7.9.2.4	Test description	706

4.7.9.2.5	Test Requirements	708

4.7.10	CSI-SINR	712

4.7.10.0	Minimum conformance requirements	712

4.7.10.0.1	Intra-frequency CSI-SINR accuracy requirements in FR1	712

4.7.10.0.1.1	Absolute CSI-SINR accuracy in FR1	712

4.7.10.0.2	Inter-frequency CSI-SINR accuracy requirements in FR1	713

4.7.10.0.2.1	Absolute accuracy of CSI-SINR in FR1	713

4.7.10.0.2.2	Relative accuracy of CSI-SINR in FR1	715

4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	716

4.7.10.1.1	Test purpose	716

4.7.10.1.2	  Test applicability	716

4.7.10.1.3	Minimum conformance requirements	716

4.7.10.1.4	Test description	716

4.7.10.1.5	Test Requirements	718

4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	721

4.7.10.2.1	Test purpose	721

4.7.10.2.2	Test applicability	721

4.7.10.2.3	Minimum conformance requirements	721

4.7.10.2.4	Test description	722

4.7.10.2.5	Test Requirements	723

4.7.11	TDCP amplitude measurement accuracy	726

4.7.11.0	Minimum conformance requirements	726

4.7.11.0.1	TDCP amplitude measurement accuracy requirements in FR1	726

4.7.11.1	TDCP amplitude measurement accuracy in EN-DC	727

4.7.11.1.1	Test purpose	727

4.7.11.1.2	Test applicability	727

4.7.11.1.3	Minimum conformance requirements	727

4.7.11.1.4	Test description	727

4.7.11.1.5	Test Requirements	729

4A	NE-DC with all NR cells in FR1	731

4A.0	General	731

4A.1	Signalling characteristics	731

4A.1.1	E-UTRAN PSCell addition	731

4A.1.1.0	Minimum conformance requirements	731

4A.1.1.0.1	E-UTRA PSCell Addition Delay Requirement	731

4A.1.1.0.2	E-UTRA PSCell Release Delay Requirement	732

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	732

4A.1.2	Active BWP switch delay	738

4A.1.2.0	Minimum conformance requirements	738

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	738

4A.2	Measurement performance requirements	742

4A.2.1	SFTD accuracy	742

4A.2.1.0	Minimum conformance requirements	742

4A.1.1.0.1	NE-DC SFTD accuracy Requirement	742

4A.2.1.1	NE-DC FR1 SFTD accuracy	744

5	EN-DC with at least one NR cell in FR2	739

5.0	General	739

5.1	Void	739

5.2	Void	739

5.3	RRC_CONNECTED state mobility	739

5.3.1	Void	739

5.3.2	RRC connection mobility control	739

5.3.2.1	Void	739

5.3.2.2	Random access	739

5.3.2.2.1	EN-DC FR2 contention based random access	739

5.3.2.2.2	EN-DC FR2 non-contention based random access	747

5.3.2.2.3	EN-DC FR2 2-step contention based random access	756

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	760

5.3.2.3	Void	764

5.4	Timing	764

5.4.1	UE transmit timing	764

5.4.1.0	Minimum Conformance Requirements	764

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	764

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	766

5.4.1.2	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	772

5.4.2	UE timer accuracy	779

5.4.3	Timing advance	779

5.4.3.0	Minimum conformance requirements	779

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	779

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	780

5.5	Signaling characteristics	785

5.5.1	Radio link monitoring	785

5.5.1.0	Minimum conformance requirements	785

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	785

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	786

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	787

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	790

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	793

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	794

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	794

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	801

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	806

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	812

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	817

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	822

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	827

5.5.1.8	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	834

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	840

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	845

5.5.1.11	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	850

5.5.2	Interruption	856

5.5.2.0	Minimum conformance requirements	856

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	856

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	856

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	857

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	858

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	859

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	860

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	864

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	868

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	874

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	879

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	883

5.5.2.7	E-UTRAN - NR FR2 interruptions at E-UTRA SRS carrier based switching	887

5.5.2.8	E-UTRAN - NR FR2 interruptions at NR SRS carrier based switching	893

5.5.3	SCell activation and deactivation delay	899

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	911

5.5.3.2 to 5.5.3.5	918

5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	918

5.5.3.7	EN-DC FR2 direct SCell activation at SCell addition of known SCell	926

5.5.3.8	EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band	930

5.5.4	UE UL carrier RRC reconfiguration delay	939

5.5.5	Link recovery procedures	939

5.5.5.0	Minimum conformance requirements	939

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	939

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	943

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	948

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	949

5.5.5.0.5	Requirements for SSB based beam failure detection for UE fulfilling relaxed measurement criteria	949

5.5.5.0.6	TRP specific Minimum conformance requirements	950

5.5.5.0.6.1	Requirements for TRP specific CSI-RS based beam failure detection	950

5.5.5.0.6.2	Requirements for CSI-RS based candidate beam detection	951

5.5.5.0.6.3	Requirements for TRP specific Beam Failure Recovery	952

5.5.5.0.6.4	Scheduling availability of UE during TRP specific beam failure detection	952

5.5.5.0.6.5	Scheduling availability of UE during TRP specific candidate beam detection	952

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	953

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	960

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	967

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	974

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	981

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	988

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	995

5.5.5.8	EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	1001

5.5.5.9	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion	1008

5.5.6	Active BWP switch delay	1015

5.5.6.1	DCI-based and time-based active BWP switch	1015

5.5.6.1.0	Minimum conformance requirements	1015

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	1015

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	1016

5.5.6.2	RRC-based active BWP switch	1016

5.5.6.2.0	Minimum conformance requirements	1016

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	1017

5.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	1024

5.5.6.3.0	Minimum conformance requirements	1024

5.5.6.3.0.1	Minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs	1024

5.5.6.3.0.1.1	Minimum conformance requirements for simultaneous DCI-based active BWP switch on multiple CCs	1024

5.5.6.3.0.1.2	Minimum conformance requirements for simultaneous timer-based active BWP switch on multiple CCs	1026

5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1026

5.5.6.4	SCell dormancy switch	1030

5.5.6.4.0	Minimum conformance requirements	1030

5.5.6.4.0.1	Minimum conformance requirements for interruptions due to SCell dormancy switch	1030

5.5.6.4.0.2	Minimum conformance requirements for interruptions due to CQI measurements during SCell dormancy	1031

5.5.6.4.0.3	Minimum conformance requirements for interruptions due to RRM measurements during SCell dormancy	1031

5.5.6.4.0.4	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	1031

5.5.6.4.0.5	Minimum conformance requirements for simultaneous DCI based BWP switch delay on multiple CCs	1032

5.5.6.4.0.6	Minimum conformance requirements for interruptions due to NR SCell dormancy switch	1034

5.5.6.4.0.7	Minimum conformance requirements for interruptions due to CSI and RRM measurements during SCell dormancy	1034

5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 Scell inside active time	1034

5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1038

5.5.6.5.0	Minimum conformance requirements	1038

5.5.6.5.0.1	Minimum conformance requirements for E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1038

5.5.6.5.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1039

5.5.7	PSCell addition and release delay	1047

5.5.7.0	Minimum conformance requirements	1047

5.5.7.1	Addition and Release Delay of NR PSCell	1047

5.5.8	Active TCI state switch delay	1051

5.5.8.0	Minimum conformance requirements	1051

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	1051

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	1052

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	1053

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	1057

5.5.9	Uplink spatial relation switch delay	1061

5.5.9.0	Minimum conformance requirements	1061

5.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	1061

5.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	1062

5.5.9.1	MAC-CE based uplink spatial relation switch	1062

5.5.9.1.1	E-UTRAN - NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1062

5.5.9.2	RRC based uplink spatial relation switch	1066

5.5.9.2.1	E-UTRAN - NR PSCell FR2 spatial relation switch associated with a known DL-RS	1066

5.5.10	UE specific CBW change	1070

5.5.10.0	Minimum conformance requirements	1070

5.5.10.0.1	Minimum conformance requirements for UE specific CBW change	1070

5.5.10.1	UE specific CBW change on FR2 NR PSCell	1071

5.5.11	Unified TCI state switch delay	1075

5.5.11.0	Minimum conformance requirements	1075

5.5.11.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	1075

5.5.11.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	1077

5.5.11.1	EN-DC FR2 MAC-CE based active joint TCI state switch	1079

5.5.11.2	EN-DC FR2 MAC-CE based active uplink TCI state switch	1086

5.5.11.3	EN-DC FR2 MAC-CE based active downlink TCI state switch	1092

5.5.12	PSCell activation and deactivation delay	1098

5.5.12.0	Minimum conformance requirements	1098

5.5.12.1	EN-DC FR2 PSCell activation and deactivation delay	1099

5.5.13	Conditional PSCell addition and release delay	1103

5.5.13.1	EN-DC FR2 Addition and Release Delay of NR PSCell	1103

5.6	Measurement procedures	1109

5.6.1	Intra-frequency measurements	1109

5.6.1.0	Minimum conformance requirements	1109

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1109

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1112

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	1113

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	1119

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	1124

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	1129

5.6.2	Inter-frequency measurements	1134

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1134

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	1136

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	1141

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	1146

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	1151

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	1156

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	1161

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	1167

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	1172

5.6.3	L1-RSRP measurement for beam reporting	1177

5.6.3.0	Minimum conformance requirements	1178

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1178

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1179

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	1182

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	1186

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1190

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	1194

5.6.4	CLI measurements	1197

5.6.4.0	Minimum conformance requirements	1197

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	1198

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	1199

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	1202

5.6.5	Measurements with autonomous gaps	1206

5.6.5.0	Minimum conformance requirements	1206

5.6.5.0.1	Minimum conformance requirements for EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1206

5.6.5.0.1.1	Introduction	1206

5.6.5.0.1.2	CGI identification of an NR cell with autonomous gaps	1206

5.6.5.0.1.3	CGI reporting delay	1207

5.6.5.1	EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1207

5.6.6	L1-SINR measurement for beam reporting	1212

5.6.6.0	Minimum conformance requirements	1212

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1212

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1215

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1217

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1220

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1224

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1230

5.6.7	CSI-RS based Intra-frequency Measurements	1234

5.6.7.0	Minimum conformance requirements	1234

5.6.7.0.1	Minimum conformance requirements for event-triggered measurement without gap	1234

5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1237

5.6.8	CSI-RS based Inter-frequency Measurements	1243

5.6.8.0	Minimum conformance requirements for Inter-frequency measurements	1243

5.6.8.1	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1245

5.7	Measurement performance requirements	1251

5.7.1	SS-RSRP	1251

5.7.1.0	Minimum conformance requirements	1251

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1251

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1252

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	1254

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	1262

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	1269

5.7.2	SS-RSRQ	1275

5.7.2.0	Minimum conformance requirements	1275

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1275

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1275

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	1277

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	1281

5.7.3	SS-SINR	1286

5.7.3.0	Minimum conformance requirements	1286

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1286

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1286

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	1288

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	1292

5.7.4	L1-RSRP	1297

5.7.4.0	Minimum conformance requirements	1297

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	1298

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	1298

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	1299

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	1300

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	1301

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	1307

5.7.5	SRS-RSRP	1313

5.7.5.0	Minimum conformance requirements	1313

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1313

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1315

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1320

5.7.5.2.1	Test purpose	1321

5.7.5.2.2	Test applicability	1321

5.7.5.2.3	Minimum conformance requirements	1321

5.7.5.2.4	Test description	1321

5.7.5.2.5	Test requirement	1322

5.7.6	L1-SINR measurement for beam reporting	1325

5.7.6.0	Minimum conformance requirements	1325

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1325

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1326

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1329

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1331

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1336

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1341

5.7.7	CSI-RSRP	1346

5.7.7.0	Minimum conformance requirements	1346

5.7.7.0.1	Intra-frequency CSI-RSRP measurement accuracy requirements	1346

5.7.7.0.2	Inter-frequency CSI-RSRP measurement accuracy requirements	1348

5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1350

5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1357

5.7.8	CSI-RSRQ	1365

5.7.8.0	Minimum conformance requirements	1365

5.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	1365

5.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	1367

5.7.8.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1369

5.7.8.1.1	Test purpose	1369

5.7.8.1.2	Test applicability	1369

5.7.8.1.3	Minimum conformance requirements	1369

5.7.8.1.4	Test description	1370

5.7.8.1.5	Test Requirements	1371

5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1373

5.7.8.2.1	Test purpose	1373

5.7.8.2.2	Test applicability	1374

5.7.8.2.3	Minimum conformance requirements	1374

5.7.8.2.4	Test description	1374

5.7.8.2.5	Test Requirements	1375

5.7.9	CSI-SINR	1378

5.7.9.0	Minimum conformance requirements	1378

5.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	1378

5.7.9.0.1.1	Absolute CSI-SINR accuracy	1378

5.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	1379

5.7.9.0.2.1	Absolute CSI-SINR accuracy	1379

5.7.9.0.2.2	Relative CSI-SINR accuracy	1380

5.7.9.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1381

5.7.9.1.1	Test purpose	1381

5.7.9.1.2	Test applicability	1381

5.7.9.1.3	Minimum conformance requirements	1381

5.7.9.1.4	Test description	1382

5.7.9.1.5	Test Requirements	1383

5.7.9.2	EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1385

5.7.9.2.1	Test purpose	1386

5.7.9.2.2	  Test applicability	1386

5.7.9.2.3	Minimum conformance requirements	1386

5.7.9.2.4	Test description	1386

5.7.9.2.5	Test Requirements	1387

6	NR standalone in FR1	1392

6.0	General	1392

6.1	RRC_IDLE state mobility	1392

6.1.1	NR cell re-selection	1392

6.1.1.0	Minimum conformance requirements	1392

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1392

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1392

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	1395

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	1395

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	1397

6.1.1.1	NR SA FR1 cell re-selection	1399

6.1.1.1.5	Test requirement	1403

6.1.1.2	NR SA FR1-FR1 cell re-selection	1405

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1409

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1416

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1421

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1428

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1434

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1439

6.1.1.9	NR SA FR1 cell re-selection for UE operating on a cell with less than 5MHz BW	1445

6.1.2	NR – E-UTRA cell re-selection	1447

6.1.2.0	Minimum conformance requirements	1447

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	1447

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	1449

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1451

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1456

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1463

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1469

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1474

6.2	RRC_INACTIVE state mobility	1480

6.2.0	General	1480

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1480

6.2.1	NR SA FR1 TA Validation for CG-SDT	1481

6.3	RRC_CONNECTED state mobility	1487

6.3.1	Handover	1487

6.3.1.0	Minimum conformance requirements	1487

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	1487

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	1487

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	1488

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	1489

6.3.1.1	NR SA FR1 handover with known target cell	1491

6.3.1.2	NR SA FR1 handover with unknown target cell	1495

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1498

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1501

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1509

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1514

6.3.1.7	NR SA FR1 synchronous DAPS handover	1520

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1528

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1533

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1542

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1548

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1557

6.3.1.13 to 6.3.1.17	1565

6.3.1.18	NR SA FR1 handover with unknown target cell operating with 12 PRB SSB bandwidth	1565

6.3.2	RRC connection mobility control	1567

6.3.2.1	RRC re-establishment	1567

6.3.2.1.0	Minimum conformance requirements	1567

6.3.2.1.1	NR SA FR1 RRC re-establishment	1569

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1573

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1577

6.3.2.2	Random access	1581

6.3.2.2.0	Minimum conformance requirements	1581

6.3.2.2.1	NR SA FR1 contention based random access	1583

6.3.2.2.2	NR SA FR1 non-contention based random access	1589

6.3.2.2.3	NR SA FR1 2-step contention based random access	1596

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1600

6.3.2.3	RRC connection release with redirection	1604

6.3.2.3.0	Minimum conformance requirements	1604

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1606

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1609

6.3.2.4	LTM PDCCH-order Random Access	1614

6.3.2.4.0	Minimum conformance requirements	1614

6.3.2.4.1	NR SA FR1 PDCCH-order RACH on neighbour cell when RACH BW is within active UL BWP	1615

6.3.2.4.2	NR SA FR1 PDCCH-ordered RACH to an inter-frequency candidate cell for LTM	1621

6.3.2.4.3	NR SA FR1 PDCCH-order RACH on neighbour cell without L1-RSRP measurement when RACH BW is within active UL BWP	1628

6.3.3	Conditional handover	1635

6.3.3.0	Minimum conformance requirements	1635

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	1635

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	1638

6.3.3.0.3	Minimum conformance requirements for SA NR-DC FR1 Conditional handover including target MCG and target SCG	1641

6.3.3.0.4	Minimum conformance requirements for NR-DC SA FR1 Conditional handover including target MCG and candidate SCG	1643

6.3.3.1	NR SA FR1 conditional handover	1645

6.3.3.2	NR SA FR1-FR1 conditional handover	1652

6.3.3.3	NR SA FR1 conditional handover including target MCG and target SCG	1659

6.3.3.4	NR SA FR1 conditional handover including target MCG and candidate SCG	1666

6.3.3.5	1675

6.3.3.6	NR SA FR1 Intra-frequency NES triggering conditional handover	1675

6.3.3.7	NR SA FR1-FR1 Inter-frequency NES-based conditional handover	1683

6.3.4	LTM PCell Switch	1692

6.3.4.0	Minimum conformance requirements	1692

6.3.4.0.1	Minimum conformance requirements for LTM PCell Switch	1692

6.3.4.1	NR SA FR1 RACH-based Intra-frequency PCell switch from FR1 to FR1	1695

6.3.4.2	NR SA FR1 RACH-based Inter-frequency LTM PCell switch from FR1 to FR1	1701

6.3.4.3	NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1	1710

6.3.4.4	NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement	1719

6.3.5	LTM PSCell Switch	1728

6.3.5.0	Minimum conformance requirements	1728

6.3.5.0.1	Minimum conformance requirements for LTM PCell Switch	1728

6.3.5.1	NR SA FR1 RACH-based intra-frequency LTM PSCell switch	1728

6.4	Timing	1736

6.4.1	UE transmit timing	1736

6.4.1.0	Minimum conformance requirements	1736

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1737

6.4.2	UE timer accuracy	1743

6.4.3	Timing advance	1743

6.4.3.0	Minimum conformance requirement	1743

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	1743

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1743

6.5	Signaling characteristics	1748

6.5.1	Radio link monitoring	1748

6.5.1.00	General	1748

6.5.1.0	Minimum conformance requirements	1750

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	1750

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	1751

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	1753

6.5.1.0.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	1754

6.5.1.0.5	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	1754

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1755

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1761

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1767

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1772

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1779

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1786

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1792

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1799

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1805

6.5.1.9.1	Test purpose	1805

6.5.1.9.2	Test applicability	1806

6.5.1.9.3	Minimum conformance requirements	1806

6.5.1.9.4	Test description	1806

6.5.1.9.5	Test requirement	1811

6.5.1.10 to 6.5.1.12	1813

6.5.1.13	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW	1813

6.5.1.14	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW	1816

6.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode	1819

6.5.2	Interruption	1822

6.5.2.0	Minimum conformance requirements	1822

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1822

6.5.2.0.2	 Interruptions at NR SRS carrier based switching	1823

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1825

6.5.2.2	SA interruptions at NR SRS carrier based switching	1830

6.5.3	SCell activation and deactivation delay	1835

6.5.3.0	Minimum conformance requirements	1835

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1835

6.5.3.0.2	Minimum conformance requirements for TRS based SCell activation	1835

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1837

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1850

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1851

6.5.3.4	NR SA FR1 direct SCell activation at SCell addition of known SCell	1853

6.5.3.5	NR SA FR1 direct SCell activation at handover with known SCell	1862

6.5.3.6 to 6.5.3.9	1871

6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	1871

6.5.3.11	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	1879

6.5.3.12 to 6.5.3.14	1882

6.5.3.15	NR SA FR1 TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX	1882

6.5.3.16	NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS in non-DRX	1890

6.5.4	UE UL carrier RRC reconfiguration delay	1897

6.5.4.0	Minimum conformance requirements	1897

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1897

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1897

6.5.5	Link recovery procedures	1904

6.5.5.0	Minimum conformance requirements	1904

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1904

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1904

6.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	1906

6.5.5.0.4	Requirements for Beam Failure Recovery in SCell	1906

6.5.5.0.5	TRP specific Minimum conformance requirements	1906

6.5.5.0.5.1	Requirements for CSI-RS based beam failure detection	1906

6.5.5.0.5.2	Requirements for CSI-RS based candidate beam detection	1907

6.5.5.0.5.3	Requirements for TRP specific Beam Failure Recovery	1908

6.5.5.0.5.4	Scheduling availability of UE during TRP specific beam failure detection	1908

6.5.5.0.5.5	Scheduling availability of FR1 UE during TRP specific candidate beam detection	1908

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1908

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1916

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1924

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1931

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1939

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1947

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1954

6.5.6	Active BWP switch delay	1962

6.5.6.1	DCI-based and time-based active BWP switch	1962

6.5.6.1.0	Minimum conformance requirements	1962

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1963

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1974

6.5.6.2	RRC-based active BWP switch	1983

6.5.6.2.0	Minimum conformance requirements	1983

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1983

6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1991

6.5.6.3.0	Minimum conformance requirements	1991

6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1991

6.5.6.4	2001

6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2001

6.5.6.5.0	Minimum conformance requirements	2001

6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2002

6.5.7	DL interruptions at switching between two uplink carriers	2009

6.5.7.0	Minimum conformance requirements	2009

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	2009

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	2015

6.5.7A	NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2020

6.5.7A.0	Minimum conformance requirements	2020

6.5.7A.1	NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA	2021

6.5.7A.2	NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA	2026

6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2031

6.5.7B.0	Minimum conformance requirements	2031

6.5.7B.1	NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA	2032

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA	2038

6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2044

6.5.7C.0	Minimum conformance requirements	2044

6.5.7C.1	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2045

6.5.7C.1.1	Test purpose	2045

6.5.7C.1.2	Test applicability	2045

6.5.7C.1.3	Minimum conformance requirements	2045

6.5.7C.1.4	Test description	2045

6.5.7C.1.4.1	Initial conditions	2045

6.5.7C.1.5	Test requirements	2047

6.5.7C.2	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2050

6.5.7C.2.1	Test purpose	2050

6.5.7C.2.2	Test applicability	2050

6.5.7C.2.3	Minimum conformance requirements	2050

6.5.7C.2.4	Test description	2051

6.5.7C.2.4.1	Initial conditions	2051

6.5.7D	 DL interruptions at UE switching across three or four uplink bands	2057

6.5.7D.0	Minimum conformance requirements	2057

6.5.7D.1	NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	2057

6.5.7D.2	NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	2062

6.5.7D.3	NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	2069

6.5.7D.4	NR SA FR1 DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs	2074

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.6.5.8	UE specific CBW change	2080

6.5.8.0	Minimum conformance requirements	2080

6.5.8.0.1	Minimum conformance requirements for UE specific CBW change	2080

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2081

6.6	Measurement procedures	2088

6.6.1	Intra-frequency measurements	2088

6.6.1.0	Minimum conformance requirements	2088

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	2088

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	2088

6.6.1.0.3	Void	2089

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	2089

6.6.1.0.5	Void	2091

6.6.1.0.6	Minimum conformance requirements for event-triggered measurement with MUSIM gap configured	2091

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	2093

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	2098

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	2103

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	2109

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	2114

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	2118

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	2122

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	2127

6.6.1.9	NR SA FR1 event triggered reporting with MUSIM gap configured	2132

6.6.1.10 to 6.6.1.12	2137

6.6.1.12	NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth	2137

6.6.1.13	NR SA FR1 event-triggered reporting tests without gap under Cell DTX	2140

6.6.2	Inter-frequency measurements	2145

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2145

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	2148

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	2154

6.6.2.3	Void	2160

6.6.2.4	Void	2160

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	2160

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	2166

6.6.2.7	Void	2172

6.6.2.8	Void	2172

6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2172

6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2177

6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	2183

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	2189

6.6.2.13	NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority	2195

6.6.2.14	NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP	2203

6.6.2.15	NR SA FR1 event triggered reporting without SSB index detection in DRX mode for UE operating on a cell with less than 5MHz BW	2211

6.6.3	2214

6.6.3.0	Minimum conformance requirements	2214

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	2214

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	2216

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	2218

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	2226

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	2232

6.6.4	L1-RSRP measurement for beam reporting	2240

6.6.4.0	Minimum conformance requirements	2240

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2240

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2240

6.6.4.0.3	Inter-cell SSB based L1-RSRP Reporting	2240

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	2241

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	2245

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	2248

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	2253

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	2256

6.6.4.6	NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX	2262

6.6.5	UTRAN inter-RAT measurement	2267

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	2267

6.6.6	CLI Measurements	2274

6.6.6.0	Minimum conformance requirements	2274

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement	2274

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	2274

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	2274

6.6.7	NR measurements with autonomous gaps	2281

6.6.7.0	Minimum conformance requirements	2281

6.6.7.0.1	Minimum conformance requirements for SA intra-frequency CGI identification of NR neighbour cell in FR1	2281

6.6.7.0.1.1	Introduction	2281

6.6.7.0.1.2	CGI identification of an NR cell with autonomous gaps	2281

6.6.7.0.1.3	CGI reporting delay	2282

6.6.7.0.2	Minimum conformance requirements for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2282

6.6.7.0.2.1	CGI identification of an E-UTRA cell with autonomous gaps	2282

6.6.7.0.2.2	CGI reporting delay	2283

6.6.7.1	SA intra-frequency CGI identification of NR neighbour cell in FR1	2283

6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2287

6.6.8	L1-SINR measurement for beam reporting	2291

6.6.8.0	Minimum conformance requirements	2291

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2291

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2291

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2291

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	2291

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	2297

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	2303

6.6.9	Idle Mode CA/DC Measurements	2309

6.6.9.0	Minimum conformance requirements	2309

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	2311

6.6.10 to 6.6.14	2321

6.6.15	Idle Mode inter-RAT CA/DC Measurements	2321

6.6.15.0	Minimum conformance requirements	2321

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2322

6.6.16	2335

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	2335

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	2337

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2343

6.6.18	SA event triggered reporting tests with concurrent gaps	2351

6.6.18.0	Minimum conformance requirements	2351

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement	2351

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement	2352

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement	2353

6.6.18.0.4	Minimum conformance requirements for PRS measurement	2354

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	2354

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	2362

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2369

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2381

6.6.19	SA event triggered reporting tests with NCSG	2390

6.6.19.0	Minimum conformance requirements	2390

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement	2390

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement	2392

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement	2393

6.6.19.0.4	Minimum conformance requirements for interruptions	2394

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	2395

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2401

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2407

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2415

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2421

6.6.20.0	Minimum conformance requirements	2421

6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC	2424

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	2428

6.6.21.0	Minimum conformance requirements	2428

6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC	2431

6.6.22	2435

6.6.23	2435

6.6.24	SA event triggered reporting tests with NeedForGap	2435

6.6.24.0	Minimum conformance requirements	2435

6.6.24.0.1	Minimum conformance requirements for intra-frequency event-triggered measurement without gap	2435

6.6.24.0.2	Minimum conformance requirements for inter-frequency event-triggered measurement without gap	2439

6.6.24.0.3	Minimum conformance requirements for measurements without gap carried out by UE supporting NeedForInterruptionInfoNR	2441

6.6.24.1	NR SA FR1 event-triggered reporting without gap with interruptions	2442

6.6.24.2	NR SA FR1-FR1 event-triggered reporting without gap with interruption with SSB index	2450

6.6.24.3	NR SA FR1-FR1 event-triggered reporting without gap with interruption	2455

6.6.24.4	NR SA FR1-FR1 event-triggered reporting without gap without interruption	2462

6.6.24.5	NR SA FR1 event-triggered reporting without gap without interruption	2468

6.6.25 SA NR - E-UTRAN event-triggered without measurement gaps	2476

6.6.25.0 Minimum conformance requirements	2476

6.6.25.0.1 Minimum conformance requirements for NR – E-UTRAN FDD measurements when no DRX is used	2476

6.6.25.0.2 Minimum conformance requirements for NR – E-UTRAN TDD measurements when no DRX is used	2477

6.6.25.1	NR SA FR1 - E-UTRAN event-triggered reporting without gap without interruption	2478

6.6.26	LTM Intra-frequency L1-RSRP measurement	2486

6.6.26.0	Minimum conformance requirements	2486

6.6.26.0.1	Minimum conformance requirements for SSB-based intra-frequency L1-RSRP measurement	2486

6.6.26.1	NR SA FR1 Intra-frequency SSB based L1-RSRP measurement	2487

6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap	2494

6.6.27.0	Minimum conformance requirements	2494

6.6.27.0.1	Minimum conformance requirements for SSB-based inter-frequency L1-RSRP measurement with gap	2494

6.6.27.1	NR SA FR1 Inter-frequency SSB based L1-RSRP measurement with measurement gap	2495

6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap	2501

6.6.28.0	Minimum conformance requirements	2501

6.6.28.0.1	Minimum conformance requirements for SSB-based inter-frequency L1-RSRP measurement without gap	2501

6.6.28.1	NR SA FR1 Inter-frequency SSB based L1-RSRP measurement without measurement gap	2503

6.7	Measurement performance requirements	2510

6.7.1	SS-RSRP	2510

6.7.1.0	Minimum conformance requirements	2510

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2510

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	2510

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	2510

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	2510

6.7.1.1	Intra-frequency measurements	2511

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	2511

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	2518

6.7.1.2	Inter-frequency measurements	2520

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	2520

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	2527

6.7.2	SS-RSRQ	2529

6.7.2.0	Minimum conformance requirements	2529

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2529

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	2529

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	2529

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	2529

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	2533

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	2533

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	2538

6.7.3	SS-SINR	2540

6.7.3.0	Minimum conformance requirements	2540

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2540

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	2540

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	2540

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	2540

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	2544

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	2544

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	2548

6.7.4	L1-RSRP	2550

6.7.4.0	Minimum conformance requirements	2550

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	2550

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	2550

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	2550

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	2550

6.7.4.1	SSB based L1-RSRP measurements	2550

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	2550

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	2555

6.7.4.2	CSI-RS based L1-RSRP measurements	2557

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	2557

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	2561

6.7.5	E-UTRAN RSRP	2563

6.7.5.0	Minimum conformance requirements	2563

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	2563

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	2564

6.7.6	E-UTRAN RSRQ	2571

6.7.6.0	Minimum conformance requirements	2571

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	2571

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	2572

6.7.7	E-UTRAN RS-SINR	2579

6.7.7.0	Minimum conformance requirements	2579

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	2579

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	2580

6.7.8	CLI Measurements	2587

6.7.8.0	Minimum conformance requirements	2587

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy	2587

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	2587

6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	2587

6.7.8.1.1	Test purpose	2587

6.7.8.1.2	Test applicability	2587

6.7.8.1.3	Minimum conformance requirements	2588

6.7.8.1.4	Test description	2588

6.7.8.1.5	Test requirement	2590

6.7.9	L1-SINR measurement for beam reporting	2597

6.7.9.0	Minimum conformance requirements	2597

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2597

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	2600

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	2602

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	2606

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	2606

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	2609

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2611

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2614

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2615

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	2618

6.7.10	CSI-RSRP	2620

6.7.10.0	Minimum conformance requirements	2620

6.7.10.1	Intra-frequency measurements	2620

6.7.10.1.1	NR SA FR1 CSI-RSRP absolute measurement accuracy	2620

6.7.10.1.2	NR SA FR1 CSI-RSRP relative measurement accuracy	2625

6.7.10.2	Inter-frequency measurements	2631

6.7.10.2.1	NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy	2631

6.7.10.2.2	NR SA FR1-FR1 CSI-RSRP relative  measurement accuracy	2638

6.7.11	CSI-RSRQ	2644

6.7.11.0	Minimum conformance requirements	2644

6.7.11.0.1	Intra-frequency CSI-RSRQ accuracy requirements	2644

6.7.11.0.1.1	Absolute CSI-RSRQ accuracy	2644

6.7.11.0.2	Inter-frequency CSI-RSRQ accuracy requirements	2645

6.7.11.0.2.1	Absolute CSI-RSRQ accuracy	2646

6.7.11.0.2.2	Relative CSI-RSRQ accuracy	2647

6.7.11.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2648

6.7.11.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2655

6.7.12	CSI-SINR	2662

6.7.12.0	Minimum conformance requirements	2662

6.7.12.0.1	Intra-frequency CSI-SINR accuracy requirements	2662

6.7.12.0.1.1	Absolute CSI-SINR accuracy	2662

6.7.12.0.2	Inter-frequency CSI-SINR accuracy requirements	2663

6.7.12.0.2.1	Absolute CSI-SINR accuracy	2664

6.7.12.0.2.2	Relative CSI-SINR accuracy	2665

6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2666

6.7.12.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2672

6.7.13 to 6.7.16	2680

6.7.17	LTM L1-RSRP measurement	2680

6.7.17.0	Minimum conformance requirements	2680

6.7.17.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	2680

6.7.17.0.2	SSB based relative L1-RSRP measurement accuracy requirements	2681

6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1	2681

6.7.17.1.1	NR SA FR1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell	2681

6.7.17.1.2	Void	2692

6.7.18	TDCP amplitude measurement accuracy	2692

6.7.18.0	Minimum conformance requirements	2692

6.7.18.0.1	TDCP amplitude measurement accuracy requirements in FR1	2692

6.7.18.1	TDCP amplitude measurement accuracy in EN-DC	2692

6.7.18.1.1	Test purpose	2692

6.7.18.1.2	Test applicability	2692

6.7.18.1.3	Minimum conformance requirements	2693

6.7.18.1.4	Test description	2693

6.7.18.1.5	Test Requirements	2694

7	NR standalone with at least one NR cell in FR2	2700

7.0	General	2700

7.1	RRC_IDLE state mobility	2700

7.1.1	NR cell re-selection	2700

7.1.1.0	Minimum conformance requirements	2700

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	2700

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	2703

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	2706

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	2707

7.1.1.1	NR SA FR2 cell re-selection	2708

7.1.1.2	NR SA FR2-FR2 cell re-selection	2716

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2722

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2727

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2732

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2739

7.2	RRC_INACTIVE state mobility	2758

7.2.1	Small Data Transmission	2758

7.2.1.0	Minimum conformance requirements	2758

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	2758

7.2.1.1	TA Validation for CG-SDT in FR2	2758

7.3	RRC_CONNECTED state mobility	2767

7.3.1	Handover	2767

7.3.1.0	Minimum conformance requirements	2767

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	2767

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	2768

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	2769

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	2770

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	2775

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	2779

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	2783

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	2793

7.3.2	RRC connection mobility control	2800

7.3.2.1	RRC re-establishment	2800

7.3.2.1.0	Minimum conformance requirements	2800

7.3.2.1.1	NR SA FR2 RRC re-establishment	2801

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	2805

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	2808

7.3.2.2	Random access	2813

7.3.2.2.0	Minimum conformance requirements	2813

7.3.2.2.2	NR SA FR2 non-contention based random access	2822

7.3.2.2.3	NR SA FR2 2-step contention based random access	2830

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2837

7.3.2.3	RRC connection release with redirection	2844

7.3.2.3.0	Minimum conformance requirements	2844

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2845

7.3.3	Conditional handover	2849

7.3.3.0	Minimum conformance requirements	2849

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2849

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2851

7.3.3.0.3	Minimum conformance requirements for NR FR2 – NR FR1 inter-frequency conditional handover	2851

7.3.3.1	NR SA FR2 conditional handover	2853

7.3.3.2	NR SA FR2-FR2 conditional handover	2859

7.3.3.3	NR SA FR2 intra-frequency NES triggering conditional handover	2866

7.3.3.4	NR SA FR2-FR1 inter-frequency NES triggering conditional handover	2870

7.3.3.5	NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	2878

7.3.4	LTM PCell Switch	2886

7.3.4.0	Minimum conformance requirements	2886

7.3.4.0.1	Minimum conformance requirements for NR FR2 LTM PCell Switch	2886

7.3.4.1	NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2	2886

7.3.4.2	NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2	2893

7.3.4.3	NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2	2901

7.3.5	LTM PSCell Switch	2909

7.3.5.0	Minimum conformance requirements	2909

7.3.5.1	NR SA FR2 RACH-based Intra-frequency LTM PSCell switch	2909

7.4	Timing	2914

7.4.1	UE transmit timing	2914

7.4.1.0	Minimum Conformance Requirements	2914

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	2914

7.4.1.1	NR SA FR2 UE transmit timing accuracy	2915

7.4.1.2	2922

7.4.1.3	NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	2922

7.4.2	UE timer accuracy	2932

7.4.3	Timing advance	2932

7.4.3.0	Minimum conformance requirements	2932

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	2932

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	2932

7.5	Signalling characteristics	2938

7.5.1	Radio link monitoring	2938

7.5.1.0	Minimum conformance requirements	2938

7.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	2938

7.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	2938

7.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	2938

7.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	2938

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	2938

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2938

7.5.1.1.1	Test purpose	2939

7.5.1.1.2	Test applicability	2939

7.5.1.1.3	Minimum conformance requirement	2939

7.5.1.1.4	Test description	2939

7.5.1.1.5	Test Requirement	2943

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2944

7.5.1.2.1	Test purpose	2944

7.5.1.2.2	Test applicability	2944

7.5.1.2.3	Minimum conformance requirement	2944

7.5.1.2.4	Test description	2944

7.5.1.2.5	Test Requirement	2950

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2951

7.5.1.3.1	Test purpose	2951

7.5.1.3.2	Test applicability	2951

7.5.1.3.3	Minimum conformance requirement	2951

7.5.1.3.4	Test description	2951

7.5.1.3.5	Test Requirement	2955

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2956

7.5.1.4.1	Test purpose	2956

7.5.1.4.2	Test applicability	2956

7.5.1.4.3	Minimum conformance requirement	2956

7.5.1.4.4	Test description	2956

7.5.1.4.5	Test Requirement	2962

7.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	2963

7.5.1.5.1	Test purpose	2963

7.5.1.5.2	Test applicability	2963

7.5.1.5.3	Minimum conformance requirement	2963

7.5.1.5.4	Test description	2963

7.5.1.5.5	Test Requirement	2968

7.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2969

7.5.1.6.1	Test purpose	2970

7.5.1.6.2	Test applicability	2970

7.5.1.6.3	Minimum conformance requirement	2970

7.5.1.6.4	Test description	2970

7.5.1.6.5	Test Requirement	2975

7.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode	2976

7.5.1.7.1	Test purpose	2976

7.5.1.7.2	Test applicability	2976

7.5.1.7.3	Minimum conformance requirement	2976

7.5.1.7.4	Test description	2976

7.5.1.7.5	Test Requirement	2980

7.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2981

7.5.1.8.1	Test purpose	2981

7.5.1.8.2	Test applicability	2981

7.5.1.8.3	Minimum conformance requirement	2981

7.5.1.8.4	Test description	2981

7.5.1.8.5	Test Requirement	2986

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	2987

7.5.1.9.1	Test purpose	2988

7.5.1.9.2	Test applicability	2988

7.5.1.9.3	Minimum conformance requirement	2988

7.5.1.9.4	Test description	2988

7.5.1.9.5	Test Requirement	2990

7.5.1.10	NR SA FR2 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	2992

7.5.2	Interruption	2998

7.5.2.0	Minimum conformance requirements	2998

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	2998

7.5.2.0.2	Minimum conformance requirements for SA interruptions at NR SRS carrier-based switching	2999

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	3001

7.5.2.1.1	Test purpose	3001

7.5.2.1.2	Test applicability	3001

7.5.2.1.3	Minimum conformance requirements	3001

7.5.2.1.4	Test description	3001

7.5.2.1.5	Test requirement	3003

7.5.2.2	SA interruptions at NR SRS carrier-based switching	3006

7.5.2.2.1	Test purpose	3006

7.5.2.2.2	Test applicability	3006

7.5.2.2.3	Minimum conformance requirements	3006

7.5.2.2.4	Test description	3006

7.5.2.2.5	Test requirement	3007

7.5.3	SCell activation and deactivation delay	3009

7.5.3.0	Minimum conformance requirements	3009

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	3009

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	3012

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	3012

7.5.3.1.1	Test purpose	3013

7.5.3.1.2	Test applicability	3013

7.5.3.1.3	Minimum conformance requirements	3013

7.5.3.1.4	Test description	3013

7.5.3.1.5	Test requirement	3014

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	3014

7.5.3.2.1	Test purpose	3015

7.5.3.2.2	Test applicability	3015

7.5.3.2.3	Minimum conformance requirements	3015

7.5.3.2.4	Test description	3015

7.5.3.2.5	Test requirement	3016

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3017

7.5.3.3.1	Test purpose	3017

7.5.3.3.2	Test applicability	3017

7.5.3.3.3	Minimum conformance requirements	3017

7.5.3.3.4	Test description	3017

7.5.3.3.5	Test requirement	3022

7.5.3.4	NR SA FR2 direct SCell activation at SCell addition of known SCell	3025

7.5.3.4.1	Test Purpose	3025

7.5.3.4.2	Test applicability	3025

7.5.3.4.3	Minimum conformance requirements	3025

7.5.3.4.4	Test description	3025

7.5.3.4.5	Test requirement	3030

7.5.3.5	NR SA FR2 direct SCell activation at handover with known SCell	3032

7.5.3.5.1	Test Purpose	3032

7.5.3.5.2	Test applicability	3032

7.5.3.5.3	Minimum conformance requirements	3032

7.5.3.5.4	Test description	3032

7.5.3.5.5	Test requirement	3037

7.5.3.6 to 7.5.3.12	3040

7.5.3.13	NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX	3040

7.5.3.13.1	Test Purpose	3040

7.5.3.13.2	Test applicability	3040

7.5.3.13.3	Minimum conformance requirements	3040

7.5.3.13.4	Test description	3040

7.5.3.13.5	Test requirement	3045

7.5.3.14	NR SA FR2 SCell Activation for known SCell in FR2 inter-band	3047

7.5.3.14.1	Test purpose	3048

7.5.3.14.2	Test applicability	3048

7.5.3.14.3	Minimum conformance requirements	3048

7.5.3.14.4	Test description	3048

7.5.3.14.5	Test requirement	3053

7.5.4	Void	3054

7.5.5	Link recovery procedures	3054

7.5.5.0	Minimum conformance requirements	3054

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	3054

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	3054

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	3055

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	3055

7.5.5.0.5	TRP specific Minimum conformance requirements	3055

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	3056

7.5.5.1.1	Test purpose	3057

7.5.5.1.2	Test applicability	3057

7.5.5.1.3	Minimum conformance requirements	3057

7.5.5.1.4	Test description	3057

7.5.5.1.5	Test requirement	3060

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	3061

7.5.5.2.1	Test purpose	3062

7.5.5.2.2	Test applicability	3062

7.5.5.2.3	Minimum conformance requirements	3062

7.5.5.2.4	Test description	3062

7.5.5.2.5	Test requirement	3066

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	3068

7.5.5.3.1	Test purpose	3068

7.5.5.3.2	Test applicability	3068

7.5.5.3.3	Minimum conformance requirements	3068

7.5.5.3.4	Test description	3068

7.5.5.3.5	Test requirement	3072

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	3073

7.5.5.4.1	Test purpose	3074

7.5.5.4.2	Test applicability	3074

7.5.5.4.3	Minimum conformance requirements	3074

7.5.5.4.4	Test description	3074

7.5.5.4.5	Test requirement	3078

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	3080

7.5.5.5.1	Test purpose	3080

7.5.5.5.2	Test applicability	3080

7.5.5.5.3	Minimum conformance requirements	3080

7.5.5.5.4	Test description	3080

7.5.5.5.5	Test requirement	3084

7.5.5.6	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX	3085

7.5.5.6.1	Test purpose	3086

7.5.5.6.2	Test applicability	3086

7.5.5.6.3	Minimum conformance requirements	3086

7.5.5.6.4	Test description	3086

7.5.5.6.5	Test requirement	3090

7.5.5.7	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX	3092

7.5.5.7.1	Test purpose	3092

7.5.5.7.2	Test applicability	3092

7.5.5.7.3	Minimum conformance requirements	3092

7.5.5.7.4	Test description	3092

7.5.5.7.5	Test requirement	3096

7.5.5.8	FFS	3098

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	3098

7.5.5.9.1	Test purpose	3098

7.5.5.9.2	Test applicability	3098

7.5.5.9.3	Minimum conformance requirements	3098

7.5.5.9.4	Test description	3098

7.5.5.9.5	Test requirements	3103

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	3104

7.5.5.10.1	Test purpose	3104

7.5.5.10.2	Test applicability	3104

7.5.5.10.3	Minimum conformance requirements	3104

7.5.5.10.4	Test description	3104

7.5.5.10.5	Test requirement	3109

7.5.6	Active BWP switch delay	3111

7.5.6.1	DCI-based and time-based active BWP switch	3111

7.5.6.1.0	Minimum conformance requirements	3111

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	3111

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	3112

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	3113

7.5.6.2	RRC-based active BWP switch	3114

7.5.6.2.0	Minimum conformance requirements	3114

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	3115

7.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	3117

7.5.6.3.0	Minimum conformance requirements	3117

7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	3120

7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3124

7.5.6.5.0	Minimum conformance requirements	3124

7.5.6.5.0.1	Minimum conformance requirements for active BWP switch on multiple SCells with non-DRX in SA	3124

7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3125

7.5.7	PSCell addition and release delay	3131

7.5.7.0	Minimum conformance requirements	3131

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	3131

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	3132

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	3132

7.5.7.1.1	Test purpose	3132

7.5.7.1.2	Test applicability	3133

7.5.7.1.3	Minimum conformance requirements	3133

7.5.7.1.4	Test description	3133

7.5.7.1.5	Test requirement	3134

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	3134

7.5.7.2.1	Test purpose	3134

7.5.7.2.2	Test applicability	3135

7.5.7.2.3	Minimum conformance requirements	3135

7.5.7.2.4	Test description	3135

7.5.7.2.5	Test requirement	3136

7.5.8	Active TCI state switch delay	3136

7.5.8.0	Minimum conformance requirements	3136

7.5.8.0.0	Known conditions for TCI state	3136

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	3137

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	3137

7.5.8.0.3	Minimum conformance requirements for MAC-CE based active TCI state switch in HST FR2	3138

7.5.8.0.5	DCI based dual DL TCI state switching delay for mDCI for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	3139

7.5.8.0.6	DCI based dual DL TCI state switching delay for sDCI for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	3140

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	3140

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	3140

7.5.8.2	NR SA FR2 RRC based active TCI state switch	3147

7.5.8.2.1	NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state	3147

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3154

7.5.8.3.1	NR SA FR2 HST active TCI state switch for a known TCI state	3154

7.5.8.3.2	NR SA FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 a known TCI state	3160

7.5.8.4	NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception	3165

7.5.8.5	NR SA FR2 Single-DCI based active TCI state switch with known target TCI states for simultaneous reception	3173

7.5.9	Uplink spatial relation switch delay	3179

7.5.9.0	Minimum conformance requirements	3179

7.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	3179

7.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	3180

7.5.9.1	MAC-CE based uplink spatial relation switch	3180

7.5.9.1.1	NR PCell FR2 spatial relation associated with known DL-RS	3180

7.5.9.2	RRC based uplink spatial relation switch	3184

7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3184

7.5.10	UE specific CBW change	3188

7.5.10.0	Minimum conformance requirements	3188

7.5.10.0.1	Minimum conformance requirements for NR FR2 UE specific CBW change of PCell with non-DRX in SA	3188

7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3189

7.5.10.1.1	Test purpose	3189

7.5.10.1.2	Test applicability	3189

7.5.10.1.3	Minimum conformance requirements	3189

7.5.10.1.4	Test description	3189

7.5.10.1.5	Test requirement	3190

7.5.11	UE UL carrier RRC reconfiguration delay	3192

7.5.11.1	UE UL carrier RRC reconfiguration delay	3192

7.5.11.1.1	Test purpose	3192

7.5.11.1.2	Test applicability	3193

7.5.11.1.3	Minimum conformance requirements	3193

7.5.11.1.4	Test description	3193

7.5.11.1.5	Test requirement	3195

7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3198

7.5.12.1	NR SA FR2 Addition and Release Delay of PSCell	3198

7.5.13	Unified TCI state switch delay	3203

7.5.13.0	Minimum conformance requirements	3203

7.5.13.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	3203

7.5.13.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	3204

7.5.13.0.3	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI for 2 known TCI states	3204

7.5.13.1	NR SA FR2 MAC-CE based active joint TCI state switch	3205

7.5.13.1.1	Test purpose	3205

7.5.13.1.2	Test applicability	3205

7.5.13.1.3	Minimum conformance requirements	3206

7.5.13.1.4	Test description	3206

7.5.13.1.5	Test requirement	3210

7.5.13.2	NR SA FR2 MAC-CE based active uplink TCI state switch	3212

7.5.13.2.1	Test purpose	3212

7.5.13.2.2	Test applicability	3212

7.5.13.2.3	Minimum conformance requirements	3212

7.5.13.2.4	Test description	3213

7.5.13.2.5	Test requirement	3216

7.5.13.3	NR SA FR2 MAC-CE based active downlink TCI state switch	3218

7.5.13.3.1	Test purpose	3218

7.5.13.3.2	Test applicability	3218

7.5.13.3.3	Minimum conformance requirements	3218

7.5.13.3.4	Test description	3219

7.5.13.3.5	Test requirement	3222

7.5.13.4 to 7.5.13.5	3224

7.5.13.6	MAC-CE based active uplink TCI state switch for single-DCI mTRP	3224

7.5.13.6.1	NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states	3224

7.5.14	PSCell activation and deactivation delay	3232

7.5.14.0	Minimum conformance requirements	3232

7.5.14.0.1	SCG Activation Delay Requirement	3232

7.5.14.0.2	SCG Deactivation Delay Requirement	3233

7.5.14.1	NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3233

7.5.14.1.1	Test purpose	3233

7.5.14.1.2	Test applicability	3233

7.5.14.1.3	Minimum conformance requirements	3233

7.6	Measurement procedures	3239

7.6.1	Intra-frequency measurements	3239

7.6.1.0	Minimum conformance requirements	3239

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	3239

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	3240

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	3242

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	3248

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	3254

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	3260

7.6.1.5	NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3266

7.6.2	Inter-frequency measurements	3270

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	3270

7.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	3270

7.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	3271

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	3273

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	3278

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	3284

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	3289

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	3295

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	3301

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	3307

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	3313

7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3319

7.6.2.10	SA event triggered reporting test without gap under non-DRX	3324

7.6.2.11	SA event triggered reporting test without gap under DRX	3328

7.6.2.12 to 7.6.2.19	3332

7.6.2.20	NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority	3332

7.6.2.21	NR SA FR2-FR2 event triggered reporting using MUSIM gap with shorter MGRP	3339

7.6.3	L1-RSRP measurement for beam reporting	3347

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	3347

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	3347

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	3347

7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting	3347

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	3349

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	3355

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	3359

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	3363

7.6.3.5	NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3366

7.6.3.6	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3372

7.6.4	CLI measurements	3379

7.6.4.0	Minimum conformance requirements	3379

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	3379

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	3380

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	3384

7.6.4.2.1	Test purpose	3384

7.6.4.2.2	Test applicability	3384

7.6.4.2.3	Minimum conformance requirements	3384

7.6.4.2.4	Test description	3384

7.6.4.2.5	Test requirement	3386

7.6.5	NR measurements with autonomous gaps	3387

7.6.5.0	Minimum conformance requirements	3387

7.6.5.0.1	Minimum conformance requirements for SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	3387

7.6.5.0.1.1	Minimum conformance requirements for interruptions when identifying CGI of an NR cell with autonomous gaps	3387

7.6.5.0.1.2	Minimum conformance requirements for NR measurements with autonomous gaps	3388

7.6.5.0.1.2.1	Introduction	3388

7.6.5.0.1.2.2	CGI identification of an NR cell with autonomous gaps	3388

7.6.5.0.1.2.3	CGI reporting delay	3389

7.6.5.1	SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	3389

7.6.6	L1-SINR measurement for beam reporting	3394

7.6.6.0	Minimum conformance requirements	3394

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	3394

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	3394

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	3394

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	3394

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	3398

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	3403

7.6.7 to 7.6.12	3409

7.6.13	UE Rx-Tx time difference measurements for PDC	3409

7.6.13.0	Minimum conformance requirements	3409

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	3409

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	3411

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	3413

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	3417

7.6.14	SA event triggered reporting tests with Pre-MG	3421

7.6.14.0	Minimum conformance requirements	3421

7.6.14.1	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation	3424

7.6.14.2	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	3430

7.6.15	SA event triggered reporting tests with concurrent gaps	3436

7.6.15.0	Minimum conformance requirements	3436

7.6.15.0.1	Minimum conformance requirements for Intra-frequency measurement	3436

7.6.15.0.2	Minimum conformance requirements for Inter-frequency measurement	3436

7.6.15.0.3	Minimum conformance requirements for PRS measurement	3437

7.6.15.1	NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3437

7.6.15.2	NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3443

7.6.15.3	NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3452

7.6.16	SA event triggered reporting tests with NCSG	3459

7.6.16.0	Minimum conformance requirements	3459

7.6.16.0.1	Minimum conformance requirements for intra-frequency measurement	3459

7.6.16.0.2	Minimum conformance requirements for inter-frequency measurement	3461

7.6.16.0.3	Minimum conformance requirements for interruptions	3462

7.6.16.1	NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX	3463

7.6.16.2	NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used (PCell in FR2)	3469

7.6.16.3	NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX	3474

7.6.17	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG in FR2	3480

7.6.17.0	Minimum conformance requirements	3480

7.6.17.0.1	Minimum conformance requirements for SA event triggered reporting with one pre-configured gap and one measurement gap	3480

7.6.17.1	NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX	3480

7.6.17.2	NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs	3483

7.6.18	SA event triggered reporting tests with concurrent gaps and NCSG	3486

7.6.18.0	Minimum conformance requirements	3486

7.6.18.0.1	Minimum conformance requirements for concurrent measurement gaps with NCSG	3486

7.6.18.1	NR SA FR2 event-triggered reporting with concurrent Type-2 measurement gap and NCSG	3488

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.7.6.19 SA event triggered reporting tests with NeedForGap in FR2	3491

7.6.19.0	Minimum conformance requirements	3491

7.6.19.0.1	Minimum conformance requirements for Intra-frequency measurements with measurement gaps	3491

7.6.19.0.2	Minimum conformance requirements for Intra-frequency measurements without measurement gaps	3494

7.6.19.0.3	Minimum conformance requirements for Inter-frequency measurements without measurement gaps	3498

7.6.19.1	NR SA FR2 event-triggered reporting without interruption outside gap	3503

7.6.19.2	NR SA FR2 SA event triggered reporting test without gap without interruption	3507

7.6.19.3	NR SA FR2 event triggered reporting test without gap without interruption under non-DRX	3510

7.6.20	LTM Intra-frequency L1-RSRP measurement	3513

7.6.20.0	Minimum conformance requirements	3513

7.6.20.0.1	Minimum conformance requirements for SSB-based intra-frequency L1-RSRP measurement	3513

7.6.20.1	NR SA FR2 Intra-frequency SSB based L1-RSRP measurement	3515

7.6.21	LTM Inter-frequency L1-RSRP measurement with measurement gap	3521

7.6.21.0	Minimum conformance requirements	3521

7.6.21.0.1	Minimum conformance requirements for SSB-based inter-frequency L1-RSRP measurement with gap	3521

7.6.21.1	NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM	3522

7.6.22	LTM Inter-frequency L1-RSRP measurement without measurement gap	3528

7.6.22.0	Minimum conformance requirements	3528

7.6.22.1	NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX	3528

7.7	Measurement performance requirements	3533

7.7.1	SS-RSRP	3533

7.7.1.0	Minimum conformance requirements	3533

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	3533

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	3533

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	3533

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	3541

7.7.1.3	Inter-frequency measurements between FR1 and FR2	3549

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	3549

7.7.1.3.2	Void	3556

7.7.2	SS-RSRQ	3556

7.7.2.0	Minimum conformance requirements	3556

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	3556

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	3556

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	3556

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	3560

7.7.3	SS-SINR	3566

7.7.3.0	Minimum conformance requirements	3566

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	3566

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	3566

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	3566

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	3571

7.7.4	L1-RSRP	3578

7.7.4.0	Minimum conformance requirements	3578

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	3578

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	3578

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	3578

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	3578

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	3578

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	3583

7.7.5	SRS-RSRP	3589

7.7.5.0	Minimum conformance requirements	3589

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	3589

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	3591

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	3596

7.7.5.2.1	Test purpose	3597

7.7.5.2.2	Test applicability	3597

7.7.5.2.3	Minimum conformance requirements	3597

7.7.5.2.4	Test description	3597

7.7.5.2.4	Test description	3597

7.7.5.2.5	Test requirement	3598

7.7.6	L1-SINR	3601

7.7.6.0	Minimum conformance requirements	3601

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	3601

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	3603

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	3605

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	3608

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	3613

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	3618

7.7.7	CSI-RSRP	3623

7.7.7.0	Minimum conformance requirements	3623

7.7.7.1	Intra-frequency measurements	3623

7.7.7.1.1	NR SA FR2 CSI-RSRP  measurement accuracy	3623

7.7.7.2	Inter-frequency measurements	3626

7.7.7.2.1	NR SA FR2-FR2 CSI-RSRP measurement accuracy	3626

7.7.8	CSI-RSRQ	3630

7.7.8.0	Minimum conformance requirements	3630

7.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	3630

7.7.8.0.1.1	Absolute CSI-RSRQ accuracy	3630

7.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	3631

7.7.8.0.2.1	Absolute CSI-RSRQ accuracy	3631

7.7.8.0.2.2	Relative CSI-RSRQ accuracy	3632

7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3633

7.7.8.1.1	Test purpose	3633

7.7.8.1.2	Test applicability	3633

7.7.8.1.3	Minimum conformance requirements	3633

7.7.8.1.4	Test description	3633

7.7.8.1.5	Test Requirements	3635

7.7.8.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3637

7.7.8.2.1	Test purpose	3637

7.7.8.2.2	Test applicability	3638

7.7.8.2.3	Minimum conformance requirements	3638

7.7.8.2.4	Test description	3638

7.7.8.2.5	Test Requirements	3639

7.7.9	CSI-SINR	3642

7.7.9.0	Minimum conformance requirements	3642

7.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	3642

7.7.9.0.1.1	Absolute CSI-SINR accuracy	3642

7.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	3643

7.7.9.0.2.1	Absolute CSI-SINR accuracy	3643

7.7.9.0.2.2	Relative CSI-SINR accuracy	3644

7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3645

7.7.9.1.1	Test purpose	3645

7.7.9.1.2	  Test applicability	3645

7.7.9.1.3	Minimum conformance requirements	3645

7.7.9.1.4	Test description	3646

7.7.9.1.5	Test Requirements	3647

7.7.9.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3649

7.7.9.2.1	Test purpose	3650

7.7.9.2.2	Test applicability	3650

7.7.9.2.3	Minimum conformance requirements	3650

7.7.9.2.4	Test description	3650

7.7.9.2.5	Test Requirements	3651

7.7.10 to 7.7.13	3654

7.7.14	L1-RSRP measurement for group-based beam reporting	3654

7.7.14.1	NR SA FR2 SSB based L1-RSRP measurement for group-based beam reporting	3654

7.7.14.2	NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off	3656

7.7.15	LTM L1-RSRP measurement	3658

7.7.15.0	Minimum conformance requirements	3658

7.7.15.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	3658

7.7.15.0.2	SSB based relative L1-RSRP measurement accuracy requirements	3659

7.7.15.1	NR SA FR2 SSB based inter-frequency L1-RSRP measurement	3660

8.0	General	3668

8.1	Void	3668

8.2.1.0	Minimum conformance requirements	3668

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	3668

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	3670

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3677

8.2.2.0	Minimum conformance requirements	3684

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	3686

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	3694

8.3.1.0	Minimum conformance requirements	3699

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	3699

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	3700

8.4.1.0	Minimum conformance requirements	3705

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	3705

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	3706

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	3712

8.4.2.0	Minimum conformance requirements	3717

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	3717

8.4.2.0.2	Void	3721

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	3721

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	3727

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	3733

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	3739

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	3745

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	3750

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	3755

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	3760

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3765

8.4.3	Inter-RAT measurements without gaps	3771

8.4.3.0	Minimum conformance requirements	3771

8.4.3.0.1	Minimum conformance requirements for inter-RAT event-triggered reporting without gap	3771

8.4.3.1	E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption	3771

8.5.1	SFTD measurement accuracy	3774

8.5.1.0	Minimum conformance requirements	3774

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	3774

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	3775

8.5.2	Inter-RAT measurement accuracy	3781

8.5.2.1	SS-RSRP	3781

8.5.2.1.0	Minimum conformance requirements	3781

8.5.2.1.1	SS-RSRP with NR FR1 target cell	3782

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	3787

8.5.2.2	SS-RSRQ	3792

8.5.2.2.0	Minimum conformance requirements	3792

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	3792

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	3795

8.5.2.3	SS-SINR	3800

8.5.2.3.0	Minimum conformance requirements	3800

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	3800

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	3803

9	NR sidelink	3808

9.1	NR sidelink in FR1	3808

9.1.1	UE transmit timing	3808

9.1.1.0	Minimum conformance requirements	3808

9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source	3808

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3808

9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source	3808

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	3809

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	3811

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	3814

9.1.2	Initiation/Cease of S-SSB transmission	3819

9.1.2.0	Minimum conformance requirements	3819

9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source	3819

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3820

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	3821

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	3826

9.1.3	Synchronization reference selection/reselection	3830

9.1.3.0	Minimum conformance requirements	3830

9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source	3830

9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source	3831

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	3833

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	3839

9.1.4	L1 SL-RSRP measurements	3844

9.1.4.0	Minimum conformance requirements	3844

9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption	3844

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	3845

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	3851

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	3857

9.1.5	Congestion control measurement	3864

9.1.5.0	Minimum conformance requirements	3864

9.1.5.0.1	Minimum conformance requirements for congestion control measurements	3864

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	3865

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	3873

9.1.6	Congestion control measurement	3879

9.1.6.0	Minimum conformance requirements	3879

9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication	3879

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	3879

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3883

10.0	General	3883

10.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	3883

10.1	RRC_CONNECTED state mobility	3884

10.1.1	Random Access	3884

10.1.1.0	Minimum conformance requirements	3884

10.1.1.0.1	Minimum conformance requirements for 4-step RA type	3884

10.1.1.0.1.1	Contention based random access	3884

10.1.1.0.1.1.1	Correct behaviour when transmitting Random Access Preamble	3884

10.1.1.0.1.1.2	Correct behaviour when receiving Random Access Response	3885

10.1.1.0.1.1.3	Correct behaviour when not receiving Random Access Response	3885

10.1.1.0.1.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	3885

10.1.1.0.1.1.6	Correct behaviour when contention Resolution timer expires	3885

10.1.1.0.1.2	Non-Contention based random access	3885

10.1.1.0.1.2.1	Correct behaviour when transmitting Random Access Preamble	3885

10.1.1.0.1.2.2	Correct behaviour when receiving Random Access Response	3886

10.1.1.0.1.2.3	Correct behaviour when not receiving Random Access Response	3886

10.1.1.0.2	Minimum conformance requirements for 2-step RA type	3886

10.1.1.0.2.1	Contention based random access	3887

10.1.1.0.2.1.1	Correct behaviour when transmitting MsgA	3887

10.1.1.0.2.1.2	Correct behaviour when receiving MsgB	3887

10.1.1.0.2.1.3	Correct behaviour when not receiving MsgB	3888

10.1.1.0.2.2	Non-Contention based random access	3888

10.1.1.0.2.2.1	Correct behaviour when transmitting MsgA	3888

10.1.1.0.2.2.2	Correct behaviour when receiving MsgB	3888

10.1.1.0.2.2.3	Correct behaviour when not receiving MsgB	3888

10.1.1.1.1	EN-DC FR1 4-step RA type contention based random access test for NR PSCell with CCA	3889

10.1.1.1.2	EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA	3897

10.1.1.1.3	EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA	3905

10.1.1.1.4	EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA	3913

10.1.2	EN-DCFR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	3922

10.2	Timing	3931

10.2.1	UE transmit timing	3931

10.2.1.0	Minimum conformance requirements	3931

10.2.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	3931

10.2.1.1	EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA	3931

10.2.2	UE timing advance	3938

10.2.2.0	Minimum conformance requirements	3938

10.2.2.0.1	Minimum conformance requirements for timing advance adjustment delay	3938

10.2.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	3938

10.2.2.1	EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3938

10.3	Signalling characteristics	3944

10.3.1	Radio link monitoring	3944

10.3.1.0	General	3944

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	3945

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	3948

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3948

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3956

10.3.1.4	Void	3961

10.3.1.5	Void	3961

10.3.2	Void	3962

10.3.3	SCell activation and deactivation delay	3962

10.3.3.0	Minimum Conformance Requirements	3962

10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA	3962

10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA	3966

10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3966

10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3975

10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3979

10.3.4	Beam failure detection and link recovery procedures	3984

10.3.4.0	General	3984

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3985

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3987

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3987

10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode	3987

10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode	3995

10.3.5	Active BWP switching	4003

10.3.5.1	EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA	4003

10.3.5.2	DCI-based and Timer-based Active BWP Switch	4010

10.3.5.2.0	Minimum conformance requirements	4010

10.3.5.2.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	4013

10.3.5.2.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA	4021

10.3.5.3	RRC-based Active BWP Switch	4029

10.3.5.3.0	Minimum conformance requirements	4029

10.3.5.3.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	4030

10.3.6	PSCell addition and release delay	4036

10.3.6.0	Minimum conformance requirements	4036

10.3.6.0.1	NR PSCell Addition Delay Requirement	4036

10.3.6.0.2	NR PSCell Release Delay Requirement	4037

10.3.6.1	EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA	4037

10.4	Measurement procedures	4044

10.4.1	Intra-frequency measurements	4044

10.4.1.0	Minimum conformance requirements	4044

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	4058

10.4.1.2	Void	4063

10.4.1.3	Void	4063

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	4063

10.4.2	Inter-frequency measurements	4069

10.4.2.0	Minimum conformance requirements	4069

10.4.2.1	Void	4075

10.4.2.2	Void	4075

10.4.2.3	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used	4075

10.4.2.4	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used	4080

10.4.2.5	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used	4087

10.4.2.6	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used	4092

10.4.2.7	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used	4099

10.4.2.8	EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used	4106

10.4.2.9	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used	4113

10.4.2.10	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used	4120

10.4.3	L1-RSRP measurement for beam reporting under CCA	4127

10.4.3.0	Minimum conformance requirements	4127

10.4.3.1	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used	4129

10.4.3.2	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used	4134

10.4.3.3	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used	4138

10.4.3.4	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used	4143

10.4.4	E-UTRAN-NR inter-RAT measurements on NR carrier frequency under CCA	4147

10.4.4.0	Minimum conformance requirements	4147

10.4.4.1	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4150

10.4.4.2	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4158

10.4.4.3	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4166

10.4.4.4	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4174

10.5	Measurement Performance	4181

10.5.1	SS-RSRP	4181

10.5.1.0	Minimum Conformance Requirements	4181

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4181

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	4181

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4182

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	4183

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	4183

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	4189

10.5.2	SS-RSRQ	4195

10.5.2.0	Minimum Conformance Requirements	4195

10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4195

10.5.2.0.2	Void	4196

10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4196

10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	4197

10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	4197

10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	4203

10.5.3	SS-SINR	4208

10.5.3.0	Minimum Conformance Requirements	4208

10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	4208

10.5.3.0.2	Void	4209

10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	4209

10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	4210

10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA	4210

10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA	4216

10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA	4222

10.5.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	4228

10.5.4.0	Minimum conformance requirements	4228

10.5.4.1	EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA	4230

10.5.5	RSSI	4234

10.5.5.0	Minimum Conformance Requirements	4234

10.5.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	4235

10.5.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	4235

10.5.5.1	EN-DC FR1 RSSI measurement accuracy on PSCC with CCA	4235

10.5.5.2	EN-DC FR1 RSSI measurement accuracy on SCC with CCA	4240

10.5.5.3	 EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA	4244

10.5.6	Channel occupancy	4249

10.5.6.0	Minimum Conformance Requirements	4249

10.5.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	4249

10.5.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	4250

10.5.6.1	EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA	4250

10.5.6.2	EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA	4254

10.5.6.3	EN-DC FR1-FR1 Channel occupancy measurement accuracy on a carrier with CCA	4259

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4265

11.0	General	4265

11.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	4265

11.1	RRC_IDLE state mobility	4266

11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4266

11.1.1.0	Minimum conformance requirements	4266

11.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection with both source and target NR carrier frequencies under CCA	4266

11.1.1.0.2	Minimum conformance requirements for for inter-frequency cell re-selection with target NR carrier frequency under CCA	4267

11.1.1.1	NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4269

11.1.1.2	NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4275

11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4282

11.1.2.0	Minimum conformance requirements	4282

11.1.2.1	NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA	4283

11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4291

11.1.3.0	Minimum conformance requirements	4291

11.1.3.1	NR SA FR1 Cell re-selection from NR carrier with target NR carrier frequency under CCA	4292

11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4300

11.1.4.0	Minimum conformance requirements	4300

11.1.4.0.1	Minimum conformance requirements for Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4300

11.1.4.1	NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4301

11.1.4.2	NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4307

11.2	RRC_CONNECTED state mobility	4313

11.2.1	Handover	4313

11.2.1.0	Minimum conformance requirements	4313

11.2.1.0.1	Minimum conformance requirements for NR FR1 - NR FR1 Handover	4313

11.2.1.1	NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4314

11.2.1.2	NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4318

11.2.1.3	NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4323

11.2.1.4	NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4328

11.2.1.5	NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4334

11.2.1.6	NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4340

11.2.1.7	NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell	4345

11.2.1.8	NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4354

11.2.2	RRC connection mobility control	4360

11.2.2.1	RRC re-establishment	4360

11.2.2.1.0	Minimum conformance requirements	4360

11.2.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1	4362

11.2.2.1.2	NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1	4366

11.2.2.1.3	NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing	4371

11.2.2.1.4	NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4376

11.2.2.2	Random Access	4382

11.2.2.2.0	Minimum conformance requirements	4382

11.2.2.2.1	NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA	4387

11.2.2.2.2	NR SA FR1 4-step RA type non-contention based random access for NR PCell with CCA	4395

11.2.2.2.3	NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA	4403

11.2.2.2.4	NR SA FR1 2-step RA type non-contention based random access for NR PCell with CCA	4411

11.2.2.3	RRC connection release with redirection	4420

11.2.2.3.0	Minimum conformance requirements	4420

11.2.2.3.1	NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4421

11.2.2.3.2	NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4427

11.3	Timing	4432

11.3.1	UE transmit timing	4432

11.3.1.0	Minimum conformance requirements	4432

11.3.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	4432

11.3.1.1	NR SA FR1 UE Transmit Timing Test with PCell under DL CCA	4433

11.3.2	UE timing advance	4440

11.3.2.0	Minimum conformance requirements	4440

11.3.2.0.1	Minimum conformance requirements for timing advance adjustment delay	4440

11.3.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	4440

11.3.2.1	NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4440

11.4	Signalling characteristics	4445

11.4.1	Radio link monitoring	4445

11.4.1.0	General	4445

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	4445

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	4446

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	4446

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	4453

11.4.2	Void	4460

11.4.3	SCell activation and deactivation delay	4460

11.4.3.0	Minimum conformance requirements	4460

11.4.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	4460

11.4.3.1	NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4460

11.4.3.2	NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4469

11.4.3.3	NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4473

11.4.4	Beam failure detection and link recovery procedures	4479

11.4.4.0	General	4479

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	4479

11.4.4.0.2	Void	4479

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	4480

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	4480

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	4480

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	4488

11.4.5	Active BWP switching	4496

11.4.5.1	NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4496

11.4.5.2	DCI-based and Timer-based Active BWP Switch	4503

11.4.5.2.0	Minimum conformance requirements	4503

11.4.5.2.1	NR SA FR1 DCI-based DL active BWP switch of PCell with non-DRX under CCA	4505

11.4.5.2.2	NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA	4513

11.4.5.3	RRC-based Active BWP Switch	4521

11.4.5.3.0	Minimum conformance requirements	4521

11.4.5.3.1	NR SA FR1 RRC-based DL active BWP switch of Cell with non-DRX under CCA	4521

11.5	Measurement procedure	4527

11.5.1	Intra-frequency measurements	4527

11.5.1.0	Minimum Conformance Requirements	4527

11.5.1.1	SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX	4528

11.5.1.2	SA FR1 Event-triggered reporting tests on PCC without gaps under DRX	4533

11.5.2	Inter-frequency measurements	4538

11.5.2.0	Minimum Conformance Requirements	4538

11.5.2.3	NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used	4538

11.5.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4544

11.5.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4551

11.5.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4557

11.5.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4563

11.5.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4569

11.5.2.9	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4574

11.5.2.10	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4580

11.5.3	Inter-RAT E-UTRAN measurements	4586

11.5.3.0	Minimum conformance requirements	4586

11.5.3.1	NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX	4587

11.5.3.2	NR SA FR1 - E-UTRAN event-triggered reporting in DRX	4592

11.5.4	L1-RSRP measurements for beam reporting	4596

11.5.4.0	Minimum conformance requirements	4596

11.5.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used	4596

11.5.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is used	4602

11.5.4.3	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used	4605

11.5.4.4	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is used	4609

11.6	Measurement Performance	4613

11.6.1	SS-RSRP	4613

11.6.1.0	Minimum Conformance Requirements	4613

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4613

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	4613

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4613

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	4613

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	4613

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4619

11.6.2	SS-RSRQ	4624

11.6.2.0	Minimum Conformance Requirements	4624

11.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4625

11.6.2.0.2	Void	4625

11.6.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4625

11.6.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	4625

11.6.2.1	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	4625

11.6.2.2	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	4630

11.6.2.3	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA	4635

11.6.2.4	NR SA FR1 Inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA	4640

11.6.3	SS-SINR	4648

11.6.3.0	Minimum Conformance Requirements	4648

11.6.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	4648

11.6.3.0.2	Void	4648

11.6.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	4648

11.6.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	4648

11.6.3.1	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	4648

11.6.3.2	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	4653

11.6.3.3	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA	4658

11.6.3.4	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA from carrier without CCA	4664

11.6.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	4674

11.6.4.0	Minimum conformance requirements	4674

11.6.4.0.1	Minimum conformance requitements for SSB-based L1-RSRP measurement  accuracy for beam reporting under CCA	4674

11.6.4.1	NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA	4674

11.6.5	RSSI	4678

11.6.5.0	Minimum Conformance Requirements	4678

11.6.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	4678

11.6.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	4678

11.6.5.1	NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA	4679

11.6.5.2	NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA	4683

11.6.5.3	NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA	4687

11.6.6	Channel occupancy	4692

11.6.6.0	Minimum Conformance Requirements	4692

11.6.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	4692

11.6.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	4692

11.6.6.1	NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4692

11.6.6.2	NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4698

11.6.6.3	NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4703

12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4712

12.1	RRC_IDLE State Mobility	4712

12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4712

12.1.1.0	Minimum Conformance Requirements	4712

12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4713

12.2	RRC_CONNECTED State Mobility	4721

12.2.1	Handover	4721

12.2.1.0	Minimum Conformance Requirements	4721

12.2.1.1	E-UTRAN - NR with CCA handover with known target cell	4722

12.3	Void	4729

12.4	Measurement Procedure	4729

12.4.1	E-UTRAN-NR inter-RAT SFTD measurements	4729

12.4.1.0	Minimum Conformance Requirements	4729

12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4730

12.4.2	E-UTRAN-NR inter-RAT measurements on NR carrier frequency under CCA	4737

12.4.2.0	Minimum Conformance Requirements	4737

12.4.2.0.1	E-UTRAN – NR measurements	4737

12.4.2.1	E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is not used under CCA	4740

12.4.2.2	E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is used under CCA	4746

12.4.2.3	E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is not used under CCA	4751

12.4.2.4	E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is used under CCA	4757

12.5	Measurement Performance	4762

12.5.1	E-UTRAN-NR SFTD	4762

12.5.1.0	Minimum conformance requirements	4762

12.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy under CCA	4763

13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4768

13.1	Void	4768

13.2	Signalling Characteristics	4768

13.2.2	SCell activation and deactivation delay	4768

13.2.2.0	Minimum conformance requirements	4768

13.2.2.0.1	Minimum conformance requirements for SCell activation and deactivation delay	4768

13.2.2.1	NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4768

13.2.2.2	NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4778

13.2.2.3	NR SA FR1 SCell Activation and Deactivation of unknown SCell under CCA	4782

13.3	Measurement Procedure	4787

13.3.1	Intra-frequency measurements	4787

13.3.1.0	Minimum Conformance Requirements	4787

13.3.1.1	NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under non-DRX	4788

13.3.1.2	NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under DRX	4795

13.3.1.3	NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under non-DRX	4802

13.3.1.4	NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under DRX	4810

13.3.2	Inter-frequency measurements	4818

13.3.2.0	Minimum Conformance Requirements	4818

13.3.2.1	Void	4818

13.3.2.2	Void	4818

13.3.2.3	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is not used	4818

13.3.2.4	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is used	4825

13.3.2.5	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is not used	4833

13.3.2.6	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is used	4840

13.3.3	L1-RSRP measurement for beam reporting for SCell under CCA	4847

13.3.3.0	Minimum Conformance Requirements	4847

13.3.3.1	NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is not used	4847

13.3.3.2	NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is used	4854

13.4	Measurement Peformance	4860

13.4.1	SS-RSRP	4860

13.4.1.0	Minimum Conformance Requirements	4860

13.4.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4860

13.4.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	4860

13.4.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4860

13.4.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	4860

13.4.1.1	NR SA FR1 SS-RSRP measurement accuracy on a carrier with CCA	4860

13.4.2	SS-RSRQ	4864

13.4.2.0	Minimum Conformance Requirements	4864

13.4.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4865

13.4.2.0.2	Void	4865

13.4.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4865

13.4.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	4865

13.4.2.1	NR SA FR1 SS-RSRQ measurement accuracy on a carrier with CCA	4865

13.4.3	SS-SINR	4874

13.4.3.0	Minimum Conformance Requirements	4874

13.4.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	4874

13.4.3.0.2	Intra-frequency relative SS-SINR measurement accuracy requirements under CCA	4874

13.4.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	4874

13.4.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	4874

13.4.3.1	NR SA FR1 SS-SINR measurement accuracy on a carrier with CCA	4874

13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4884

13.4.4.0	Minimum Conformance Requirements	4884

13.4.4.0.1	SSB-based L1-RSRP measurement absolute accuracy under CCA	4884

13.4.4.0.2	SSB-based L1-RSRP measurement relative accuracy under CCA	4884

13.4.4.1	NR SA FR1 L1-RSRP measurement accuracy for beam reporting on a carrier with CCA	4884

13.4.5	RSSI	4891

13.4.5.0	Minimum Conformance Requirements	4891

13.4.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	4891

13.4.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	4891

13.4.5.0.3	RSSI measurement report mapping	4891

13.4.5.1	NR SA FR1 RSSI measurement accuracy on a carrier with CCA	4891

13.4.5.2	NR SA FR1-FR1 RSSI measurement accuracy on a carrier with CCA	4896

13.4.6	Channel Occupancy	4901

13.4.6.0	Minimum Conformance Requirements	4901

13.4.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	4901

13.4.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	4901

13.4.6.1	NR SA FR1 channel occupancy measurement accuracy on a carrier with CCA	4901

13.4.6.2	NR SA FR1-FR1 channel occupancy measurement accuracy on a carrier with CCA	4907

14	NR standalone tests for Satellite access	4912

14.0	General	4912

14.0.1	Principle of testing GSO and NGSO scenarios	4912

14.0.2	Principle of testing different RRM requirements	4912

14.0.3	Principle of testing different ephemeris formats	4913

14.0.4	General setup for SIB19	4915

14.0.5	Initial test environment conditions for RRM NTN tests	4915

14.1	RRC_IDLE state mobility	4916

14.1.0	Minimum conformance requirements	4916

14.1.0.1	Measurements of intra-frequency NR cells	4916

14.1.0.2	Measurements of inter-frequency NR cells	4918

14.1.1	NR SA FR1 Cell Reselection for Satellite Access	4921

14.1.2	NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	4926

14.1.3	NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	4931

14.1.4	NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	4935

14.1.5	NR SA FR1-FR1 Cell Reselection for Satellite Access	4939

14.1.6	NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	4944

14.1.7	NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	4948

14.1.8	NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	4953

14.1.9	NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access	4957

14.1.10	NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access	4963

14.2	RRC_CONNECTED state mobility	4969

14.2.1	Handover for SAN	4969

14.2.1.0	Minimum conformance requirements	4969

14.2.1.0.1	Minimum conformance requirements for handover	4969

14.2.1.0.2	Minimum conformance requirements for conditional handover	4970

14.2.1.1	NR SA FR1 Handover for Satellite Access	4972

14.2.1.2	NR SA FR1-FR1 Handover for Satellite Access	4976

14.2.1.3	NR SA FR1 Time-based Conditional Handover for Satellite Access	4979

14.2.1.4	NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access	4987

14.2.1.5	NR SA FR1 Distance-based Conditional Handover for Satellite Access	4994

14.2.1.6	NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access	4998

14.2.2	RRC Connection Mobility Control for SAN	5003

14.2.2.1	RRC Re-establishment for SAN	5003

14.2.2.1.0	Minimum conformance requirements for RRC Re-establishment for Satellite Access	5003

14.2.2.1.1	NR SA FR1 RRC Re-establishment for Satellite Access	5005

14.2.2.1.2	NR SA FR1-FR1 RRC Re-establishment for Satellite Access	5009

14.2.2.4	NR SA FR1 RACH-based Hard Satellite switching with re-synchronization for Satellite Access	5014

14.2.2.5	NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access	5018

14.3	Timing for Satellite Access	5023

14.3.1	UE transmit timing for Satellite Access	5023

14.3.1.0	Minimum conformance requirements	5023

14.3.1.1	NR SA FR1 UE transmit timing accuracy for Satellite Access	5024

14.3.2	Timing Advance for Satellite Access	5031

14.3.2.0	Minimum conformance requirements	5031

14.3.2.1	NR SA FR1 Timing Advance adjustment accuracy for Satellite Access	5031

14.4	Signalling characteristics	5036

14.4.1	Radio link monitoring for Satellite Access	5036

14.4.1.0	General	5036

14.4.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring for Satellite Access	5037

14.4.1.0.2	Minimum conformance requirements for CSI-RS based Radio Link Monitoring for Satellite Access	5039

14.4.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access	5042

14.4.1.2	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access	5047

14.4.1.3	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access	5054

14.4.1.4	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access	5060

14.4.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access	5067

14.4.1.6	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access	5073

14.4.1.7	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access	5079

14.4.1.8	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access	5084

14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	5090

14.4.2.00	General	5090

14.4.2.0	Minimum conformance requirements	5091

14.4.2.0.1	Minimum conformance requitements for SSB based beam failure detection	5091

14.4.2.0.2	Measurement restriction for SSB based beam failure detection	5092

14.4.2.0.3	Minimum conformance requitements for CSI-RS based beam failure detection	5093

14.4.2.0.4	Measurement restriction for CSI-RS based beam failure detection	5094

14.4.2.0.5	Minimum requirement for L1 indication	5095

14.4.2.0.6	Minimum conformance requitements for SSB based candidate beam detection	5095

14.4.2.0.7	Measurement restriction for SSB based candidate beam detection	5096

14.4.2.0.8	Minimum conformance requitements for CSI-RS based candidate beam detection	5097

14.4.2.0.9	Measurement restriction for CSI-RS based candidate beam detection	5098

14.4.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access	5098

14.4.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access	5107

14.4.2.3	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access	5116

14.4.2.4	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access	5124

14.4.3	Active BWP Switch for SAN	5133

14.4.3.1	DCI-based and Timer-based Active BWP Switch	5133

14.4.3.1.0	Minimum conformance requirements	5133

14.4.3.1.1	NR SA FR1 DCI-based DL active BWP switch with non-DRX for Satellite Access	5134

14.4.3.2	RRC-based Active BWP Switch	5143

14.4.3.2.0	Minimum conformance requirements	5143

14.4.3.2.1	NR SA FR1 RRC-based DL active BWP switch with non-DRX for Satellite Access	5143

14.4.4	UE specific CBW change for SAN	5149

14.4.4.0	Minimum conformance requirements	5149

14.4.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for Satellite Access	5150

14.5	Measurement procedure	5154

14.5.1	Intra-frequency Measurements for SAN	5154

14.5.1.0	Minimum conformance requirements	5154

14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps	5155

14.5.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps	5158

14.5.1.1	NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access	5160

14.5.1.2	NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access	5163

14.5.1.3	NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access	5167

14.5.1.4	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	5171

14.5.1.5	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access	5175

14.5.1.6	NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	5181

14.5.2	Inter-frequency Measurements for SAN	5185

14.5.2.0	Minimum conformance requirements	5185

14.5.2.1	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	5189

14.5.2.2	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access	5195

14.5.2.3	NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access	5201

14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access	5207

14.5.2.5	Void	5213

14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	5213

14.5.2.7	NR SA FR1-FR1 Event triggered reporting without gap under non-DRX for Satellite Access	5219

14.5.2.8	NR SA FR1-FR1 Event triggered reporting without gap under DRX for Satellite Access	5222

14.5.3	L1-RSRP measurement for beam reporting for SAN	5227

14.5.3.0	Minimum conformance requirements	5227

14.5.3.0.1	Minimum conformance requirements for SSB based L1-RSRP measurement for beam reporting	5227

14.5.3.0.2	Minimum conformance requirements for CSI-RS based L1-RSRP measurement for beam reporting	5228

14.5.3.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for Satellite Access	5230

14.5.3.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for Satellite Access	5234

14.5.3.3	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for Satellite Access	5237

14.5.3.4	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for Satellite Access	5241

14.6	Measurement Performance Requirements	5244

14.6.1	SS-RSRP for SAN	5244

14.6.1.0	Minimum conformance requirements	5244

14.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements for FR1 SAN	5244

14.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements for FR1 SAN	5245

14.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements for FR1 SAN	5245

14.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements for FR1 SAN	5246

14.6.1.1	NR SA FR1 SS-RSRP measurement accuracy for Satellite Access	5247

14.6.1.2	NR SA FR1-FR1 SS-RSRP measurement accuracy for Satellite Access	5251

14.6.2	SS-RSRQ for SAN	5256

14.6.2.0	Minimum conformance requirements	5256

14.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements for FR1 SAN	5256

14.6.2.0.2	Inter-frequency absolute SS-RSRQ measurement accuracy requirements for FR1 SAN	5257

14.6.2.0.3	Inter-frequency relative SS-RSRQ measurement accuracy requirements for FR1 SAN	5257

14.6.2.1	NR SA FR1 SS-RSRQ measurement accuracy for Satellite Access	5258

14.6.2.2	NR SA FR1-FR1 SS-RSRQ measurement accuracy for Satellite Access	5263

14.6.3	SS-SINR Requirements for SAN	5267

14.6.3.0	Minimum conformance requirements	5267

14.6.3.0.1	Intra-Frequency SS-SINR accuracy requirements in FR1	5267

14.6.3.0.2	Inter-Frequency SS-SINR accuracy requirements in FR1	5268

14.6.3.1	NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access	5269

14.6.3.2	NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access	5274

14.6.4	L1-RSRP Beam Reporting Measurement Requirements for SAN	5279

14.6.4.0	Minimum conformance requirements	5279

14.6.4.0.1	Minimum conformance requirements for SSB based L1-RSRP Reporting	5279

14.6.4.0.2	Minimum conformance requirements for CSI-RS based L1-RSRP Reporting	5281

14.6.4.1	NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access	5282

14.6.4.2	NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access	5287

15	FFS	5292

16	NR standalone tests in FR1 for RedCap	5294

16.0	General	5294

16.1	RRC_IDLE state mobility	5294

16.1.1	NR cell re-selection for RedCap	5294

16.1.1.0	Minimum conformance requirements	5294

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	5294

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	5296

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	5300

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	5303

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	5306

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	5314

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	5319

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	5324

16.1.1.5	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5328

16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5336

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5342

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5350

16.1.2	NR – E-UTRA cell re-selection for RedCap	5358

16.1.2.0	Minimum conformance requirements	5358

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	5358

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	5360

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection	5360

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1 Rx UE	5362

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 Rx UE	5368

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1 Rx UE	5372

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2 Rx UE	5378

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5382

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5388

16.2	RRC_INACTIVE state mobility for RedCap	5392

16.2.1	Configured Grant based Small Data Transmissions for RedCap	5392

16.2.1.0	Minimum conformance requirements	5392

16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	5392

16.2.1.1	NR SA FR1 CG-SDT for 1 Rx UE	5394

16.2.1.2	NR SA FR1 CG-SDT for 2 Rx UE	5398

16.3	RRC_CONNECTED state mobility for RedCap	5401

16.3.1	Handover for RedCap	5401

16.3.1.0	Minimum conformance requirements	5401

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	5401

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	5401

16.3.1.1	NR SA FR1 handover with known target cell for 1 Rx UE	5402

16.3.1.2	NR SA FR1 handover with known target cell for 2 Rx UE	5405

16.3.1.3	NR SA FR1 handover with unknown target cell for 1 Rx UE	5409

16.3.1.4	NR SA FR1 handover with unknown target cell for 2 Rx UE	5415

16.3.1.5	NR SA FR1-FR1 handover with unknown target cell for 1 Rx UE	5420

16.3.1.6	NR SA FR1-FR1 handover with unknown target cell for 2 Rx UE	5426

16.3.1.7	NR SA FR1 - E-UTRA handover with known target cell for 1 Rx UE	5433

16.3.1.8	NR SA FR1 - E-UTRA handover with known target cell for 2 Rx UE	5438

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	5443

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	5447

16.3.2	RRC connection mobility control for RedCap	5451

16.3.2.1	RRC re-establishment for RedCap	5451

16.3.2.1.0	Minimum conformance requirements	5451

16.3.2.1.1	NR SA FR1 RRC Re-establishment for 1 Rx UE	5451

16.3.2.1.2	NR SA FR1 RRC Re-establishment for 2 Rx UE	5454

16.3.2.1.3	NR SA FR1-FR1 RRC Re-establishment for 1 Rx UE	5457

16.3.2.1.4	NR SA FR1-FR1 RRC Re-establishment for 2 Rx UE	5460

16.3.2.1.5	NR SA FR1 RRC Re-establishment for 1 Rx UE without serving cell timing	5463

16.3.2.1.6	NR SA FR1 RRC Re-establishment for 2 Rx UE without serving cell timing	5466

16.3.2.2	Random access for RedCap	5469

16.3.2.2.0	Minimum conformance requirements	5469

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5470

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5473

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5476

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5479

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5482

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5485

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5488

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5491

16.3.2.3	RRC connection release with redirection for RedCap	5494

16.3.2.3.0	Minimum conformance requirements	5494

16.3.2.3.1	NR SA FR1-FR1 RRC connection release with redirection for 1 Rx UE	5494

16.3.2.3.2	NR SA FR1-FR1 RRC connection release with redirection for 2 Rx UE	5498

16.3.2.3.3	NR SA FR1 - E-UTRA RRC connection release with redirection for 1 Rx UE	5501

16.3.2.3.4	NR SA FR1 - E-UTRA RRC connection release with redirection for 2 Rx UE	5506

16.4	Timing for RedCap	5511

16.4.1	UE transmit timing for RedCap	5511

16.4.1.0	Minimum conformance requirements	5511

16.4.1.1	NR SA FR1 NR UE Transmit Timing accuracy for 1 Rx UE	5512

16.4.1.2	NR SA FR1 NR UE Transmit Timing accuracy for 2 Rx UE	5515

16.4.2	5518

16.4.3	Timing advance for RedCap	5518

16.4.3.0	Minimum conformance requirement	5518

16.4.3.1	NR SA FR1 timing advance adjustment accuracy for 1 Rx UE	5518

16.4.3.2	NR SA FR1 timing advance adjustment accuracy for 2 Rx UE	5521

16.5	Signalling characteristics for RedCap	5524

16.5.1	Radio link monitoring for RedCap	5524

16.5.1.0	Minimum requirements	5524

16.5.1.0.1	General	5524

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	5528

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	5529

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5530

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5535

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5538

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5543

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5547

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5552

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5557

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5562

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5566

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5569

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5572

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5575

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5579

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5582

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5585

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5588

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	5591

16.5.2.0	Minimum requirements	5591

16.5.2.0.1	General	5591

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	5593

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	5595

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5598

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5602

16.5.2.3	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5607

16.5.2.4	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5613

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5618

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5623

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5626

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5630

16.5.3	Active BWP switch delay for RedCap	5635

16.5.3.1	DCI-based and timer-based active BWP switch for RedCap	5635

16.5.3.1.0	Minimum conformance requirements	5635

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	5636

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	5641

16.5.3.2	RRC-based active BWP switch for RedCap	5646

16.5.3.2.0	Minimum conformance requirements	5646

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	5647

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	5652

16.5.4	UE specific CBW change for RedCap	5657

16.5.4.0	Minimum conformance requirements	5657

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	5657

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	5658

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	5660

16.6	Measurement procedures for RedCap	5662

16.6.1	Intra-frequency measurements for RedCap	5663

16.6.1.0	Minimum conformance requirements	5663

16.6.1.0.1	General	5663

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	5664

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	5666

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	5667

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	5673

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	5679

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	5681

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5684

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5687

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5690

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5693

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5696

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5702

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5707

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5710

16.6.2	Inter-frequency measurements for RedCap	5713

16.6.2.0	Minimum conformance requirements	5713

16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps	5713

16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps	5715

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5718

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5723

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5729

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5734

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5738

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5742

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5746

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5750

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5755

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5761

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5766

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5771

16.6.3	Inter-RAT measurements for RedCap	5776

16.6.3.0	Minimum conformance requirements	5776

16.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	5776

16.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	5779

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX for 1 Rx UE	5782

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX for 2 Rx UE	5788

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX for 1 Rx UE	5793

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX for 2 Rx UE	5799

16.6.4	L1-RSRP measurement for beam reporting for RedCap	5805

16.6.4.0	Minimum conformance requirements	5805

16.6.4.0.1	General	5805

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5810

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5813

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5816

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5820

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5823

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5826

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5829

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5832

16.6.5	NR measurements with autonomous gaps for RedCap	5835

16.6.5.0	Minimum conformance requirements	5835

16.6.5.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	5835

16.6.5.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	5836

16.6.5.0.3	Minimum conformance requirements for CGI identification of an E-UTRA cell with autonomous gaps	5837

16.6.5.1	NR SA FR1 CGI identification of NR neighbour cell for 1 Rx UE	5838

16.6.5.2	NR SA FR1 CGI identification of NR neighbour cell for 2 Rx UE	5841

16.6.5.3	NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 1 Rx UE	5844

16.6.5.4	NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 2 Rx UE	5852

16.7	Measurement performance requirements for RedCap	5857

16.7.1	SS-RSRP for RedCap	5857

16.7.1.0	Minimum conformance requirements	5857

16.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	5857

16.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	5858

16.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	5859

16.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	5860

16.7.1.1	Intra-frequency measurements for RedCap for 1 Rx UE	5861

16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	5861

16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	5867

16.7.1.2	Intra-frequency measurements for RedCap for 2 Rx UE	5869

16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	5869

16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	5876

16.7.1.3	Inter-frequency measurements for RedCap for 1 Rx UE	5878

16.7.1.3.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	5878

16.7.1.3.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	5885

16.7.1.4	Inter-frequency measurements for RedCap for 2 Rx UE	5887

16.7.1.4.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	5887

16.7.1.4.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	5893

16.7.2	SS-RSRQ for RedCap	5895

16.7.2.0	Minimum conformance requirements	5895

16.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	5895

16.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	5896

16.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	5896

16.7.2.1	NR SA FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE	5897

16.7.2.2	NR SA FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE	5901

16.7.2.3	Inter-frequency measurement accuracy for 1 Rx UE	5905

16.7.2.3.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE	5905

16.7.2.3.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 1 Rx UE	5909

16.7.2.4	Inter-frequency measurement accuracy for 2 Rx UE	5911

16.7.2.4.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE	5911

16.7.2.4.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 2 Rx UE	5915

16.7.3	SS-SINR for RedCap	5917

16.7.3.0	Minimum conformance requirements	5917

16.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	5917

16.7.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	5918

16.7.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	5918

16.7.3.1	NR SA FR1 SS-SINR absolute measurement accuracy for 1 Rx UE	5919

16.7.3.2	NR SA FR1 SS-SINR absolute measurement accuracy for 2 Rx UE	5923

16.7.3.3	Inter-frequency measurement accuracy for 1 Rx UE	5927

16.7.3.3.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 1 Rx UE	5927

16.7.3.3.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for 1 Rx UE	5931

16.7.3.4	Inter-frequency measurement accuracy for 2 Rx UE	5933

16.7.3.4.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 2 Rx UE	5933

16.7.3.4.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for 2 Rx UE	5937

16.7.4	L1-RSRP measurement for beam reporting for RedCap	5938

16.7.4.0	Minimum conformance requirements	5938

16.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	5939

16.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	5939

16.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	5940

16.7.4.1	SSB based L1-RSRP measurements for 1 Rx UE	5942

16.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 1 Rx UE	5942

16.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 1 Rx UE	5947

16.7.4.2	SSB based L1-RSRP measurements for 2 Rx UE	5949

16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 2 Rx UE	5949

16.7.4.2.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 2 Rx UE	5953

16.7.4.3	CSI-RS based L1-RSRP measurements for 1 Rx UE	5955

16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 1 Rx UE	5955

16.7.4.3.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 1 Rx UE	5959

16.7.4.4	CSI-RS based L1-RSRP measurements for 2 Rx UE	5961

16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 2 Rx UE	5961

16.7.4.4.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 2 Rx UE	5965

16.7.5	E-UTRAN RSRP for RedCap	5967

16.7.5.0	Minimum conformance requirements	5967

16.7.5.0.1	E-UTRAN RSRP absolute accuracy	5967

16.7.5.1	NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 1 Rx UE	5968

16.7.5.2	NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 2 Rx UE	5972

16.7.6	E-UTRAN RSRQ for RedCap	5976

16.7.6.0	Minimum conformance requirements	5976

16.7.6.0.1	E-UTRAN RSRQ absolute accuracy	5976

16.7.6.1	NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 1 Rx UE	5977

16.7.6.2	NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 2 Rx UE	5982

17	NR standalone tests in FR2 for RedCap	5987

17.1	RRC_IDLE state mobility for RedCap	5987

17.1.1	NR cell re-selection for RedCap	5987

17.1.1.0	Minimum conformance requirements	5987

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	5987

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	5989

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	5991

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	5993

17.1.1.1	NR SA FR2 Cell reselection for 2 Rx UE	5995

17.1.1.2	NR SA FR2-FR2 Cell reselection for 2 Rx UE	6001

17.1.1.3	NR SA FR2 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	6007

17.1.1.4	NR SA FR2-FR2 Cell reselection for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	6012

17.2	RRC_INACTIVE state mobility for RedCap	6019

17.2.1	Configured Grant based Small Data Transmissions for RedCap	6019

17.2.1.0	Minimum conformance requirements	6019

17.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	6019

17.2.1.1	NR SA FR2 TA validation for CG-SDT for 2 Rx UE	6020

17.3	RRC_CONNECTED state mobility for RedCap	6023

17.3.1	Handover for RedCap	6023

17.3.1.0	Minimum conformance requirements	6023

17.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	6023

17.3.1.1	NR SA FR2 handover with unknown target cell for 2 Rx UE	6025

17.3.1.2	NR SA FR2-FR2 handover with unknown target cell for 2 Rx UE	6027

17.3.2	RRC connection mobility control for RedCap	6034

17.3.2.1	RRC re-establishment for RedCap	6034

17.3.2.1.0	Minimum conformance requirements	6034

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	6034

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	6036

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	6038

17.3.2.2	Random access for RedCap	6041

17.3.2.2.0	Minimum conformance requirements	6041

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	6041

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	6041

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access for 2 Rx UE	6042

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access for 2 Rx UE	6045

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access for 2 Rx UE	6048

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access for 2 Rx UE	6051

17.3.2.3	RRC Connection Release with Redirection for RedCap	6054

17.3.2.3.0	Minimum conformance requirements	6054

17.3.2.3.0.1	Minimum conformance requirements for FR2-FR2 RRC connection release with redirection	6054

17.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection for 2 Rx UE	6054

17.4	Timing for RedCap	6056

17.4.1	UE transmit timing for RedCap	6056

17.4.1.0	Minimum Conformance Requirements	6057

17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	6057

17.4.1.1	NR SA FR2 NR UE Transmit Timing accuracy for 2 Rx UE	6058

17.4.2	UE timer accuracy for RedCap	6061

17.4.3	Timing advance for RedCap	6061

17.4.3.0	Minimum conformance requirements	6061

17.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	6061

17.4.3.1	NR SA FR2 timing advance adjustment accuracy for 2 Rx UE	6061

17.5	Signalling characteristics for RedCap	6066

17.5.1	Radio link monitoring for RedCap	6066

17.5.1.0	Minimum requirements	6066

17.5.1.0.1	General	6066

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	6069

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	6071

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	6073

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	6076

17.5.1.3	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	6081

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	6084

17.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	6087

17.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	6090

17.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	6095

17.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	6098

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions for 2 Rx UE	6102

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	6105

17.5.2.0	Minimum conformance requirements	6105

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	6105

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	6105

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	6106

17.5.2.1	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	6106

17.5.2.2	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	6111

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	6116

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	6120

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	6124

17.5.3	Active BWP switch delay for RedCap	6128

17.5.3.1	DCI-based and time-based active BWP switch for RedCap	6128

17.5.3.1.0	Minimum conformance requirements for DCI-based and time-based active BWP switch	6128

17.5.3.1.1	NR SA FR2 DCI-based and Timer-based DL active BWP switch for 2 Rx UE	6129

17.5.3.2	RRC-based active BWP switch for RedCap	6136

17.5.3.2.0	Minimum conformance requirements for RRC-based active BWP switch	6136

17.5.3.2.1	NR SA FR2 RRC-based DL active BWP switch for 2 Rx UE	6136

17.5.4	Active TCI state switch delay for RedCap	6142

17.5.4.0	Minimum conformance requirements	6142

17.5.4.0.1	Known conditions for TCI state	6142

17.5.4.0.2	Minimum conformance requirements for MAC-CE based active TCI state switch	6142

17.5.4.0.3	Minimum conformance requirements for RRC based active TCI state switch	6142

17.5.4.1	MAC-CE based active TCI state switch for RedCap	6142

17.5.4.2	RRC based active TCI state switch for RedCap	6145

17.5.4.2.1	NR SA FR2 RRC-CE based active TCI state switch for a known TCI state for 2 Rx UE	6145

17.5.5	Uplink spatial relation switch delay for RedCap	6148

17.5.5.1	MAC-CE based Spatial Relation switch for RedCap	6148

17.5.5.1.0	Minimum conformance requirements MAC-CE based uplink spatial relation switch delay	6148

17.5.5.1.1	NR SA FR2 MAC-CE based spatial relation switch associated with known DL-RS for 2 Rx UE	6149

17.5.5.2	RRC based spatial relation switch for RedCap	6152

17.5.5.2.0	Minimum conformance requirements RRC based uplink spatial relation switch delay	6152

17.5.5.2.1	NR SA FR2 RRC-based spatial relation switch associated with a known DL-RS for 2 Rx UE	6153

17.5.6	UE specific CBW change for RedCap	6156

17.5.6.0	Minimum conformance requirements	6156

17.5.6.0.1	Minimum conformance requirements for UE specific CBW change	6156

17.5.6.1	NR SA FR2 UE specific CBW change on PCell for 2 Rx UE	6156

17.6	Measurement procedures for RedCap	6162

17.6.1	Intra-frequency measurements for RedCap	6162

17.6.1.0	Minimum conformance requirements	6162

17.6.1.0.1	General	6162

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	6163

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	6165

17.6.1.1	NR SA FR2 event triggered reporting without gap in non-DRX for 2 Rx UE	6166

17.6.1.2	NR SA FR2 event triggered reporting without gap in DRX for 2 Rx UE	6170

17.6.1.3	NR SA FR2 event triggered reporting with gap in non-DRX for 2 Rx UE	6171

17.6.1.4	NR SA FR2 event triggered reporting with gap in DRX for 2 Rx UE	6178

17.6.2	Inter-frequency measurements for RedCap	6181

17.6.2.0	Minimum conformance requirements	6181

17.6.2.1	NR SA FR2-FR2 event triggered reporting in non-DRX for 2 Rx UE	6183

17.6.2.2	NR SA FR2-FR2 event triggered reporting in DRX for 2 Rx UE	6187

17.6.2.3	NR SA FR2-FR2 event triggered reporting with SSB time index detection in non-DRX for 2 Rx UE	6192

17.6.2.4	NR SA FR2-FR2 event triggered reporting with SSB time index detection in DRX for 2 Rx UE	6197

17.6.3	L1-RSRP measurement for beam reporting for RedCap	6202

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	6202

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	6202

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	6202

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement in non-DRX for 2 Rx UE	6202

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement in DRX for 2 Rx UE	6203

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement in non-DRX for 2 Rx UE	6206

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement in DRX for 2 Rx UE	6208

17.6.4	NR Measurements with autonomous gaps for RedCap	6211

17.6.4.0	Minimum conformance requirements	6211

17.6.4.0.1	Minimum conformance requirements for CGI identification of an NR cell with autonomous gaps	6211

17.6.4.1	NR SA FR2-FR2 CGI identification of NR neighbour cell for 2 Rx UE	6212

17.7	Measurement performance requirements for RedCap	6218

17.7.1	SS-RSRP for RedCap	6218

17.7.1.0	Minimum conformance requirements	6218

17.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	6218

17.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	6218

17.7.1.1	NR SA FR2 SS-RSRP measurement accuracy for 2 Rx UE	6218

17.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy for 2 Rx UE	6223

17.7.2	SS-RSRQ for RedCap	6230

17.7.2.0	Minimum conformance requirements	6230

17.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	6230

17.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	6230

17.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy for 2 Rx UE	6230

17.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy for 2 Rx UE	6233

17.7.3	L1-RSRP measurement for beam reporting for RedCap	6236

17.7.3.0	Minimum conformance requirements	6236

17.7.3.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	6236

17.7.3.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	6236

17.7.3.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	6236

17.7.3.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	6237

17.7.3.1	NR SA FR2 SSB based L1-RSRP measurement accuracy for 2 Rx UE	6237

17.7.3.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy for 2 Rx UE	6241

17.7.4	SS-SINR for RedCap	6245

17.7.4.0	Minimum conformance requirements	6245

17.7.4.0.1	Intra-frequency SS-SINR measurement accuracy requirements	6245

17.7.4.0.2	Inter-frequency SS-SINR measurement accuracy requirements	6245

17.7.4.1	NR SA FR2 SS-SINR measurement accuracy for 2 Rx UE	6245

17.7.4.2	NR SA FR2-FR2 SS-SINR measurement accuracy for 2 Rx UE	6248

18	E-UTRA - NR inter-RAT with E-UTRA serving cell for RedCap	6252

18.1	RRC_IDLE state mobility for RedCap	6252

18.1.1	Inter-RAT cell re-selection for RedCap	6252

18.1.1.0	Minimum conformance requirements	6252

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA cell re-selection to higher priority NR target Cell for 2 Rx UE	6252

18.2	RRC_CONNECTED state mobility for RedCap	6256

18.2.1	Handover for RedCap	6256

18.2.1.0	Minimum conformance requirements	6256

18.2.1.1	E-UTRA - NR SA FR1 handover for 2 Rx UE	6257

18.2.2.0	Minimum conformance requirements	6260

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	6260

18.2.2.1	E-UTRA - NR SA FR1 RRC connection release with redirection for 2 Rx UE	6261

18.3	Measurement procedure for RedCap	6268

18.3.1	E-UTRA - NR Measurements for RedCap	6268

18.3.1.0	Minimum conformance requirements	6268

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	6268

18.3.1.1	E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in non-DRX	6271

18.3.1.2	E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in DRX	6275

18.3.1.3	E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in non-DRX	6279

18.3.1.4	E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in DRX	6283

18.3.1.5	E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in non-DRX	6287

18.3.1.6	E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in DRX	6289

18.3.1.7	E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in non-DRX	6292

18.3.1.8	E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in DRX	6294

19	NR standalone tests for ATG	6297

19.2	RRC_CONNECTED state mobility	6297

19.2.1	Handover for ATG	6297

19.2.1.0	Minimum conformance requirements	6297

19.2.1.0.1	Minimum conformance requirements for NR FR1 – NR FR1 handover	6297

19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	6298

19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	6303

19.2.2	Conditional Handover for ATG	6308

19.2.2.0	Minimum conformance requirements	6308

19.2.2.0.1	Minimum conformance requirements for NR FR1 – NR FR1 conditional handover	6308

19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	6310

19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	6315

19.4	Signalling characteristics for ATG	6321

19.4.1	Radio link monitoring for ATG	6321

19.4.1.0	Minimum conformance requirements	6321

19.4.1.0.1	Minimum conformance requirements for SSB based RLM	6321

19.4.1.0.2	Minimum conformance requirements for CSI-RS based RLM	6322

19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6323

19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6327

19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6331

The rate of correct events observed during repeated tests shall be at least 90 % with the confidence level of 95%.19.4.1.4	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6335

19.6	Measurement Performance requirements for ATG	6339

19.6.1	SS-RSRP for ATG	6339

19.6.1.0	Minimum conformance requirements	6339

19.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	6339

19.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	6339

19.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	6339

19.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	6339

19.6.1.1	Intra-frequency measurements for ATG	6340

19.6.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for ATG	6340

19.6.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for ATG	6341

19.6.1.2	Inter-frequency measurements for ATG	6343

19.6.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG	6343

19.6.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG	6344

19.6.2	SS-RSRQ for ATG	6346

19.6.2.0	Minimum conformance requirements	6346

19.6.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	6346

19.6.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	6346

19.6.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	6346

19.6.2.1	NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG	6346

19.6.2.2	Inter-frequency SS-RSRQ measurements for ATG	6348

19.6.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG	6348

19.6.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG	6349

19.6.3	SS-SINR for ATG	6351

19.6.3.0	Minimum conformance requirements	6351

19.6.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	6351

19.6.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	6351

19.6.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	6351

19.6.3.1	NR SA FR1 SS-SINR absolute measurement accuracy for ATG	6351

19.6.3.2	Inter-frequency SS-SINR measurements for ATG	6352

19.6.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG	6352

19.6.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG	6354

19.6.4	L1-RSRP measurement for beam reporting for ATG	6355

19.6.4.0	Minimum conformance requirements	6355

19.6.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	6355

19.6.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	6355

19.6.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	6355

19.6.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	6356

19.6.4.1	SSB based L1-RSRP measurements for ATG	6356

19.6.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG	6356

19.6.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG	6357

19.6.4.2	CSI-RS based L1-RSRP measurements for ATG	6359

19.6.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG	6359

19.6.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG	6360

-	Table 6.7.4.2.2.5-1 is replaced by Table 19.6.4.2.1.5-1.19.6.5 L1-SINR measurement for beam reporting for ATG	6362

19.6.5.0	Minimum conformance requirements	6362

19.6.5.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured	6362

19.6.5.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	6362

19.6.5.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	6362

19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR for ATG	6362

19.6.5.1.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG	6362

19.6.5.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured L1-SINR relative measurement accuracy for ATG	6363

[Editor's note: duplication!]	6365

19.6.5.1.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG	6365

19.6.5.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy for ATG	6366

[Editor's note: duplication!]	6367

19.6.5.2	NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG	6367

19.6.5.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy for ATG	6369

19.6.5.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy for ATG	6369

[Editor's note: duplication!]	6370

19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR for ATG	6370

19.6.5.3.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG	6370

19.6.5.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy for ATG	6372

-	Table 6.7.9.3.1.5-1 is replaced by Table 19.6.5.3.1.5-1.[Editor's note: duplication!]	6373

19.6.5.3.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG	6373

Annex A (normative): RRM test configurations	6377

A.1	Reference measurement channels	6377

A.1.0	General	6377

A.1.1	PDSCH	6377

A.1.1.1	FDD	6377

A.1.1.2	TDD	6378

A.1.2	CORESET for RMSI scheduling	6381

A.1.2.1	FDD	6381

A.1.2.2	TDD	6382

A.1.3	CORESET for RMC scheduling	6385

A.1.3.1	FDD	6385

A.1.3.2	TDD	6386

A.1.4	CSI-RS	6390

A.1.4.1	FDD	6390

A.1.4.2	TDD	6393

A.1.4A	CSI-RS for tracking	6398

A.1.4A.1	FR1	6398

A.1.4A.1.1	FDD	6398

A.1.4A.1.2	TDD	6399

A.1.4A.2	FR2	6400

A.1.4A.2.1	TDD	6400

A.1.4B	CSI-IM configurations	6401

A.1.4B.1	FDD	6401

A.1.4B.2	TDD	6401

A.1.5	TDD UL/DL configuration	6403

A.1.6	PUSCH	6404

A.1A	Reference measurement channels under CCA	6405

A.1A.0	General	6405

A.1A.1	PDSCH	6405

A.1A.1.1	FDD	6405

A.1A.2	CORESET for RMSI scheduling	6406

A.1A.2.1	TDD	6406

A.1A.3	CORESET for RMC scheduling	6407

A.1A.3.1	TDD	6407

A.1A.4	TDD UL/DL configuration	6408

A.1A.4.1	TDD	6408

A.1A.5	RMC burst transmission model	6408

A.2	Reference OCNG configuration	6408

A.2.1	Generic OFDMA channel noise generator (OCNG)	6408

A.3	Reference SSB configuration	6411

A.3.1	SSB configuration for FR1	6411

A.3.2	SSB configuration for FR2	6411

A.3A	Reference SSB configuration under CCA	6412

A.3A.1	SSB configuration under CCA for FR1	6412

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	6412

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	6412

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	6413

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	6413

A.3B	Reference SSB configuration for RedCap	6414

A.3B.1	SSB configuration for FR1	6414

A.3B.2	SSB configuration for FR2	6415

A.4	Reference SMTC configuration	6415

A.4A	Reference SMTC configuration for RedCap	6415

A.5	Reference DRX configurations	6416

A.6	EN-DC test setup	6416

A.6.1	E-UTRA serving cell parameters	6416

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	6416

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	6418

A.6A	NR FR1-FR2 test setup	6420

A.6B	EN-DC test setup with unlicensed bands	6420

A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	6420

A.7	Reference PRACH configurations	6421

A.7.1	PRACH configurations for FR1	6421

A.7.1A	PRACH configurations for FR1 under CCA	6423

A.7.1A.1	FR1 PRACH configuration 1 under CCA	6423

A.7.1A.2	FR1 PRACH configuration 2 under CCA	6423

A.7.2	PRACH configurations for FR2	6424

A.7A	Reference MsgA configurations	6426

A.7A.1	MsgA configurations for FR1	6426

A.7A.1A	MsgA configurations for FR1 under CCA	6428

A.7A.1A.1	FR1 MsgA configuration 1 under CCA	6428

A.7A.1A.2	FR1 MsgA configuration 2 under CCA	6429

A.7A.2	MsgA configurations for FR2	6431

A.8	Reference BWP configurations	6433

A.8.1	Downlink BWP configurations	6433

A.8.2	Uplink BWP configurations	6433

A.8A	Reference BWP configurations for RedCap	6434

A.8A.1	Downlink BWP configurations	6434

A.8A.2	Uplink BWP configurations	6434

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	6434

A.9.1	Setup 1: Single AoA in Rx beam peak direction	6435

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	6435

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	6435

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	6435

A.9.3	Setup 3: 2 AoAs	6435

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	6436

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	6436

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	6436

A.9.7	Setup 7: 3 AoAs	6437

A.10	TCI State Configuration	6437

A.10.1	Introduction	6437

A.10.2	TCI states	6437

A.10A	Unified TCI State Configuration	6437

A.10A.1	Introduction	6437

A.10A.2	DLorJoint TCI states	6438

A.10A.3	UL TCI states	6438

A.10B	LTM Candidate TCI State Configuration	6439

A.10B.1	Introduction	6439

A.10B.2	LTM candidate DLorJoint TCI states	6439

A.10B.3	LTM candidate UL TCI states	6439

A.11	NR sidelink communication	6439

A.11.1	Introduction	6439

A.11.2	Reference resource pool configurations for NR Sidelink Communication	6440

A.11.3	Reference measurement channels for NR Sidelink Communication	6443

A.12	Discovery Burst Transmission Window configuration under CCA	6444

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	6444

A.13	Channel bandwidth (CBW) configurations	6444

A.13.1	DL UE specific CBW	6444

A.13.2	UL UE specific CBW	6444

A.14	Satellite access configuration	6445

A.14.2	Satellite specific configuration for neighbour cell	6445

A.15	SRS Configurations	6446

A.15.1	SRS Configuration for SCS=15kHz	6446

A.15.2	SRS Configuration for SCS=30kHz	6447

A.15.3	SRS Configuration for SCS=30kHz	6448

A.16	CSI-RS configurations for RRM	6448

A.16.1	FDD	6448

A.16.2	TDD	6449

A.17	Reference Cell DTX configurations	6451

A.17.1	Cell DTX Configuration 1: Cell DTX cycle = 160 ms and TAT = Infinity	6451

A.18	Spatial Relation Configuration	6451

A.18.1	Introduction	6451

A.18.2	Spatial Relation	6451

Annex B (normative): Conditions for RRM requirements applicability for operating bands	6452

B.1	Conditions for NR RRC_IDLE state mobility	6452

B.1.1	Introduction	6452

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6452

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6453

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6455

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6456

B.1.8	Conditions for measurements on NR intra-frequency cells for cell re-selection under CCA	6456

B.1.9	Conditions for measurements on NR inter-frequency cells for cell re-selection under CCA	6456

B.2	Conditions for NR RRC_CONNECTED state	6456

B.2.1	Introduction	6456

B.2.2	Conditions for NR intra-frequency measurements	6457

B.2.3	Conditions for NR inter-frequency measurements	6457

B.2.4	Conditions for NR L1-RSRP reporting	6458

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6458

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6459

B.2.5	Conditions for RRC connection release with redirection to NR	6460

B.2.6	Conditions for UE transmit timing	6461

B.2.6.1	Conditions for SSB based UE transmit timing	6461

B.2.7	Conditions for NR intra-frequency measurements for RedCap	6462

B.2.8	Conditions for NR inter-frequency measurements for RedCap	6464

B.2.9	Conditions for NR intra-frequency measurements under CCA	6466

B.2.10	Conditions for NR inter-frequency measurements under CCA	6467

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6467

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6467

B.2.12 to B.2.16	6467

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6467

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6468

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6468

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6468

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6468

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6468

B.3	RRM requirement exceptions	6469

B.3.1	Introduction	6469

B.3.2	Receiver sensitivity relaxation for CA	6469

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6469

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6469

B.3.2.2.1	Inter-band carrier aggregation	6469

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6469

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6469

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6470

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6470

B.3.2.4.1	Intra-band contiguous carrier aggregation	6470

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6470

B.3.3	Receiver sensitivity relaxation for DC	6470

B.3.4	Receiver sensitivity relaxation for SUL	6470

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6470

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6470

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6470

B.4	Conditions for NR sidelink	6471

B.4.1	Test parameters for GNSS signals	6471

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6471

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6471

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6472

B.5	High level test procedure for SAN RRM tests	6472

Annex C (normative): Downlink physical channels and propagation conditions	6473

C.1	Downlink physical channels	6473

C.1.1	General	6473

C.1.2	Default downlink signal levels	6473

C.1.3	Default connection setup	6474

C.2	Propagation conditions	6474

C.2.0	General	6474

C.2.1	No interference	6475

C.2.2	Static propagation conditions	6475

C.2.2.0	General	6475

C.2.2.1	UE receiver with 2Rx antenna connectors	6475

C.2.2.2	UE receiver with 4Rx antenna connectors	6475

C.2.3	Multi-path fading propagation conditions	6476

Annex D (normative): Deviations from standard test configuration	6477

D.1	Test cases with different numerologies	6477

D.2	EN-DC test cases with different EN-DC configurations	6477

D.2.0	General	6477

D.2.1	Principle of testing	6477

D.3	Carrier aggregation test cases with different CA configurations	6477

D.3.0	General	6477

D.3.1	Principle of testing	6477

D.4	Antenna connection for 4Rx capable UEs	6477

D.4.0	General	6477

D.4.1	Principle of testing	6478

D.4.1.1	Single carrier tests	6478

D.4.1.2	Carrier aggregation tests	6479

D.4.1.3	EN-DC tests	6479

D.4.2	Antenna connection	6479

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	6479

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	6479

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	6479

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	6480

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	6480

D.4A.1	Introduction	6480

D.4A.2	Principle of Testing	6480

D.4A.2.1	Single Carrier Tests	6480

D.4A.2.2	Carrier Aggregation Tests	6481

D.4A.2.3	EN-DC Tests	6481

D.4A.2.4	Antenna connection for bands where 2RX is supported	6481

D.4A.2.5	Antenna connection for bands where 4RX is supported	6481

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	6481

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	6481

D.4B	Antenna connection for 8Rx capable UEs	6482

D.4B.1	Introduction	6482

D.4B.1	Principle of testing	6482

D.4B.1.1	Single carrier tests	6482

D.4B.1.2	Carrier aggregation tests	6482

D.4B.1.3	EN-DC tests	6482

D.4B.2	Antenna connection	6483

D.4B.2.1	Antenna connection for NR bands where 2Rx is supported	6483

D.4B.2.2	Antenna connection for NR bands where 4Rx is supported	6483

D.4B.2.3	Antenna connection for bands where 8Rx is supported	6483

D.4B.2.4	Antenna connection for E-UTRA bands where 2Rx is supported	6483

D.4B.2.5	Antenna connection for E-UTRA bands where 4Rx is supported	6483

D.4B.2.6	Antenna connection for E-UTRA bands where 8Rx is supported	6483

D.5	Test Cases with Different Channel Bandwidths	6483

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	6483

D.5.1.1	Introduction	6483

D.5.1.2	Principle of testing	6483

D.6	Test Cases for Synchronous and Asynchronous DC Operations	6484

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	6484

D.6.1.1	Introduction	6484

D.6.1.2	Principle of Testing	6484

D.7	CCA model	6484

D.7.1	Introduction	6484

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	6484

D.7.2.1	DL CCA model	6484

D.7.2.2	UL CCA model	6485

D.7.3	Void	6486

D.8	Test Cases in SA and EN-DC Operations	6486

D.8.0	General	6486

D.8.1	Principle of testing	6486

Annex E (normative): Cell configuration mapping	6487

E.0	General	6487

E.1	Test frequency selection	6487

E.1.0	General	6487

E.1.1	E-UTRA PCell for EN-DC test cases	6487

E.1.2	Test cases with one NR cell	6487

E.1.3	Test cases with more than one NR cell	6487

E.1.3.1	Intra-frequency test cases	6487

E.1.3.2	Inter-frequency test cases	6487

E.1.4	Carrier aggregation test cases	6488

E.1.4.1	Inter-band carrier aggregation	6488

E.1.4.2	Intra-band contiguous carrier aggregation	6488

E.1.4.3	Intra-band non-contiguous carrier aggregation	6488

E.1.5	E-UTRA – NR inter RAT test cases	6488

E.1.6	Intra-band EN-DC test cases	6488

E.1.6.1	Intra-band non-contiguous EN-DC	6488

E.1.6.1.1	Inter frequency neighbour cell	6488

E.1.6.2	Intra-band contiguous EN-DC	6488

E.1.6.1.1	E-UTRA PCell	6488

E.1.7	NR sidelink test cases	6489

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	6489

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	6493

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	6497

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	6504

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	6509

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	6510

E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10	6511

E.9	Cell configuration mapping for SA FR1 Shared Spectrum test cases in Chapter 11	6514

E.13	6524

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	6524

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	6530

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	6532

E.17	Cell configuration mapping for SA FR1 test cases for ATG in Chapter 19	6533

Annex F (normative): Measurement uncertainties and test tolerances	6535

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	6535

F.1.1	Acceptable uncertainty of test system (normative)	6535

F.1.1.1	Measurement of test environments	6535

F.1.1.2	Measurement of RRM requirements	6535

F.1.2	Interpretation of measurement results (normative)	6616

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	6616

F.1.3.1	Measurement of test environments	6616

F.1.3.2	Measurement of RRM requirements	6616

Annex G (normative): Statistical testing	6806

G.1	General	6806

G.2	Statistical testing of delay and UE measurement performance in RRM tests	6806

G.2.1	General	6806

G.2.2	Design of the test	6806

G.2.3	Numerical definition of the pass fail limits	6806

G.2.4	Pass fail decision rules	6807

G.2.5	Void	6808

G.2.6	Test conditions for delay tests and UE measurement performance	6808

G.2.7	Additional considerations for tests in bands subject to CCA failure	6808

G.3	Statistical testing of NR sidelink CBR measurement tests	6809

G.3.1	General	6809

G.3.2	Design of the test	6809

G.3.3	Numerical definition of the pass fail limits	6810

G.3.4	Pass fail decision rules	6810

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	6811

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	6811

G.5.1	Algorithm to calculate pass and fail probabilities	6811

G.5.2	Method of designing pass/fail limit to approach given risk probability	6814

Annex H (normative): Default message contents for RRM	6819

H.1	Void	6819

H.2	System information blocks message content exceptions	6819

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	6819

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	6821

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	6822

H.3	RRC message content exceptions	6823

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	6823

H.3.2	RRC messages and information elements contents exceptions for handover	6849

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	6850

H.3.4	E-UTRA RRC messages and information elements contents exceptions	6851

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	6862

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	6865

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	6871

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	6874

H.3.9	RRC messages and information elements contents exceptions for UL timing	6875

H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access	6876

H.3.11	RRC messages and information elements contents exceptions for UL Tx switching	6877

H.3.12	RRC messages and information elements contents exceptions for L1/L2-triggered mobility	6894

Annex I (normative): RRM OTA procedures	6905

I.0	Test applicability per permitted test method	6905

I.1	Direct far field (DFF)	6905

I.1.1	RX beam peak direction search	6905

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	6905

I.2	Direct far field (DFF) simplification	6905

I.2.1	RX beam peak direction search	6905

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	6906

I.3	Indirect far field (IFF)	6906

I.3.1	RX beam peak direction search	6906

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	6906

I.3A	Enhanced indirect far field (Enhanced IFF)	6906

I.3A.1	RX beam peak direction search	6906

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	6906

I.4	Procedures to search test directions for RRM FR2	6906

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	6906

I.5	Re-Positioning Approach for RRM FR2	6907

I.5.1	1 AoA Test Cases	6908

I.5.2	2 AoA Test Cases	6909

Annex J (informative): Change history	6911