# Title:	Test Tolerances for RACH based Inter-frequency PCell switch from FR2 to FR2

# Source:	Mediatek

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RACH based Inter-frequency PCell switch from FR2 to FR2.

The test case in question, from TS 38.533, is:

- 7.3.4.3, NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2

The applicability of the analysis is:

- UE Power class 3

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v18.9.0.

#### A.7.3.4.3	RACH-based Inter-frequency LTM PCell switch from FR2 to FR2

##### A.7.3.4.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 RACH-based inter-frequency PCell switch specified in clause 6.3.1 for both with and without early TCI state activation.

##### A.7.3.4.3.2	Test Parameters

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on a different frequency than the PCell. Test configurations are given in table A.7.3.4.3.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.7.3.4.3.2-2 and A.7.3.4.3.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of five successive time periods, with time durations of T1, T2, T3, and T4, respectively. Measurement gap pattern gp0 is configured.

During T1, for Test 1A, 1B,2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in table A.7.3.4.3.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in table A.7.3.4.3.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 100 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell and the field of Timing Advance Command is set to 0.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	Cell 2 continuously schedules PUSCH for the UE.

-	T4 ends upon the reception of PRACH at Cell 2.

Table A.7.3.4.3.2-1: Inter-frequency cell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.4.3.2-2: General test parameters for Inter-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -15 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| includeBeamMeasurements |  |  | True |  |  |  |  |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Measurement gap pattern ID |  |  | gp0 |  |  |  | As specified in table 9.1.2-1 |
| Measurement gap offset |  |  | 39 |  |  |  |  |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0.3 s |  |  |  |  |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.2 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.7.3.4.3.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T5 | T1 ~ T5 |
| NR RF Channel Number |  |  |  | 1 | 2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |
| BWchannel |  |  | MHz | 100: NPRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NPRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |
|  |  |  | dB | 5 | 5 |
|  |  |  | dB | 5 | 5 |
| SSB_RP |  |  | dBm/SCS | -90.7 | -90.7 |
| IoNote3 |  |  | dBm/ 95.04 MHz | -60.5 | -60.5 |
| Propagation condition |  |  | - | AWGN | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.7.3.4.3.3	Test Requirements

The UE shall start to transmit RACH to Cell 2 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90 %.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3 ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU,

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B,

-	TLTM-IU = 20 ms.

-	TLTM-RRC-processing = 10 ms if UE does not support ltm-FastProcessingConfig-r18, otherwise TLTM-RRC-processing =0 ms

-	TLTM-processing = 10 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support ltm-FastUE-Processing-r18 capability.

# 3. Discussion

In the test there are two cells: Cell 1 and Cell 2. Cell 1 is FR2 PCell, Cell 2 is an FR2 neighbour cell on a different frequency than the PCell. The test consists of 3 test configurations and each configuration has 4 tests: Test 1A, 1B, 2A and 2B and 4 time periods: T1, T2, T3 and T4. Test 2A and 2B skip T3 if the UE is incapable of Early DL Synch.

Observations

In T1, UE performs inter-frequency measurement on Cell 2. UE shall meet the accuracy requirement in clause 10.1.2 of TS 38.133 to report an Event A3 on Cell 2 to Cell 1.

In T2, UE performs inter-frequency L1-RSRP measurement on Cell 2. UE shall meet the measurement and absolute accuracy requirement in clauses 9.15 and 10.1.20B of TS 38.133 to report a valid L1-RSRP of Cell 2 to Cell 1.

In T3, UE receives candidate cell TCI state activation MAC CE for Cell 2. UE shall meet the delay requirement in clause 8.25 of TS 38.133 to complete the activation of TCI state within the delay.

In T4, UE receives LTM cell switch command MAC CE for Cell 2 and transmits PRACH on Cell 2 no later than cell switch delay. UE shall meet the delay requirement in clause 6.3 of TS 38.133.

It is reasonable that UE shall meet the same accuracy requirement for L1 and L3 inter-frequency measurement during the whole test, ensuring that

- the consistency of the network triggers the cell switch from Cell 1 (Source Cell) to Cell 2 (Target Cell): the measurement of NR Cell 2 is always better than NR Cell 1 compared it to the signalled A3 offset, hence, the Event A3 entering conditions are satisfied since the end of T1, and

- the UE is capable to continuously perform the periodic L1-RSRP measurement report on Cell 2 to Cell 1 since the end of T2.

# 3.1	Measurement and delay

UE shall do the L3 measurement and report of both Cell 1 and Cell2 at the end of T1. UE also shall do L1-RSRP measurement and report of Cell 2 periodically from the beginning of T2. Additionally, UE shall follow the delay requirements of TCI state activation at the beginning of T3 and the cell switch at the beginning of T4 based on test procedure.

The effect of uncertainties on the test verdict needs to be checked. The decisions can then be made on whether Test Tolerances need to be applied.

Overall, after the discussion above, the measurement and delay requirement be discussed in section 4.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.7.3.4.3.2-1, A.7.3.4.3.2-2 and A.7.3.4.3.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The angle of arrival of the radiated signals is also recorded.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1.

In this test case at the start of T1, Cell 2 is a detectable cell, and the UE is expected to send an event triggered measurement report. The criterion is Event A3: Neighbour becomes amount of offset better than SpCell. The UE makes a measurement of NR Cell 2 (neighbour) relative to NR Cell 1 (PSCell) and compares it to the signalled A3 offset. A further step is done to calculate SS-RSRP power difference including the offset:

- (Cell 2 SS-RSRP – Cell 1 SS-RSRP) – Ofn (A3 offset)

Meanwhile, during T1, these two calculations also be included for the inter-frequency accuracy requirement:

- |SSB_RP1dBm - SSB_RP2dBm|  27dB

- |Channel 1_Io -Channel 2_Io |  20 dB

c) Uncertainties

The Test system provides 2 cells, one on each of 2 frequencies, with AWGN. We propose to control the following parameters:

- AWGN absolute power on carrier frequency 1, Noc averaged over BWConfig uncertainty: ±5.65* dB

- AWGN absolute power on carrier frequency 1, Noc for PRBs #RBJ-RBJ+19 uncertainty: ±5.65* dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc averaged over BWConfig uncertainty: ±0.3 dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc for PRBs #RBJ-RBJ+19 uncertainty: ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc averaged over BWConfig uncertainty: ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc for PRBs #RBJ-RBJ+19 uncertainty: ±0.3 dB

*These values are applicable for frequencies <= 40.8 GHz.

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers #RBJ to #RBJ+19 for 120kHz SCSSSB. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE SS-RSRP relative accuracy: Value taken from TS 38.133

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

<< Extracts from TS 38.133 v18.9.0 >>

##### 10.1.5.1.2	Relative SS-RSRP Accuracy

The relative SS-RSRP accuracy is defined as the SS-RSRP measured from one cell on a frequency in FR2 compared to the SS-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in table 10.1.5.1.2-1 are valid under the following conditions:

-	Conditions defined in TS 38.101-2 [19] clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to annex B.2.3 for a corresponding Band for each relevant SSB.

--	|SSB_RP1dBm - SSB_RP2dBm|  27dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.2-1: SS-RSRP Inter-frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

<< clauses skipped >>

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5, 6 |  |
| Conditions | Rx Beam Peak | n257 | -108.3 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | -105.9 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-4 |
|  |  | n258 | -108.3 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 | -106.1 |  |  |
|  |  | n259 |  |  | -106.5 |  | -118.5+Y5 |  |  |  |
|  |  | n260 | -105.3 |  | -107.5 | -123.8+Y4 |  |  |  |  |
|  |  | n261 | -108.3 | -111.8 | -110.1 | -125.8+Y4 |  | -105.9 |  |  |
|  |  | n262 | -103.3 | -106.6 | -104.6 | -119.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -100.3 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | -97.9 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-4 |
|  |  | n258 | -100.3 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 | -98.1 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |  |  |
|  |  | n260 | -97.3 |  | -94.9 | -111.8+Z4 |  |  |  |  |
|  |  | n261 | -100.3 | -100.8 | -99.2 | -116.8+Z4 |  | -97.9 |  |  |
|  |  | n262 | -95.1 | -94.7 | -91.5 | -107.7+Z4 |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Normal conditions are selected.

To ensure this relative SS-RSRP accuracy, the conditions of Io, SSB Es/Iot in Table 10.1.5.1.2-1, and conditions of SSB_RP and SSB Es/Iot in Table B.2.3-2 of section Annex B.2.2 for both NR Cell 1 and Cell 2 need to be fulfilled.

In addition, Cell 2 needs to be detectable so that the requirement of inter-frequency measurement can apply. The condition that an inter-frequency cell is detectable is defined in TS 38.133 clause 9.3.2.

### 9.3.2	Requirements applicability

The requirements in clause 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding band,

-	SS-RSRQ related side conditions given in clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding band,

-	SS-SINR related side conditions given in clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding band.

The side conditions mentioned above refer to Annex B.3.2, which is already considered.

During T2, the UE shall be able to perform inter-frequency L1-RSRP measurements on Cell 2. The Cell 2 needs to be considered as known and the SSB resource configured for L1-RSRP measurements is measurable so that the requirement of inter- frequency L1-RSRP measurement can apply. The conditions are defined in TS 38.133 9.15.2.

<< Extracts from TS 38.133 v18.9.0 >>

### 9.15.2	Requirements Applicability

The requirements in clause 9.15 apply if the following conditions are met:

-	The SSB configured for inter-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for RRM measurement in one of the measurement objects configured in the measObjectNR.

-	The inter-frequency neighbor cell is known.

-	The SSB resources configured for inter-frequency L1-RSRP measurements are measurable.

An SSB resource configured for inter-frequency L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.19E.1 and 10.1.20B.1 for FR1 and FR2, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to annex B.2.4.1 for a corresponding band.

The inter-frequency neighbor cell is considered as known if the following conditions are met in this requirement:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.3.

Otherwise, the cell is unknown.

<< clauses skipped >>

### 10.1.20B		LTM Inter-frequency L1-RSRP accuracy requirements for FR2

#### 10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements

##### 10.1.20B.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute SSB based L1-RSRP accuracy in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in table 10.1.20B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20B.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

<< clauses skipped >>

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | -120.4+ Y6 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-3 |
|  |  | n258 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 | -120.6+ Y6 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |  |
|  |  | n260 | -104.3 |  | -106.5 | -122.8+Y4 |  |  |  |  |
|  |  | n261 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  | -120.4+ Y6 |  |  |
|  |  | n262 | -102.3 | -105.6 | -103.6 | -118.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | -112.4+Z6 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-3 |
|  |  | n258 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 | -112.6+Z6 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |  |  |
|  |  | n260 | -96.3 |  | -93.9 | -110.8+Z4 |  |  |  |  |
|  |  | n261 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  | -112.4+Z6 |  |  |
|  |  | n262 | -94.3 | -93.7 | -90.5 | -106.7+Z4 |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

One of the conditions for the NR Cell 2 to be considered as known mentioned in above, refers to TS 38.133 clause 9.3, which has already been considered.

To ensure UE is capable to perform inter-frequency L1-RSRP measurement, the conditions of Io, SSB Es/Iot in Table 10.1.20B.1.1-1, and conditions of SSB_RP and SSB Es/Iot in Table B.2.4.1-2 of section Annex B.2.4 for NR Cell 2 need to be fulfilled.

Next, the condition of known TCI state needs to be considered, because the UE shall complete the activation of TCI state within delay during T3. The condition is defined in the following extract from TS 38.133 8.25.2.

<< Extracts from TS 38.133 v18.9.0 >>

### 8.25.2	Known TCI state conditions

The candidate downlink TCI state in the TCI state activation command is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	The MAC CE command indicating the activation of the TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received.

-	The target downlink TCI state remains detectable during the downlink TCI state activation period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period

-	SNR of the downlink TCI state ≥ -3 dB

Otherwise, the candidate downlink TCI state is unknown.

And during T4, the conditions of known cell and known joint/separate TCI state(s) need to be considered, because the UE shall perform the cell switch from Cell 1 to Cell 2 after receiving the cell switch command. The conditions are defined in the following extract from TS 38.133 6.3.

### 6.3.1	LTM PCell Cell Switch

#### 6.3.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to a target cell indicated in LTM cell switch command. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before the cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB,

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

The condition of a known cell is the cell remains detectable refer to clause 9.3 for inter-frequency cell, which is already considered.

In summary, during T1 the following conditions shall be satisfied.

- SSB Es/Iot of Cell 1 and Cell 2 at baseband shall be no less than -4dB,

- SSB_RP of Cell 1 and Cell 2 shall be no less than -105.5dBm/120kHz (Rx beam peak, PC3 and n259) including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2,

- Io of Cell 1 and Cell2 shall be no less than -105.5dBm/120kHz (Rx beam peak, PC3 and n259) and no larger than -50dBm/BWchannel including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2,

- UE measured (Cell 2 RSRP - Cell 1 RSRP - A3-Offset) shall be larger than 0 dB,

- |SSB_RP1dBm - SSB_RP2dBm|  27dB, and

- |Channel 1_Io -Channel 2_Io |  20 dB.

During T2~T4 the following conditions shall be satisfied.

- SSB Es/Iot of Cell 1 at baseband shall be no less than -4dB,

- SSB_RP of Cell 1 shall be no less than -105.5dBm/120kHz (Rx beam peak, PC3 and n259) including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2,

- Io of Cell 1 shall be no less than -105.5dBm/120kHz (Rx beam peak, PC3 and n259) and no larger than -50dBm/BWchannel including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2,

- SSB Es/Iot of Cell 2 at baseband shall be no less than -3dB,

- SSB_RP of Cell 2 shall be no less than -104.5dBm/120kHz (Rx beam peak, PC3 and n259) including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2,

- Io of Cell 2 shall be no less than -104.5dBm/120kHz (Rx beam peak, PC3 and n259) and no larger than -50dBm/BWchannel including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2,

- UE measured (Cell 2 RSRP - Cell 1 RSRP - A3-Offset) shall be larger than 0 dB,

- |SSB_RP1dBm - SSB_RP2dBm|  27dB, and

- |Channel 1_Io -Channel 2_Io |  20 dB.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and also the side conditions that should be met for the UE to be tested correctly. The 6 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. Cell 1 and Cell 2 Es/Iot at UE baseband is calculated according to TR 38.903 A.2.4, and applied Io calculated according to TR 38.903 A.2.5.

The test system uncertainties are combined root-sum-square as they are uncorrelated. More detailed explanation is given in the accompanying spreadsheet, and in TR 38.903 A.2.7.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE-related uncertainties where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

e) Determine parameters to offset

During T1, UE is expected to perform inter-frequency measurement. The nominal conditions in TS 38.133 Table A.7.3.4.3.2-3 gives SS-RSRP values, as measured by the UE. More details are available in the attached spreadsheet.

The Event A3 measurement report trigger requirement in TS 38.331 clause 5.5.4.4 is “Event A3 (Neighbour becomes offset better than SpCell), inequality A3-1 (Entering condition) is: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off”, and the equation reduces to:

Mn > Mp + Off. Putting in values and rearranging gives -90.7dBm - (-90.7dBm) > -15dB.

The uncertainties do not take NR Cell 1 and Cell 2 Ês/Iot, SSB_RP and lo power outside the allowed range under the conditions of inter-frequency measurement requirement; therefore, no offsets are required.

The inequality defined above ensures that Cell 2 SS-RSRP – Cell 1 SS-RSRP > A3 offset. There is no need for offset to the A3-offset.

The Es/Iot and SSB_RP of both NR Cells and the Io on NR RF channel 1 and channel 2 fulfill the required side conditions taking into account the uncertainties, therefore no offsets are required.

During T2, UE is expected to perform SSB-based L1-RSRP inter-frequency measurement on NR Cell 2 and report a valid L1-RSRP report of Cell 2.

The uncertainties do not take NR Cell 2 Ês/Iot, SSB_RP and Io power outside the allowed range under the conditions of inter-frequency L1-RSRP measurement requirement; therefore, no offsets are required.

During T3, UE is expected to activate the TCI state of NR Cell 2.

The uncertainties do not take NR Cell 2 Ês/Iot power outside the allowed range under the condition of LTM TCI state activation requirement; therefore, no offsets are required.

During T4, UE is expected to transmit the PRACH triggered by cell switch command.

The uncertainties do not take NR Cell 2 Ês/Iot, SSB_RP power outside the allowed range under the condition of LTM cell switch delay requirement; therefore, no offsets are required.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/IotBB, Applied SSB_RP, Applied SSB_RP difference , Applied Io, Applied Io difference and Event A3 entering condition). The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable. It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.