3GPP TSG-RAN5 Meeting #107	R5-251890

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	FR2 MU - PC7 analysis for ACS and IBB test cases

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to analyse the measurement uncertainty, test tolerances and relaxations applicable to PC7 for ACS and IBB test cases.

# Discussion

## Relaxations

The relaxation baseline analysis for ACS and IBB test cases is captured in [1]. There, in FR2a, the agreed maximum feasible interferer power is set to -52.8dBm. In this test cases, ACS Case 1 and IBB, the power level of the interferer, Pinterferer, depends on the REFSENS power level which in the end also depends on the power class. For PC7, the REFSENS per polarization is -82.3dBm/50MHz and -79.3dBm/100MHz. The required Pinterferer is equal to REFSENS per polarization + 34.5dB resulting in a Pinterferer power of -52.8dBm/50MHz and -49.8dBm/100MHz. Therefore, in case of 100MHz, required Pinterferer for PC7 in ACS Case 1 and IBB test cases will be 3dB higher than the maximum power supported so 3dB relaxation will be required. All these calculations are summarised in table below.

| Frequency range | Feasible Interferer power (dBm) | Interferer level requirement for 50 MHz CHBW (dBm) | Interferer level requirement for 100 MHz CHBW (dBm) | Relaxation for 50 MHz CHBW (dB) | Relaxation for 100 MHz CHBW (dB) |
| --- | --- | --- | --- | --- | --- |
| FR2a | -52.8 | -52.8 | -49.8 | N/A | 3dB |

Table 2.1-1. Feasible interferer power level and proposed relaxation required for PC7 in ACS Case 1 and IBB tests.

As the most relevant in ACS and IBB testing is the relative power between the interferer and the wanted signal, in case relaxation is required, it should be applied to both the interferer and the wanted signal. This approach is already followed in these test cases for other power classes and frequency ranges where relaxations are already required so same approach should be followed now for PC7 in Fra.

Proposal 1. For testing PC7 in ACS Case 1 and IBB test cases in FR2a bands, for 100MHz bandwidth, apply 3dB relaxation to both the interferer and the wanted signal.

## MU and TT

If relaxations are applied as proposed in section 2.1, taking also into account the general MU analysis made for PC7 in [2]and [3] , for ACS and IBB test cases it is proposed to reuse MU and TT definition for PC3 in FR2a.

Proposal 2. For testing PC7 in ACS Case 1 and IBB test cases in FR2a bands, for 100MHz bandwidth, reuse corresponding MU and TT defined for PC3.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. For testing PC7 in ACS Case 1 and IBB test cases in FR2a bands, for 100MHz bandwidth, apply 3dB relaxation to both the interferer and the wanted signal.

Proposal 2. For testing PC7 in ACS Case 1 and IBB test cases in FR2a bands, for 100MHz bandwidth, reuse corresponding MU and TT defined for PC3.

# References

- R5-197497, “Discussion on testability of ACS and IBB”, Rohde & Schwarz, Anritsu, Keysight, RAN5#84

- R5-246230, “PC7 FR2 MU - Discussion on non-antenna related MU factors”, Keysight, RAN5#105

- R5-246537, “PC7 MUs based on antenna array assumptions”, Keysight, RAN5#105

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.6.0 (2025-03)

- TS 38.101-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v17.17.0 (2025-03)