3GPP TSG-RAN5 Meeting #107	R5-251940

# St. Julians, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0362 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the applicability of eCall CEN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several discrepancies between the parameters listed in Table 4.1 and their numbering in TS 34.229-2 and typos in C176 and C177. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 4.1, update all instances of A.4.5/67 with A.12/67 and wrong references in C176 and C177. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

< Unchanged sections omitted >

## 4.1	Applicabilities for test cases specified in TS 34.229-1

Table 4.1: Applicability of TS 34.229-1 tests

| Condition | Test Case Selection Expression | Comment |
| --- | --- | --- |
| C00 | Void |  |
| … | … | … |
| C148 | IF A.12/54 AND [3] A.10/17 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and automatic type of eCall |
| C149 | IF A.12/55 AND [3] A.10/16 AND NOT A.12/67 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and manual type of eCall and not subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C150 | IF A.12/55 AND [3] A.10/17 AND NOT A.12/67 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and automatic type of eCall and not subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C151 | IF A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | E-UTRA and not UE category M1 |
| C152 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and preconditions and WLAN |
| C153 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/4 AND A.15/3 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and preconditions and WLAN |
| C154 | IF A.3A/61 AND [73] A.4.4-2/32 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | SM-over-IP sender and UE configured to use SMS over IP and UE is category M1 |
| C155 | IF A.3A/62 AND [73] A.4.4-2/32 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | SM-over-IP receiver and UE configured to use SMS over IP and UE is category M1 |
| C156 | Void | - |
| C157 | IF A.3A/50 AND A.15/1 AND A.4/2B AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND A.12/57 THEN R ELSE N/A | MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 and Session Timer |
| C158 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND A.12/57 THEN R ELSE N/A | MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 and Session Timer |
| C159 | IF A.3A/50 AND A.15/1 AND A.4/2B AND A.18/1 AND A.16/19 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | UE supports MTSI and MTSI speech and initiating a session and E-UTRA and USSI and not UE category M1 |
| C160 | IF A.3A/50 AND A.15/1 AND A.4/2B AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| C161 | IF A.6a/5 AND A.18/3 AND A.3A/50 AND A.4/16 AND A.15/1 THEN R ELSE N/A | MTSI and MTSI speech and SIP Digest without TLS and preconditions and Fixed Broadband |
| C162 | IF [73] A.4.2.1.1-1/4 AND A.18/1 AND [3] A.20/90 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and E-UTRA and no USIM test execution and not UE category M1 |
| C163 | IF A.3A/50 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and preconditions and UE category M1 |
| C164 | IF A.3A/50 AND A.15/1 AND A.4/2B AND NOT A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and initiating a session and no preconditions and E-UTRA and not UE category M1 |
| C165 | IF A.3A/50 AND A.15/1 AND A.18/1 AND NOT A.4/16 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | UE supports MTSI and MTSI speech and E-UTRA and no preconditions and not UE category M1 |
| C166 | Void |  |
| C167 | IF A.3A/50 AND A.15/1 AND A.15/2 AND NOT A.15/10 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI speech, AMR wideband and not MTSI speech, EVS and preconditions and E-UTRA and not UE category M1 |
| C168 | IF A.3A/64 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | Concatenated SM-over-IP sender and E-UTRA and UE is not category M1 |
| C169 | IF A.3A/65 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | Concatenated SM-over-IP receiver and E-UTRA and UE is not category M1 |
| C170 | IF A.3A/50 AND A.15/1 AND A.4/16 AND [93] A.4.1-3/2 THEN R ELSE N/A | UE supports MTSI and MTSI speech and preconditions and EN-DC |
| C171 | IF A.6a/2 AND A.18/1 AND A.15/14 NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS security and E-UTRA and IMS data channel and not UE category M1 |
| C172 | IF A.12/55 AND [3] A.10/16 AND A.12/67 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and manual type of call and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C173 | IF A.12/55 AND [3] A.10/17 AND A.12/67 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and Automatic type of eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C174 | IF A.12/54 AND [3] A.10/16 AND A.12/67 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and manual type of eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C175 | IF A.12/54 AND [3] A.10/17 AND A.12/67 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and automatic type of eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C176 | IF A.12/54 AND [3] A.10/16 AND [3] A.10/19 AND A.12/67 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and Manual type of eCall initiation and capable of triggering a Test eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| C177 | IF A.12/55 AND [3] A.10/16 AND [3] A.10/19 AND A.12/67 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation and capable of triggering a Test eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |

<End of modified section>