3GPP TSG-RAN5 Meeting #107	R5-251969

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0085 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of Beamlock test function to support locking two TX beams from two panels simultaneously |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing support locking multiple TX beams from multiple panels simultaneously |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE to the UBF messages in clauses 5.4.2 and 5.4.3 to allow locking of multiple TX beams simultaneously from Rel-18 onwards. Changes in this CR essentially implement Proposal 1 and example in R5-251677 endorsed in RAN5#106. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support locking multiple TX beams from multiple panels simultaneously will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Response LS to RAN4 in R5-251970 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 5.4.2	Activate Beamlock procedure

#### 5.4.2.1	Initiation

The SS requests the UE to activate beamlock by transmitting an ACTIVATE BEAMLOCK message.

#### 5.4.2.2	Reception of ACTIVATE BEAMLOCK message by UE

When UE receives ACTIVATE BEAMLOCK message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state:

2>	if UE Beamlock test Function = 01

3>	Lock the UE antenna pattern with Tx only

NOTE: For UEs Release 18 and above supporting STxMP, lock all active UE antenna patterns

2>	else if UE Beamlock test Function = 10

3>	Lock the UE antenna pattern with Rx only

2>	else if UE Beamlock test Function = 11

3>	Lock the UE antenna pattern with both TxRx

2>	Transmit ACTIVATE BEAMLOCK COMPLETE message

1>	else:

2>	the UE behaviour is unspecified.

### 5.4.3	Deactivate Beamlock procedure

#### 5.4.3.1	Initiation

The SS requests the UE to deactivate beamlock by transmitting a DEACTIVATE BEAMLOCK message. The SS should do this when the UE is in RRC_CONNECTED state.

#### 5.4.3.2	Reception of DEACTIVATE BEAMLOCK message by UE

When UE receives DEACTIVATE BEAMLOCK message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Beamlock test function is active:

2>	unlock the UE antenna pattern and transmit DEACTIVATE BEAMLOCK COMPLETE message;

NOTE: For UEs Release 18 and above supporting STxMP, unlock all active UE antenna patterns

1> else:

2>	the UE behaviour is unspecified.

#### 5.4.3.3	Release of antenna beamlock by UE

When the UE leaves the RRC_CONNECTED state, the UE shall:

1>	if the UE is operating in FR2 AND the UE Beamlock test Function is active

2>	unlock the UE antenna pattern;

NOTE: For UEs Release 18 and above supporting STxMP, unlock all active UE antenna patterns

-------- End of changes --------