3GPP TSG-RAN5 Meeting #107	R5-251991

# Malta, MT, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3878 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of RRM distance-based conditional Handover test cases in FR1 for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update of RRM distance-based conditional Handover test cases in FR1 for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s note is removed - Test requirement is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cae 19.2.2.1 & 19.2.2.2 will be still incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.2.2.1, 19.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1

Editor's Note: This test case is incomplete for the following configurations:

-	Message contents is TBD

-	TT analysis is missing.

19.2.2.1.1	Test purpose

To verify the requirememt for the NR FR1-NR FR1 intra-frequency distance-based conditional handover for ATG requirements specified in 38.133 [6] 6.1E.2.2.

19.2.2.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards and supporting conditional handover.

19.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.2.1.

19.2.2.1.4	Test description

19.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 19.2.2.1.4.1-1.

Table 19.2.2.1.4.1-1: NR ATG SA FR1 distance-based conditional handover test configurations

| Config | Description |
| --- | --- |
| 19.2.2.1-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.2.1-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.2.1-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.2.1.4.1-2

Table 19.2.2.1.4.1-2: Initial conditions for NR ATG SA FR1 distance-based conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 19.2.2.1.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 19.2.2.1.4.1-3 and 19.2.2.1.5-1 below.

Table 19.2.2.1.4.1-3: General test parameters NR ATG SA FR1 Intra-frequency distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 |  |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) at T1 start |  |  | (0, 0, 3000) | Set by AT command |
| UE moving speed |  | km/h | (1200, 0, 0) | Set by AT command |
| referenceLocation1-r17.condEventD1-r17 |  | m | (-4600, 0, 0) | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | (14479, 0, 0) | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | 200 | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | 200 | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | not barred | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 15 |  |
| T2 |  | s | 6 |  |

19.2.2.1.4.2	Test procedure

The test scenario comprises of 2 NR carrier and two cells, Cell 1 and Cell 2, on each carrier respectively. General parameters and Cell-specific parameters for Cell 1 and Cell 2 are given in Table 19.2.2.1.4.1-3 and 19.2.2.1.5-1 respectively. Measurement gap (gap pattern #0) is configured in the test case.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

Time interval T1 needs to start immediately after the UE position and moving speed are configured via AT commands. The UE is configured to measure intra-frequency neighbour cell. The RRC message implying distance-based handover to Cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2. where

- TRRC = 10ms, is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

From start of T2, the Cell 2 becomes detectable and offset better than Cell 1 and location condition event condEventD1-r17 is is satisfied. During T2, the UE performs measurement on Cell 2 and evaluates the execution condition, and starts handover procedure when execution condition is satisifed. The UE shall sent PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the start of T2. The Interruption length Tinterrupt shall also be verified in T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 19.2.2.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. UE position and moving speed are configured on the UE via AT commands. Immediately after that, T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedbacks sent by the UE.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008).

4.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure distanced-based CHO execution condition and measurement gap pattern #0 for the UE.

5.	The UE shall transmit an RRCReconfigurationComplete message.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 19.2.2.1.5-1. T2 starts.

7.	If

- the UE transmits the PRACH preambles to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the beginning of time period T2,

and

- no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = Tinterrupt +k1 for test configuration 19.2.2.1-1

-	X = Tinterrupt for test configuration 19.2.2.1-2

-	X = 2∙Tinterrupt for test configuration 19.2.2.1-3.

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

9.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

19.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

TBD

19.2.2.1.5	Test requirements

Table 19.2.2.1.5-1 defines the primary level settings for all tests.

Table 19.2.2.1.5-1: Cell specific test parameters for NR ATG SA FR1 Intra-frequency distance-based conditional handover from FR1 to FR1

| Parameter |  | Test configuration | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2, 3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1, 2 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1, 2 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  | Config 1, 2, 3 | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  | Config 1, 2, 3 |  | OP.1 |  |  |  |
| SMTC Configuration |  | Config 1, 2, 3 |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1, 2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1, 3 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  | Config 1, 2, 3 |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP | Config 1, 2, 3 |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1, 2, 3 | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | Config 1, 2, 3 | dBm/ 15kHz | -98 |  |  |  |
| Note2 |  | Config 1, 2 | dBm/ SCS | -98 |  |  |  |
|  |  | Config 3 |  | -95 |  |  |  |
|  |  | Config 1, 2, 3 | dB | 8+TT | -3.3+TT | -Infinity | 2.36+TT |
|  |  | Config 1, 2, 3 | dB | 8+TT | 8+TT | -Infinity | 11+TT |
| SSB_RP |  | Config 1, 2 | dBm/ SCS | -90+TT | -90+TT | -Infinity | -87+TT |
|  |  | Config 3 |  | -87+TT | -87+TT | -Infinity | -84+TT |
| IoNote3 |  | Config 1, 2 | dBm/ 9.36MHz | -61.41+TT | -57.06+TT | -61.41+TT | -57.06+TT |
|  |  | Config 3 | dBm/ 38.16MHz | -55.31+TT | -50.96+TT | -55.31+TT | -50.96+TT |
| Propagation condition |  | Config 1, 2 | - | AWGN + 2412HzNote4 |  |  |  |
|  |  | Config 3 |  | AWGN + 5556HzNote5 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: 2412Hz is the maximum value of doppler with carrier frequency of 2170MHz. The specific doppler shift trajectory is up to test system’s design considering of BS location and UE GNSS emulation. Note 5: 5556Hz is the maximum value of doppler with carrier frequency of 5GHz. The specific doppler shift trajectory is up to test system’s design considering of BS location and UE GNSS emulation. |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in TS 38.133 [6] clause 6.1E.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay TRRC = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

At start of T2,

distance to source cell reference location is (3000)2+(1200*1000/3600*15-(-4600))2  = 10057.8m, and D1-1 = 10000m

distance to target cell reference location is (3000)2+(1200*1000/3600*15-(14479))2  = 9942.4m, and D1-2 = 10000m

i.e. D1-1 and D1-2 conditions are fulfilled at start of T2 with >=50m location margin.

Tmeasure = max(600 + 200 ms, 0) = 800 ms;

Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 800ms + 62ms + 10ms = 872 ms.

19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1

Editor's Note: This test case is incomplete for the following configurations:

-	Message contents is TBD

-	TT analysis is missing.

19.2.2.2.1	Test purpose

To verify the requirememt for the NR FR1-NR FR1 inter-frequency distance-based conditional handover for ATG requirements specified in 38.133 [6] 6.1E.2.2.

19.2.2.2.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards and supporting distance-based conditional handover.

19.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.2.2.

19.2.2.2.4	Test description

19.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 19.2.2.2.4.1-1.

Table 19.2.2.2.4.1-1: NR ATG SA FR1 distance-based conditional handover test configurations

| Config | Description |
| --- | --- |
| 19.2.2.2-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.2.2-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.2.2-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.2.2.4.1-2

Table 19.2.2.2.4.1-2: Initial conditions for NR ATG SA FR1 distance-based conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 19.2.2.2.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 19.2.2.2.4.1-3 and 19.2.2.2.5-1 below.

Table 19.2.2.2.4.1-3: General test parameters NR ATG SA FR1 Inter-frequency distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) at T1 start |  |  | (0, 0, 3000) | Set by AT command |
| UE moving speed |  | km/h | (1200, 0, 0) | Set by AT command |
| referenceLocation1-r17.condEventD1-r17 |  | m | (-4600, 0, 0) | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | (14479, 0, 0) | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | 200 | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | 200 | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | not barred | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 16 |  |

19.2.2.2.4.2	Test procedure

The test scenario comprises of 2 NR carrier and two cells, Cell 1 and Cell 2, on each carrier respectively. General parameters and Cell-specific parameters for Cell 1 and Cell 2 are given in Table 19.2.2.2.4.1-3 and 19.2.2.2.5-1 respectively. Measurement gap (gap pattern #0) is configured in the test case.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

Time interval T1 needs to start immediately after the UE position and moving speed are configured via AT commands. During T1, the UE is configured to measure inter-frequency neighbour cell and Gap pattern ID gp0. The RRC message implying distance-based handover to Cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2. where

- TRRC = 10ms, is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

From start of T2, the Cell 2 becomes detectable and offset better than Cell 1 and after 9976ms of T2, location condition event condEventD1-r17 is satisfied. During T2, the UE performs measurement on Cell 2 and evaluates the execution condition, and starts handover procedure when execution condition is satisifed. The UE shall sent PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the start of T2. The Interruption length Tinterrupt shall also be verified in T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 19.2.2.2.5-1. Propagation conditions are set according to Annex C clause C.2.2. UE position and moving speed are configured on the UE via AT commands. Immediately after that, T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedbacks sent by the UE.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008).

4.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure distanced-based CHO execution condition and measurement gap pattern #0 for the UE.

5.	The UE shall transmit an RRCReconfigurationComplete message.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 19.2.2.2.5-1. T2 starts.

7.	If

- the UE transmits the PRACH preambles to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the beginning of time period T2,

and

- no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = Tinterrupt +k1 for test configuration 19.2.2.2-1

-	X = Tinterrupt for test configuration 19.2.2.2-2

-	X = 2∙Tinterrupt for test configuration 19.2.2.2-3.

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

9.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

19.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

TBD

19.2.2.2.5	Test requirements

Table 19.2.2.2.5-1 defines the primary level settings for all tests.

Table 19.2.2.2.5-1: Cell specific test parameters for NR ATG SA FR1 Intra-frequency distance-based conditional handover from FR1 to FR1

| Parameter |  | Test configuration | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2, 3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1, 2 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1, 2 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  | Config 1, 2, 3 | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  | Config 1, 2, 3 |  | OP.1 |  |  |  |
| SMTC Configuration |  | Config 1, 2, 3 |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1, 2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1, 3 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  | Config 1, 2, 3 |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP | Config 1, 2, 3 |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1, 2, 3 | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | Config 1, 2, 3 | dBm/ 15kHz | -98 |  |  |  |
| Note2 |  | Config 1, 2 | dBm/ SCS | -98 |  |  |  |
|  |  | Config 3 |  | -95 |  |  |  |
|  |  | Config 1, 2, 3 | dB | 8+TT | -3.3+TT | -Infinity | 2.36+TT |
|  |  | Config 1, 2, 3 | dB | 8+TT | 8+TT | -Infinity | 11+TT |
| SSB_RP |  | Config 1, 2 | dBm/ SCS | -90+TT | -90+TT | -Infinity | -87+TT |
|  |  | Config 3 |  | -87+TT | -87+TT | -Infinity | -84+TT |
| IoNote3 |  | Config 1, 2 | dBm/ 9.36MHz | -61.41+TT | -57.06+TT | -61.41+TT | -57.06+TT |
|  |  | Config 3 | dBm/ 38.16MHz | -55.31+TT | -50.96+TT | -55.31+TT | -50.96+TT |
| Propagation condition |  | Config 1, 2 | - | AWGN + 2412HzNote4 |  |  |  |
|  |  | Config 3 |  | AWGN + 5556HzNote5 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: 2412Hz is the maximum value of doppler with carrier frequency of 2170MHz. The specific doppler shift trajectory is up to test system’s design considering of BS location and UE GNSS emulation. Note 5: 5556Hz is the maximum value of doppler with carrier frequency of 5GHz. The specific doppler shift trajectory is up to test system’s design considering of BS location and UE GNSS emulation. |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 later than 9976 ms and less than 10048 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90 %.

NOTE:	The handover delay is defined in TS 38.133 [6] clause 6.1E.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay TRRC = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

At 9976 ms after start of T2,

distance to source cell reference location is (3000)2+(1200*1000/3600*14.976-(-4600))2  = 10050.2m, and D1-1 = 10000m

distance to target cell reference location is (3000)2+(1200*1000/3600*14.976-(14479))2  = 9949.08m, and D1-2 = 10000m

i.e. D1-1 and D1-2 conditions are fulfilled at start of T2 with >=50m location margin.

Tmeasure = max(600 + 200 ms, 9976 ms) = 9976 ms;

Tinterrupt = 62 ms; TCHO_execution = 10 ms.

This gives a total of 9976 ms + 62 ms + 10 ms = 10048 ms.

(3000)2+(1200*1000/3600*15-(-4600))2

(3000)2+(1200*1000/3600*15-(14479))2

19.4	Signalling characteristics for ATG

## << End of changes >>