Source:		Sporton

Title:	Discussion on test point selection for EVM equalizer spectral flatness CA in FR2

Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the EVM equalizer spectrum flatness test case is analyzed.

Discussion

The EVM equalizer spectrum flatness CA is a measure for the ripple inside the transmission bandwidth of an UE and is defined in clauses 6.4A.2.4 of TS 38.101-2 [1].

The minimum requirement is specified as follows:

#### 6.4A.2.4	EVM equalizer spectrum flatness

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the EVM equalizer spectrum flatness requirements are specified in clause 6.4.2.4 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

In LTE, EVM equalizer spectrum flatness is tested under extreme conditions. As there is no minimum conformance requirement for extreme conditions for NR FR2, EVM equalizer spectrum flatness should be tested with normal conditions.

Proposal 1: Define test environment as Normal for EVM equalizer spectrum flatness CA in FR2.

Test Subcarrier Spacing

In order to maximize the tested bandwidth the smallest SCS is selected as it provides the highest spectrum efficiency.

Proposal 2: Select smallest SCS for EVM equalizer spectrum flatness CA in FR2.

Test Frequencies

In LTE & 5G FR1, EVM equalizer spectrum flatness is tested across Low, Mid, and High frequency ranges under single carrier. Since there is currently no TP analysis for EVM equalizer spectrum flatness in CA scenarios, it is proposed to reference the TP analysis from the FR2 EVM CA test case, and reuse the same low and high frequency ranges for NR CA EVM equalizer spectrum flatness testing.

Proposal 3: Test EVM equalizer spectrum flatness CA for Low range and High range frequencies in FR2.

Test Channel Bandwidth

In LTE & 5G FR1, EVM equalizer spectrum flatness is tested across Lowest, 5 MHz and Highest channel bandwidths under single carrier. Since there is currently no TP analysis for EVM equalizer spectrum flatness in CA scenarios, it is proposed to reference the TP analysis from the FR2 EVM CA test case, and reuse the same Lowest and Highest channel bandwidths for NR CA EVM equalizer spectrum flatness testing.

Proposal 4: Test EVM equalizer spectrum flatness CA for Lowest and Highest channel bandwidth in FR2.

Test Modulations and waveforms

In LTE EVM equalizer spectrum flatness is tested with QPSK modulation at maximum output power. In NR for FR2 different requirements other modulations (clause 6.4A.2.4).

Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for DFT-s-OFDM and CP-OFDM both waveforms should be tested. Select QPSK as a modulation for test case 6.4A.2.4 analog to LTE.

Proposal 5: Test EVM equalizer spectrum flatness with CP-OFDM QPSK and DFT-s-OFDM QPSK for test case 6.4A.2.4.

RB allocation

In LTE, EVM equalizer spectrum flatness is tested with full allocation to test all equalizer coefficients. The same approach is followed for NR.

Proposal 6: Test EVM equalizer spectrum flatness with Outer Full allocation.

Conclusion

To sum up, the following selection of test points is proposed for EVM equalizer spectrum flatness CA in FR2:

Proposal 1: Define test environment as Normal for EVM equalizer spectrum flatness in FR2.

Proposal 2: Select smallest SCS for EVM equalizer spectrum flatness in FR2.

Proposal 3: Test EVM equalizer spectrum flatness CA for Low range and High range frequencies in FR2.

Proposal 4: Test EVM equalizer spectrum flatness CA for Lowest and Highest channel bandwidth in FR2.

Proposal 5: Test EVM equalizer spectrum flatness with CP-OFDM QPSK and DFT-s-OFDM QPSK for test case 6.4.2.4.

Proposal 6: Test EVM equalizer spectrum flatness with Outer Full allocation.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2 CA.

Test case 6.4A.2.4

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 1 | 2 | 8 |

- References

- R5-187589, Discussion on test point selection for EVM equalizer spectrum flatness in FR2, 3GPP TSG RAN5, December 2018

- R5-192568, Discussion on Test Point analysis for FR2 EVM CA test cases, 3gpp TSG RAN5, February 2019

- 3GPP TS 36.521-1 v18.8.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing