3GPP TSG-RAN WG5 Meeting #107	R5-252078

# Malta, MT, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3283 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to ATG Tx TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - ATG Tx TCs need to be updated according to RAN4 latest spec and some format issues are founded in ATG TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - ATG Tx TCs 6.2J.1, 6.2J.2, 6.3J.2 and 6.4J.1 minimum conformance requirement parts have been updated. - Editorial correction to the format of TC 6.5J.2.2 sub-clause headers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current ATG minimum conformance requirements will not be aligned with RAN4 latest spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2J.1, 6.2J.2, 6.3J.2, 6.4J.1, 6.5J.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Align with RAN4 spec TS 38.101-1 i90 version |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2J	Transmitter power for ATG

6.2J.1	UE maximum output power for ATG

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

- MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

6.2J.1.1	Test purpose

To verify that the error of the NR ATG UE maximum output power for the ATG UE does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2J.1.2	Test applicability

This test case applies to all types of NR ATG UE release 18 and forward.

6.2J.1.3	Minimum conformance requirements

For the ATG UE, the rated maximum output power is reported via UE capability maxOutputPowerATG-r18 at maximum modulation order reported by ATG UE and full PRB configurations within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). UE capability maxOutputPowerATG-r18 is an integer value in the range 23 to 40 dBm.

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the measured maximum output power Pmax,c,AC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,AC reported by the ATG UE.

For ATG UE with antenna array indicating the capability antennaArrayType-r18, the measured maximum output power Pmax,c,TABC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,TABC reported by the ATG UE.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2J.1.

<< Unchanged Parts Skipped >>

6.2J.2	Configured transmitted power

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

- MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.6.2J.2.1	Test purpose

To verify the measured NR ATG UE configured maximum output power PUMAX,f,c for the ATG UE is within the specified bounds.

6.2J.2.2	Test applicability

This test case applies to all types of NR ATG UE release 18 and forward.

6.2J.2.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, Prated,c,AC or Prated,c,TABC}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by ATG specific the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[6]; It’s noted that the actual PEMAX,c value is (9 + field value) in ATG cell, according to p-Max IE definition in TS 38.331 [6];

Prated,c,AC is the rated maximum output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability maxOutputPowerATG-r18 for ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18;

Prated,c,TABC is the rated maximum output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability maxOutputPowerATG-r18 for ATG UE with antenna array indicating the capability antennaArrayType-r18.

TREF and Teval are specified in Table 6.2J.2.3-0. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2J.2.3-0: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  – T(PCMAX_L,f,c)  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2J.2.3-1.

Table 6.2J.2.3-1: ATG PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 40 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -19 ≤ PCMAX,c < 8 | 7.0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2J.2.

<< Unchanged Parts Skipped >>

6.3J.2	Transmit OFF power for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT are FFS for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (equal to PC1).

- MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

- The referenced clause 6.3J.3 is FFS

6.3J.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3J.2.2	Test applicability

The requirements of this test apply in test case 6.3J.3 Transmit ON/OFF time mask for ATG to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.2.3	Minimum conformance requirements

The Transmit OFF power is defined as the mean power at each transmit antenna connector or each TAB connector in a duration of at least one sub-frame (1ms) excluding any transient periods. The Transmit OFF power shall not exceed the values specified in Table 6.3J.2.3-1.

Table 6.3J.2.3-1: Transmit OFF power for ATG

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.2 and 6.3.2.

<< Unchanged Parts Skipped >>

6.4J.1	Frequency error for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU and TT are FFS

Test procedure and Test requirements are FFS

6.4J.1.1	Test purpose

Same test purpose as in 6.4.1.1

6.4J.1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.4J.1.3	Minimum conformance requirements

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency per layer shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [6].

NOTE 1:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.1.

<< Unchanged Parts Skipped >>

6.5J.2.2	Spectrum emission mask for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

Test Requirements and Test procedure are FFS.

- MU and TT are FFS

6.5J.2.2.1	Test purpose

Same test purpose as in 6.5.2.1.1.

6.5J.2.2.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.5J.2.2.3	Minimum conformance requirements

If the actual transmission power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in clause 6.5.2.2 apply; if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB. For ATG UE, the requirements for spectrum emission mask are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

NOTE:	This scaling factor is only applicable to ATG airborne UE.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5J.2.2.

6.5J.2.2.4	Test description

Same test purpose as in 6.5.2.1.4

6.5J.2.2.4.1	Initial conditions

Same initial conditions as in 6.5.2.1.4.1

6.5J.2.2.4.2	Test procedure

FFS

6.5J.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.5J.2.1.5	Test requirement

FFS

<< END OF CHANGES >>