3GPP TSG-RAN WG5 Meeting #107	R5-252100

# Malta, MT, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3289 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to AMPR TC for n28 PC2 40MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n28_PC2_40MHz_CBW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AMPR TC for n28 PC2 and 40MHz needs to be update as per the new requirements from n28 PC2 40MHz WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following parts of Minimum conformance requirements of AMPR TC 6.2.3 for band n28 PC2 40MHz are updated: - 6.2.3.3.1 General - 6.2.3.3.13 A-MPR for NS_18 - 6.2.3.3.33 (new) A-MPR for NS_17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Align with RAN4 spec TS 38.101-1 j10 version |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. When IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n1213, n85 | 3, 5, 10, 15 |  | Clause 6.2.3.3.3212 |
|  |  | n13, n14 | 5, 10 |  |  |
| NS_07 | 6.5.2.3.3.4, 6.5.3.3.5.26 | n13 | 5,10 | Table 6.2.3.3.29-1 | Table 6.2.3.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 3, 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 3, 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 3, 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n8313 | 3, 5, 10 | Table 5.3.2-1 | 6.2.3.3.33 |
| NS_18 | 6.5.3.3.3.3 | n28, n8313, n109 | 3, 5 |  | Table 6.2.3.3.13-1, A1 Table 6.2.3.3.13-3, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 Table 6.2.3.3.13-3, A2 |
|  |  |  | 25, 30, 40 |  | Table 6.2.3.3.13-1, A3, A4, A5 Table 6.2.3.3.13-3, A3, A4, A5, A6 |
| NS_21 | 6.5.2.3.3.9, 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20, 25, 30, 35 | Table 5.3.2-1 | Clause 6.2.3.3.3111 |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 10, 15, 20, 25, 30, 35, 40, 50 | Table 6.2.3.3.17-1 Table 6.2.3.3.17-311 Table 6.2.3.3.17-5 | Table 6.2.3.3.17-2 Table 6.2.3.3.17-411 Table 6.2.3.3.17-6 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 Table 6.2.3.3.18-3 | Table 6.2.3.3.18-2 Table 6.2.3.3.18-4 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 and n98 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_62 | 6.5.3.3.3.28 | n54 | 5 |  | N/A |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz, 10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [6] absent is default for all NR bands. NOTE 9: Void. NOTE 10: FFS NOTE 11: Applicable only for power class 1 operation. NOTE 12: Applicable only for power class 1 operation on band n85. NOTE 13: 3 MHz channel bandwidth is not applicable. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n31 | NS_01 |  |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n54 | NS_01 | NS_62 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n72 | NS_01 |  |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n85 | NS_01 | NS_06 |  |  |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| n100 | NS_01 |  |  |  |  |  |  |  |
| n101 | NS_01 |  |  |  |  |  |  |  |
| n1042 | NS_01 |  |  |  |  |  |  |  |
| n105 | NS_01 |  |  |  |  |  |  |  |
| n106 | NS_01 |  |  |  |  |  |  |  |
| n109 | NS_01 | NS_18 |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

<< Unchanged Parts Skipped >>

6.2.3.3.13	A-MPR for NS_18

Table 6.2.3.3.13-0: Band n28, n83 and n109 25, 30, 40MHz A-MPR regions for NS_18 (power class 3)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~748 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~748 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

Table 6.2.3.3.13-1: A-MPR for NS_18 (power class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Table 6.2.3.3.13-2: A-MPR regions for NS_18 (power class 2)

| Channel Bandwidth, MHz | Frequency range of UL channel bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~748 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~748 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Table 6.2.3.3.13-3: A-MPR for NS_18 for 1Tx and 2Tx (power class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | 3.5 | 10 | 5 | 2 |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | 4 | 11 | 5 | 3 |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | 4 | 11 | 5.5 | 4 |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | 4 | 11 | 6 | 4.5 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | 4 | 11 | 6 | 4.5 |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | 5.5 | 12.5 | 7 | 4.5 |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | 5.5 | 12.5 | 7 | 5 |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | 5.5 | 12.5 | 7 | 5 |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | 5.5 | 12.5 | 8 | 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.13.

<< Unchanged Parts Skipped >>

6.2.3.3.32	A-MPR for NS_06

For power class 1 operation on band n85 A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.32.

6.2.3.3.33	A-MPR for NS_17

Table 6.2.3.3.33-1: A-MPR regions for NS_17 (power class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | ≥5.4 | A2 |

Table 6.2.3.3.33-2: A-MPR for NS_17 for 1Tx and 2Tx (power class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3 | ≤ 3.5 |
|  | QPSK | ≤ 3 | ≤ 4 |
|  | 16 QAM | ≤ 3.5 | ≤ 5 |
|  | 64 QAM | ≤ 4 | ≤ 5 |
|  | 256 QAM |  | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤6 |
|  | 16 QAM | ≤ 5 | ≤6 |
|  | 64 QAM | ≤ 5 | ≤6 |
|  | 256 QAM |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.33.

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

<< END OF CHANGES >>