3GPP TSG-RAN5 Meeting #107	R5-252112

# Malta, MT, May 19 - 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.918 | CR | 0007 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of typo in TR 38.918 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2025-4-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The SST value in the Test Procedure is not align with the configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SST value and corresponding comment to ensure the test procedure use the correct slice. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test procedure is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

A.2	5G NR /URSP Configuration and Application Mapping

A.2.1	5G NR /URSP Configuration

A.2.1.1	5G NR / URSP Configuration / Signalling

A.2.1.1.1	Definition

The provisioning of URSP rules from network to UE is one of the fundamental functional requirements for network slicing.

A.2.1.1.2	Test Purpose

To verify that UE could support the provisioning of URSP rules from network to UE over NAS messages.

A.2.1.1.3	Test Parameters

Void.

A.2.1.1.4	Test Description

A.2.1.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.1.1.4.2	Test Procedure

1.	UE is switched on.

2.	Steps 2 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed.

3.	The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Configured NSSAI.

4.	The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message.

5.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to configure URSP rules.

6.	The UE transmits an UL NAS TRANSPORT message including UE Policy Container and a MANAGE UE POLICY COMPLETE message is included.

7.	The SS transmits an RRC Release message.

Table A.2.1.1.4.2-1: REGISTRATION ACCEPT (step 3)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | '001'B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table A.2.1.1.4.2-2: DL NAS TRANSPORT (step 5)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | '0101'B | UE policy container |  |
| Payload container |  | See Table A.2.1.1.4.2-3 |  |  |

Table A.2.1.1.4.2-3: MANAGE UE POLICY COMMAND (step 5)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B |  | OS App Id type |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of 'OS App Id' in bytes |  |  |
| OS App Id |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.2.2	5G NR / Mapping Application to Network Slicing

A.2.2.1	5G NR / Mapping Application to Network Slicing / DNN

A.2.2.1.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.1.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of DNN.

A.2.2.1.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.1.4	Test Description

A.2.2.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.1.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.1.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.1.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B |  | Match-all type |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |

Table A.2.2.1.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

Table A.2.2.1.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.2.2.2	5G NR / Mapping Application to Network Slicing / APP ID

A.2.2.2.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.2.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of OS App Id.

A.2.2.2.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.2.4	Test Description

A.2.2.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.2.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching OS App Id value in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B |  | OS App Id type |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B |  | Match-all type |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |

Table A.2.2.2.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |

Table A.2.2.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |

A.2.2.3	5G NR / Mapping Application to Network Slicing / FQDN

A.2.2.3.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.3.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of destination FQDN.

A.2.2.3.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.3.4	Test Description

A.2.2.3.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.3.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.3.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching destination FQDN value in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.3.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10010001'B |  | Destination FQDN |
| Traffic descriptor component |  |  |  |  |
| destination FQDN length |  | Set to the actual length of ' destination FQDN value' in bytes |  |  |
| destination FQDN value |  | pc_Des_FQDN |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B |  | Match-all type |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |

Table A.2.2.3.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |

Table A.2.2.3.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |

A.2.2.4	5G NR / Mapping Application to Network Slicing / IP 3 Tuples

A.2.2.4.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.4.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of IP 3 Tuples.

A.2.2.4.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.4.4	Test Description

A.2.2.4.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.4.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.4.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching IP 3 tuples value in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.4.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '01010010'B |  | IP 3 tuple type |
| Traffic descriptor component |  |  |  |  |
| Information Bitmap |  | '00001101'B |  | IPv4 |
|  |  | '00001110'B |  | IPv6 |
| IP Address |  | pc_IP_Address |  |  |
| Protocol identifier/next header type |  | pc_Protocol_ID |  |  |
| Single remote port |  | pc_Sinlge_Remote_Port |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B |  | Match-all type |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |

Table A.2.2.4.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |

Table A.2.2.4.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |

A.2.2.5	5G NR / Mapping Application to Network Slicing / Connection Capabilities

A.2.2.5.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.5.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of Connection Capabilities.

A.2.2.5.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.5.4	Test Description

A.2.2.5.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.5.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.5.4.2-1.

2.	Using the Application Client Simulator, generate data traffic.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.5.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10010000'B | Connection capabilities type |  |
| Traffic descriptor component |  |  |  |  |
| Connection capabilities |  | '00001000'B | Internet |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |

Table A.2.2.5.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |

Table A.2.2.5.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |

A.2.2.6	5G NR / Mapping Application to Network Slicing / URSP Update

A.2.2.6.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.6.2	Test Purpose

To verify that UE could support the update of URSP rules and the mapping of applications to right network slicing based on the updated URSP rules.

A.2.2.6.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.6.4	Test Description

A.2.2.6.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.6.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.1.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

7.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

8.	The UE transmits an UL NAS TRANSPORT message including a MANAGE UE POLICY COMPLETE message.

9.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

10.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

11.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.6.4.2-1: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

Table A.2.2.6.4.2-2: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 10)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  | step 4 |
|  |  | 0x000003 |  | step 19 |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.2.2.6.4.2-3: MANAGE UE POLICY COMMAND (step 7)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000003 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.2.2.6.4.2-4: UL NAS Transport (step 9)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000003 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

A.3	5G NR / Service Performance Testing with Network Slicing

A.3.1	5G NR / Service Performance / Single Network Slicing

A.3.1.1	5G NR / Service Performance / Single Application with Single Network Slicing

A.3.1.1.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.1.1.2	Test Purpose

To measure the performance of single service in application layer of the 5G NR UE with single network slicing.

A.3.1.1.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for UL.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.1.1.4	Test Description

A.3.1.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.1.1.4.2	Test Procedure

1.	UE is switched on.

2.	Steps 2 to 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed.

3.	Using the Application Client Simulator, begin uplink data transfer. Wait for 15 seconds and then start to measure the throughput result and latency in transport layer. (This is iteration 1) Continue data transfer for 1 minutes.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput and latency across three iterations.

6.	Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed.

7.	UE is brought back to operation.

8.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the MANAGE UE POLICY COMMAND message is defined in Table A.3.1.1.4.2-1.

9.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 8.

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

13.	Wait for 15 seconds and then start to measure the throughput result and latency in [transport layer] in parallel (This is iteration 1). Continue data transfer for 1 minute.

14.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

15. 	Calculate and record the average application layer data throughput and latency across three iterations.

16.	Observe that the throughput in step 15 is higher than the throughput benchmark in step 5 and the latency in step 15 is lower than the latency benchmark in step 5.

Table A.3.1.1.4.2-1: MANAGE UE POLICY COMMAND (step 8)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.3.1.2	5G NR / Service Performance / Multiple Applications with Single Network Slicing

A.3.1.2.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.1.2.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with single network slicing.

A.3.1.2.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. For single APP A at step 5-8 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block. For APP A and APP B running simultaneously at step 9-11 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.1.2.4	Test Description

A.3.1.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.1.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the MANAGE UE POLICY COMMAND message is defined in Table A.3.1.1.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the simulated APP A, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in application layer (This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	Using the Application Client Simulator to simulate APP B, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

10.	Using the simulated APP B, begin uplink data transfer from UE to the Application Server Simulator.

11.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

12.	Observe that the throughput in step 11 is not lower than the throughput benchmark in step 8.

Table A.3.1.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000002 |  |  |

Table A.3.1.2.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |

Table A.3.1.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |

A.3.2	5G NR / Service Performance / Multiple Network Slicing

A.3.2.1	5G NR / Service Performance / Multiple Applications with Multiple Network Slicing

A.3.2.1.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.2.1.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with multiple network slicing.

A.3.2.1.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. For single APP A or APP B at step 8-15 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block. For APP A and APP B running simultaneously at step 16-19 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.2.1.4	Test Description

A.3.2.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.2.1.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  REGISTRATION ACCEPT message is defined in Table A.3.2.1.4.2-1 and MANAGE UE POLICY COMMAND message is defined in Table A.3.2.1.4.2-2.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching DNN value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP B, generate traffic matching DNN value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

6.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

7.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

8.	Using the simulated APP A, begin uplink data transfer from UE to the Application Server Simulator.

9.	Wait for 15 seconds and then start to measure the throughput result in application layer (This is iteration 1). Continue data transfer for 1 minute.

10.	Repeat step 9 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

11.	Calculate and record the average application layer data throughput across three iterations.

12.	Stop the data transfer with simulated APP A.

13.	Using the simulated APP B, begin uplink data transfer from UE to the Application Server Simulator.

14.	Wait for 15 seconds and then start to measure the latency in application layer (This is iteration 1). Continue data transfer for 1 minute.

15.	Repeat step 14 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

16.	Calculate and record the average application layer latency across three iterations.

17.	Restart the data traffic of APP A. APP A and APP B generate data traffic simultaneously.

18.	Repeat steps 9 to 11 to get the average application layer data throughput of the data traffic of simulated APP A

19.	Repeat steps 14 to 16 to get the average application layer latency of the data traffic of simulated APP B in parallel with step 18.

20.	Observe that the throughput in step 18 is not lower than the throughput benchmark in step 11 and the latency in step 19 is not higher than the latency benchmark in step 16.

Table A.3.2.1.4.2-1: REGISTRATION ACCEPT (step 1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | '001'B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | default S-NSSAI |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000002 |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | default S-NSSAI |  |
| Length of S-NSSAI contents |  | '00000100'B |  |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000002 |  |  |

Table A.3.2.1.4.2-2: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B | DNN Type |  |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B | DNN Type |  |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000002 |  |  |

Table A.3.2.1.4.2-3: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | eMBB |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

Table A.3.2.1.4.2-4: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.3.2.1.4.2-5: UL NAS Transport (step 6)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-2 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific_2nd |  |  |  |

Table A.3.2.1.4.2-6: PDU SESSION ESTABLISHMENT ACCEPT (step 7)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific_2nd |  |  |

A.3.3	5G NR / Service Performance / URSP Update

A.3.3.1	5G NR / Service Performance / Single Network Slicing / URSP update

A.3.3.1.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.1.2	Test Purpose

To measure the performance of single services in application layer of the 5G NR UE with the change of network slicing.

A.3.3.1.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for UL.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.1.4	Test Description

A.3.3.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.3.1.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.2.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP A, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in transport layer (This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator to simulate APP A, begin uplink data transfer from UE to the Application Server Simulator.

13.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

14.	Observe that the throughput in step 13 is not lower than the throughput benchmark in step 8.

Table A.3.3.1.4.2-1: UL NAS Transport (step 3, 10)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  | step 3 |  |
|  | 0x000003 |  | step 10 |  |

Table A.3.3.1.4.2-2: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 11)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  | step 4 |
|  |  | 0x000003 |  | step 11 |

Table A.3.3.1.4.2-3: MANAGE UE POLICY COMMAND (step 9)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000003 |  |  |

A.3.3.1A	5G NR / Service Performance / Single Network Slicing / URSP update / Different Resource Allocation

A.3.3.1A.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.1A.2	Test Purpose

To measure the performance of single services in application layer of the 5G NR UE with the different resource allocation of the same slice.

A.3.3.1A.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block at step 5-7. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 at step 9-13.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.1A.4	Test Description

A.3.3.1A.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.3.1A.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.2.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP A, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in transport layer (This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator to simulate APP A, begin uplink data transfer from UE to the Application Server Simulator.

13.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

14.	Observe that the throughput in step 13 is two times of the throughput benchmark in step 8.

Table A.3.3.1A.4.2-1: UL NAS Transport (step 3, 10)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | eMBB |  |  |
| SD | 0x000001 |  | step 3 |  |
|  | 0x000003 |  | step 10 |  |

Table A.3.3.1A.4.2-2: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 11)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  | step 4 |
|  |  | 0x000003 |  | step 11 |

Table A.3.3.1A.4.2-3: MANAGE UE POLICY COMMAND (step 9)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000003 |  |  |

A.3.3.2	5G NR / Service Performance / Multiple Network Slicing / URSP update

A.3.3.2.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.2.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with the change of network slicing.

A.3.3.2.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for UL.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.2.4	Test Description

A.3.3.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.3.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  REGISTRATION ACCEPT message is defined in Table A.3.2.1.4.2-1 and MANAGE UE POLICY COMMAND message is defined in Table A.3.2.2.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP B, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

6.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value are selected.

7.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

8.	Using the simulated APP A and APP B, begin uplink data transfer from UE to the Application Server Simulator.

9.	Wait for 15 seconds and then start to measure the throughput result of simulated APP A and latency result of simulated APP B in application layer (This is iteration 1). Continue data transfer for 1 minute.

10.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

11.	Calculate and record the average application layer data throughput of simulated APP A and latency  of simulated APP B across three iterations.

12.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

13.	Repeat step 2-4 with the message exception defined in Table A.3.2.2.4.2-5 and A.3.2.2.4.2-6

14.	Repeat step 5-7 with the message exception defined in Table A.3.2.2.4.2-5 and A.3.2.2.4.2-6

15.	Repeat step 8-11 to get the average application layer data throughput of simulated APP A and latency of simulated APP B in parallel.

16.	Observe that the throughput in step 15 is not lower than the throughput benchmark in step 11 and the latency in step 15 is not higher than the latency benchmark in step 11.

Table A.3.3.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000002 |  |  |

Table A.3.3.2.4.2-2: UL NAS Transport (step 3, step 6)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | eMBB | step 3 |  |
|  | '00000010'B | URLLC | step 6 |  |
| SD | 0x000001 |  |  |  |

Table A.3.3.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 7)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB | step 4 |
|  |  | '00000010'B | URLLC | step 7 |
| SD |  | 0x000001 |  |  |

Table A.3.3.2.4.2-4: MANAGE UE POLICY COMMAND (step 12)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000002 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |

Table A.3.3.2.4.2-5: UL NAS Transport (step 13, step 14)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | eMBB |  |  |
| SD | 0x000002 |  | step 13 |  |
|  | 0x000001 |  | step 14 |  |

Table A.3.3.2.4.2-6: PDU SESSION ESTABLISHMENT ACCEPT (step 13, step 14)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000002 |  | step 13 |
|  |  | 0x000001 |  | step 14 |

<End of modified section>