3GPP TSG-RAN5 Meeting #107	R5-252121

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1516 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SIB33 for IoT NTN new RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inplement proposal 4 of R5-252120. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ephemeris for IoT NTN newly added RRM multi-cells TCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cases can’t be finalised. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depend on R5-252120 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

#### 8.4.6.2.1	Ephemeris for elevation angle 30º

All NB-IoT NTN RRM test cases defined in in section 13 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 8.4.6.2.1-1a to 8.4.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites or both GSO and NGSO satellites, ephemeris values in Table 8.4.6.2.1-3a or 8.4.6.2.1-3d shall be used.

-	Tables 8.4.6.2-4a to 8.4.6.2-4d shall only be used if the test case under test is only applicable to GEO satellites.

Table 8.4.6.2.1-1a: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-1b: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 8393544277 |  |  |  |  |
| eccentricity-r17 |  | 0 |  |  |  |  |
| periapsis-r17 |  | 266532360 |  |  |  |  |
| longitude-r17 |  | 224998499 |  |  |  |  |
| inclination-r17 |  | 2125399 |  |  |  |  |
| meanAnomaly-r17 |  | 118496066 |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |

Table 8.4.6.2.1-1c: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-18) – NB-IoT NTN Neighbour cell Ephemeris Information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -25235934 |  |  |
| positionY-r17 | 20178903 |  |  |
| positionZ-r17 | 2814664 |  |  |
| velocityVX-r17 | 298 |  |  |
| velocityVY-r17 | -241 |  |  |
| velocityVZ-r17 | 4398 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.78 ms and Doppler equal to +0.11 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-1d: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-18) – NB-IoT NTN Neighbour cell Ephemeris Information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 137910436 |  |  |
| longitude-r17 | 105383807 |  |  |
| inclination-r17 | TBD |  |  |
| meanAnomaly-r17 | 32981819 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-2a: Void

Table 8.4.6.2.1-2b: Void

Table 8.4.6.2.1-3a: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 |  |  |
| positionY-r17 | 4895460 |  |  |
| positionZ-r17 | 1188458 |  |  |
| velocityVX-r17 | 8506 |  |  |
| velocityVY-r17 | -23314 |  |  |
| velocityVZ-r17 | 118253 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-3b: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |
| semiMajorAxis-r17 | 273626880 |  |  |  |
| eccentricity-r17 | 769300 |  |  |  |
| periapsis-r17 | 198830479 |  |  |  |
| longitude-r17 | 89834718 |  |  |  |
| inclination-r17 | 65473992 |  |  |  |
| meanAnomaly-r17 | 7822790 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |

Table 8.4.6.2.1-3c: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-18) – NB-IoT NTN Neighbour cell Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2470752 |  |  |
| positionY-r17 | 3858366 |  |  |
| positionZ-r17 | 3624634 |  |  |
| velocityVX-r17 | 31542 |  |  |
| velocityVY-r17 | -67516 |  |  |
| velocityVZ-r17 | 94597 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-3d: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-18) – NB-IoT NTN Neighbour cell Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 272189533 |  |  |
| eccentricity-r17 | 715030 |  |  |
| periapsis-r17 | 199909097 |  |  |
| longitude-r17 | 89257064 |  |  |
| inclination-r17 | 64358188 |  |  |
| meanAnomaly-r17 | 27947120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4a: SystemInformationBlockType31-NB – NB-IoT NTN Serving cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4b: SystemInformationBlockType31-NB – NB-IoT NTN Serving cell Ephemeris Information orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 200160368 |  |  |
| longitude-r17 | 24831613 |  |  |
| inclination-r17 | 5309231 |  |  |
| meanAnomaly-r17 | 66742914 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4c: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17) – NB-IoT NTN Neighbour cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4d: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17) – NB-IoT NTN Neighbour cell Ephemeris Information (orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

<<< END OF CHANGES 1>>>