3GPP TSG-RAN5 Meeting #107	R5-252136

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	5.3.15.9

Source:	Keysight Technologies, Anritsu

Title:	Discussion on MU and TT for ATG RF testing

Document for:	Discussion and Endorsement

# Introduction

The aim of this paper is to continue MU and TT analysis for Air-To-Ground (ATG) devices made in [1]. In particular, this paper will resubmit proposals made for OFF power test case, Out-of-band blocking test case and spurious test cases. In addition, a proposal will be made for the EVM test case.

# Discussion

## Background

Proposals 1 to 8 made in [2] were endorsed two meetings ago. From them, proposals 1 to 4 were generic while proposals 5 to 8 were test case specific. Therefore, to help understanding the proposals to be made in this document, new or resubmitted, it is worth to recover here those endorsed proposals that will condition them:

Proposal 1. In the dynamic range analysis of the test systems to support ATG, the discrete value of the nominal attenuation and the tolerance in the frequency response of the extra attenuators need to be considered. Proposal 2. If a test system requires extra attenuation to run ATG test cases, RAN5 shall consider the highest required attenuation to be added to all ATG test cases. That is, the same level of extra attenuation will be used for the MU and TT analysis of all the ATG test cases. Proposal 3. Create an AP to ATG UE manufacturers to clarify the expected implementation in terms of number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors. Proposal 4. While it is unclear the number of UL connectors for ATG, for requirements that apply to the sum of the mean power from all antenna connectors or all TAB connectors, limit MTSU definition for ATG assuming no more than 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, otherwise MTSU will be FFS.

## Test cases specific analysis

#### OFF power test case

- With all the background presented and following the endorsed proposals, we have analyzed the test requirement in our test systems and concluded that, for ATG UEs with maximum nominal power in the range {31dBm (PC1) < maxOutputPowerATG-r18 ≤ 40dBm}, MTSU and TT should be increased, and even certain relaxation would be required depending on the CBW to test compared to regular OFF power test case 6.3.2, otherwise, values from test case 6.3.2 can apply, assuming required attenuators remain connected to enable testing automation and test system protection of ATG devices.

| - MTSU proposal | - TT Proposal | - Relaxation Proposal |
| --- | --- | --- |
| For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): Same as 6.3.2 for each UE antenna/TAB connector Otherwise, for each UE antenna/TAB connector: f ≤ 3.0GHz ±2.15 dB, BW ≤ 40MHz ±2.35 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±2.45 dB, BW ≤ 40MHz ±2.55 dB, 40MHz < BW ≤ 80MHz ±2.85 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.65 dB, BW ≤ 20MHz ±2.75 dB, 20MHz < BW ≤ 80MHz ±2.85 dB, 80MHz < BW ≤ 100MHz | For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): Same as 6.3.2 for each UE antenna/TAB connector Otherwise, for each UE antenna/TAB connector: f ≤ 3.0GHz 2.15 dB, BW ≤ 40MHz 2.35 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 2.45 dB, BW ≤ 100MHz | For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): 0dB Otherwise: 3dB, BW ≤ 25MHz 6dB, 25MHz < BW ≤ 50MHz 9dB, 50MHz < BW ≤ 100MHz |

Table 2.2.1-1. MTSU, TT and relaxations proposal for ATG OFF power test case 6.3J.2.

- Proposal 1. For ATG OFF power test case 6.3J.2, define MTSU, TT and relaxation according to proposal in Table 2.2.1-1.

- So far, previous proposal has not been agreed due to reluctancy to accept adding relaxations to the test requirements in FR1, encouraging to find out alternatives to avoid it. However, for this particular test case, it must be noted that OFF power is measured as part of the Transmit ON/OFF time mask test case. In that testing scenario, the UE will need to set to transmit to its maximum power, therefore to protect the system it cannot be avoided the presence of extra attenuation unless the tester is natively designed to support maximum possible ATG UE power.

#### Out-of-band blocking test case

This is the test case requiring the highest power in DL, in particular, for CW signal, required power in Range 3 is -15dBm at DUT plane operating at frequencies from 1MHz up to 12.75GHz. This is quite remarkable high DL power compared to other test cases. For example, it is 15dB higher than the highest DL power required for ATG Maximum input level test 7.4J. Therefore, the presence of any extra attenuation would have a significant impact on the maximum power a test system can transmit at DUT plane.

After evaluating our test systems under all premises presented in section 2.1, we conclude that, for ATG UEs with maximum nominal power in the range {31dBm (PC1) < maxOutputPowerATG-r18 ≤ 40dBm}, on the need of certain relaxation to apply to the CW power. In particular 9dB.

In this type of test cases, the most important is the power difference between the interferer and the wanted signal so if CW signal power level is decreased by certain relaxation, the wanted signal power level should be decreased accordingly. However, the required wanted signal power level is P ≤ REFSENS +9dB. In no case, wanted signal should be set below REFSENS. Therefore, in those cases, either should not be tested for Range 3 or tested with less power reduction in the wanted signal. Given only highest channel BW will be tested for Range 3, in practice there will no deviation in the delta power between the interferer and the wanted signal vs the delta required, we propose to test them in the way captured in Table 2.2.2-1.

| - Relaxation Proposal - (CW) | - Relaxation Proposal - (Wanted Signal) |
| --- | --- |
| For Range 3 for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1): 9dB Otherwise: 0dB | For Range 3 for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1): 6dB, BW ≤ 10MHz 7dB, BW = 15MHz 9dB, 15MHz < BW ≤ 100MHz Otherwise: 0dB |

Table 2.2.2-1. Relaxations proposal for ATG Out-of-band blocking test case 7.6J.3, Range 3.

- Proposal 2. For ATG Out-of-band blocking test case 7.6J.3, Range 3, apply relaxations according to proposal in Table 2.2.2-1.

- So far, previous proposal has not been agreed due to reluctancy to accept adding relaxations to the test requirements in FR1, encouraging to find out alternatives to avoid it. However, for this particular test case, the UE is required to transmit close to its maximum power during the test and therefore to protect the system it cannot be avoided the presence of extra attenuation unless the tester is natively designed to support maximum possible ATG UE power.

- It has been even suggested the use of circulators but that is not an option for this particular test case given circulators are narrow banded but CW interferer in this test case will need to transmit from 1MHz up 12.75GHz while current ATG bands will be limited to operation from 1.71GHz to 5GHz.

#### Spurious Emissions test cases

We have been evaluating our test systems with all the considerations and endorsed proposals made in [2] and concluded that for spurious test cases for ATG, MU and TT can be leveraged from the equivalent non-ATG tests limiting, in case of Spurious Tx test, to ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors. Such a condition does not apply to Rx Spurious test case where requirements apply per connector.

Proposal 3. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, in ATG General Spurious test 6.5J.3.1, define MU and TT same as defined for General Spurious test 6.5.3.1.

Proposal 4. For ATG UEs, in ATG Rx Spurious test 7.9J, define MU and TT same as defined for Rx Spurious test 7.9.

#### Error Vector Magnitude test case

During last meeting, error vector magnitude for ATG test case 6.4J.2.1 was defined. However, MU and test procedure were left FFS given it is unclear to operate with devices with more than one antenna/TAB connector.

Minimum EVM requirements for ATG extracted from TS 38.101-1 [3], just refer to requirement defined in clause 6.4.2.1. In that section, given there is no clarification related to the UE antennas/TAB connectors, the only assumption that can be made is that requirement shall apply per UL transmitting antenna/TAB connector.

Observation 1. Minimum EVM requirements for ATG refer to general minimum EVM requirements where there is no clarification related to the UE antennas/TAB connectors. Therefore, the only assumption that can be made is that the requirements shall apply per UL transmitting antenna/TAB connector.

Based on previous observation and assumption, it is proposed to update test case procedure indicating that EVM measurement needs to be repeated per UL transmitting antenna/TAB connector while clarifying that when adjusting the UE power to set it close to minimum power, Pmin is defined in section 6.3J.1 and applies to the sum power at all UE antenna/TAB connectors.

Proposal 5. for ATG test case 6.4J.2.1, update test definition to apply the requirement per UL antenna/TAB connector.

Proposal 6. for ATG test case 6.4J.2.1, update test definition to clarify how to adjust UE UL power when it is required to be set close to Pmin, by referring to minimum power defined for ATG in section 6.3J.1 applicable to the sum power at all UE antenna/TAB connectors.

With test definition updated as per proposals and assumptions above, we have been evaluating the measurement uncertainty in our test systems and concluded in the MU and TT proposed in the following table, where TT is calculated using the same formula used for other EVM test cases and where there is no limitation or restriction regarding maxOutputPowerATG-r18 supported by the UE but where there is a restriction regarding the number of UE antenna/TAB connectors due to power split is assumed through all connectors. MU analysis for more than 4 UL antenna/TAB connectors is FFS.

| - MTSU proposal | - TT Proposal |
| --- | --- |
| For ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors: For up to 256QAM: f ≤ 6.0GHz, BW ≤ 100MHz PUL = Maximum Output Power PUSCH, PUCCH, PRACH: ±2 % at each UE antenna/TAB connector PUL = Minimum Output Power PUSCH, PUCCH, PRACH: ±3.0 % at each UE antenna/TAB connector Absolute Uplink power measurement same as 6.3J.1. Relative Uplink power measurement same as 6.3J.4.2. | For ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors: For up to 64QAM 0% For 256QAM f ≤ 6.0GHz, BW ≤ 100MHz 0.6%, PUL = Maximum Output Power, 1.1%, PUL = Minimum Output Power |

Table 2.2.4-1. MTSU, TT and relaxations proposal for ATG EVM test case 6.4J.2.1.

- Proposal 7. For ATG EVM test case 6.4J.2.1, define MU and TT according to proposal in Table 2.2.4-1.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Minimum EVM requirements for ATG refer to general minimum EVM requirements where there is no clarification related to the UE antennas/TAB connectors. Therefore, the only assumption that can be made is that the requirements shall apply per UL transmitting antenna/TAB connector.

Proposal 1. For ATG OFF power test case 6.3J.2, define MTSU, TT and relaxation according to proposal in Table 2.2.1-1.

Proposal 2. For ATG Out-of-band blocking test case 7.6J.3, Range 3, apply relaxations according to proposal in Table 2.2.2-1.

Proposal 3. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, in ATG General Spurious test 6.5J.3.1, define MU and TT same as defined for General Spurious test 6.5.3.1.

Proposal 4. For ATG UEs, in ATG Rx Spurious test 7.9J, define MU and TT same as defined for Rx Spurious test 7.9.

Proposal 5. for ATG test case 6.4J.2.1, update test definition to apply the requirement per UL antenna/TAB connector.

Proposal 6. for ATG test case 6.4J.2.1, update test definition to clarify how to adjust UE UL power when it is required to be set close to Pmin, by referring to minimum power defined for ATG in section 6.3J.1 applicable to the sum power at all UE antenna/TAB connectors.

Proposal 7. For ATG EVM test case 6.4J.2.1, define MU and TT according to proposal in Table 2.2.4-1.

# References

- R5-251676, “Discussion on MU and TT for ATG RF testing”, Keysight, Anritsu, RAN5#106, Feb’25.

- R5-246445, “Discussion on MU and TT for ATG RF testing”, Keysight, RAN5#105, Nov’24.

- TS 38.101-1, “User Equipment (UE) Radio transmission and reception; Part 1: Range 1 Standalone”, v18.9.0 (2025-03)

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v18.6.0 (2025-03)

- R5-195141, “MU for sum of power”, Anritsu, RAN5#83, May’19.