3GPP TSG-RAN5 Meeting #107	R5-252137

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3292 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ATG - Tx Tests - General updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress has been made on MU and TT analysis in R5-252136. ATG test cases defined so far need to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test definition updated for the following ATG Tx tests: -6.3J.2 -6.4J.2.1 -6.5J.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J.2, 6.4J.2.1, 6.5J.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 6.3J.1	Minimum output power for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

## <<< Skip unchanged sections >>>

### 6.3J.2	Transmit OFF power for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referenced clause 6.3J.3 is FFS

6.3J.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3J.2.2	Test applicability

The requirements of this test apply in test case 6.3J.3 Transmit ON/OFF time mask for ATG to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.2.3	Minimum conformance requirements

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The Transmit OFF power is defined as the mean power at each transmit antenna connector or each TAB connector in a duration of at least one sub-frame (1ms) excluding any transient periods. The Transmit OFF power shall not exceed the values specified in Table 6.3J.2.3-1.

Table 6.3J.2.3-1: Transmit OFF power for ATG

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.2 and 6.3.2.

6.3J.2.4	Test description

This test is covered by clause 6.3J.3 Transmit ON/OFF time mask for ATG.

6.3J.2.5	Test requirement

The requirement for the Transmit OFF power for ATG is the mean power at each antenna connector (for ATG UE with omni-directional antennas) or at each TAB connector (for ATG UE with antenna array) and shall not exceed the values specified in Table 6.3J.2.5-1.

Table 6.3J.2.5-1: Transmit OFF power for ATG

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+ R+TT | 28.815 |
| 35 | -50+ R+TT | 33.855 |
| 40 | -50+ R+TT | 38.895 |
| 45 | -50+ R+TT | 43.575 |
| 50 | -50+ R+TT | 48.615 |
| 60 | -50+ R+TT | 58.35 |
| 70 | -50+ R+TT | 68.07 |
| 80 | -50+ R+TT | 78.15 |
| 90 | -50+ R+TT | 88.23 |
| 100 | -50+ R+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3J.2.5-2 |  |  |

Table 6.3J.2.5-2: Test Tolerance (Transmit OFF power for ATG)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 2.15dB | 2.45dB |
| 40MHz < BW ≤ 100MHz | 2.35dB | 2.45dB |

Table 6.3J.2.5-3: Relaxation required (R)

| BW ≤ 25MHz | 25MHz < BW ≤ 50MHz | 50MHz < BW ≤ 100MHz | Condition |
| --- | --- | --- | --- |
| 3dB | 6dB | 9dB | For ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1). Otherwise, 0dB relaxation required. |

## <<< Skip unchanged sections >>>

#### 6.4J.2.1	Error Vector Magnitude for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

- Test procedure and even minimum requirements are FFS in case of devices with more than one antenna/TAB connector.

6.4J.4.2.1.1	Test purpose

Same test purpose as in 6.4.2.1.1.

6.4J.4.2.1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional.only applicable if indicating support.

6.4J.4.2.1.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.1.3 shall apply for ATG UE except for the  requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.4.2.1.4	Test description

Same test description as in clause 6.4.2.1.4 with the following exceptions:

- Pmin is defined in clause 6.3J.1.

- In steps 1.4 and 2.5 where Uplink power control window is used, power control shall be applied to the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

- In steps 1.3, 1.5, 2.4, 2.6 and 3.7 where EVM is measured, measure the EVM and power for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array). For antenna connector or TAB connector with power smaller than Pmin, the EVM measurement is discarded.

6.4J.4.2.1.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin, shall not exceed the values in Table 6.4J.4.2.1.5-1.

The PUSCH, derived in Annex E.4.6.2, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin, shall not exceed the values in Table 6.4J.4.2.1.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4J.4.2.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 + TT |
| QPSK | % | 17.5 + TT |
| 16 QAM | % | 12.5 + TT |
| 64 QAM | % | 8 + TT |
| 256 QAM | % | 3.5 + TT |
| Note 1: TT is defined in Table 6.4.2.1.5-2. |  |  |

Table 6.4J.4.2.1.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | FFS |
| QPSK | % | FFS |
| 16 QAM | % | FFS |
| 64 QAM | % | FFS |
| 256 QAM | % | FFS |

The PUCCH EVM derived in Annex E.5.9.2 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin shall not exceed 17.5 %.

The PRACH EVM derived in Annex E.6.9.2 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin shall not exceed 17.5%.

## <<< Skip unchanged sections >>>

#### 6.5J.3.1	General spurious emissions for ATG

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

6.5.3J.1.1	Test purpose

To verify that NR ATG UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5.3J.1.2	Test applicability

This test case applies to all types of NR ATG UE release 18 and forward.

6.5.3J.1.3	Minimum conformance requirements

For ATG UE, the minimum conformance requirements shall be the same as in clause 6.5.3.1.3. For ATG UE, the requirements for Spurious emissions are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5J.3

6.5.3J.1.4	Test description

6.5.3J.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.5.3J.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3J.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | OuterFull |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.3J.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5.3J.1.4.3 with no exceptions.

6.5.3J.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.3J.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the power of the transmitted signal at each antenna connector with a measurement filter of bandwidths according to Table 6.5.3J.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3J.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5.3J.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.5.3J.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5.3J.1.5-1.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3J.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3J.1.5-1 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5.3J.1.5-1.

Table 6.5.3J.1.5-1: General spurious emissions test requirements for ATG UG

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 2 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

## <<< END OF CHANGES >>>