3GPP TSG-RAN5 Meeting #107	R5-252144

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3299 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction for combo CA_n41A-n77A for PC2 and PC1.5 minimum and test requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In test configuration table, test IDs 3, 4 and 5 cover the cross-band isolation. However, in test requirements tables, they are incorrectly referred using different test IDs. For PC1.5, format of cross band isolation table for PC1.5 is incorrect, with table title embedded in the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Numbered as Test IDs 3, 4 and 5 the cross-band isolation Test IDs in Table 7.3A.1_1.5-1a and Table 7.3A.1_1.5-1b. Reformated Table 7.3A.0.6-3b. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.6, 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 7.3A.0.6	Reference sensitivity exceptions due to cross band isolation for CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part which belongs to NR band of the same NR CA configuration due to cross band isolation issues. The reference sensitivity degradation for the victim band due to cross band isolation is specified only for the specific uplink and downlink test points specified in Table 7.3A.0.6-1 for either PC3 and PC2 NR CA from a PC3 aggressor NR UL band, and for PC2 NR CA, in Table 7.3A.0.6-3a from a PC2 aggressor NR UL band, and in Table 7.3A.0.6-3b from a PC1.5 aggressor NR single band uplink.

In Tables 7.3A.0.6-1 the following terminology is used to define the source of cross-band isolation interference:

-	"ACLR1" indicates that the first adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

-	"ACLR2" indicates that the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

">ACLR2" indicates that neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

Table 7.3A.0.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | n3 | 1922.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n1 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | ACLR1 |
| n1 | n41 | 1955 | 50 | 15 | 128 (RBstart=142) | 2501 | 10 | 6.1 | >ACLR2 |
| n1 | n41 | 1970 | 20 | 15 | 100 (RBstart=6) | 2546 | 100 | 0.7 | >ACLR2 |
| n3 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2501 | 10 | 0.7 | >ACLR2 |
| n3 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2546 | 100 | 0.7 | >ACLR2 |
| n30 | n66 | 2310 | 10 | 15 | 20 (RBstart=0) | 2197.5 | 5 | 8.3 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | >ACLR2 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 18.1 | >ACLR2 |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | >ACLR2 |
| n411 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 10.5 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.5 | ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 10 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n78 | n71 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 2 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 2.6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 2.6 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.6-2: Void

Table 7.3A.0.6-3: Void

Table 7.3A.0.6-3a: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 2.3 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 10.5 | >ACLR2 |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 3.1 | >ACLR2 |
| n77 | n2 | 3305 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.0 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 6.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 6.5 | >ACLR2 |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 1.0 | >ACLR2 |
| n79 | n41 | 4450 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 3.5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 5 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 5.6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 5.6 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.6-3b: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 13.3 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 9.0 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 9.0 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.A.

## <<< Skip unchanged sections >>>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1 for CA configuration with single uplink carrier indicating PC2 in TS 38.508-2 table A.4.3.2A.4.1-3 but without note 4 applied in table 5.5A.3.1-1.

For inter-band carrier aggregation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier with note 4 applied in table 5.5A.3.1-1 and UE indicating power class 2 in TS 38.508-2 Table A.4.3.2A.4.1-3, and Table 7.3A.1_1.5-1b for PC1.5 2UL CA configuration and CA configuration with single uplink carrier indicating power class 1.5 in TS 38.508-2 table A.4.3.2A.4.1-3.

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter-band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 9, Note 10, Note 11) |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | 1 | n1 | 5 | REF_victim +23 |
|  |  | n3 | 5 | REF_aggressor |
|  | 2 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +3 |
|  | 3 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +19.7 |
| CA_n1A-n8A | 1 | n1 | 5 | REF_victim +6 |
|  |  | n8 | 5 | REF_aggressor |
| CA_n1A-n28A | 1 | n1 | 5 | REF_victim +10.2 |
|  |  | n28 | 5 | REF_aggressor |
|  | 2 | n1 | 50 | REF_victim +1.1 |
|  |  | n28 | 5 | REF_aggressor |
| CA_n1A-n41A | 1 | n1 | 50 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +6.1 |
|  | 2 | n1 | 20 | REF_aggressor |
|  |  | n41 | 100 | REF_victim +0.7 |
|  | 3 | n1 | 5 | REF_victim +18.1 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n1A-n77A | 1 | n1 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +27.1 |
|  | 2 | n1 | 20 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n1 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.9 |
| CA_n1A-n78A | 1 | n1 | 5 | REF_victim +8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n2A-n48A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 10 | REF_victim +1.9 |
|  | 2 | n2 | 5 | REF_victim +12 |
|  |  | n48 | 20 | REF_aggressor |
| CA_n2A-n66A | 1 | n2 | 5 | REF_victim +20 |
|  |  | n66 | 5 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +4 |
| CA_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n2 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n2 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n2 | 5 | REF_victim +6.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 20 | REF_victim +3.7 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +26 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n8A | 1 | n3 | 10 | REF_aggressor |
|  |  | n8 | 5 | REF_victim +8 |
|  | 2 | n3 | 10 | REF_victim +6.4 |
|  |  | n8 | 5 | REF_aggressor |
| CA_n3A-n41A | 1 | n3 | 5 | REF_victim +8.2 |
|  |  | n41 | 10 | REF_aggressor |
|  | 2 | n3 | 5 | REF_victim +0.6 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n3 | 5 | REF_victim +26.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +8.0 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n78A | 1 | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
|  | 4 | n3 | 5 | REF_victim +26 |
|  |  | n78 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n5A-n66A | 1 | n5 | 5 | REF_victim +30 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.5 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +1.4 |
|  | 3 | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 4 | n5 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 5 | n5 | 20 | REF_victim +2.7 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n5 | 5 | REF_victim +5.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +8.3 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n5 | 5 | REF_victim +5.5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n5A-n78A | 1 | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.5 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n5 | 5 | REF_victim +8.3 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n7A-n78A | 1 | n7 | 5 | REF_victim +4.5 |
|  |  | n78 | 100 | REF_aggressor |
| CA_n8A-n78A | 1 | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.8 |
|  | 2 | n8 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n8 | 5 | REF_victim +8.3 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n14A-n77A | 1 | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 2 | n14 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 3 | n14 | 5 | REF_victim + 5.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 4 | n14 | 5 | REF_victim + 31 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n14 | 10 | REF_victim + 28 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n20A-n78A | 1 | n20 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.8 |
|  | 2 | n20 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n20 | 5 | REF_victim + 11 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n25A-n66A | 1 | n66 | 5 | REF_aggressor |
|  |  | n25 | 5 | REF_victim +20 |
|  | 2 | n66 | 5 | REF_victim +23 |
|  |  | n25 | 5 | REF_aggressor |
|  | 3 | n66 | 5 | REF_victim +4 |
|  |  | n25 | 5 | REF_aggressor |
| CA_n25A-n77A | 1 | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n25 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n25 | 5 | REF_victim +6.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n25 | 40 | REF_victim +1.1 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n25 | 5 | REF_victim +26 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n25 | 5 | REF_victim +8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n25 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n25A-n78A | 1 | n25 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n25 | 10 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n25 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
|  | 4 | n25 | 5 | REF_victim +26 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n26A-n66A | 1 | n26 | 5 | REF_victim + 30 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n26A-n70A | 1 | n26 | 5 | REF_victim + 30 |
|  |  | n70 | 5 | REF_aggressor |
| CA_n28A-n40A | 1 | n28 | 5 | REF_victim +37.8 |
|  |  | n40 | 5 | REF_aggressor |
|  | 2 | n28 | 20 | REF_victim +30.3 |
|  |  | n40 | 20 | REF_aggressor |
| CA_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 2 | n28 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 3 | n28 | 5 | REF_victim +0.7 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A-n78A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.4 |
|  | 2 | n28 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +0.7 |
|  | 3 | n28 | 5 | REF_victim + 31 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n29A-n71A | 1 | n29 | 5 | REF_victim + 17.5 |
|  |  | n71 | 20 | REF_aggressor |
| CA_n30A-n66A | 1 | n30 | 10 | REF_aggressor |
|  |  | n66 | 15 | REF_victim + 8.3 |
| CA_n30A-n77A | 1 | n30 | 5 | REF_victim + 10.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n30 | 10 | REF_victim + 8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n30 | 5 | REF_victim + 8 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n39A-n41A | 1 | n39 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +8.1 |
|  | 2 | n39 | 5 | REF_aggressor |
|  |  | n41 | 100 | REF_victim +3.3 |
|  | 3 | n39 | 5 | REF_victim +4.3 |
|  |  | n41 | 10 | REF_aggressor |
|  | 4 | n39 | 40 | REF_victim + 0.8 |
|  |  | n41 | 10 | REF_aggressor |
|  | 5 | n39 | 40 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +3.3 |
|  | 6 | n39 | 5 | REF_victim +1.6 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n40A-n77A | 1 | n40 | 10 | REF_victim +10.4 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n40 | 100 | REF_victim +0.9 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n40 | 10 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 4 | n40 | 100 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n41A-n66A | 1 | n41 | 100 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +10.5 |
| CA_n41A-n71A | 1 | n41 | 10 | REF_victim + 10.8 |
|  |  | n71 | 5 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim + 1.4 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n41A-n77A | 1 | n41 | 10 | REF_victim +10.4 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim +6.3 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.3 |
|  | 4 | n41 | 10 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n48A-n66A | 1 | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim + 5.0 |
|  | 2 | n48 | 5 | REF_victim +27.1 |
|  |  | n66 | 5 | REF_aggressor |
|  | 3 | n48 | 100 | REF_victim + 13.8 |
|  |  | n66 | 40 | REF_aggressor |
|  | 4 | n48 | 5 | REF_victim + 1.9 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n48A-n70A | 1 | n48 | 10 | REF_aggressor |
|  |  | n70 | 15 MHz UL /25 MHz DL | REF_victim +26 |
| CA_n66A-n71A | 1 | n66 | 5 | REF_victim +5 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n66A-n77A | 1 | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n66 | 20 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n66 | 5 | REF_victim +31 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n66A-n78A | 1 | n66 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n66 | 20 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n66 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
| CA_n70A-n71A | 1 | n70 | 25 | REF_victim +4.1 |
|  |  | n71 | 5 | REF_aggressor |
|  | 2 | n70 | 5 | REF_victim +9.9 |
|  |  | n71 | 5 | REF_aggressor |
|  | 3 | n70 | 5 | REF_victim +5 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n71A-n77A | 1 | n71 | 5 | REF_victim +5.5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n71A-n78A | 1 | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.4 |
|  | 2 | n71 | 5 | REF_victim +5.5 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +2 |
|  | 2 | n78 | 10 | REF_victim +2.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +2.6 |
|  |  | n79 | 100 | REF_aggressor |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 10: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 11: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter-band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 8, Note 9, Note 10) |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | 1 | n1 | 5 | REF_victim +17.8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n2A-n77A | 4 | n2 | 5 | REF_victim +9.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 20 | REF_victim +6.7 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +32.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +19.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +20.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n2 | 5 | REF_victim +1.0 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n3A-n41A | 1 | n3 | 5 | REF_victim +18.4 |
|  |  | n41 | 10 | REF_aggressor |
|  | 2 | n3 | 5 | REF_victim +2.3 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n3A-n77A | 4 | n3 | 5 | REF_victim +31.9 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n3 | 5 | REF_victim +8.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n3 | 40 | REF_victim +1.0 |
|  |  | n77 | 40 | REF_aggressor |
| CA_n3A-n78A | 6 | n3 | 5 | REF_victim + 8.1 |
|  |  | n78 | 5 | REF_aggressor |
|  | 7 | n3 | 40 | REF_victim + 1.0 |
|  |  | n78 | 40 | REF_aggressor |
| CA_n5A-n77A | 5 | n5 | 20 | REF_victim +4.3 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n5 | 5 | REF_victim +8.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +18.6 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n14A_n77A | 4 | n14 | 5 | REF_victim + 34 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n14 | 10 | REF_victim + 31 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A_n77A | 3 | n28 | 5 | REF_victim +19.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 4 | n28 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n28 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n28 | 5 | REF_victim +31 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A_n78A | 3 | n28 | 5 | REF_victim + 31 |
|  |  | n78 | 10 | REF_aggressor |
|  | 4 | n28 | 30 | REF_victim + 11.7 |
|  |  | n78 | 30 | REF_aggressor |
| CA_n30A-n77A | 1 | n30 | 5 | REF_victim + 13.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n30 | 10 | REF_victim + 10.6 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n30 | 5 | REF_victim + 17.6 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n41A-n77A | 1 | n41 | 10 | REF_victim +13.2 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim +8.8 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.5 |
|  | 4 | n41 | 10 | REF_victim+6.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim+6.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n41A-n79A | 1 | n41 | 100 | REF_aggressor |
|  |  | n79 | 10 | REF_victim +3.1 |
|  | 2 | n41 | 10 | REF_victim +3.5 |
|  |  | n79 | 100 | REF_aggressor |
| CA_n66A-n77A | 4 | n66 | 5 | REF_victim +34.33 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +11.27 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +5 |
|  | 2 | n78 | 10 | REF_victim +5.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +5.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 4 | n78 | 100 | REF_aggressor |
|  |  | n79 | 100 | REF_victim +5 |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 9: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 10: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

Table 7.3A.1_1.5-1b: Reference sensitivity requirement for inter band CA with PC1.5 single UL carrier or PC1.5 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 3, Note 4, Note 5) |
| --- | --- | --- | --- | --- |
| CA_n2A-n77A | 6 | n2 | 5 | REF_victim +35.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +26.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +28.0 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n77A | 8 | n3 | 5 | REF_victim +10.7 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n5A-n77A | 7 | n5 | 5 | REF_victim +25.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n5 | 5 | REF_victim +24.3 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n25A-n77A | 6 | n25 | 5 | REF_victim +37.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n25 | 5 | REF_victim +29.8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n25 | 5 | REF_victim +25.8 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A_n77A | 7 | n28 | 5 | REF_victim +37 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n41A-n77A | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +13.3 |
|  | 4 | n41 | 10 | REF_victim +9.0 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim +9.0 |
|  |  | n77 | 100 | REF_aggressor |
|  | 10 | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +14.5 |
|  | 11 | n41 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +5.5 |
|  | 12 | n41 | 10 | REF_victim +17.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 13 | n41 | 100 | REF_victim +7.9 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n61A-n77A | 4 | n61 | 5 | REF_victim +40.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n61 | 5 | REF_victim +26.0 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n71A-n77A | 2 | n71 | 5 | REF_victim +16.0 |
|  |  | n77 | 10 | REF_aggressor |

Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2). Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 4: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 5: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports.

## <<< END OF CHANGES >>>