3GPP TSG-RAN5 Meeting #107	R5-252264

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	5.3.21.9

Source: 	Apple

Title: 	Impact of beam correspondence test method without UE beam lock

Document for:	Discussion and Endorsement

# Introduction

There have been several discussions in RAN5 meetings regarding the RACH configuration for the UE Conformance testing for FR2 beam correspondence during initial access and RRC_Inactive.

The goal is to consider the beam lock function during initial access for beam correspondence test.

This paper provides Apple’s view and highlights the disadvantages associated to the ‘test method without UE beam lock function’ for beam correspondence test during initial access and RRC_Inactive.

# Discussion

## Impact of test method without UE Beam Lock

UE beam lock function is needed to ensure the UE preamble transmission are from UE optimal Tx beam. The test method without considering the UE beam lock function can have following drawbacks for beam correspondence test during initial access and RRC_Inactive:

- Beam correspondence test without UE beam lock function violates the basic assumption [1] made by RAN4.

- Without the UE beam lock function, the UE can switch between fine and course beam. This can cause the fluctuation in PRACH Tx power measurements results.

- Without the UE beam lock function, the UE may attempt to change the beam during the measurement period. Consequently, measurement samples may be obtained from sub-optimal and optimal beams. This may lead to inaccuracies in PRACH Tx power measurement results [2].

- The absence of the UE beam lock function may also lead to inaccurate measurement results if the TE collection of several UE RACH preambles is performed for a duration exceeding the measurement period. TE is unable to distinguish preambles from sub-optimal and optimal beams, potentially enabling compromised UE to satisfy the test requirement as depicted in Figure 1.

Figure 1 : PRACH Tx Power at TE

Observation 1: There are several disadvantages of defining the test method without using UE beam lock:

- This violates the fundamental assumption [1] made by RAN4 when deciding the minimum requirement.

- It may lead to unstable test results.

- This approach may result in inaccurate PRACH TX power measurement results due to preambles from the sub-optimal beam during measurement period.

- TE is unable to distinguish between preambles from sub-optimal and optimal beams, potentially allowing compromised UE to meet the test requirement.

Based on the above observations, we re-iterate the proposals from [3]:

Proposal 1: To only allow the UE beam lock function (UBF) to ensure reliable and repeatable measurement results for beam correspondence test during initial access and RRC_Inactive.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: There are several disadvantages of defining the test method without using UE beam lock:

- This violates the fundamental assumption [1] made by RAN4 when deciding the minimum requirement.

- It may lead to unstable test results.

- This approach may result in inaccurate PRACH TX power measurement results due to preambles from the sub-optimal beam during measurement period.

- TE is unable to distinguish between preambles from sub-optimal and optimal beams, potentially allowing compromised UE to meet the test requirement.

Based on the above observations, we re-iterate the proposals from [3]:

Proposal 1: To only allow the UE beam lock function (UBF) to ensure reliable and repeatable measurement results for beam correspondence test during initial access and RRC_Inactive.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- R5-247075,“Discussion on FR2 beam correspondence for initial access”, RAN5#105, Nov 18 – Nov 22, 2024

- R5-244945, “Discussion on FR2 beam correspondence during initial access”, RAN4#104, Aug 19 – Aug 23, 2024