3GPP TSG-RAN5 Meeting #107	R5-252314

# St Julian, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1152 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 256QAM requirements in 6.2A.2.0.6 MPR PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 6.2A.2.0.6 missing 256QAM requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.2A.2.0.6 Maximum output power reduction for power class 5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.0.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.0	Minimum conformance requirements

< Start of Change >

##### 6.2A.2.0.6	Maximum output power reduction for power class 5

For power class 5, MPR (except 256 QAM) specified in sub-clause 6.2A.2.0.4 applies for intra-band contiguous UL CA and intra-band non-contiguous UL CA.

For 256 QAM, MPR specified in sub-clause 6.2A.2.0.3 applies for intra-band contiguous UL CA and intra-band non-contiguous UL CA.

For inter-band carrier aggregation with uplink assigned to two NR bands, MPR for each configured UL band in the UL CA band combination is:

MPRinter-band_CA = max(MPRSingleBand, MPRPA-PA)

Where:

MPRSingleBand is the MPR specified in clause 6.2.2.3.5 for the allocation and modulation type in that band

MPRPA-PA is the maximum of the MPR values specified per band combination in Table 6.2A.2.0.6-1 and applies only when both bands have non-zero UL RB allocations, 0 dB otherwise.

Table 6.2A.2.0.6-1: MPRPA-PA for Inter-band CA in FR2 for PC5

| NR CA Band | Value (dB) | Condition |
| --- | --- | --- |
| CA_n257A-n259A | Max(0, 6 - 10*log10(Max(1.0, LRB,min*12*SCS/1e6))) | LRB,min = Min (LRB,n257 , LRB,n259 ), where LRB,n is the number of non-zero power UL RBs in band ‘n’ |

< End of Change >