3GPP TSG-RAN5 Meeting #107	R5-252330

# St Julian’s, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0586 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability for new further NR coverage enhancement test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add applicability for new Further NR coverage enhancements test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case applicability and selection expressions for 7.1.1.1.23 Random access procedure / Msg1 repetition / Contention-free 7.1.1.3.22 UE power headroom reporting / delta power class reporting 7.1.1.3.23 UE power headroom reporting / PCMAX for assumed PUSCH reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for Further NR coverage enhancements remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.1-2a & Table 4.2-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... 38.508-2 CR ...0815 TS/TR ... 38.523-1 CR ... 4978, 4979, 4980 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 4.1-2a: Applicability of Protocol conformance Layer 2 test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability Condition | Applicability Comment |
| --- | --- | --- | --- | --- |
| 7 | Layer 2 |  |  |  |
| 7.1 | NR Layer 2 |  |  |  |
| 7.1.1 | MAC |  |  |  |
| 7.1.1.1 | Random Access Procedures |  |  |  |
| 7.1.1.1.1 | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.1a | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.2 | Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.3 | Random access procedure / Successful / SI request | Rel-15 | C21 | UEs supporting 5G Core |
| 7.1.1.1.4 | Random access procedure / Successful / Beam Failure / Preamble selected by MAC itself / non-Contention Free RACH procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.5 | Random access procedure / Successful / Supplementary Uplink | Rel-15 | C28 | UEs supporting 5GS and NR SUL and supplemental uplink with dynamic switch |
| 7.1.1.1.6 | Random access procedure / Successful / Temporary C-RNTI Based / Preamble selected by MAC itself | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.7 | Random access procedure / 2-step RACH / RA_TYPE selection | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.8 | Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.9 | Random access procedure / Successful / 2-step RACH/C-RNTI Based / Preamble selected by MAC itself | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.9a | Random access procedure / 2-step RACH / Successful / RRC_IDLE | Rel-16 | C135A | UEs supporting 5G Core and 2-Step RACH |
| 7.1.1.1.10 | Random access procedure / 2-step RACH/not complete/ RA_TYPE to 4-stepRA | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.10a | Random access procedure / 2-step RACH/ Fallback for CBRA | Rel-16 | C135A | UEs supporting 5G Core and 2-Step RACH |
| 7.1.1.1.11 | Random access procedure / Successful / Slice specific RACH configuration | Rel-17 | C262 | UEs supporting slice-based RACH partitioning and slice-based RACH prioritisation |
| 7.1.1.1.12 | Random access procedure / Successful / ra-PrioritizationForSlicing | Rel-17 | C263 | UEs supporting slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| 7.1.1.1.13 | Random access procedure / Successful / Slice specific RACH configuration / 2-step RACH | Rel-17 | C264 | UEs supporting 2-Step RACH, slice-based RACH partitioning and slice-based RACH prioritisation |
| 7.1.1.1.14 | Random access procedure / Successful / ra-PrioritizationForSlicingTwoStep / 2-step RACH | Rel-17 | C265 | UEs supporting 2-Step RACH, slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| 7.1.1.1.15 | Random access procedure / RedCap UE / SI request | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.1.16 | Random access procedure / RedCap UE identification / Msg3-based / CCCH1 | Rel-17 | C212a | UEs supporting 5G Core and RedCap and RRC_INACTIVE |
| 7.1.1.1.17 | Random access procedure / RedCap UE identification | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.1.18 | Random access procedure / Msg3 repetition indication / Random access resources selection | Rel-17 | C211 | UEs supporting 5G Core and repetition of Message 3 PUSCH |
| 7.1.1.1.19 | Random access procedure / Successful / Beam Failure / Unified TCI | Rel-17 | C311 | UEs supporting 5GS and unified TCI state operation with joint DL/UL TCI update for intra-cell beam management |
| 7.1.1.1.20 | Random access procedure / Msg1 repetition / Contention-based | Rel-18 | C414 | UEs supporting 5G Core and PRACH repetition |
| 7.1.1.1.21 | Random access procedure / Msg1 repetition / Msg1-based SI request | Rel-18 | C414 | UEs supporting 5G Core and PRACH repetition |
| 7.1.1.1.22 | Random access procedure / Msg1 repetition / BWP only configured with RACH resource for coverage enhancement | Rel-18 | C351 | UEs supporting 5GS and PRACH repetition |
| 7.1.1.1.23 | Random access procedure / Msg1 repetition / Contention-free | Rel-18 | C414 | UEs supporting 5G Core and PRACH repetition |
| 7.1.1.1.24 | Random access procedure / eRedCap UE identification / Msg3-based / CCCH1 | Rel-18 | C389 | UEs supporting 5G Core and eRedCap and RRC_INACTIVE |
| 7.1.1.1.25 | Random access procedure / eRedCap UE identification / Msg1-based / Contention based / Msg3-based / CCCH / Correct handling of contention Resolution with large bandwidth | Rel-18 | C380 | UEs supporting 5G Core and eRedCap |
| 7.1.1.1.26 | Random access procedure / eRedCap UE identification / Msg1-based / Contention free random access procedure | Rel-18 | C380 | UEs supporting 5G Core and eRedCap |
| 7.1.1.1.27 | Random access procedure / eRedCap UE / SI request | Rel-18 | C380 | UEs supporting 5G Core and eRedCap |
| 7.1.1.1.29 | Random access procedure / Successful / LTM cell switch / DL assignment on the PDCCH / UL grant | Rel-18 | C405 | UEs supporting 5G Core and unified TCI with joint DL/UL TCI-state indication and MAC-CE activated joint LTM TCI states and TA indication in cell switch command and RACH-Less LTM with dynamic grant |
| 7.1.1.1.31 | NR NTN Enhancements / Random access procedure / Coverage Enhancement | Rel-18 | C408 | UEs supporting 5G Core and NR NTN access and Msg4 HARQ Repetition |
| 7.1.1.2 | Downlink Data Transfer |  |  |  |
|  | ## <<< Rows skipped here>>> |  |  |  |
| 7.1.1.3.19 | Correct handling of MAC control information / Buffer Status / UL resources are allocated / Refined BSR / Padding BSR | Rel-18 | C357 | UEs supporting 5G Core and using the refined buffer size table for BSR |
| 7.1.1.3.20 | Correct handling of MAC control information / Delay Status / Delay status reporting | Rel-18 | C352 | UEs supporting 5GS and delay status reporting |
| 7.1.1.3.20a | Correct handling of MAC control information / Delay Status / Refined delay status reporting | Rel-18 | C398 | UEs supporting 5GS and delay status reporting and refined buffer size table |
| 7.1.1.3.21 | Correct handling of MAC control information / Delay Status / Triggering scheduling request | Rel-18 | C352 | UEs supporting 5GS and delay status reporting |
| 7.1.1.3.22 | UE power headroom reporting / delta power class reporting | Rel-18 | C4AA | UEs supporting 5G Core and delta power class reporting |
| 7.1.1.3.23 | UE power headroom reporting / PCMAX for assumed PUSCH reporting | Rel-18 | C4BB | UEs supporting 5G Core and dynamic waveform switch PHR |
| 7.1.1.3.26 | Multi-cell PUSCH scheduling with a single DCI / FDRA field based |  |  |  |

## <<< Text skipped here>>>

## 4.2	Protocol conformance test cases applicability conditions

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| C01 | IF A.4.1-3/2 THEN R ELSE N/A | UEs supporting EN-DC |
|  | <<< Rows skipped here>>> |  |
| C413 | IF A.4.1-5/1 AND A.4.3.7-1/24 AND A.4.3.7-1/78 THEN R ELSE N/A | UEs supporting 5G Core and SNPN and equivalent SNPNs for cell (re)selection |
| C414 | IF A.4.1-5/1 AND A.4.3.2-1/160 THEN R ELSE N/A | UEs supporting 5G Core and PRACH repetition |
| C4AA | IF A.4.1-5/1 AND A.4.3.2-1/2AA THEN R ELSE N/A | UEs supporting 5G Core and delta power class reporting |
| C4BB | IF A.4.1-5/1 AND A.4.3.2-1/2BB THEN R ELSE N/A | UEs supporting 5G Core and dynamic waveform switch PHR |

>>>

Note to Secretary: The A.4.3.2-1/2AA  & A.4.3.2-1/2BB needs to be resolved based on 38.508-2 CR 0815

<<<