3GPP TSG-RAN5 Meeting #107	R5-252352

# Malta, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3324 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of frequency error CA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the frequency error CA TC 6.4A.1.1, the UL RB allocation for intra-band contiguous CA is a non-contiguous allocation. For intra-band contiguous CA, the SCell can drop, since the power is not limited via p-Max. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UL RB allocation has been changed to point to the default RB allocation of Tx intra-band contiguous CA test cases in Table 6.1A-1a. For CA_n5B, an editor's note has been added that the RB allocation is FFS, since the default RB allocation exceeds the REFSENS UL allocation for this band configuration. Message exceptions for p-Max have been specified for intra-band contiguous CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.1.1, 6.4A.1.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.4A.1.1	Frequency error for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The uplink RB allocation for CA_n5B is FFS, since the default RB allocation exceeds the REFSENS UL allocation.

6.4A.1.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency for 2UL CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.4A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.1.0.

6.4A.1.1.4	Test description

6.4A.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in Table 6.4A.1.1.4.1-1 to table 6.4A.1.1.4.1-3. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Mid range for PCC and SCC (NOTE 3) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg for both PCC and SCC |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band belongs to inter-band CA combination. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

Table 6.4A.1.1.4.1-2: Intra band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |  |
|  |  |  |  | PCC | SCC |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | Outer Full (NOTE 2) |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: .The specific configuration of each RB allocation is defined in Table 6.1A-1a. |  |  |  |  |  |

Table 6.4A.1.1.4.1-3: Intra band non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | One frequency chosen from Low range or High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |  |
|  |  |  |  | PCC | SCC |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 for each carrier which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band belongs to intra-band CA combination. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure 3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The DL and UL Reference Measurement channels are set according to Tables 6.4A.1.1.4.1-1 to 6.4A.1.1.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.4A.1.1.4.3

6.4A.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.4A.1.1.4.3

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 6.4A.1.1.4.1-1 to 6.4A.1.1.4.1-3 as appropriate on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.4A.1.1.4.1-1 to 6.4A.1.1.4.1-3 as appropriate on both PCC and SCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in subclauses 7.3A.1.5. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the Frequency Error on PCC and SCC using Global In-Channel Tx-Test (Annex E) respectively. For TDD slots with transient periods are not under test.

8.	For UEs supporting DSS, repeat steps 1~7 on the applicable bands as specified in Section 5.4.2.1 with message contents being according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-61 and Table 4.6.3-62 with condition DSS.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.4A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.4A.1.1.4.3-1 FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.4A.1.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA for CA_n41C, CA_n78C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 11 |  | Power class 3 |
|  | 15 |  | Power class 2 |

Table 6.4A.1.1.4.3-3: FrequencyInfoUL-SIB for intra-band contiguous CA for CA_n48B

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 |

Table 6.4A.1.1.4.3-4: FrequencyInfoUL-SIB for intra-band contiguous CA for CA_n77C, CA_n79C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 11 |  | Power class 3 |

6.4A.1.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + TT) for each test point

For inter-band CA, PPM refers to each CC UL frequency.

For intra-band contiguous CA, PPM refers to PCC UL frequency.

For intra-band non-contiguous CA, PPM refers to each CC UL frequency.

Table 6.4A.1.1.5-1: Test Tolerance for frequency error

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 15Hz | 15Hz |
| 40MHz < BW ≤ 100MHz | 15Hz | 15Hz |

< End of changes >