3GPP TSG-RAN WG5 Meeting #107	R5-252413

# St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4907 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PIXIT for testing of variable Tx-Rx spacing for eMTC and NB-IoT for NTN bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 has specified variable Tx-Rx spacing for NTN bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds new PIXITs and associated condition for special conformance testing of variable Tx-Rx spacing for NTN bands for eMTC and NB-IoT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Protocol conformance related to variable Tx-Rx spacing for NTN bands cannot be performed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1, 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 9.1	E-UTRAN PIXIT

<<Unchanged text skipped>>

Table 9.1-2: E-UTRAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eTDDsubframeConfig | TDD_SubframeAssignment_Type | 1 |  | TDD uplink-downlink subframe configuration |
| px_ePrimaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_ePrimaryFrequencyBand |
| px_eSecondaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_eSecondaryFrequencyBand |
| px_EUTRA_CA_BandCombination | CA_BandCombination_Type | CA_1C |  | Band combination for CA test cases |
| px_EUTRA_DC_BandCombination | DC_BandCombination_Type | DC_1A-3A |  | Band combination for DC test cases |
| px_eMaxNumberROHC_ContextSessions | MaxNumberROHC_ContextSessions_Type | Cs16 |  | Maximum number of ROHC context sessions |
| px_MFBI_FrequencyBand | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI test case scenario. |
| px_MFBI_BandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | E-UTRAN channel bandwidth used on px_MFBI_FrequencyBand. Applied to MFBI test case scenario. |
| px_OverlappingNotSupportedFrequencyBand_MFBI | integer | 27 |  | A not supported E-UTRA frequency band that is overlapping with a supported band (px_MFBI_FrequencyBand). Applied to MFBI test case scenario. |
| px_OverlappingSupportedFrequencyBand_MFBI | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI enhancement test case scenario. |
| px_NoTDD_LCR_onLTE | boolean | false |  | Only applicable to UE supporting (at least) LTE and LCR TDD. If set to true, the UE is configured not to report support of LCR TDD (IE utraTDD128 in UECapabilityInformation) when being camped on an LTE cell with no detectable LCR TDD cell in the environment |
| px_ntn_variable_txrx_separation | boolean | false |  | eMTC NTN UE configured to handle variable Tx-Rx frequency separation for NTN bands. If the value is FALSE it is assumed that the UE is configured for default Tx-Rx frequency separation |

<<Unchanged text skipped>>

## 9.3	NB-IoT PIXIT

Table 9.3-1: NB-IoT PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_NB_OperationMode | NB_OperationMode_Type | standalone | inband_samePCI, inband_differentPCI, guardband, standalone | NB-IoT Operation Mode |
| px_DoAttachWithoutPDN | boolean | false |  | UE is configured to do Attach without PDN; if the value is FALSE it is assumed that the UE is configured to do Attach with PDN |
| px_ModifyBearerResources | boolean | false |  | NB-IoT UE configured to handle modification of bearer resources including dedicated bearers (PDN of type "IP") |
| px_NS_Value | integer | 1 |  | Network Signalling value as defined in TS 36.101 [34] clause 6.2.4F and used in TS 36.508 [3] clause 8.1.3.1 to select the NB-IoT frequencies. |
| px_ntn_variable_txrx_separation | boolean | false |  | NB-IoT NTN UE configured to handle variable Tx-Rx frequency separation for NTN bands. If the value is FALSE it is assumed that the UE is configured for default Tx-Rx frequency separation |

END OF CHANGES >>