3GPP TSG-RAN5 Meeting #107	R5-252454

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3475 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test procedure for IMS MO call release in 5GC - release IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the IMS data channel (IMS DC) test cases, IMS data channel may be added simutaniously with Voice/Video Call so when SS reelase the IMS Call, the resource for IMS DC also need to release together. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Release the Qos flow and DRB for IMS DC when the MO Call release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource for IMS DC may maintain after call release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.9.17	Test procedure for IMS MO call release in 5GC

4.9.17.1	Scope

The purpose of this procedure is to make a UE initiated release of an ongoing IMS call.

4.9.17.2	Procedure description

4.9.17.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A on the NR Cell with an active IMS call.

4.9.17.2.2	Procedure sequence

Table 4.9.17.2.2-1: IMS MO call release in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-2 | Generic test procedure for MO release of speech call, steps 1-2, as defined in annex A.7 of TS 34.229-5 [47] are performed. | - | - | - | - |
| 3 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Steps 4 and 5 can occur in any order. | - | - | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

4.9.17.2.3	Specific message contents

Table 4.9.17.2.3-1: RRCReconfiguration (step 3, Table 4.9.17.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  | IMS_VOICE |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| } |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  | IMS_VIDEO |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS video bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS video bearer | entry 2 |  |
| } |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  | IMS_ VOICE AND IMS_BDC |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS data channel bearer | entry 2 |  |
| } |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 3 entries |  | IMS_VIDEO AND IMS_BDC |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS video bearer | entry 2 |  |
| DRB-Identity[3] |  | DRB-Identity linked to the IMS data channel bearer | entry 3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry |  | IMS_VOICE |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| } |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  | IMS_VIDEO |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  | IMS_ VOICE AND IMS_BDC |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 3 entries |  | IMS_VIDEO AND IMS_BDC |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| LogicalChannelIdentity[3] |  | Same value as drb-Identity[3] above | entry 3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |
| IMS_BDC | If this QoS rule is used to setup an IMS bootstrap data channel session |

Table 4.9.17.2.3-2: PDU SESSION MODIFICATION COMMAND (step 3, Table 4.9.17.2.2-1)

| Derivation Path: Table 4.7.2-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  |  |  |  |
| QoS rule[1] |  |  |  | IMS_VOICE, IMS_VIDEO |
| QoS rule identifier |  | ‘00000011’B | QoS rule 3 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| QoS rule[2] |  |  |  | IMS_VIDEO |
| QoS rule identifier |  | ‘00000100’B | QoS rule 4 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| QoS rule[3] |  |  |  | IMS_BDC |
| QoS rule identifier |  | ‘00000101’B | QoS rule 5 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| Authorized QoS flow descriptions |  |  |  |  |
| QoS flow descriptions[1] |  |  |  | IMS_VOICE, IMS_VIDEO |
| QFI |  | ‘00000111’B | QFI 7 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |
| QoS flow descriptions[2] |  |  |  | IMS_VIDEO |
| QFI |  | ’00001000’B | QFI 8 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |
| QoS flow descriptions[3] |  |  |  | IMS_BDC |
| QFI |  | ’00001001’B | QFI 9 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |
| IMS_BDC | If this QoS rule is used to setup an IMS bootstrap data channel session |

<End of modified section 1>