3GPP TSG-RAN5 Meeting #106	R5-252477

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4993 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of XR UTO-UCI test case 7.1.1.6.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the discussion paper R5-250570, the unused bits in UTO-UCI is not checked.Therefore, test procedure and test purpose needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the unused bits in UTO-UCI check in test procedure and test purpose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_XR_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.6.8	Correct handling of UL grant / Unused transmission occasion / Configured grant type 1

7.1.1.6.8.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { the UE determines that buffered data can be transmitted using N CG occasion (N>0, N<nrofBitsInUTO-UCI-r18) and UE transmits for the first time in CG occasion m }

then { The first N-X-1 bits of UTO-UCI are set to 0 (used) in CG occasion m+X (0≤X≤N). }

}

7.1.1.6.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clause 5.8.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.8.2]

RRC configures the following parameter when UTO-UCI (as specified in clause 9.3 in TS 38.213 [6]) is configured for a configured grant:

-	nrofBitsInUTO-UCI: number of bits in a UTO-UCI bitmap.

For a configured uplink grant, the MAC entity shall:

1>	if its associated configured grant is configured with UTO-UCI and it has not been indicated to the lower layers as unused for PUSCH transmission; or

1>	if its associated configured grant is not configured with UTO-UCI:

2>	if it is associated with a multi-PUSCH configured grant and meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]; or

2>	if it is not associated with a multi-PUSCH configured grant:

3>	consider it available for use.

The MAC entity shall not include the UL-SCH resource of a configured uplink grant not available for use in its procedures (e.g. in clause 5.4.4).

For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI.

7.1.1.6.8.3	Test description

7.1.1.6.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.6.8.3.2	Test procedure sequence

Table 7.1.1.6.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCReconfiguration message to configure UL configured grant type 1 and UTO-UCI for 8 transmission occasions. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message to indicate successful configuration. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a DL MAC PDU containing 4 RLC SDUs of size 99 bytes. (Note 1) | <-- | MAC PDU (4 RLC SDUs) | - | - |
| - | EXCEPTION: Step 4 is repeated 4 times. Testcase is considered passed when UE transmits UTO-UCI with first 4-m-1 bits set to 0, where m is the index of transmission occasion, i.e. m = [0,1,2,3]. | - | - | - | - |
| 4 | Check: Does the UE transmit UTO-UCI indicating unused transmission occasion(s) along with UL MAC PDU on scheduled transmission occasion? | --> | MAC PDU (RLC SDU) UTO-UCI | 1 | P |
| Note 1: According to the setting parameters in Table 7.1.1.6.8.3.3-2, TB size for configured grant type 1 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). |  |  |  |  |  |

7.1.1.6.8.3.3	Specific message contents

Table 7.1.1.6.8.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.8.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.8.3.3-2: CellGroupConfig (Table 7.1.1.6.8.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 1 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym40x14 |  | 15kHz |
| periodicity | Sym80x14 |  | 30kHz |
| periodicity | Sym160x14 |  | 60kHz |
| periodicity | Sym320x14 |  | 120kHz |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 9 |  |  |
| timeDomainAllocation | 0 | Reference TS 38.508-1 [4], Table 4.6.3-122 |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 2 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD, FR1_TDD |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB=9 PRB, RBstart = 0and NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.2. | FR2_TDD |
| antennaPort | 0 |  |  |
| precodingAndNumberOfLayers | 0 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 18 |  | FR1_FDD, FR1_TDD |
|  | 25 |  | FR2_TDD |
| pathlossReferenceIndex | 0 |  |  |
| } |  |  |  |
| uto-UCI-Config-r18 SEQUENCE { |  |  |  |
| nrofBitsInUTO-UCI-r18 | 8 |  |  |
| betaOffsetUTO-UCI-r18 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup SEQUENCE { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | n4 |  | FR1 |
|  | n8 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 | Start symbol(S)=0, Length(L)=14 | FR1 |
| startSymbolAndLength | 0001110 | S=0, L=2 | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>