3GPP TSG-RAN5 Meeting #107	R5-252492

# St Julian’s, Malta, 19th May – 23rd May 2025

Title:							Analysis on common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation

Source:			Anritsu, Keysight, Apple Trading, Huawei, HiSilicon

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the previous meeting, there was a discussion on how to correct the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation in Table 6.1A-1b [1] based on the following RF Action Point [2], and we reached a rough agreement [3]. This paper provides our analysis on the more concrete common UL configuration.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#105.22 | RF | Evaluate and apply any corrections needed to the common uplink configuration for FR1 intra-band contiguous CA (non-contiguous RB) in TS 38.521-1 Clause 6.1. | Apple, Anritsu, Keysight | R5-247730 | RAN5#107 | Open |

# 2.	Discussion

For FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation, RB allocation is judged as Inner, Outer1, or Outer2 from UL configuration and minimum guard band as defined in Annex A.1. Since μ = 0, 1, 2 and channel raster = 15 kHz, 100 kHz are used for calculating minimum guard band as shown in Annex A.2 and Annex A.3, there are 6 different patterns of conditions to judge RB allocation: {μ = 0, Raster 15 kHz}, {μ = 0, Raster 100 kHz}, {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}, {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}. In the current core requirements [4], applicable μ can be observed as below based on the detailed information summarized in Annex B.

- μ = 0 will only apply if testing CABW combinations including CCx ≤ 5 MHz and CCy ≤ 50 MHz.

- μ = 1 will only apply in case of:

  - CABW combinations including CCx ≤ 5 MHz and CCy > 50 MHz for any band.

  - 10 MHz < CABW ≤ 25 MHz in case of band n5 and n71 whose channel raster is 100 kHz.

  - 25 MHz < CABW ≤ 35 MHz in case of band n71 whose channel raster is 100 kHz.

- μ = 2 otherwise.

In [3], the following proposals were agreed to determine the common UL configuration, but it was not concretely clarified which patterns should be considered and what RB allocation should be adopted for a given CABW combination.

Option 1: For each CABW combination, determine the common UL configuration optimized for each of the patterns of conditions that can apply to the given CABW.

Option 2: For each CABW combination, determine the common UL configuration that can be used across all operating bands. Define one pattern of RB allocation in Table 6.1A-1b [1] which satisfies the patterns of conditions that can apply to the given CABW.

Proposal 1: Select Option 2 to determine the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Proposal 2: For each CABW combination, adopt the first RB allocation which is a valid allocation for all the patterns of conditions that can apply to the given CABW by moving RB inwards from the outermost possible RB for the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Based on the above observation on μ for each CABW combination and Option 2 showing how to determine the common UL configuration, there are two types of concrete methods to select the patterns of conditions for a given CABW combination as follows. Regarding “can be used across all operating bands” in Option 2, Option 2-1 assumes all bands currently defined, and Option 2-2 assumes all bands including the future. Below, the difference between these options is marked in red text.

Option 2-1: Select the patterns of conditions optimized for the operating bands currently defined in [4].

- CABW combinations including CCx ≤ 5 MHz and CCy ≤ 50 MHz.

  - {μ = 0, Raster 15 kHz}, {μ = 0, Raster 100 kHz}

- CABW combinations including CCx ≤ 5 MHz and CCy > 50 MHz.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}

- CABW ≤ 35 MHz, except for the cases where previous conditions apply.

  - {μ = 1, Raster 100 kHz}, {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

- Otherwise.

  - {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

Option 2-2: Select the patterns of conditions considering the possibility that any band does not support SCS = 60 kHz for CC ≥ 10 MHz can be added in the future.

- CABW combinations including CCx ≤ 5 MHz and CCy ≤ 50 MHz.

  - {μ = 0, Raster 15 kHz}, {μ = 0, Raster 100 kHz}

- CABW combinations including CCx ≤ 5 MHz and CCy > 50 MHz.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}

- Otherwise.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}, {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

For comparison, UL configurations considering the patterns of conditions proposed by the above two options are shown in Annex C, and Table 1 shows the difference between these options, where the difference is defined as (RBStart1 for Option 2-2 – RBStart1 for Option 2-1) + (RBStart2 for Option 2-1 – RBStart2 for Option 2-2).

Table 1: The number of channel bandwidth combinations for each difference between Option 2-1 and Option 2-2

| BWchannel_CA (MHz) | Difference | RB allocation (Inner) | RB allocation (Outer1) | RB allocation (Outer2) |
| --- | --- | --- | --- | --- |
| 20 | 0 | 3 | 3 | 3 |
| 25 | 0 | 6 | 6 | 6 |
| 30 | 0 | 4 | 4 | 6 |
|  | 1 | 2 | 2 | 0 |
| 35 | 0 | 2 | 2 | 4 |
|  | 1 | 2 | 2 | 0 |
| 40 | 0 | 6 | 6 | 6 |
| 45 | 0 | 2 | 2 | 2 |
| 50 | 0 | 4 | 4 | 8 |
|  | 1 | 0 | 2 | 0 |
|  | 2 | 2 | 2 | 0 |
|  | 4 | 2 | 0 | 0 |
| 55 | 0 | 0 | 0 | 2 |
|  | 1 | 0 | 2 | 0 |
|  | 2 | 1 | 0 | 0 |
|  | 4 | 1 | 0 | 0 |
| 60 | 0 | 3 | 4 | 10 |
|  | 1 | 3 | 6 | 0 |
|  | 2 | 4 | 0 | 0 |
| 70 | 0 | 4 | 4 | 8 |
|  | 1 | 3 | 2 | 0 |
|  | 2 | 1 | 2 | 0 |
| 80 | 0 | 4 | 2 | 6 |
|  | 1 | 2 | 4 | 0 |
| 90 | 0 | 4 | 4 | 4 |
| 100 | 0 | 4 | 4 | 4 |
| 105 | 0 | 1 | 1 | 2 |
|  | 1 | 1 | 1 | 0 |
| 110 | 0 | 5 | 4 | 10 |
|  | 1 | 5 | 6 | 0 |
| 115 | 0 | 2 | 2 | 4 |
|  | 1 | 2 | 2 | 0 |
| 120 | 0 | 7 | 7 | 9 |
|  | 1 | 2 | 2 | 0 |
| 125 | 0 | 2 | 2 | 2 |
| 130 | 0 | 8 | 8 | 8 |
| 140 | 0 | 5 | 7 | 7 |
|  | 1 | 2 | 0 | 0 |
| 150 | 0 | 5 | 6 | 6 |
|  | 1 | 1 | 0 | 0 |
| 160 | 0 | 5 | 3 | 5 |
|  | 1 | 0 | 2 | 0 |
| 170 | 0 | 4 | 4 | 4 |
| 180 | 0 | 3 | 3 | 3 |
| 190 | 0 | 2 | 2 | 2 |
| 200 | 0 | 1 | 1 | 1 |
| Total | 0 | 96 | 95 | 132 |
|  | 1 | 25 | 33 | 0 |
|  | 2 | 8 | 4 | 0 |
|  | 4 | 3 | 0 | 0 |

Option 2-1 is optimized to the current definition in [4], so more accurate RB allocation can be obtained by this approach. RB are allocated as far out as possible so that it can be tested under almost the worst-case conditions. However, when a new intra-band contiguous CA configuration does not support 60 kHz with channel raster = 15 kHz or CABW > 35 MHz is added in the future, 28% of Inner RB allocation and Outer1 RB allocation need to be updated. As a result, the corresponding RB allocation of all existing bands will be impacted, and it might lead to the need for revalidation.

On the other hand, Option 2-2 is stable for any addition of intra-band contiguous CA configuration does not support 60 kHz with channel raster = 15 kHz or CABW > 35 MHz in the future. However, 28% of Inner RB allocation and Outer1 RB allocation are allocated inward from those in Option 2-1, and there is a maximum difference of 4 RBs. As a result, these RB allocations are tested under the conditions far from the most strict.

In summary, there is a trade-off between the accuracy of RB allocation for testing under the strict condition and the future expandability of operating bands. For testing under stricter conditions while avoiding a risk of future revalidation as much as possible, we focus on the distribution of the difference that increases significantly from CABW = 50 MHz and above. In case of CABW ≤ 45 MHz, there is only a maximum difference of 1 RB at few cases, so the impact due to the issue of RB allocation accuracy in Option 2-2 is acceptable. In case of CABW ≥ 50 MHz, the impact due to the issue is too large to be ignored, and it can be expected that the probability of adding intra-band contiguous CA configuration does not support 60 kHz in RAN4 decreases as CABW increases. Therefore, we propose the following Option 2-3 as a method to select the patterns of conditions that balances appropriate accuracy of RB allocation and risk for the future band addition. The concrete UL configuration of Option 2-3 is shown in Table C-3.

Option 2-3: Select the patterns of conditions considering the possibility that a band does not support SCS = 60 kHz for CC ≥ 10 MHz can be added in the future up to CABW = 45 MHz.

- CABW combinations including CCx ≤ 5 MHz and CCy ≤ 50 MHz.

  - {μ = 0, Raster 15 kHz}, {μ = 0, Raster 100 kHz}

- CABW combinations including CCx ≤ 5 MHz and CCy > 50 MHz.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}

- CABW ≤ 45 MHz, except for the cases where previous conditions apply.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}, {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

- Otherwise.

  - {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

Proposal 1: Select Option 2-3 to determine the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Proposal 2: Adopt Table C-3 as the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Although we assume it is unlikely, there is a risk that a band not considered in Option 2-3 could be added by RAN4. In order to ensure that the common UL configuration can be maintained even in such case, we propose to attach an Excel file that optimizes the RB allocation to [5] as a part of the TP analysis for MPR test case, and to add an editor’s note to [1] for clarifying when the RB allocation needs to be updated.

Proposal 3: Attach an Excel file that optimizes the RB allocation for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation to [5] as a part of the TP analysis for 6.2A.2.1.

Proposal 4: Add an editor’s note to [1] for clarifying when the RB allocation needs to be updated.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals.

Option 2-3: Select the patterns of conditions considering the possibility that a band does not support SCS = 60 kHz for CC ≥ 10 MHz can be added in the future up to CABW = 45 MHz.

- CABW combinations including CCx ≤ 5 MHz and CCy ≤ 50 MHz.

  - {μ = 0, Raster 15 kHz}, {μ = 0, Raster 100 kHz}

- CABW combinations including CCx ≤ 5 MHz and CCy > 50 MHz.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}

- CABW ≤ 45 MHz, except for the cases where previous conditions apply.

  - {μ = 1, Raster 15 kHz}, {μ = 1, Raster 100 kHz}, {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

- Otherwise.

  - {μ = 2, Raster 15 kHz}, {μ = 2, Raster 100 kHz}

Proposal 1: Select Option 2-3 to determine the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Proposal 2: Adopt Table C-3 as the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Proposal 3: Attach an Excel file that optimizes the RB allocation for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation to [5] as a part of the TP analysis for 6.2A.2.1.

Proposal 4: Add an editor’s note to [1] for clarifying when the RB allocation needs to be updated.

# 4.	References

[1] TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, V18.6.0 (2025-03)

[2] R5-247730, “On Common UL config for FR1 intra-band non-contiguous CA scenarios”, Apple Benelux B.V., RAN5#105

[3] R5-251530, “Analysis on common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation”, Anritsu, Keysight, Apple Trading, RAN5#106

[4] TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, V19.1.0 (2025-03)

[5] TR 38.903, “Derivation of test points for radio transmission and reception; User Equipment (UE) conformance test cases”, V18.6.0 (2025-03)

# Annex A.1	Definition of RB allocations of Inner, Outer1, and Outer2

In the minimum conformance requirements for MPR in 6.2A.2.0.4 [1], Inner, Outer1, and Outer2 RB allocations are defined as below. RB allocation is judged by the relationships between RBStart,Low, RBStart_CA, RBStart,High, and the relationships between NRB_alloc, BWChannel_CA, BWgap. These parameters are calculated from UL configuration and minimum guard band that is shown in Annex A.2.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc, RBStart_CA, BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

# Annex A.2	Definition of minimum guard band and common μ

In 5.3A.3 Minimum guard band and transmission bandwidth configuration for CA [1], the common μ, which is used to calculate minimum guard band, is defined as below. From the common subcarrier spacing supported in the operating band for both channel bandwidths, the μ corresponding to the largest common subcarrier spacing is selected as the common μ. Or μ = 1 is used if there is no common subcarrier spacing.

The aggregated channel bandwidth, BWChannel_CA, is defined as

BWChannel_CA = Fedge,high - Fedge,low (MHz).

The lower bandwidth edge Fedge, low and the upper bandwidth edge Fedge,high of the aggregated channel bandwidth are used as frequency reference points for transmitter and receiver requirements and are defined by

Fedge,low = FC,low - Foffset,low

Fedge,high = FC,high + Foffset,high

The lower and upper frequency offsets depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carrier and are defined as

Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

BWGB = max(BWGB,Channel(k))

NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.3.2-1 for the lowest and highest assigned component carrier, SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier respectively. SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k) use the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and BWGB,Channel(k) is the minimum guard band for carrier k according to Table 5.3.3-1 for the said μ value. In case there is no common μ value for both of the channel bandwidths, μ=1 is used for SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k).

# Annex A.3	Definition of nominal channel spacing

In 5.4A.1 Channel spacing for CA [1], nominal channel spacing between two adjacent NR component carriers is defined as below for each of the two types of channel raster.

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with a 100 kHz channel raster:

Nominal channel spacing=BWChannel(1)+BWChannel(2)-2GBChannel(1)-GBChannel(2)0.60.3 [MHz]

while for NR operating bands with 15 kHz channel raster:

Nominal channel spacing=BWChannel(1)+BWChannel(2)-2GBChannel(1)-GBChannel(2)0.015*2n+10.015*2n [MHz]

with n = µ0

# Annex B	Channel bandwidths, maximum CABW, and channel raster for each NR band

From Table 5.3.5-1, Table 5.4.2.3-1, and Table 5.5A.1-1 [4], the following information is summarized in the table below. Note that channel bandwidths, which are applicable only to downlink or not supported in the corresponding intra-band contiguous 2UL CA configuration, are omitted.

- Supported channel bandwidths and SCS for each NR band

- Maximum aggregated bandwidth for intra-band contiguous 2UL CA using the NR band

- Channel raster for each NR band

Table B-1: Channel bandwidths, maximum CABW, and channel raster for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Max CABW (MHz) | ΔFRaster (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| n1 | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 40 | 100 |
|  | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n2 | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 100 |
|  | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n3 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  | 75 | 100 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |  |
| n5 | 15 | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 25 | 100 |
|  | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  | 70 | 100 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |  |
| n25 | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 100 |
|  | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 |  | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |  | 50 | 100 |
|  | 30 |  |  | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
| n40 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  | 100 | 100 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| n41 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  | 190 | 15 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| n46 | 15 |  |  | 10 |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  | 160 | 15 |
|  | 30 |  |  | 10 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |  |  |
|  | 60 |  |  | 10 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |  |  |
| n48 | 15 |  | 5 | 10 | 15 | 20 |  | 30 |  | 40 |  |  |  |  |  |  |  | 140 | 15 |
|  | 30 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  | 50 | 100 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  | 35 | 100 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  | 200 | 15 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| n78 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  | 200 | 15 |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| n79 | 15 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  | 200 | 15 |
|  | 30 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |  |  |
|  | 60 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |  |  |
| n96 | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  | 160 | 15 |
|  | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |  |  |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |  |  |
| n102 | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  | 160 | 15 |
|  | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |  |  |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |  |  |
| n104 | 15 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  | 200 | 15 |
|  | 30 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
|  | 60 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |  |

# Annex C	UL configurations for each option

UL configurations for Options 2-x can be obtained by the attached excel file as below. Note that RB allocation is optimized by the procedure of Proposal 2 in [3].

Table C-1: UL configuration for Option 2-1

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 24 | 1@24 | 1@22 | 63 | 1@21 | 1@58 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 33 | 1@36 | 1@16 | 62 | 1@21 | 1@30 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 23 | 1@57 | 1@0 | 63 | 1@21 | 1@4 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 24 | 1@24 | 1@22 | 79 | 1@24 | 1@77 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 42 | 1@45 | 1@34 | 79 | 1@26 | 1@52 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 41 | 1@45 | 1@6 | 79 | 1@26 | 1@25 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 23 | 1@84 | 1@0 | 79 | 1@28 | 1@0 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 38 | 1@22 | 1@16 | 74 | 1@13 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 38 | 1@22 | 1@2 | 74 | 1@13 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 51 | 1@51 | 1@49 | 96 | 1@31 | 1@74 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 50 | 1@54 | 1@24 | 95 | 1@32 | 1@47 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 50 | 1@57 | 1@0 | 96 | 1@31 | 1@20 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 44 | 1@23 | 1@20 | 92 | 1@15 | 1@36 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 48 | 1@26 | 1@11 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 42 | 1@31 | 1@0 | 92 | 1@15 | 1@9 | 150 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 61 | 1@62 | 1@43 | 113 | 1@36 | 1@69 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 61 | 1@62 | 1@16 | 113 | 1@36 | 1@42 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 56 | 1@31 | 1@20 | 108 | 1@18 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 58 | 1@30 | 1@7 | 108 | 1@18 | 1@20 | 178 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 51 | 1@51 | 1@49 | 129 | 1@42 | 1@118 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 68 | 1@72 | 1@33 | 128 | 1@42 | 1@63 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 50 | 1@111 | 1@0 | 129 | 1@42 | 1@10 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 44 | 1@23 | 1@20 | 124 | 1@20 | 1@57 | 204 | 1@0 | 1@77 |
|  |  | 20+20 | 64 | 1@35 | 1@15 | 124 | 1@20 | 1@30 | 204 | 1@0 | 1@50 |
|  |  | 30+10 | 42 | 1@58 | 1@0 | 124 | 1@20 | 1@3 | 204 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 78 | 1@78 | 1@76 | 145 | 1@47 | 1@112 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 72 | 1@37 | 1@34 | 142 | 1@23 | 1@55 | 232 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 55 | 1@51 | 1@53 | 164 | 1@51 | 1@162 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 86 | 1@90 | 1@69 | 160 | 1@53 | 1@106 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 86 | 1@90 | 1@15 | 160 | 1@53 | 1@52 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 54 | 1@163 | 1@0 | 164 | 1@53 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 48 | 1@23 | 1@22 | 158 | 1@23 | 1@77 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@44 | 1@33 | 156 | 1@26 | 1@52 | 258 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@44 | 1@6 | 156 | 1@26 | 1@25 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 46 | 1@84 | 1@0 | 158 | 1@28 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 82 | 1@78 | 1@80 | 180 | 1@58 | 1@158 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 76 | 1@37 | 1@36 | 176 | 1@28 | 1@77 | 288 | 1@0 | 1@105 |
| 60 | 15 | 10+50 | 54 | 1@51 | 1@52 | 191 | 1@51 | 1@189 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 106 | 1@105 | 1@104 | 195 | 1@64 | 1@152 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@108 | 1@51 | 192 | 1@64 | 1@95 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 104 | 1@113 | 1@0 | 195 | 1@64 | 1@42 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 53 | 1@218 | 1@0 | 191 | 1@80 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 46 | 1@23 | 1@21 | 186 | 1@23 | 1@91 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 98 | 1@50 | 1@47 | 190 | 1@31 | 1@74 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 100 | 1@53 | 1@24 | 188 | 1@31 | 1@46 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@31 | 1@19 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 46 | 1@111 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 104 | 1@105 | 1@102 | 228 | 1@74 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 124 | 1@126 | 1@89 | 228 | 1@74 | 1@141 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 125 | 1@126 | 1@34 | 228 | 1@74 | 1@85 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 103 | 1@168 | 1@0 | 228 | 1@74 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 96 | 1@50 | 1@46 | 224 | 1@36 | 1@96 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 120 | 1@62 | 1@43 | 224 | 1@36 | 1@69 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 120 | 1@62 | 1@15 | 224 | 1@36 | 1@41 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 96 | 1@86 | 1@0 | 224 | 1@36 | 1@14 | 368 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 141 | 1@145 | 1@125 | 260 | 1@85 | 1@184 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 141 | 1@146 | 1@70 | 260 | 1@86 | 1@129 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 142 | 1@144 | 1@15 | 260 | 1@85 | 1@74 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 136 | 1@72 | 1@61 | 256 | 1@42 | 1@91 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 136 | 1@72 | 1@33 | 256 | 1@42 | 1@63 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@42 | 1@36 | 422 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 160 | 1@163 | 1@106 | 293 | 1@97 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 159 | 1@164 | 1@52 | 292 | 1@97 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 154 | 1@81 | 1@51 | 288 | 1@48 | 1@85 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 154 | 1@81 | 1@24 | 288 | 1@48 | 1@58 | 478 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 176 | 1@182 | 1@87 | 325 | 1@108 | 1@162 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 96 | 1@50 | 1@46 | 320 | 1@50 | 1@158 | 536 | 1@0 | 1@216 |
|  |  | 50+50 | 172 | 1@90 | 1@42 | 320 | 1@53 | 1@79 | 532 | 1@0 | 1@132 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 74 | 1@37 | 1@35 | 324 | 1@37 | 1@160 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 124 | 1@64 | 1@60 | 340 | 1@56 | 1@160 | 564 | 1@0 | 1@216 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 46 | 1@23 | 1@21 | 324 | 1@23 | 1@160 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 348 | 1@50 | 1@172 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 150 | 1@77 | 1@73 | 356 | 1@59 | 1@158 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 356 | 1@59 | 1@130 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@100 | 1@62 | 354 | 1@59 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@100 | 1@33 | 354 | 1@59 | 1@73 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 192 | 1@100 | 1@6 | 356 | 1@59 | 1@47 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 150 | 1@143 | 1@0 | 356 | 1@59 | 1@19 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 94 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 44 | 1@252 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 74 | 1@37 | 1@35 | 352 | 1@37 | 1@174 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 124 | 1@64 | 1@60 | 372 | 1@62 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 124 | 1@184 | 1@0 | 372 | 1@62 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 72 | 1@238 | 1@0 | 352 | 1@98 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 96 | 1@50 | 1@46 | 376 | 1@50 | 1@186 | 648 | 1@0 | 1@272 |
|  |  | 30+90 | 150 | 1@77 | 1@73 | 388 | 1@65 | 1@180 | 646 | 1@0 | 1@244 |
|  |  | 40+80 | 206 | 1@105 | 1@101 | 390 | 1@64 | 1@152 | 646 | 1@0 | 1@216 |
|  |  | 50+70 | 210 | 1@109 | 1@80 | 388 | 1@64 | 1@124 | 644 | 1@0 | 1@188 |
|  |  | 60+60 | 212 | 1@109 | 1@52 | 390 | 1@65 | 1@97 | 648 | 1@0 | 1@161 |
|  |  | 70+50 | 210 | 1@109 | 1@24 | 388 | 1@64 | 1@68 | 644 | 1@0 | 1@132 |
|  |  | 80+40 | 204 | 1@116 | 1@0 | 390 | 1@64 | 1@41 | 646 | 1@0 | 1@105 |
|  |  | 90+30 | 150 | 1@171 | 1@0 | 388 | 1@65 | 1@13 | 646 | 1@0 | 1@77 |
|  |  | 100+20 | 94 | 1@227 | 1@0 | 376 | 1@86 | 1@0 | 648 | 1@0 | 1@50 |
| 125 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 25+100 | 124 | 1@64 | 1@60 | 404 | 1@64 | 1@200 | 676 | 1@0 | 1@272 |
|  |  | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 150 | 1@77 | 1@73 | 422 | 1@70 | 1@202 | 702 | 1@0 | 1@272 |
|  |  | 40+90 | 206 | 1@105 | 1@101 | 422 | 1@70 | 1@174 | 702 | 1@0 | 1@244 |
|  |  | 50+80 | 228 | 1@118 | 1@98 | 422 | 1@70 | 1@147 | 700 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@118 | 1@70 | 422 | 1@70 | 1@118 | 702 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@118 | 1@43 | 422 | 1@70 | 1@91 | 702 | 1@0 | 1@161 |
|  |  | 80+50 | 228 | 1@118 | 1@14 | 422 | 1@70 | 1@63 | 700 | 1@0 | 1@132 |
|  |  | 90+40 | 204 | 1@144 | 1@0 | 422 | 1@70 | 1@35 | 702 | 1@0 | 1@105 |
|  |  | 100+30 | 148 | 1@200 | 1@0 | 422 | 1@70 | 1@7 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 206 | 1@105 | 1@101 | 456 | 1@76 | 1@197 | 758 | 1@0 | 1@272 |
|  |  | 50+90 | 248 | 1@127 | 1@117 | 456 | 1@75 | 1@169 | 756 | 1@0 | 1@244 |
|  |  | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@76 | 1@141 | 758 | 1@0 | 1@216 |
|  |  | 70+70 | 248 | 1@127 | 1@61 | 454 | 1@76 | 1@113 | 756 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@128 | 1@34 | 456 | 1@76 | 1@86 | 758 | 1@0 | 1@161 |
|  |  | 90+50 | 248 | 1@127 | 1@5 | 456 | 1@75 | 1@57 | 756 | 1@0 | 1@132 |
|  |  | 100+40 | 206 | 1@171 | 1@0 | 456 | 1@76 | 1@30 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 260 | 1@132 | 1@128 | 488 | 1@81 | 1@191 | 812 | 1@0 | 1@272 |
|  |  | 60+90 | 268 | 1@137 | 1@108 | 490 | 1@81 | 1@163 | 814 | 1@0 | 1@244 |
|  |  | 70+80 | 266 | 1@137 | 1@80 | 488 | 1@81 | 1@135 | 812 | 1@0 | 1@216 |
|  |  | 80+70 | 266 | 1@137 | 1@52 | 488 | 1@81 | 1@107 | 812 | 1@0 | 1@188 |
|  |  | 90+60 | 266 | 1@137 | 1@24 | 490 | 1@81 | 1@80 | 814 | 1@0 | 1@161 |
|  |  | 100+50 | 260 | 1@144 | 1@0 | 488 | 1@81 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 286 | 1@146 | 1@126 | 524 | 1@87 | 1@186 | 870 | 1@0 | 1@272 |
|  |  | 70+90 | 284 | 1@146 | 1@98 | 522 | 1@87 | 1@158 | 868 | 1@0 | 1@244 |
|  |  | 80+80 | 284 | 1@146 | 1@70 | 522 | 1@87 | 1@130 | 868 | 1@0 | 1@216 |
|  |  | 90+70 | 284 | 1@146 | 1@42 | 522 | 1@87 | 1@102 | 868 | 1@0 | 1@188 |
|  |  | 100+60 | 286 | 1@146 | 1@15 | 524 | 1@87 | 1@75 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@155 | 1@117 | 556 | 1@92 | 1@180 | 924 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@155 | 1@89 | 556 | 1@92 | 1@152 | 924 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@155 | 1@61 | 556 | 1@92 | 1@124 | 924 | 1@0 | 1@216 |
|  |  | 100+70 | 304 | 1@155 | 1@33 | 556 | 1@92 | 1@96 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@165 | 1@108 | 588 | 1@98 | 1@174 | 980 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@165 | 1@80 | 588 | 1@98 | 1@146 | 980 | 1@0 | 1@244 |
|  |  | 100+80 | 322 | 1@165 | 1@52 | 588 | 1@98 | 1@118 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 340 | 1@174 | 1@98 | 622 | 1@104 | 1@169 | 1036 | 1@0 | 1@272 |
|  |  | 100+90 | 340 | 1@174 | 1@70 | 622 | 1@104 | 1@141 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@183 | 1@89 | 656 | 1@109 | 1@163 | 1092 | 1@0 | 1@272 |

Table C-2: UL configuration for Option 2-2

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 24 | 1@24 | 1@22 | 63 | 1@21 | 1@58 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 33 | 1@36 | 1@16 | 62 | 1@21 | 1@30 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 23 | 1@57 | 1@0 | 63 | 1@21 | 1@4 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 24 | 1@24 | 1@22 | 79 | 1@24 | 1@77 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 42 | 1@45 | 1@34 | 79 | 1@26 | 1@52 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 41 | 1@45 | 1@6 | 79 | 1@26 | 1@25 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 23 | 1@84 | 1@0 | 79 | 1@28 | 1@0 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 38 | 1@22 | 1@16 | 74 | 1@13 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 38 | 1@22 | 1@2 | 74 | 1@13 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 51 | 1@51 | 1@49 | 96 | 1@31 | 1@74 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 50 | 1@54 | 1@24 | 95 | 1@32 | 1@47 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 49 | 1@58 | 1@0 | 96 | 1@31 | 1@20 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 42 | 1@23 | 1@19 | 90 | 1@15 | 1@35 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 48 | 1@26 | 1@11 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 42 | 1@31 | 1@0 | 90 | 1@15 | 1@8 | 150 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 60 | 1@63 | 1@43 | 112 | 1@37 | 1@69 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 61 | 1@62 | 1@16 | 112 | 1@37 | 1@42 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 56 | 1@31 | 1@20 | 108 | 1@18 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 56 | 1@31 | 1@7 | 108 | 1@18 | 1@20 | 178 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 51 | 1@51 | 1@49 | 129 | 1@42 | 1@118 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 68 | 1@72 | 1@33 | 128 | 1@42 | 1@63 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 50 | 1@111 | 1@0 | 129 | 1@42 | 1@10 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 44 | 1@23 | 1@20 | 124 | 1@20 | 1@57 | 204 | 1@0 | 1@77 |
|  |  | 20+20 | 64 | 1@35 | 1@15 | 124 | 1@20 | 1@30 | 204 | 1@0 | 1@50 |
|  |  | 30+10 | 42 | 1@58 | 1@0 | 124 | 1@20 | 1@3 | 204 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 78 | 1@78 | 1@76 | 145 | 1@47 | 1@112 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 72 | 1@37 | 1@34 | 142 | 1@23 | 1@55 | 232 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 51 | 1@51 | 1@49 | 162 | 1@51 | 1@160 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 86 | 1@90 | 1@69 | 160 | 1@53 | 1@106 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 86 | 1@90 | 1@15 | 160 | 1@53 | 1@52 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 50 | 1@167 | 1@0 | 162 | 1@55 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 44 | 1@23 | 1@20 | 156 | 1@23 | 1@76 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@44 | 1@33 | 156 | 1@26 | 1@52 | 258 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@44 | 1@6 | 156 | 1@26 | 1@25 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 42 | 1@86 | 1@0 | 156 | 1@29 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 78 | 1@78 | 1@76 | 179 | 1@58 | 1@157 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 72 | 1@37 | 1@34 | 174 | 1@29 | 1@77 | 288 | 1@0 | 1@105 |
| 60 | 15 | 10+50 | 52 | 1@51 | 1@50 | 190 | 1@51 | 1@188 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 104 | 1@105 | 1@102 | 194 | 1@64 | 1@151 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@108 | 1@51 | 192 | 1@64 | 1@95 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 102 | 1@115 | 1@0 | 194 | 1@64 | 1@41 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 51 | 1@220 | 1@0 | 190 | 1@81 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 44 | 1@23 | 1@20 | 184 | 1@23 | 1@90 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 96 | 1@50 | 1@46 | 190 | 1@31 | 1@74 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 100 | 1@53 | 1@24 | 188 | 1@31 | 1@46 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@31 | 1@19 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 44 | 1@112 | 1@0 | 184 | 1@42 | 1@0 | 314 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 104 | 1@105 | 1@102 | 228 | 1@74 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 123 | 1@127 | 1@89 | 226 | 1@75 | 1@140 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 123 | 1@127 | 1@33 | 226 | 1@75 | 1@84 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 103 | 1@168 | 1@0 | 228 | 1@74 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 96 | 1@50 | 1@46 | 224 | 1@36 | 1@96 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 118 | 1@63 | 1@43 | 222 | 1@37 | 1@69 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 118 | 1@63 | 1@15 | 222 | 1@37 | 1@41 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 96 | 1@86 | 1@0 | 224 | 1@36 | 1@14 | 368 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 141 | 1@145 | 1@125 | 259 | 1@86 | 1@184 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 141 | 1@146 | 1@70 | 260 | 1@86 | 1@129 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 141 | 1@145 | 1@15 | 259 | 1@86 | 1@74 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 136 | 1@72 | 1@61 | 254 | 1@42 | 1@90 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 136 | 1@72 | 1@33 | 256 | 1@42 | 1@63 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 136 | 1@72 | 1@6 | 254 | 1@42 | 1@35 | 422 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 160 | 1@163 | 1@106 | 293 | 1@97 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 159 | 1@164 | 1@52 | 292 | 1@97 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 154 | 1@81 | 1@51 | 288 | 1@48 | 1@85 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 154 | 1@81 | 1@24 | 288 | 1@48 | 1@58 | 478 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 176 | 1@182 | 1@87 | 325 | 1@108 | 1@162 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 96 | 1@50 | 1@46 | 320 | 1@50 | 1@158 | 536 | 1@0 | 1@216 |
|  |  | 50+50 | 172 | 1@90 | 1@42 | 320 | 1@53 | 1@79 | 532 | 1@0 | 1@132 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 72 | 1@37 | 1@34 | 322 | 1@37 | 1@159 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 124 | 1@64 | 1@60 | 340 | 1@56 | 1@160 | 564 | 1@0 | 1@216 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 44 | 1@23 | 1@20 | 322 | 1@23 | 1@159 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 348 | 1@50 | 1@172 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 148 | 1@77 | 1@72 | 354 | 1@59 | 1@157 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 192 | 1@100 | 1@89 | 354 | 1@59 | 1@129 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@100 | 1@62 | 354 | 1@59 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@100 | 1@33 | 354 | 1@59 | 1@73 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 192 | 1@100 | 1@6 | 354 | 1@59 | 1@46 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 148 | 1@144 | 1@0 | 354 | 1@59 | 1@18 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 94 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 42 | 1@253 | 1@0 | 322 | 1@113 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 72 | 1@37 | 1@34 | 350 | 1@37 | 1@173 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 124 | 1@64 | 1@60 | 372 | 1@62 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 124 | 1@184 | 1@0 | 372 | 1@62 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 70 | 1@239 | 1@0 | 350 | 1@99 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 96 | 1@50 | 1@46 | 376 | 1@50 | 1@186 | 648 | 1@0 | 1@272 |
|  |  | 30+90 | 150 | 1@77 | 1@73 | 388 | 1@65 | 1@180 | 646 | 1@0 | 1@244 |
|  |  | 40+80 | 204 | 1@105 | 1@100 | 388 | 1@65 | 1@152 | 646 | 1@0 | 1@216 |
|  |  | 50+70 | 210 | 1@109 | 1@80 | 388 | 1@64 | 1@124 | 644 | 1@0 | 1@188 |
|  |  | 60+60 | 212 | 1@109 | 1@52 | 390 | 1@65 | 1@97 | 648 | 1@0 | 1@161 |
|  |  | 70+50 | 210 | 1@109 | 1@24 | 388 | 1@64 | 1@68 | 644 | 1@0 | 1@132 |
|  |  | 80+40 | 204 | 1@116 | 1@0 | 388 | 1@65 | 1@41 | 646 | 1@0 | 1@105 |
|  |  | 90+30 | 148 | 1@172 | 1@0 | 388 | 1@65 | 1@13 | 646 | 1@0 | 1@77 |
|  |  | 100+20 | 94 | 1@227 | 1@0 | 376 | 1@86 | 1@0 | 648 | 1@0 | 1@50 |
| 125 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 25+100 | 124 | 1@64 | 1@60 | 404 | 1@64 | 1@200 | 676 | 1@0 | 1@272 |
|  |  | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 150 | 1@77 | 1@73 | 422 | 1@70 | 1@202 | 702 | 1@0 | 1@272 |
|  |  | 40+90 | 206 | 1@105 | 1@101 | 422 | 1@70 | 1@174 | 702 | 1@0 | 1@244 |
|  |  | 50+80 | 228 | 1@118 | 1@98 | 422 | 1@70 | 1@147 | 700 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@118 | 1@70 | 422 | 1@70 | 1@118 | 702 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@118 | 1@43 | 422 | 1@70 | 1@91 | 702 | 1@0 | 1@161 |
|  |  | 80+50 | 228 | 1@118 | 1@14 | 422 | 1@70 | 1@63 | 700 | 1@0 | 1@132 |
|  |  | 90+40 | 204 | 1@144 | 1@0 | 422 | 1@70 | 1@35 | 702 | 1@0 | 1@105 |
|  |  | 100+30 | 148 | 1@200 | 1@0 | 422 | 1@70 | 1@7 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 206 | 1@105 | 1@101 | 456 | 1@76 | 1@197 | 758 | 1@0 | 1@272 |
|  |  | 50+90 | 248 | 1@127 | 1@117 | 456 | 1@75 | 1@169 | 756 | 1@0 | 1@244 |
|  |  | 60+80 | 248 | 1@128 | 1@89 | 456 | 1@76 | 1@141 | 758 | 1@0 | 1@216 |
|  |  | 70+70 | 248 | 1@127 | 1@61 | 454 | 1@76 | 1@113 | 756 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@128 | 1@34 | 456 | 1@76 | 1@86 | 758 | 1@0 | 1@161 |
|  |  | 90+50 | 248 | 1@127 | 1@5 | 456 | 1@75 | 1@57 | 756 | 1@0 | 1@132 |
|  |  | 100+40 | 204 | 1@172 | 1@0 | 456 | 1@76 | 1@30 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 260 | 1@132 | 1@128 | 488 | 1@81 | 1@191 | 812 | 1@0 | 1@272 |
|  |  | 60+90 | 266 | 1@137 | 1@107 | 490 | 1@81 | 1@163 | 814 | 1@0 | 1@244 |
|  |  | 70+80 | 266 | 1@137 | 1@80 | 488 | 1@81 | 1@135 | 812 | 1@0 | 1@216 |
|  |  | 80+70 | 266 | 1@137 | 1@52 | 488 | 1@81 | 1@107 | 812 | 1@0 | 1@188 |
|  |  | 90+60 | 266 | 1@137 | 1@24 | 490 | 1@81 | 1@80 | 814 | 1@0 | 1@161 |
|  |  | 100+50 | 260 | 1@144 | 1@0 | 488 | 1@81 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 286 | 1@146 | 1@126 | 522 | 1@87 | 1@185 | 870 | 1@0 | 1@272 |
|  |  | 70+90 | 284 | 1@146 | 1@98 | 522 | 1@87 | 1@158 | 868 | 1@0 | 1@244 |
|  |  | 80+80 | 284 | 1@146 | 1@70 | 522 | 1@87 | 1@130 | 868 | 1@0 | 1@216 |
|  |  | 90+70 | 284 | 1@146 | 1@42 | 522 | 1@87 | 1@102 | 868 | 1@0 | 1@188 |
|  |  | 100+60 | 286 | 1@146 | 1@15 | 522 | 1@87 | 1@74 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@155 | 1@117 | 556 | 1@92 | 1@180 | 924 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@155 | 1@89 | 556 | 1@92 | 1@152 | 924 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@155 | 1@61 | 556 | 1@92 | 1@124 | 924 | 1@0 | 1@216 |
|  |  | 100+70 | 304 | 1@155 | 1@33 | 556 | 1@92 | 1@96 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@165 | 1@108 | 588 | 1@98 | 1@174 | 980 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@165 | 1@80 | 588 | 1@98 | 1@146 | 980 | 1@0 | 1@244 |
|  |  | 100+80 | 322 | 1@165 | 1@52 | 588 | 1@98 | 1@118 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 340 | 1@174 | 1@98 | 622 | 1@104 | 1@169 | 1036 | 1@0 | 1@272 |
|  |  | 100+90 | 340 | 1@174 | 1@70 | 622 | 1@104 | 1@141 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@183 | 1@89 | 656 | 1@109 | 1@163 | 1092 | 1@0 | 1@272 |

Table C-3: UL configuration for Option 2-3

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 24 | 1@24 | 1@22 | 63 | 1@21 | 1@58 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 33 | 1@36 | 1@16 | 62 | 1@21 | 1@30 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 23 | 1@57 | 1@0 | 63 | 1@21 | 1@4 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 24 | 1@24 | 1@22 | 79 | 1@24 | 1@77 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 42 | 1@45 | 1@34 | 79 | 1@26 | 1@52 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 41 | 1@45 | 1@6 | 79 | 1@26 | 1@25 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 23 | 1@84 | 1@0 | 79 | 1@28 | 1@0 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 38 | 1@22 | 1@16 | 74 | 1@13 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 38 | 1@22 | 1@2 | 74 | 1@13 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 51 | 1@51 | 1@49 | 96 | 1@31 | 1@74 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 50 | 1@54 | 1@24 | 95 | 1@32 | 1@47 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 49 | 1@58 | 1@0 | 96 | 1@31 | 1@20 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 42 | 1@23 | 1@19 | 90 | 1@15 | 1@35 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 48 | 1@26 | 1@11 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 42 | 1@31 | 1@0 | 90 | 1@15 | 1@8 | 150 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 60 | 1@63 | 1@43 | 112 | 1@37 | 1@69 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 61 | 1@62 | 1@16 | 112 | 1@37 | 1@42 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 56 | 1@31 | 1@20 | 108 | 1@18 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 56 | 1@31 | 1@7 | 108 | 1@18 | 1@20 | 178 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 51 | 1@51 | 1@49 | 129 | 1@42 | 1@118 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 68 | 1@72 | 1@33 | 128 | 1@42 | 1@63 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 50 | 1@111 | 1@0 | 129 | 1@42 | 1@10 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 44 | 1@23 | 1@20 | 124 | 1@20 | 1@57 | 204 | 1@0 | 1@77 |
|  |  | 20+20 | 64 | 1@35 | 1@15 | 124 | 1@20 | 1@30 | 204 | 1@0 | 1@50 |
|  |  | 30+10 | 42 | 1@58 | 1@0 | 124 | 1@20 | 1@3 | 204 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 78 | 1@78 | 1@76 | 145 | 1@47 | 1@112 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 72 | 1@37 | 1@34 | 142 | 1@23 | 1@55 | 232 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 55 | 1@51 | 1@53 | 164 | 1@51 | 1@162 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 86 | 1@90 | 1@69 | 160 | 1@53 | 1@106 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 86 | 1@90 | 1@15 | 160 | 1@53 | 1@52 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 54 | 1@163 | 1@0 | 164 | 1@53 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 48 | 1@23 | 1@22 | 158 | 1@23 | 1@77 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@44 | 1@33 | 156 | 1@26 | 1@52 | 258 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@44 | 1@6 | 156 | 1@26 | 1@25 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 46 | 1@84 | 1@0 | 158 | 1@28 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 82 | 1@78 | 1@80 | 180 | 1@58 | 1@158 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 76 | 1@37 | 1@36 | 176 | 1@28 | 1@77 | 288 | 1@0 | 1@105 |
| 60 | 15 | 10+50 | 54 | 1@51 | 1@52 | 191 | 1@51 | 1@189 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 106 | 1@105 | 1@104 | 195 | 1@64 | 1@152 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@108 | 1@51 | 192 | 1@64 | 1@95 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 104 | 1@113 | 1@0 | 195 | 1@64 | 1@42 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 53 | 1@218 | 1@0 | 191 | 1@80 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 46 | 1@23 | 1@21 | 186 | 1@23 | 1@91 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 98 | 1@50 | 1@47 | 190 | 1@31 | 1@74 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 100 | 1@53 | 1@24 | 188 | 1@31 | 1@46 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@31 | 1@19 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 46 | 1@111 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 104 | 1@105 | 1@102 | 228 | 1@74 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 124 | 1@126 | 1@89 | 228 | 1@74 | 1@141 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 125 | 1@126 | 1@34 | 228 | 1@74 | 1@85 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 103 | 1@168 | 1@0 | 228 | 1@74 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 96 | 1@50 | 1@46 | 224 | 1@36 | 1@96 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 120 | 1@62 | 1@43 | 224 | 1@36 | 1@69 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 120 | 1@62 | 1@15 | 224 | 1@36 | 1@41 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 96 | 1@86 | 1@0 | 224 | 1@36 | 1@14 | 368 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 141 | 1@145 | 1@125 | 260 | 1@85 | 1@184 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 141 | 1@146 | 1@70 | 260 | 1@86 | 1@129 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 142 | 1@144 | 1@15 | 260 | 1@85 | 1@74 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 136 | 1@72 | 1@61 | 256 | 1@42 | 1@91 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 136 | 1@72 | 1@33 | 256 | 1@42 | 1@63 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@42 | 1@36 | 422 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 160 | 1@163 | 1@106 | 293 | 1@97 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 159 | 1@164 | 1@52 | 292 | 1@97 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 154 | 1@81 | 1@51 | 288 | 1@48 | 1@85 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 154 | 1@81 | 1@24 | 288 | 1@48 | 1@58 | 478 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 176 | 1@182 | 1@87 | 325 | 1@108 | 1@162 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 96 | 1@50 | 1@46 | 320 | 1@50 | 1@158 | 536 | 1@0 | 1@216 |
|  |  | 50+50 | 172 | 1@90 | 1@42 | 320 | 1@53 | 1@79 | 532 | 1@0 | 1@132 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 74 | 1@37 | 1@35 | 324 | 1@37 | 1@160 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 124 | 1@64 | 1@60 | 340 | 1@56 | 1@160 | 564 | 1@0 | 1@216 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 46 | 1@23 | 1@21 | 324 | 1@23 | 1@160 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 348 | 1@50 | 1@172 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 150 | 1@77 | 1@73 | 356 | 1@59 | 1@158 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 356 | 1@59 | 1@130 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@100 | 1@62 | 354 | 1@59 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@100 | 1@33 | 354 | 1@59 | 1@73 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 192 | 1@100 | 1@6 | 356 | 1@59 | 1@47 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 150 | 1@143 | 1@0 | 356 | 1@59 | 1@19 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 94 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 44 | 1@252 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 74 | 1@37 | 1@35 | 352 | 1@37 | 1@174 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 124 | 1@64 | 1@60 | 372 | 1@62 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 124 | 1@184 | 1@0 | 372 | 1@62 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 72 | 1@238 | 1@0 | 352 | 1@98 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 96 | 1@50 | 1@46 | 376 | 1@50 | 1@186 | 648 | 1@0 | 1@272 |
|  |  | 30+90 | 150 | 1@77 | 1@73 | 388 | 1@65 | 1@180 | 646 | 1@0 | 1@244 |
|  |  | 40+80 | 206 | 1@105 | 1@101 | 390 | 1@64 | 1@152 | 646 | 1@0 | 1@216 |
|  |  | 50+70 | 210 | 1@109 | 1@80 | 388 | 1@64 | 1@124 | 644 | 1@0 | 1@188 |
|  |  | 60+60 | 212 | 1@109 | 1@52 | 390 | 1@65 | 1@97 | 648 | 1@0 | 1@161 |
|  |  | 70+50 | 210 | 1@109 | 1@24 | 388 | 1@64 | 1@68 | 644 | 1@0 | 1@132 |
|  |  | 80+40 | 204 | 1@116 | 1@0 | 390 | 1@64 | 1@41 | 646 | 1@0 | 1@105 |
|  |  | 90+30 | 150 | 1@171 | 1@0 | 388 | 1@65 | 1@13 | 646 | 1@0 | 1@77 |
|  |  | 100+20 | 94 | 1@227 | 1@0 | 376 | 1@86 | 1@0 | 648 | 1@0 | 1@50 |
| 125 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 25+100 | 124 | 1@64 | 1@60 | 404 | 1@64 | 1@200 | 676 | 1@0 | 1@272 |
|  |  | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 150 | 1@77 | 1@73 | 422 | 1@70 | 1@202 | 702 | 1@0 | 1@272 |
|  |  | 40+90 | 206 | 1@105 | 1@101 | 422 | 1@70 | 1@174 | 702 | 1@0 | 1@244 |
|  |  | 50+80 | 228 | 1@118 | 1@98 | 422 | 1@70 | 1@147 | 700 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@118 | 1@70 | 422 | 1@70 | 1@118 | 702 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@118 | 1@43 | 422 | 1@70 | 1@91 | 702 | 1@0 | 1@161 |
|  |  | 80+50 | 228 | 1@118 | 1@14 | 422 | 1@70 | 1@63 | 700 | 1@0 | 1@132 |
|  |  | 90+40 | 204 | 1@144 | 1@0 | 422 | 1@70 | 1@35 | 702 | 1@0 | 1@105 |
|  |  | 100+30 | 148 | 1@200 | 1@0 | 422 | 1@70 | 1@7 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 206 | 1@105 | 1@101 | 456 | 1@76 | 1@197 | 758 | 1@0 | 1@272 |
|  |  | 50+90 | 248 | 1@127 | 1@117 | 456 | 1@75 | 1@169 | 756 | 1@0 | 1@244 |
|  |  | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@76 | 1@141 | 758 | 1@0 | 1@216 |
|  |  | 70+70 | 248 | 1@127 | 1@61 | 454 | 1@76 | 1@113 | 756 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@128 | 1@34 | 456 | 1@76 | 1@86 | 758 | 1@0 | 1@161 |
|  |  | 90+50 | 248 | 1@127 | 1@5 | 456 | 1@75 | 1@57 | 756 | 1@0 | 1@132 |
|  |  | 100+40 | 206 | 1@171 | 1@0 | 456 | 1@76 | 1@30 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 260 | 1@132 | 1@128 | 488 | 1@81 | 1@191 | 812 | 1@0 | 1@272 |
|  |  | 60+90 | 268 | 1@137 | 1@108 | 490 | 1@81 | 1@163 | 814 | 1@0 | 1@244 |
|  |  | 70+80 | 266 | 1@137 | 1@80 | 488 | 1@81 | 1@135 | 812 | 1@0 | 1@216 |
|  |  | 80+70 | 266 | 1@137 | 1@52 | 488 | 1@81 | 1@107 | 812 | 1@0 | 1@188 |
|  |  | 90+60 | 266 | 1@137 | 1@24 | 490 | 1@81 | 1@80 | 814 | 1@0 | 1@161 |
|  |  | 100+50 | 260 | 1@144 | 1@0 | 488 | 1@81 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 286 | 1@146 | 1@126 | 524 | 1@87 | 1@186 | 870 | 1@0 | 1@272 |
|  |  | 70+90 | 284 | 1@146 | 1@98 | 522 | 1@87 | 1@158 | 868 | 1@0 | 1@244 |
|  |  | 80+80 | 284 | 1@146 | 1@70 | 522 | 1@87 | 1@130 | 868 | 1@0 | 1@216 |
|  |  | 90+70 | 284 | 1@146 | 1@42 | 522 | 1@87 | 1@102 | 868 | 1@0 | 1@188 |
|  |  | 100+60 | 286 | 1@146 | 1@15 | 524 | 1@87 | 1@75 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@155 | 1@117 | 556 | 1@92 | 1@180 | 924 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@155 | 1@89 | 556 | 1@92 | 1@152 | 924 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@155 | 1@61 | 556 | 1@92 | 1@124 | 924 | 1@0 | 1@216 |
|  |  | 100+70 | 304 | 1@155 | 1@33 | 556 | 1@92 | 1@96 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@165 | 1@108 | 588 | 1@98 | 1@174 | 980 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@165 | 1@80 | 588 | 1@98 | 1@146 | 980 | 1@0 | 1@244 |
|  |  | 100+80 | 322 | 1@165 | 1@52 | 588 | 1@98 | 1@118 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 340 | 1@174 | 1@98 | 622 | 1@104 | 1@169 | 1036 | 1@0 | 1@272 |
|  |  | 100+90 | 340 | 1@174 | 1@70 | 622 | 1@104 | 1@141 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@183 | 1@89 | 656 | 1@109 | 1@163 | 1092 | 1@0 | 1@272 |