3GPP TSG-RAN5 Meeting #107 	R5-252512

# St Julian's, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0821 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new capabilities for 16Tx or 32Tx ports of CSI-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS38.306, the minimum number of CSI-RS Tx prots is defined in 8. This means the number of CSI-RS Tx ports with more than 8 ports is optional. However, the applicability conditions are same as mandatory funtion. From TS 38.306 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new items related to CSI-RS Tx ports to A.4.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE not supporting 16 or 32 ports will be tested in 16Tx or 32Tx test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 CR 0969 TS 38.522 CR 0600 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |
| 234 | Support of PUCCH Msg4 HARQ-ACK repetition | 38.306 5.4 | Rel-18 | pc_msg4_harq_Repetition_r18 | No |  |  |
| 235 | Support of type I codebook with at least 16 ports per CSI-RS resource | 38.306, 4.2.7.2 | Rel-15 | pc_typeI_codebook_with_16ports | No |  |  |
| 236 | Support of type I codebook with at least 32 ports per CSI-RS resource | 38.306, 4.2.7.2 | Rel-15 | pc_typeI_codebook_with_32ports | No |  |  |

<<End of change>>