3GPP TSG-RAN5 Meeting #107	R5-252531

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3490 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of LogicalChannelIdentity, Uu-RelayRLC-ChannelID and SL-RLC-ChannelID for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Tables of LogicalChannelIdentity, Uu-RelayRLC-ChannelID and SL-RLC-ChannelID need to be updated for uu&PC5 Relay RLC Channel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Table 4.6.3-67, Table 4.6.3-202C and Table 4.6.6-27A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

–	LogicalChannelIdentity

Table 4.6.3-67: LogicalChannelIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelIdentity | 1 |  | SRB1 |
| LogicalChannelIdentity | 2 |  | SRB2 |
| LogicalChannelIdentity | 3 |  | SRB3 |
| LogicalChannelIdentity | 4 |  | SRB4 |
| LogicalChannelIdentity | n+3 |  | DRBn |
| LogicalChannelIdentity | 2m+18 |  | MRBm AND PTP |
| LogicalChannelIdentity | 2m+19 |  | MRBm AND PTM |
| LogicalChannelIdentity | n+20 |  | uuRelaySRBn |
| LogicalChannelIdentity | n+24 |  | uuRelayDRBn |
| LogicalChannelIdentity | n+10 |  | PC5RelaySRBn |
| LogicalChannelIdentity | n+14 |  | PC5RelayDRBn |

| Condition | Explanation |
| --- | --- |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |
| DRBn | Establishment of DRBn; n=1..16 |
| MRBm | Establishment of MRBm; m = 1.. (16-n) |
| PTM | RLC entity is used for receving PTM transmission |
| PTP | RLC entity is used for receving PTP transmission |
| uuRelaySRBn | Establishment of Uu RelayRLC Channel for SRBn; n=0..3 |
| uuRelayDRBn | Establishment of Uu RelayRLC Channel for DRBn; n=1..16 |
| PC5RelaySRBn | Establishment of PC5 RelayRLC Channel for SRBn; n=1..3 |
| PC5RelayDRBn | Establishment of PC5 RelayRLC Channel for DRBn; n=1..16 |

< Unchanged Section Skipped >

–	Uu-RelayRLC-ChannelID

Table 4.6.3-202C: Uu-RelayRLC-ChannelID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Uu-RelayRLC-ChannelID-r17 | n+1 |  | uuRelaySRBn |
|  | n+4 |  | uuRelayDRBn |

| Condition | Explanation |
| --- | --- |
| uuRelaySRBn | Establishment of Uu RelayRLC Channel for SRBn; n=0..3 |
| uuRelayDRBn | Establishment of Uu RelayRLC Channel l for DRBn; n=1..16 |

< Unchanged Section Skipped >

–	SL-RLC-ChannelID

Table 4.6.6-27A: SL-RLC-ChannelID

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-ChannelID-r17 |  | n+1 |  | PC5RelaySRBn |
|  |  | n+4 |  | PC5RelayDRBn |

| Condition | Explanation |
| --- | --- |
| PC5RelaySRBn | Establishment of PC5 RelayRLC Channel for SRBn; n=1..3 |
| PC5RelayDRBn | Establishment of PC5 RelayRLC Channel for DRBn; n=1..16 |

<End of Changed Section>