3GPP TSG-RAN5 Meeting #107	R5-252568

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5005 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.2.5 Sidelink Relay / L2 U2N / Relay UE / System information acquisition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Added of new test case 12.3.1.2.5 Sidelink Relay / L2 U2N / Relay UE / System information acquisition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition new test case 12.3.1.2.5 Sidelink Relay / L2 U2N / Relay UE / System information acquisition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0589 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.2.5	Sidelink Relay / L2 U2N / Relay UE / System information acquisition

12.3.1.2.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and acting as a L2 U2N Relay UE }

ensure that {

when { UE receives a RemoteUEInformationSidelink message including sl-RequestedSIB-List set to setup }

then { UE transmits a UuMessageTransferSidelink message }

}

(2)

with { UE in NR RRC_CONNECTED state and acting as a L2 U2N Relay UE }

ensure that {

when { UE is receiving the updated SIB(s) from network which has been requested by the connected L2 U2N Remote UE }

then { UE transmits a UuMessageTransferSidelink message }

}

(3)

with { UE in NR RRC_CONNECTED state and acting as a L2 U2N Relay UE }

ensure that {

when { UE is receiving the updated SIB1 from network }

then { UE transmits a UuMessageTransferSidelink message }

}

12.3.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331, clause 5.8.9.8.3 and 5.8.9.9.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.8.9.8.3]

The L2 U2N Relay UE shall:

…

1>	if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List:

2>	if the sl-RequestedSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List:

4>	perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2;

3>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedSIB-List is set to release:

3>	release received SIB request in sl-RequestedSIB-List.

[TS 38.331, clause 5.8.9.9.2]

The L2 U2N Relay UE initiates the Uu message transfer procedure when at least one of the following conditions is met:

1>	upon receiving Paging message related to the connected L2 U2N Remote UE from network (including Paging message within RRCReconfiguration message);

1>	upon acquisition of the SIB(s) requested by the connected L2 U2N Remote UE (as indicated in sl-RequestedSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated SIB(s) from network which has been requested by the connected L2 U2N Remote UE;

1>	upon unsolicited SIB1 forwarding to the connected L2 U2N Remote UE or upon receiving the updated SIB1 from network;

For each associated L2 U2N Remote UE, the L2 U2N Relay UE shall set the contents of UuMessageTransferSidelink message as follows:

1>	include sl-PagingDelivery if the Paging message received from network containing the ue-Identity of the L2 U2N Remote UE;

1>	include sl-SIB1-Delivery if any of the conditions for initiating Uu message transfer procedure related to SIB1 are met;

1>	include sl-SystemInformationDelivery if any of the conditions for initiating Uu message transfer procedure related to System Information are met;

1>	submit the UuMessageTransferSidelink message to lower layers for transmission.

NOTE:	The L2 U2N Relay UE may perform unsolicited forwarding of SIB1 to the L2 U2N Remote UE based on UE implementation.

12.3.1.2.5.3	Test Description

12.3.1.2.5.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1.

-	System information combination NR-31 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used.

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Remote UE

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using condition Monitoring_Restricted_Model_A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4] on NR Cell 1, subclause 4.4A.

12.3.1.2.5.3.2	Test procedure sequence

Table 12.3.1.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 2 | The SS-NW transmits an RRCRelease message to NR-SS-UE1 via the UE. | <-- | Uu Relay RLC Channel: RRCRelease | - | - |
| 3 | The UE relays the RRCRelease message received to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCRelease | - | - |
| 4 | The NR-SS-UE1 transmits an RemoteUEInformationSidelink message with sl-RequestedSIB-List set to setup. | <-- | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 5 | Check: Dose the UE transmit a UuMessageTransferSidelink message with sl-SystemInformationDelivery? | --> | PC5 RRC: UuMessageTransferSidelink | 1 | P |
| 6 | SS-NW change NR Cell 1 SIB12. | - | - | - | - |
| 7 | Wait for 2.1* modification period second for the SS to transmit modified system information.(Note 1) | - | - | - | - |
| 8 | Check: Dose the UE transmit a UuMessageTransferSidelink message with sl-SystemInformationDelivery? | --> | PC5 RRC: UuMessageTransferSidelink | 2 | P |
| 9 | SS-NW change NR Cell 1 SIB1. | - | - | - | - |
| 10 | Wait for 2.1* modification period second for the SS-NW to transmit modified system information.(Note 1) | - | - | - | - |
| 11 | Check: Dose the UE transmit a UuMessageTransferSidelink message with sl-SIB1-Delivery? | --> | PC5 RRC: UuMessageTransferSidelink | 3 | P |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

12.3.1.2.5.3.3	Specific message contents

Table 12.3.1.2.5.3.3-1: RemoteUEInformationSidelink (Step 4, Table 12.3.1.2.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-2A with condition RequestedSIB_Setup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RemoteUEInformationSidelink-r17 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| remoteUEInformationSidelink-r17 SEQUENCE { |  |  |  |
| sl-RequestedSIB-List-r17 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (maxSIB-MessagePlus1-r17)) OF SL-SIB-ReqInfo-r17 { |  |  |  |
| SL-SIB-ReqInfo-r17[1] | sib12 |  |  |
| } |  |  |  |
| sl-PagingInfo-RemoteUE-r17 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.2.5.3.3-2: UuMessageTransferSidelink (Step 5 and 8, Table 12.3.1.2.5.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-8 with condition SystemInformation

Table 12.3.1.2.5.3.3-3: SIB12 for NR Cell 1 (step 8, Table 12.3.1.2.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-r16 ::= SEQUENCE { |  |  |  |
| segmentContainer-r16 | OCTET STRING (CONTAINING SIB12-IEs or segment of SIB12-IEs) | Table 12.3.1.2.5.3.3-3 |  |
| } |  |  |  |

Table 12.3.1.2.5.3.3-4: SIB12-IEs (Table 12.3.1.2.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A with condition L2RelayUE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| t400-r16 | ms1500 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.2.5.3.3-5: SIB1 for NR Cell 1 (step 9, Table 12.3.1.2.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ims-EmergencySupport | Not present |  |  |
| } |  |  |  |

Table 12.3.1.2.5.3.3-6: UuMessageTransferSidelink (Step 11, Table 12.3.1.2.5.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-8 with condition SIB1

<End of Changed Section>