3GPP TSG-RAN5 Meeting #107	R5-252579

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3499 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE-CapabilityRequestFilterCommon for NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | [TS38.331] UE-CapabilityRequestFilterCommon ::= SEQUENCE { mrdc-Request SEQUENCE { omitEN-DC ENUMERATED {true} OPTIONAL, -- Need N includeNR-DC ENUMERATED {true} OPTIONAL, -- Need N includeNE-DC ENUMERATED {true} OPTIONAL -- Need N } OPTIONAL, -- Need N includeNR-DC Only if this field is present, the UE supporting NR-DC shall indicate support for NR-DC in band combinations and include feature set combinations which are applicable to NR-DC. [cl.5.6.1.4] 1> compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received; As provided above, includeNR-DC is required to be include for the UE to report NR-DC band combination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | includeNR-DC is included for all NR-DC in the UE capabiltiy enquiry |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.4-46A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### –	UECapabilityEnquiry

Table 4.6.1-31: UECapabilityEnquiry

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |  |
| ue-CapabilityRAT-RequestList |  | UE-CapabilityRAT-RequestList |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| ue-CapabilityEnquiryExt |  | OCTET STRING Containing UECapabilityEnquiry-v1560-IEs |  | NR-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR-DC | Used in NR-DC test cases |

Table 4.6.4-46AA: UECapabilityEnquiry-v1560-IEs

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry-v1560-IEs ::= SEQUENCE { |  |  |  |
| capabilityRequestFilterCommon | UE-CapabilityRequestFilterCommon |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### –	UE-CapabilityRequestFilterCommon

Table 4.6.4-46A: UE-CapabilityRequestFilterCommon

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRequestFilterCommon ::= SEQUENCE { |  |  |  |
| mrdc-Request SEQUENCE { |  |  |  |
| omitEN-DC | Not checked |  |  |
| includeNR-DC | true |  | NR-DC |
| includeNE-DC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR-DC | Used in NR-DC test cases |

<End of Change>