3GPP TSG-RAN5 Meeting #107	R5-252703

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3377 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of OFF power and time mask test cases for 4Tx UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add following output power dynamics test cases for UE supporting 4Tx UL MIMO: - 6.3D.2_2 Transmit OFF power for UL MIMO for UE supporting 4Tx - 6.3D.3_2 Transmit ON/OFF time mask for UL MIMO for UE supporting 4Tx |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum requirements for Transmit ON/OFF time mask for UL MIMO is updated to also cover 4Tx UL MIMO case. - Test cases mentioned above are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3D.2_2 (new), 6.3D.3.3, 6.3D.3_2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.3D.2_2	Transmit OFF power for UL MIMO for UE supporting 4Tx

6.3D.2_2.1	Test purpose

Same as the test purpose given in clause 6.3D.2.1

6.3D.2_2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO.

6.3D.2_2.3	Minimum conformance requirements

Same as the minimum conformance requirements given in clause 6.3D.2.3.

6.3D.2_2.4	Test description

This test is covered by clause 6.3D.3_2 Transmit ON/OFF time mask for UL MIMO for UE supporting 4Tx.

6.3D.2_2.5	Test requirement

The requirement for the transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3D.2_2.5-1.

Table 6.3D.2_2.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 35 | -50+TT | 33.855 |
| 40 | -50+TT | 38.895 |
| 45 | -50+TT | 43.575 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 70 | -50+TT | 68.07 |
| 80 | -50+TT | 78.15 |
| 90 | -50+TT | 88.23 |
| 100 | -50+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3D.2_2.5-2 |  |  |

Table 6.3D.2_2.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

<End of Change 1>

<Start of Change 2>

6.3D.3.3	Minimum conformance requirements

For UE supporting UL MIMO, the ON/OFF time mask requirements in subclause 6.3.3.2.3 apply to each transmit antenna connector.

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in subclause 6.3.3.2.3 apply to each transmit antenna connector with the UL MIMO configurations specified in Table 6.3D.3.3-1.

Table 6.3D.3.3-1: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1, the requirements in TS 38.101-1 [2] clause 6.3.3 apply when TxD is not indicated, and the requirements in TS 38.101-1 [2] clause 6.3G.3 apply when TxD is indicated.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.3

<Unchanged contents are omitted>

Table 6.3D.3_1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3D.3_2	Transmit ON/OFF time mask for UL MIMO for UE supporting 4Tx

6.3D.3_2.1	Test purpose

Same as the test purpose given in clause 6.3D.3.1.

6.3D.3_2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO.

6.3D.3_2.3	Minimum conformance requirements

Same as the minimum conformance requirements given in clause 6.3D.3.3.

6.3D.3_2.4	Test description

6.3D.3_2.4.1	Initial condition

Same as the initial condition given in clause 6.3D.3.4.1 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.2a for TE diagram and Figure A.3.2.5.6 for UE diagram.

6.3D.3_2.4.2	Test procedure

Same as the test procedure given in clause 6.3D.3.4.2 with following exceptions:

-	Step 1 is replaced by following step:

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3D.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slots 8 for15kHz SCS, on slots 8 and 18 for 30kHz SCS and on slots 17 and 37 for 60kHz SCS. The PDCCH DCI format 0_1 is specified with the condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

6.3D.3_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO and following exceptions.

Table 6.3D.3_2.4.3-1: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  |  |
|  | ms10 |  | SCS15 |
| nrofDownlinkSlots | 6 |  | SCS15 |
|  | 6 |  | SCS30 |
|  | 14 |  | SCS60 |
| nrofDownlinkSymbols | 10 |  | SCS15 |
|  | 6 |  | SCS30 |
|  | 12 |  | SCS60 |
| nrofUplinkSlots | 3 |  | SCS15 SCS30 |
|  | 4 |  | SCS60 |
| nrofUplinkSymbols | 4 |  | SCS30 |
|  | 2 |  | SCS15, |
|  | 8 |  | SCS60 |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

Table 6.3D.3_2.4.3-2: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k2 | 4 |  | SCS15 SCS30 |
|  | 6 |  | SCS60 |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | SCS15 |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | SCS30 |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

Table 6.3D.3_2.4.3-3: P-Max

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | Power class 1.5 |

6.3D.3_2.5	Test requirement

The requirement for the power measured in steps 2, 3 and 4 of the test procedure shall not exceed the values specified in Table 6.3D.3_2.5-1.

Table 6.3D.3_2.5-1: General ON/OFF time mask

|  | BW (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power | Same as test ID 1 in Table 6.2D.2.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth of OFF power is specified in Table 6.3D.3.2.5-2 NOTE 2: TT for each frequency and channel bandwidth of ON power is specified in Table 6.2D.1.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3D.3_2.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

<End of Change 2>