3GPP TSG-RAN5 Meeting #107	R5-252714

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3505 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configurations for 2-step RACH RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Description and configuration values of msgA-PUSCH-NrofPorts in MSGA RMCs in 38.133/38.533 don’t match. - Description of msgA-PUSCH-NrofPorts says that “configure 1 port per CDM group”. - Configuration value “1” is used. Then 2 ports, rather than 1 port, are configured per CDM group as specified in 38.331. msgA-PUSCH-NrofPorts 0 indicates 1 port per CDM group, 1 indicates 2 ports per CDM group. If the field is absent then 4 ports per CDM group are used (see TS 38.213 [13], clause 8.1A).. |
| Summary of change: |  | - Configuration value of msgA-PUSCH-NrofPorts in Table 7.3.1-14A is changed to “0” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Default configuration is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2506686 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

–	MsgA-PUSCH-Config

Table 7.3.1-14A: MsgA-PUSCH-Config

| Derivation Path: Table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 2 |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| msgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos1 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 0 |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha1 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | 1 |  |  |
| nrofInterlacesPerMsgA-PO-r16 | 1 |  |  |
| } |  |  |  |
| msgA-TransformPrecoder-r16 | Not present |  |  |
| msgA-DeltaPreamble-r16 | 1 | Actual value = 2dB |  |
| } |  |  |  |

<End of Change 1>