3GPP TSG-RAN5 Meeting #107	R5-252715

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3934 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 2-step RACH RMCs in Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Description and configuration values of msgA-PUSCH-NrofPorts in MSGA RMCs in 38.133/38.533 don’t match. - Description of msgA-PUSCH-NrofPorts says that “configure 1 port per CDM group”. - Configuration value “1” is used. Then 2 ports, rather than 1 port, are configured per CDM group as specified in 38.331. msgA-PUSCH-NrofPorts 0 indicates 1 port per CDM group, 1 indicates 2 ports per CDM group. If the field is absent then 4 ports per CDM group are used (see TS 38.213 [13], clause 8.1A).. |
| Summary of change: |  | - Configuration value of msgA-PUSCH-NrofPorts in Table A.7A.1-1 and A.7A.2-1 are changed to “0” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Default configuration is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7A.1, A.7A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2506686 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7A	Reference MsgA configurations

This clause provides the typical PRACH and PUSCH configurations for MsgA used for all RRM test cases defined in this test specification. Parameters not listed in this clause can be derived from the configuration of each test.

A.7A.1	MsgA configurations for FR1

Table A.7A.1-1 defines the MsgA configurations for FR1. Each of the MsgA configurations defined in Table A.7A.1-1 have different applicabilities:

-	MsgA.1 FR1 for SSB-based 2-step contention based random access in FR1.

-	MsgA.2 FR1 for SSB-based 2-step non-contention based random access in FR1.

Table A.7A.1-1 Parameters for MsgA Configurations for FR1

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR1 | MsgA.2 FR1 |  |
| msgA-prach-ConfigurationIndex | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 14 | 14 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 0 | 0 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 2dB | 2dB | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

<End of Change 1>

<Start of Change 2>

A.7A.2	MsgA configurations for FR2

Table A.7A.2-1 defines the MsgA configurations for FR2. Each of the MsgA configurations defined in Table A.7A.2-1 have different applicabilities:

-	MsgA.1 FR2 for SSB-based contention based random access in FR2.

-	MsgA.2 FR2 for SSB-based non-contention based random access in FR2.

Table A.7A.2-1 Parameters for MsgA Configurations for FR2

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR2 | MsgA.2 FR2 |  |
| msgA-prach-ConfigurationIndex | 190 | 190 | Preamble Format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 10 | 10 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 0 | 0 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 2 | 2 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

<End of Change 2>