3GPP TSG-RAN5 Meeting #107	R5-252827

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1160 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case of beam correspondence for IDLE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test configuration and test procedure is missing for testing of beam correspondence for IDLE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the genral test procedure of EIRP spherical coverage for IDLE mode in Annex K.1.5a Adding the test procedures for beam correspondence for IDLE Addign the PRACH configuration for beam correspondence for IDLE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The beam correspondence for IDLE can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3, K.1.5a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.6.3	Beam Correspondence in RRC_INACTIVE and initial access

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The test case is incomplete for Test procedure, Message Contents and Test Requirement.

- TP Analysis is pending

6.6.3.1	Test purpose

The purpose of this test is to verify the UE RACH MSG1 performance and uplink spatial coverage of the UE in expected directions is acceptable.

Transmission of the wrong power increases interference or transmission errors in the uplink channel.

6.6.3.2	Test applicability

This test case applies to all types of NR UEs release 18 and forward.

6.6.3.3	Minimum conformance requirements

The minimum conformance requirements for beam correspondence in RRC_INACTIVE and initial access are same as specified in section 6.2.1.1.3.3.

For the beam correspondence requirement for UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

-	If a UE meets UE beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE

The normative reference for this requirement is TS 38.101-2 [3] clause 6.6.3.

6.6.3.4	Test description

6.6.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.6.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.4.1-1: Test Configuration Table

| Default Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 | Normal |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 | Low range, Mid Range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 | 100 MHz |
| Test SCS as specified in Table 5.3.5-1 | 120 kHz |
| PRACH preamble format |  |
| PRACH Configuration Index | 143 |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.6.3.4.3.

6.6.3.4.2	Test procedure

1.	The side conditions for SSB-based L1-RSRP measurements are applied as per clause 6.6.1.3.3.1.1 for PC3.

2.	Measure UE EIRP value for each grid point according to the EIRP spherical coverage procedure defined in Annex K.1.5a, and obtain a cumulative distribution function (CDF) of all EIRP dBm values. Alternatively, UE EIRP measurement for each grid point could be done according to Tx Fast spherical coverage procedure defined in Annex K.1.5.1. The accumulated measuring duration is 1ms. EIRP is calculated considering both polarizations, theta and phi.

3.	Identify the EIRP dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 6.6.3.5.

6.6.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with following exceptions.

Table 6.6.3.4.3-1: RACH-ConfigCommon

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneEighth | n4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |

Table 6.6.3.4.3-2: RACH-ConfigGeneric (in Table 6.6.3.4.3-1)

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 143 |  |  |
| preambleReceivedTargetPower | -118 |  |  |
| preambleTransMax | n200 |  |  |
| ra-ResponseWindow | sl1 |  |  |
| } |  |  |  |

6.6.3.5	Test requirement

The defined %-tile EIRP in measurement distribution derived in step 3 shall exceed the values specified in Table 6.6.3.5-1 to Table 6.6.3.5-4.

Table 6.6.3.5-1: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 30.0-TT |
| n258 | 30.0-TT |
| n260 | 28.0-TT |
| n261 | 30.0-TT |

Table 6.6.3.5-1a: Test Tolerance (UE spherical coverage for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | FFS dB | FFS dB |

Table 6.6.3.5-2: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 16.0-TT |
| n258 | 16.0-TT |
| n260 |  |
| n261 | 16.0-TT |

Table 6.6.3.5-3: UE spherical coverage for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 9.5-TT-MBs,n |  |  |  |  |  |
| 2 | n258 |  | 9.5-TT-MBs,n |  |  |  |  |
| 3 | n259 |  |  | 3.8-TT-MBs,n |  |  |  |
| 4 | n260 |  |  |  | 6-TT-MBs,n |  |  |
| 5 | n261 |  |  |  |  | 9.5-TT-MBs,n |  |
| 6 | n257, n261 | 9.5-TT-MBs,n |  |  |  | 9.5-TT-MBs,n | MBs,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 6-TT-MBs,n | 9.5-TT-MBs,n | MBs,n relaxation is 0 dB for n260 |
| Note 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.6.3.5-3a: Test Tolerance (UE spherical coverage for Power class 3)

| Test Metric | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| Max device size ≤ 30 cm | FFS dB | FFS dB | FFS dB |

Table 6.6.3.5-4: FFS

Table 6.6.3.5-5: UE spherical coverage for power class 5

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 20.0-TT-MBs,n |
| n258 | 20.4-TT-MBs,n |
| Note 1: MBs,n = 0 for single band UE. For multi-band UEs, MBs,n is defined in table 6.2.1.1.3.5-4. |  |

Table 6.6.3.5-5a: Test Tolerance (UE spherical coverage for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | FFS dB |

Table 6.6.3.5-6: FFS

Table 6.6.3.5-7: UE spherical coverage for power class 7

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 3.5-TT-MBs,n |  |  |  |  |  |
| 2 | n258 |  | 3.5-TT-MBs,n |  |  |  |  |
| 3 | n261 |  |  |  |  | 3.5-TT-MBs,n |  |
| 4 | n257, n261 | 3.5-TT-MBs,n |  |  |  | 3.5-TT-MBs,n | MBs,n relaxation is 0 dB |
| Note 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.6.3.5-7a: Test Tolerance (UE spherical coverage for Power class 7)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | FFS dB |

< Unchanged Text Skipped >

K.1.5	EIRP spherical coverage

The EIRP results from the TX beam peak search procedures of K.1.1, using the minimum number of grid points as described in Annex M.2.1 can be re-used for EIRP spherical coverage.

In case a coarse beam peak grid is used for TX beam peak search, using the minimum number of grid points defined in Annex M.3.1.1, the EIRP results can be re-used for EIRP spherical coverage.

K.1.5.0	Tx Spherical Coverage Method

In case a separate test is performed for EIRP spherical coverage, the procedure as per K.1.1 should be followed using the minimum number of grid points defined in Annex M.3.1.1 for spherical coverage.

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maximum(EIRP(PolLink=), EIRP(PolLink=)) for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1, to account for the denser grid point distribution near the poles. In case of Clenshaw-Curtis weights, when just a single measurement at the poles is performed, the PDF probability contributions need to be scaled by M*W()/W(=90°) to account for the M longitudes at those two grid points. When using constant density grids, these corrections are not needed.

K.1.5.1	Tx Fast Spherical Coverage Method

K.1.5.1.1	Introduction

The Fast Spherical Coverage Method is a test method providing an optimized test time for Tx spherical coverage measurements. This method is applicable to constant density and constant step size grid type. Instead of measuring all grid points as per Annex M, as required by the test procedure defined in Annex K.1.5, this method requires only a reduced number of grid points to be measured.

K.1.5.1.2	Description

To use this method, apply the following steps

1)	During the EIRP Spherical coverage measurements, calculate the EIRP result for the grid point as EIRPspherical = Max(EIRP(PolLink= θ), EIRP(PolLink= ϕ)) starting with Ngrid, meas, PASS =0. If the EIRPspherical value is above the Min EIRP spherical coverage limit increase Ngrid, meas, PASS by 1.

2)	Calculate the percentage of total grid points measured thus far above the EIRP spherical coverage requirement limit Ngrid, meas, PASS compared to the total number of grid points on the measurement grid Ngrid,total.

3)	If the percentage calculated in step 2) is equal to or higher than (100 - nth percentile for EIRP spherical coverage)%, pass the device, otherwise continue to step 4. If all grid points have been measured, calculate the CDF for all grid points and pass the UE if the derived %-tile EIRP in measurement distribution exceeds the requirement. Otherwise fail the UE.

4)	Advance to the next grid point and repeat the steps until measurements within zenith range 0º≤ θ ≤[90]º have been completed

NOTE 1: For test systems where the device repositioning approach outlined in Annex N is applied, the grid points of up to a zenith of [90]° are allowed to be measured in the first hemisphere before the device needs to be placed in the second orientation.

#### K.1.5.1.3	Measurement uncertainties

Same as when test procedure described in clause K.1.5.0 is used.

K.1.5a	EIRP spherical coverage for IDLE mode

In this EIRP spherical coverage for IDLE mode procedure, the minimum number of grid points defined in Annex M.3.1.1 for spherical coverage is used.

The measurement procedure includes the following steps:

1)	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-7 [3] to mount the DUT inside the QZ.

2)	Position the DUT in DUT Orientation 1 from Tables N.2-1 through N.2-7 [3].

3)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

4)	SS activates the UE Beamlock_IDLE Function (UBF_IDLE) by performing the procedure as specified in TS 38.509 [10] clause 5.4a.2.

5)	SS transmits an RRCRelease message.

6)	SS transmits a Paging message. The UE’s Random access procedure is initiated.

7)	Through its beam correspondence procedure, DUT refines its TX beam by autonomously choosing the corresponding TX beam for PRACH preamble transmission using downlink reference signals to transmit in the direction of the incoming DL signal. DUT doesn’t transmit PRACH preamble in this step.

8)	The DUT locks the TX beam and starts transmiting the PRACH preambles.

9)	SS detects UE’s transmit power on the PRACH occasions. If on PRACH occasion n the transmit power exceeds the Pmin specified in 6.3.1.5, SS measures the mean power Pmeas (PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) starting from PRACH occasion n until accumulated measurement period achieves 1ms.

10)	Calculate EIRP (PolMeas= PolLink=) by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas= PolLink=).

11)	Measure the mean power Pmeas (PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment.

12)	Calculate EIRP (PolMeas= PolLink=) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,φ, and frequency to the measured power Pmeas (PolMeas= PolLink=).

13)	Calculate total EIRP(PolLink=) = EIRP(PolMeas= PolLink=) + EIRP(PolMeas= PolLink=).

14)	SS transmits RAR and completes the Random access procedure, and ensures the UE is in state RRC_CONNECTED.

15)	SS deactivates the UE Beamlock_IDLE Function (UBF_IDLE) by performing the procedure as specified in TS 38.509 [10] clause 5.4a.3.

16)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization.

17)	Repeat steps 4 through 16 and get the result of total EIRP(PolLink=) = EIRP(PolMeas= PolLink=) + EIRP(PolMeas= PolLink=)

18)	Advance to the next grid point and repeat steps 3 through 17 until measurements within zenith range 0o≤≤90o  have been completed

19)	After the measurements within zenith range 0o≤≤90o have been completed and

a) if the re-positioning concept is applied to the TX test cases, position the device in DUT Orientation 2 (either Options 1 or 2) from Tables N.2-1 through N.2-7 [3] for the Alignment Option selected in Step 1. For the TX beam peak search in the second hemisphere, perform steps 3 through 15 for the range of zenith angles 90o>≥0o.

b) if the re-positioning concept is not applied to the TX test cases, continue steps 3 through 15 for the range of zenith angles 90o<≤180o

20)	The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maximum(EIRP(PolLink=), EIRP(PolLink=)) for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1, to account for the denser grid point distribution near the poles. In case of Clenshaw-Curtis weights, when just a single measurement at the poles is performed, the PDF probability contributions need to be scaled by M*W()/W(=90°) to account for the M longitudes at those two grid points. When using constant density grids, these corrections are not needed.

The Fast Spherical Coverage Method specified in K.1.5.1 could be applied.

< End of Changes >