3GPP TSG-RAN5 Meeting #107	R5-252924

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5465 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 22.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. TP1, 3 and 4 mentions wrong RRC state 2. At step 21 procedure is not aligned with the message 3. Step 26 refers to the wrong test step 4. In Table 22.1.6.3.3-3: RRCConnectionRelease-NB the value of pur-ConfigID-r16 IE is of wrong type and mandatory IE npdcch-Config-r16 is marked as not present. 5. Typo in derivation path for Table 22.1.6.3.3-4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Corrected RRC state in TP1 2. Aligned procedure with message at step 21 3. Corrected step reference in step 26 4. Corrected specific message contents for Table 22.1.6.3.3-3 RRCConnectionRelease-NB 5. Corrected typo |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain incorrect and specific message incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.1.6	NB-IoT / NTN / User Plane CIoT Optimization / PUR

##### 22.1.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_SUSPENDED state with pur-Config }

ensure that {

when { UE has data to be sent in the UL using PUR}

then { UE sends RRCEarlyDataRequest-NB using the PUR uplink grant, and the associated HARQ information }

}

(2)

with { UE in E-UTRA RRC_IDLE state and PUR-timeAlignmentTimer running and having previously sent an RRCResumeRequest-NB message using PUR}

ensure that {

when { UE receives RRCConnectionSetup-NB }

then { UE responds with RRCConnectionSetupComplete-NB }

}

(3)

with { UE in E-UTRA RRC_SUSPENDED state with pur-config available and having sent RRCConnectionResumeRequest-NB without using PUR}

ensure that {

when { The SS sends RRCConnectionSetup-NB}

then { UE replies with RRCConnectionSetuoComplete-NB including pur-ConfigID }

}

(4)

with { UE in E-UTRA RRC_SUSPENDED

state and PUR-timeAlignmentTimer has expired }

ensure that {

when { The UE has UL data to send}

then { UE sends RRCConnectionResumeRequest-NB including the resumeIdentity and AS security context stored and no EDT data on DTCH and without using the PUR uplink grant }

}

##### 22.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.321 clauses 5.4.7.1 and 5.4.7.2 and TS 36.331 clause 5.3.3.1c and 5.3.3.4. Unless otherwise stated these are Rel-17 requirements

[TS 36.321 clause 5.4.7.1]

Transmission using PUR is initiated by the RRC layer. When transmission using PUR is initiated, RRC layer provides MAC with the following information:

-	PUR-RNTI;

-	Duration of PUR response window pur-ResponseWindowTimer;

-	UL grant information.

If the MAC entity has a PUR-RNTI, the MAC entity shall for each TTI for which RRC layer has provided uplink grant for transmission using PUR:

-	deliver the uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer, which starts at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes. While pur-ResponseWindowTimer is running, the MAC entity shall:

-	if the PDCCH transmission is addressed to the PUR-RNTI and contains an UL grant for a retransmission:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes.

-	if L1 ACK for transmission using PUR is received from lower layers; or

-	if PDCCH transmission is addressed to the PUR-RNTI and the MAC PDU is successfully decoded:

-	stop pur-ResponseWindowTimer;

-	if L1 ACK for transmission using PUR is received from lower layers or the MAC PDU contains only Timing Advance Command MAC control element:

-	indicate to upper layers the transmission using PUR was successful;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	else if fallback indication for PUR is received from lower layers:

-	stop pur-ResponseWindowTimer;

-	indicate to upper layers PUR fallback indication is received;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	if the pur-ResponseWindowTimer expires:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

[TS 36.321, clause 5.4.7.2]

MAC entity may be configured with timer pur-TimeAlignmentTimer by upper layers as specified in TS 36.331 [8], clause 5.3.8.3, TS 36.331 clause 5.3.3.1c.

The MAC entity shall:

-	when pur-TimeAlignmentTimer configuration is received from upper layers:

-	start or restart pur-TimeAlignmentTimer.

-	when pur-TimeAlignmentTimer is released by upper layers:

-	stop the pur-TimeAlignmentTimer, if running.

-	when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5] and if a NTA has been stored or maintained:

-	if the Timing Advance Command MAC control element or PDCCH indicating timing advance adjustment is addressed with a PUR-RNTI:

-	apply the Timing Advance Command or the timing advance adjustment;

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted.

-	upon considering a Random Access procedure successfully completed:

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted;

-	if a temporary NTA has been stored, delete the stored temporary NTA.

-	upon considering a Random Access procedure unsuccessfully completed, if a temporary NTA has been stored:

-	set the NTA to the stored temporary NTA;

-	delete the stored temporary NTA.

Upon request from upper layers, MAC entity shall indicate whether pur-TimeAlignmentTimer is running.

[TS 36.331 clause 5.3.1.1]

In case of CP-EDT or CP transmission using PUR, the data are appended in the RRCEarlyDataRequest and RRCEarlyDataComplete messages, if available, and sent over SRB0. In case of UP-EDT or UP transmission using PUR, security is re-activated prior to transmission of RRC message using the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure and the radio bearers are re-established. The uplink data are transmitted ciphered on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. In the downlink, the data, if available, are transmitted on DTCH multiplexed with the RRCConnectionRelease message on DCCH. In response to a request for EDT or transmission using PUR, E-UTRA/EPC or E-UTRA/5GC may also choose to establish or resume the RRC connection.

[TS 36.331, clause 5.3.3.1]

Figure 5.3.3.1-3: RRC connection resume (suspended RRC connection or RRC_INACTIVE), or UP-EDT fallback or fallback from UP transmission using PUR to RRC connection resume, successful

Figure 5.3.3.1-4: RRC connection resume (suspended RRC connection or RRC_INACTIVE) or UP-EDT fallback or fallback from UP transmission using PUR to RRC connection establishment, successful

Figure 5.3.3.1-5: RRC connection resume or UP-EDT or UP transmission using PUR, network reject (suspended RRC connection or RRC_INACTIVE) or release (suspended RRC connection)

Figure 5.3.3.1-6: RRC connection resume (RRC_INACTIVE), network release or suspend or UP-EDT or UP transmission using PUR, successful

Figure 5.3.3.1-7: CP-EDT or CP transmission using PUR, successful

Figure 5.3.3.1-7a: CP transmission using PUR, successful

Figure 5.3.3.1-8: CP-EDT fallback or fallback from CP transmission using PUR to RRC connection establishment, successful

Figure 5.3.3.1-9: CP-EDT or CP transmission using PUR, network reject

The purpose of this procedure is to establish an RRC connection, to resume a suspended RRC connection, to move the UE from RRC_INACTIVE to RRC_CONNECTED, to perform EDT or to perform transmission using PUR. RRC connection establishment involves SRB1 (and SRB1bis for NB-IoT) establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to E-UTRAN.

E-UTRAN applies the procedure as follows:

-	When establishing an RRC connection:

-	to establish SRB1 and, for NB-IoT, SRB1bis;

-	When resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

-	to restore the AS configuration from a stored context including resuming SRB(s) and DRB(s);

-	When performing EDT;

-	When performing transmission using PUR.

[TS 36.331, clause 5.3.3.1c]

A BL UE, UE in CE or NB-IoT UE can initiate transmission using PUR when all of the following conditions are fulfilled:

1>	the UE has a valid PUR configuration for the serving cell as specified in 5.3.3.20;

1>	the UE has a valid timing alignment value as specified in 5.3.3.19;

1>	the upper layers request establishment of an RRC connection; or the upper layers request resumption of an RRC connection and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess;

1>	for CP transmission using PUR, the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS configured for PUR.

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the establishment or resumption request is suitable for transmission using PUR.

[TS 36.331, clause 5.3.3.4]

2>	if the RRCConnectionSetup is not in response to transmission using PUR and the UE has a stored pur-Config including pur-ConfigID:

3>	include the stored pur-ConfigID;

##### 22.1.6.3	Test description

22.1.6.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

The UE is configured to use PUR to establish or resume the RRC connection whenever it is possible as per TS 36.331 clause 5.3.3.1c.

Preamble

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode B and configured to return no data in UL according to [18] in Ncell 1.

22.1.6.3.2	Test procedure sequence

Table 22.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | SS transmits 1 IP packet message containing downlink user data of 1 octet. | <-- | IP Packet (containing 1 octet user data) | - | - |
| - | EXCEPTION: In parallel with below steps the parallel behaviour defined in table 22.1.5.3.2-2 might happen each time when the UE goes from RRC_IDLE to RRC_CONNECTED state. | - | - | - | - |
| 2 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer > IP PDU delay). (Note 1) | <-- | RRCConnectionRelease-NB | - | - |
| 3 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 1 and the EDT data on DTCH ? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, EDT data on DTCH) | 1 | P |
| 4 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 5 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 6 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 7 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 8 | SS transmits 1 IP packet message containing downlink user data of 1 octet. | <-- | IP Packet (containing 1 octet user data) | - | - |
| 9 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer > IP PDU delay). (Note 1) | <-- | RRCConnectionRelease-NB | - | - |
| 10 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 8 and the EDT data on DTCH using PUR? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, EDT data on DTCH) | 2, 3 | P |
| 11 | The SS sends RRCConnectionSetup-NB | <-- | RRCConnectionSetup-NB | - | - |
| 12 | The UE sends RRCConnectionSetupComplete-NB. | --> | RRCConnectionSetupComplete-NB | - | - |
| 13 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 14 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 15 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 16 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 17 | SS transmits 1 IP packet message containing downlink user data of 1 octet. | <-- | IP Packet (containing 1 octet user data) | - | - |
| 18 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer< IP PDU delay). (Note 1) | <-- | RRCConnectionRelease-NB | - | - |
| 19 | Check: Does the UE send RRCConnectionResumeRequest-NB? | --> | RRCConnectionResumeRequest-NB | 4 | P |
| 20 | The SS sends RRCConnectionResume-NB | <-- | RRCConnectionResume-NB | - | - |
| 21 | The UE sends RRCConnectionResumeComplete -NB with SERVICE REQUEST . | --> | RRC: RRCConnectionResumeComplete -NB NAS: SERVICE REQUEST | - | - |
| 22-25 | Steps 5 to 8 of test procedure in Table 8.1.5A.9.3-1is TS 36.508. | - | - | - | - |
| 26 | The UE loops back the IP packet received in Step 17. | --> | IP packet (containing 1 octet user data) | - | - |
| Note 1: pur-TimeAlignmentTimer = pur-TimeAlignmentTimer-r16 * periodicityAndOffset-r16 * H-SFN duration (10.24s) = 81.92s |  |  |  |  |  |

Table 22.1.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | The UE sends PURConfigurationRequest | --> | RRC: PURConfigurationRequest-NB | - | - |

22.1.6.3.3	Specific message contents

Table 22.1.6.3.3-1: CLOSE UE TEST LOOP (preamble and step 6 and 15)

| Derivation path: TS 36.508 Table 4.7A-3, Condition: UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| IP PDU delay | '00000101 'B | 5 seconds delay | Preamble and Step 6 |
| IP PDU delay | '01011010 'B | 90 seconds delay | Step 15 |

Table 22.1.6.3.3-2: SystemInformationBlockType2-NB (Ncell1 in all steps of Table 22.1.3.3.2-2)

| Derivation Path: TS 36.508 clause 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| earlySecurityReactivation-r16 | True |  |  |
| up-PUR-EPC-r16 | True |  |  |
| } |  |  |  |

Table 22.1.6.3.3-3: RRCConnectionRelease-NB (steps 2, 9 and 18, Table 22.1.6.3.2-1)

| Derivation path: TS 36.508 table 8.1.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | rrc-Suspend |  |  |
| resumeIdentity-r13 | BIT STRING (SIZE(40)) |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v1430 | Not present |  |  |
| extendedWaitTime-CPdata-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| drb-ContinueROHC-r15 | Not present |  |  |
| nextHopChainingCount-r15 | 0 |  | Step 2 |
| nextHopChainingCount-r15 | 1 |  | Step 9 |
| nextHopChainingCount-r15 | 2 |  | Step 18 |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pur-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pur-ConfigID-r16 | '00000000000000000001’B |  |  |
| pur-TimeAlignmentTimer-r16 | 1 |  |  |
| pur-NRSRP-ChangeThreshold-r16 | Not present |  |  |
| pur-ImplicitReleaseAfter-r16 | n8 |  |  |
| pur-RNTI-r16 | ’0001 0001 0001 0001’B |  |  |
| pur-ResponseWindowTimer-r16 | pp32 |  |  |
| pur-StartTimeParameters-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-r16 CHOICE { |  |  |  |
| periodicity8 | 1 |  |  |
| } |  |  |  |
| startSFN-r16 | 0 |  |  |
| startSubframe-r16 | 0 |  |  |
| hsfn-LSB-Info-r16 | 0 |  |  |
| } |  |  |  |
| pur-NumOccasions-r16 | infinite |  |  |
| pur-PhysicalConfig-r16 SEQUENCE { |  |  |  |
| carrierConfig-r16 SEQUENCE { |  |  |  |
| dl-CarrierConfig-r13 SEQUENCE { |  |  |  |
| dl-CarrierFreq-r13 | Downlink EARFCN of NCell 1 |  |  |
| downlinkBitmapNonAnchor-r13 | Not present |  |  |
| dl-GapNonAnchor-r13 | Not present |  |  |
| inbandCarrierInfo-r13 | Not present |  |  |
| nrs-PowerOffsetNonAnchor-v1330 | Not present |  |  |
| dl-GapNonAnchor-v1530 | Not present |  |  |
| dl-CarrierFreq-v1550 | Not present |  |  |
| } |  |  |  |
| ul-CarrierConfig-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Uplink EARFCN of NCell 1 |  |  |
| tdd-UL-DL-AlignmentOffset-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| npusch-NumRUsIndex-r16 | 0 |  |  |
| npusch-NumRepetitionsIndex-r16 | 0 |  |  |
| npusch-SubCarrierSetIndex-r16 CHOICE { |  |  |  |
| khz3dot75 | 0 |  |  |
| } |  |  |  |
| npusch-MCS-r16 CHOICE { |  |  |  |
| singleTone | 1 |  |  |
| } |  |  |  |
| p0-UE-NPUSCH-r16 | 0 |  |  |
| alpha-r16 | al0 |  |  |
| npusch-CyclicShift-r16 | n0 |  |  |
| npdcch-Config-r16 | Table 8.1.6.3-3 in TS 36.508. | NPDCCH-ConfigDedicated-NB-DEFAULT |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.6.3.3-4: PURConfigurationRequest-NB (step 1 Table 22.1.6.3.2-2)

| Derivation path: TS 36.508 Table 8.1.6.1-2a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PURConfigurationRequest-NB-r16 ::= SEQUENCE { |  |  |  |
| purConfigurationRequest-r16 SEQUENCE { |  |  |  |
| pur-ConfigRequest-r16 CHOICE { |  |  |  |
| pur-SetupRequest | Not checked. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |