3GPP TSG RAN5 Meeting #107	R5-252929

St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda Item:	6.4.7

Source:	Apple Solutions

Title:	FR2 FBG5 CA bandwidth support inclusion in the UE capability enquiry

Document for:	Discussion

# 1	Introduction

Verizon has a requirement for UE to include FR2 FBG5 CA bandwidth support in UE capability information message. This is Rel-17 feature and is currently not handled/validated in any of the UE Capability related test cases defined in [1].

TS 38.331: UE-Capability Request Filter Common field descriptions

fallbackGroupFiveRequest

Only if this field is present, the UE supporting FR2 CA bandwidth class from fallback group 5 shall include band combinations with FR2 CA bandwidth class from fallback group5, and shall omit band combinations with FR2 CA bandwidth class from fallback group 2 or 3 (see TS 38.101-2 [39]) with same or lower capabilities.

# 2	Discussion

Propose new test scenario to include “fallbackGroupFiveRequest_r17”  IE in the UE capability enquiry message.

# 3	Proposal

Proposal-1:Update the existing test case (TC 8.1.5.1.1 or TC 8.1.5.9.1) to include FR2 FBG5 CA bandwidth support related IE in the UE capability enquiry.

Proposal-2: Implement a new test (TC 8.1.5.1.x or TC 8.1.5.9.x) to include FR2 FBG5 IE in the UE capability enquiry. A new test might be required if the test applicability needs to be changed/modified.

# 4	References

- TS38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol