# Title:	Test Tolerances for NR SA FR1 RRC re-establishment for Satellite Access

# Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for NR SA FR1 RRC re-establishment for Satellite Access.

The test case in question, from TS 38.533, is:

- 14.2.2.1.1, NR SA FR1 RRC Re-establishment for Satellite Access

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Test prototype definition in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v17.17.0 Annex A.

##### A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1

A.14.2.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. These tests will verify the requirements in clause 6.2C.1.

The test parameters are given in table A.14.2.2.1.1.1-1, table A.14.2.2.1.1.1-2 and table A.14.2.2.1.1.1-3  below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, Cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

The UE shall be provided with the valid information about the SAN serving the each cell in the test before the test.

Table A.14.2.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.2.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell 1 |  |
|  | Neighbour cells |  | 1, 2 | Cell 2 |  |
| Final condition | Active cell |  | 1, 2 | Cell 2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
| N310 |  | - | 1, 2 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.2 |  |
| DRX cycle length |  | s | 1, 2 | OFF |  |
| PRACH configuration |  |  | 1, 2 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2 | 5 |  |
| T2 |  | ms | 1, 2 | 640 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1C in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1C.5 in TS 38.133 ) |
| T3 |  | s | 1, 2 | 2 |  |

Table A.14.2.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | 1 | SSC.1 |  |  | NSC.1 |  |  |
|  |  | 2 | SSC.2 |  |  | NSC.2 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP confgiuration |  | 1, 2 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
|  | dB | 1, 2 | 1.54 | -infinity | -infinity | -3.79 | 4 | 4 |
| Note2 | dBm/SCS | 1, 2 | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |
|  | dB | 1, 2 | 7 | -infinity | -infinity | 4 | 4 | 4 |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -91 | -infinity | -infinity | -94 | -94 | -94 |
| Io | dBm/9.36 MHz | 1, 2 | -60.74 | -64.59 | -64.59 | -60.74 | -64.59 | -64.59 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.14.2.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to Cell 2 for sending the RRCReestablishmentRequest message to Cell 2.

The RRC re-establishment delay to a known NR intra frequency cell shall be less than 1.6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90 %.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 200 ms

TSI = 1280 ms, provided that SIB1 and SIB19 are scheduled with 20 ms period; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 1545 ms, allow 1.6 s in the test case.

# 3. 	Discussion

The test case consists of 2 test configurations, with identical power distribution, and with 3 time periods: T1, T2 and T3. To analyse the test case, it is helpful to visualise these as pie-charts showing the distribution of power, as below:

Power distribution for test A.14.2.2.1.1 in 38.133 v17.17.0

During T1, NR Cell 1 and NR Cell 2 are both present and NR Cell 1 is 3 dB stronger than NR Cell 2.

During T2 and T3, NR Cell 1 does not exist.

The AWGN remains the same absolute power during T1, T2 and T3, the design of the test case requires NR Cell 1 and NR Cell 2 to be detectable during T1, but the relative SS-RSRP difference between NR Cell 1 and NR Cell 2 does not appear to be critical.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.14.2.2.1.1.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

The key parameters appear in section a) of the accompanying spreadsheet. The layout for NR cell 1 and NR cell 2 is similar to Table A.14.2.2.1.1.1-3 in TS 38.133, but the AWGN used to model the other NR cells is given a separate set of columns. This allows the spreadsheet calculations to be done in a consistent way.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In general, all the values are listed for each time interval T1, T2 and T3 but as Cell 1 is not present during T2 and T3 those entries are greyed out.

In this test case at the start of T2, NR Cell 1 disappears, and the UE is expected to re-establish the link using NR Cell 2.

c) Uncertainties

The SS provides AWGN, 2 NR intra-frequency cells to the UE. We propose to control the following parameters:

- AWGN Absolute power on cell 1 frequency, Noc ±1.5 dB

- Ratio of NR cell 1 signal to noise, Ês1 / Noc ±0.3 dB

- Ratio of NR cell 2 signal to noise, Ês2 / Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary. For these test cases the signals are not faded, and the values for Ês / Noc are chosen to be similar to other intra-frequency test cases. The absolute level of Noc is not critical, and is specified as ±1.5 dB.

A diagram giving the powers of NR Cell 1 and NR Cell 2 during T1, T2 and T3 is provided in section 3 of this document. This is the most appropriate view to analyse the critical parameters for this test.

During T1, the UE is in RRC_CONNECTED state and both Cell 1 and Cell 2 are present, and must be detectable. The relevant conditions are defined in the following extract from TS 38.133:

### 9.2C.2	Requirements applicability

The requirements in clause 9.2C apply, provided:

-	The cell being identified or measured is detectable.

-	Valid information for the satellite serving the target cell has been provided

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.2C for FR1 and 10.1.3C for FR2-NTN, for a corresponding band,

-	SS-RSRQ related side conditions given in clauses 10.1.7C for FR1 and 10.1.8C for FR2-NTN, for a corresponding band,

-	SS-SINR related side conditions given in clauses 10.1.12C for FR1 and 10.1.13C for FR2-NTN, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.17 for a corresponding band.

To find the relevant conditions for SS-RSRP, we have to select the relevant part of TS 38.133 clause 10.1.2C.1.1, which is:

##### 10.1.2C.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute SS-RSRP accuracy in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in table 10.1.2C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-5 [43] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to annex B.2.17 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.2C.1.1-1: SS-RSRP Intra-frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_SAB_FR1_A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2A. |  |  |  |  |  |  |  |

To ensure this absolute SS-RSRP accuracy, the conditions of Io, SSB Es/Iot, and conditions of SSB_RP and SSB Es/Iot in section Annex B.2.17 need to be fulfilled.

## B.2.17	Conditions for NR intra-frequency measurements for satellite access

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in table B.2.17-1 for FR1 NR cells.

Table B.2.17-1: Conditions for intra-frequency measurements in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -127 | -124 | -6 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3.5.2A. |  |  |  |  |

The conditions for FR2-NTN NR cells depend on the EISREFSENS_50M declared by the vendor as described in clause 10.3.2 in [42] and on the UE VSAT Type. The minimum SSB_RP for the different bands shall be calculated for intra frequency measurements according to the formulation provided in clause B.2.1.7 of this specification.

During T2 and T3, Cell 2 is present, and must be detectable. Since the test verdict is only dependent on the UE being able send PRACH, the relevant conditions are defined in the following extract from TS 38.133:

#### 4.2C.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

If Srxlev > SIntraSearchP and Squal > SIntraSearchQ, and the distance between UE and serving cell reference location or serving cell moving reference location is smaller than distanceThresh if the distanceThresh is configured (see TS 38.304[1]) and UE has location information, then the UE is not required to perform measurement of intra-frequency.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Kmulti_SMTC * Tdetect,NR_Intra when that Treselection= 0 if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or within Kmulti_SMTC *  Tdetect,NR_Intra_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementLEO-r17 is enabled. An intra frequency cell is considered to be detectable according to the conditions defined in annex B.1.6 for a corresponding Band.

<< clauses skipped >>

## B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in table B.1.6-1 for FR1 NR cells.

Table B.1.6-1: Conditions for intra-frequency cell re-selection in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -124 | -121 | -4 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3.5.2A. |  |  |  |  |

In conclusion, the following conditions need to be satisfied:

NR Cell 1:

- SSB Es/Iot ≥ -6dB

- -121dBm/15kHz ≤ Io ≤ -50dBm/CBW

- SS-RSRP ≥ -127dBm/15kHz

NR Cell 2:

- SSB Es/Iot ≥ -6dB, during T1

- SSB Es/Iot ≥ -4dB, during T2 and T3

- -121dBm/15kHz ≤ Io ≤ -50dBm/CBW

- SS-RSRP ≥ -127dBm/15kHz, during T1

- SS-RSRP ≥ -124dBm/15kHz, during T2 and T3

These conditions are verified in the analysis spreadsheet.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Source and Reference” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 RSRP, so the sensitivity factor is 1.000 during T1. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to NR Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the NR Cell 2 Es/Noc has an effect on NR Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example,

- During T1, the effect of NR Cell 2 Es/Noc uncertainty on NR Cell 1 Es/Iot is x 0.715

These factors can also be derived intuitively, by looking at the pie chart for T1 in step a). For example, NR Cell 2 forms 29.5% / (29.5%+11.7%) of the total interference to NR Cell 1, which is 0.715. A change in the power of NR Cell 2 Es/Noc alone is slightly diluted in the overall interference.

Although in this case the sensitivity factor value of 0.715 is close to 1, it would be wrong to approximate because any change in geometry factors could make the approximation invalid.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1, T2 and T3.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T1 for both NR Cell 1 and NR Cell 2 the uncertainties do not take Io, Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required.

During T2 and T3, the uncertainties do not take NR Cell 2 Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

As no offsets have been applied, all values are the same as in section b).

In general, all the values are listed for each time interval T1, T2 and T3 but as NR Cell 1 is not present during T2 and T3 those entries are greyed out (the formulae are left in place for use by future test cases).

g) Check controlled parameters Min/Max

It is not necessary to calculate all parameters during each time interval T1, T2 and T3, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 14.2.2.1.1 configuration 1 refers to the GSO satellite access configuration. In addition, test 14.2.2.1.1 also contains another test configuration with identical power levels for NGSO satellite access. There is no test tolerance or measurement uncertainty difference between both cases, so the above analysis can also be leveraged to the NGSO test configuration. There are no other BW/SCS configurations defined.