# Title:	Test Tolerances for 5G NR UE transmit timing accuracy Tests for Satellite Access

Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM transmit timing accuracy tests for satellite access.

The test case in question, from TS 38.533, is:

- 14.3.1.1, NR SA FR1 UE transmit timing accuracy for Satellite Access

This document describes the process to derive the Test Tolerances for this test case.

# 2. 	Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.17.0:

#### A.14.3.1.1	NR UE Transmit Timing Test for FR1

##### A.14.3.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the reference cell and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1C.2. Supported test configurations are shown in table A.14.3.1.1.1-1.

Table A.14.3.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

The test consists a single NR cell (PCell). Table A.14.3.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in table A.14.3.1.1.1-3.

Table A.14.3.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2 | 1 | 1 |
| Serving satellite configuration |  | 1 | SSC.1 |  |
|  |  | 2 | SSC.2 |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle | ms | 1,2 | N/A | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1,2 | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1,2 | OP.1 |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |
| TRS configuration |  | 1,2 | TRS.1.1 FDD |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2 | -98 | -98 |
| Note2 | dBm/SCS | 1,2 | -98 | -98 |
|  |  | 1,2 | 3 | 3 |
|  |  | 1,2 | 3 | 3 |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 | -95 |
| IoNote3 | dBm/9.36 MHz | 1,2 | -65.2 | -65.2 |
| Propagation condition |  | 1,2 | AWGN |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 | SRSConf.2Note6 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: DRX related parameters are given in table A.3.3.8-1 NOTE 6: SRS configs are given in table A.14.3.1.1.1-3 |  |  |  |  |

Table A.14.3.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 |  |
| ResourceSet | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 14 | 14 | Matches NPRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl320, 3 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.14.3.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up PCell according to parameters given in table A.14.3.1.1.1-2.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin)  of the first detected path of DL SSB.

a.	The NTA_offset value (in Tc units) is 25600

b.	The NTA,adjcommon value is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation.

c.	The NTA,adjUE value is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters.

d.	The Te_NTN values depend on the DL and UL SCS for which the test is being run and are given in table 7.1C.2-1

e.	The TGNSS_margin counts for the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. TGNSS_margin=327,68×Tc

3)	If the NTN parameters are configured as GSO scenario, the test system shall adjust the timing of the DL path by values given in table A.14.3.1.1.2-1. If the NTN parameters are configured as NGSO scenario, the test system shall adjust the timing of the DL path according to the serving-satellite-ephemeris-related higher-layers parameters.

Table A.14.3.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1C.2 Table 7.1C.2.1-1 until the UE transmit timing offset is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin) respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin) of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

# 3. 	Calculation of Test Tolerances

The test consists of two subtests with one cell configured; the difference between the Subtests is related to DRX cycle and other parameters. Each Subtest consists of one time period.

The downlink timing of Cell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting SRS used as a measurement reference facilitating the SS timing estimation. The test requires the UE to pass in one of the supported test configurations in FR1, NGSO or GSO. If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2.

## 3.1	Additional timing components to be considered for NR-NTN

The UE transmission timing error shall be less than or equal to  Te, which is specified in TS 38.133 Table 7.1C.2-1:

### 7.1C.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN:

-	is specified in table 7.1C.2-1 for FR1-NTN.

-	is specified in table 7.1C.2-2 and table 7.1C.2-3 for VSAT UE in FR2-NTN.

This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc.

The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna.

NTA for PRACH is defined as 0. NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3C was applied. or after the last update in NTA,adjcommon or NTA,adjUE.

The value of NTA offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA offsetis defined in table 7.1.2-2 for FR1-NTN.

NTA offset is defined in table 7.1C.2-4 for VSAT UE in FR2-NTN.

NTA,adjcommon and NTA,adjUE are defined in TS 38.211 [6].

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

In addition, in the test requirement, there is an additional timing component, TGNSS_margin, which is the margin for the GNSS positioning error considered in the core requirement, which needs to be subtracted for the test requirement, as the UE will be provided with location through any preconfigured means (38.533 section 14.0.5), and the location is not expected to be changed throughout the test. In consequence, there is no need to consider this component in the measurement uncertainty analysis.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin)  of the first detected path of DL SSB.

a.	The NTA_offset value (in Tc units) is 25600

b.	The NTA,adjcommon value is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation.

c.	The NTA,adjUE value is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters.

d.	The Te_NTN values depend on the DL and UL SCS for which the test is being run and are given in table 7.1C.2-1

e.	The TGNSS_margin counts for the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. TGNSS_margin=327,68×Tc

From 38.533 section 14.0.5:

### 14.0.5	Initial test environment conditions for RRM NTN tests

The following initial conditions apply to all RRM satellite access specified in this section, unless otherwise specified in the test:

1.  UE location according to TS 38.508-1 [14] clause 7.6.1 is provided to the UE through any preconfigured means.

2.  Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [14] Tables 7.6.2.1-1a and 7.6.2.1-1b for GSO at 30º elevation angle conditions for GSO configurations or Tables 7.6.2.1-2a and 7.6.2.1-2b for NGSO (LEO-600) at 30º elevation angle conditions for NGSO (LEO-600) configurations or Tables 7.6.2.1-3a and 7.6.2.1-3b for NGSO (LEO-1200) at 30º elevation angle conditions for NGSO (LEO-1200) configurations. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [14] clause 7.6.3.1.

3.  Deactivate UE prediction of satellite trajectory by any preconfigured means.

We observe that for NR-NTN there are two new UE timing components to consider, NTA,adjcommon and NTA,adjUE, in addition to all other legacy components. These new components are defined in 38.213 section 4.2 as follows:

Using higher-layer ephemeris parameters for a serving satellite or atg-gNB-Location for ATG, if provided, a UE pre-compensates the two-way transmission delay on the service link or on the link between gNB and UE for ATG based on NTA,adjUE that the UE determines using the serving satellite position or gNB location for ATG and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2

where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

Further, according to 38.508-1 section 7.6.2, satellite ephemeris remains constant throughout the test. This implies that satellite remains in a fixed position in the sky. Since the UE doesn’t move either, both UE and satellite are static elements in the scenario, always at the same distance from each other, and thus always experiencing constant relative time variation.

Therefore, from UE perspective time will pass for both elements at the same rate of 1 second per second (or rather, 1 subframe per subframe). In this scenario, delta time will remain constant throughout the test:

Delta time= t-tepoch=constant

Every time the UE decodes SIB19, it will evaluate its epoch time against the UE internal time, and the result will remain constant over time, independently of how often SIB19 is broadcasted.

Since all other three parameters are signalled (and thus static, reference 38.508-1 Table 7.6.3.1-1), they will also remain constant over time. We can therefore conclude that:

Delaycommont=constant

And then:

Delaycommont=constant→NTA,adjcommon=constant

Table 7.6.3.1-1: SIB19 for RRM tests (single cell test cases)

| Derivation Path: Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s900 |  | GSO |
|  | s5 |  | NGSO |
| cellSpecificKoffset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | 0 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | Not present |  |  |
| ntn-PolarizationUL-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  | [TC_EF_SV] |
| positionX-r17 | See cl. 7.6.2 |  |  |
| positionY-r17 | See cl. 7.6.2 |  |  |
| positionZ-r17 | See cl. 7.6.2 |  |  |
| velocityVX-r17 | See cl. 7.6.2 |  |  |
| velocityVY-r17 | See cl. 7.6.2 |  |  |
| velocityVZ-r17 | See cl. 7.6.2 |  |  |
| } |  |  |  |
| orbital-r17 SEQUENCE { |  |  | [TC_EF_OP] |
| semiMajorAxis-r17 | See cl. 7.6.2 |  |  |
| eccentricity-r17 | See cl. 7.6.2 |  |  |
| periapsis-r17 | See cl. 7.6.2 |  |  |
| longitude-r17 | See cl. 7.6.2 |  |  |
| inclination-r17 | See cl. 7.6.2 |  |  |
| meanAnomaly-r17 | See cl. 7.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |
| ntn-NeighCellConfig-r17 | Not present |  | [1CELL] |
|  | See cl. 7.6.3.2 |  | [2CELLS] |
|  | See cl. 7.6.3.2 |  | [3CELLS] |
| } |  |  |  |

Based on 38.211 section 4.3.1, NTA,adjUE is computed by the UE based on its own position and satellite ephemeris information broadcasted via SIB19:

### 4.3.1	Frames and subframes

Downlink, uplink, and sidelink transmissions are organized into frames with  duration, each consisting of ten subframes of  duration. The number of consecutive OFDM symbols per subframe is Nsymbsubframe,μ=NsymbslotNslotsubframe,μ. Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 – 4 and half-frame 1 consisting of subframes 5 – 9.

There is one set of frames in the uplink and one set of frames in the downlink on a carrier.

Uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon given by clause 4.2 of [5, TS 38.213] is derived from the higher-layer parameters ta-Common, ta-CommonDrift, and ta-CommonDriftVariant if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE given by clause 4.2 of [5, TS 38.213] is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters if configured, or is computed by the UE based on UE position and gNB location provided by atg-gNB-Location if configured, otherwise NTA,adjUE=0.

Figure 4.3.1-1: Uplink-downlink timing relation.

Since both UE position and satellite ephemeris are kept constant throughout the test, the computed NTA,adjUE is expected to remain constant as well:

NTA,adjUE=fUEposition,Satelliteephemeris=fconstant,constant=constant

In conclusion, both NTA,adjcommon and NTA,adjUE remain constant throughout the test. This implies that none of them is expected to contribute towards UE timing error uncertainty.

## 3.2	Absolute timing error uncertainty analysis for NR-NTN

Based on the above analysis, the measurement uncertainty of the test system is the only additional effect to be considered. According to 38.533 annex F.1.1.2, the Test tolerance for uplink signal transmit timing relative to downlink for FR1-NTN is 352*Tc, considered to be a reasonable uncertainty value for the Test system measurement of transmit timing. This was derived from taking 25% of the tightest UE core requirement for FR1-NTN, which is 22*64*Tc for SCS of 30KHz, giving a ±5.5*64*Tc = 352*Tc uncertainty.

For Test 1 and Test 2 (10MHz Channel BW) the test requirements are specified in 38.133 Table 7.1C.2-1. The formula to obtain the modified requirements is:

Formula: (Upper limit + TT, Lower limit – TT):

In numbers: (±29*64*Tc ±5.5*64*Tc) = ± 34.5*64*Tc

These values are therefore can be used in 38.533 test requirements Table 14.3.1.1.5-4.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 7*64*Tc) |
| Uplink signal transmit timing relative to downlink for satellite access | Tc | ±352 | Calculated as 25% of tightest UE core requirement for FR1 for satellite access (25% of 22*64*Tc) |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| Timing Advance Adjustment accuracy | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

## 3.3	Autonomous timing adjustment analysis for NR-NTN

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference timing exceeds ±Te the UE is required to adjust its timing to within ±Te. The reference timing shall be before the downlink timing.

When not in DRX (Test 1), all adjustments made to the UE uplink timing shall follow the following rules, extracted from TS 38.133 v17.17.0 clause 7.1C.2.1:

#### 7.1C.2.1	Gradual timing adjustment

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second.

3a)	For a FR1-NTN UE or a FR2-NTN fixed UE, the maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200 ms, shall be Tq_NTN per 200 ms.

3b)	 For a FR2-NTN mobile UE , the maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200 ms, shall be 3 * cTN per 200 ms.

Where, the maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN are specified in table 7.1C.2.1-1.

Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | N/A | N/A |
| FR2-NTN | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

In the above core requirements, we notice the blue highlighted part is excluded from meeting the autonomous timing adjustment requirements, Tp_NTN, Tq_NTN. As we have analyzed before, both NTA,adjcommon and NTA,adjUE remain constant throughout the test, and are therefore not expected to create any additional measurement uncertainty. The above rules 1, 2 and 3a can be performed on SRS timing error measurements without any consideration related to these two timing components.

For the autonomous timing adjustment test two additional effects need to be considered. Both the allowed UE frequency error and the UE time adjustment quantisation can affect the result of the test, and it is not possible to test the UE transmit timing accuracy in isolation from these core requirements. They are therefore considered when setting the test tolerances and the test requirements.

### 3.3.1 Relative transmit timing measurement uncertainty

To evaluate above core requirements, the test equipment needs to measure the relative timing error between two independent SRS transmissions from the UE. The relative measurement uncertainty, according to 38.533 annex F.1.1.2 is ±88*Tc = ±1.375*64*Tc. This was derived from taking 25% of the tightest UE core requirement for FR1-NTN, which is 5.5*64*Tc. Since the FR1-NTN core requirement is the same as FR1, the same entry in Table F.1.1.2-1 can be used:

| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| --- | --- | --- | --- |

In addition to this measurement uncertainty, the UE frequency error and the UE time adjustment quantization need to be analyzed separately.

### 3.3.2 Allowed UE frequency error

In addition to the effect of test system uncertainty for relative transmit timing, the effect of allowed UE frequency error specified in 38.101-5 clause 6.4.1 should be considered:

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Excluding variations on both NTA,adjcommon and NTA,adjUE, the effect of allowed UE frequency error on the timing adjustment rules is calculated as follows:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be Tq_NTN = 5.5*64*Tc.

As the SRS is configured to be transmitted every 1ms (FDD duplex configuration with SCS 15kHz), the UE timing change from frequency error could be (0.001s x (0.1 x 10-6)) = 0.1ns ≈ 0.2*Tc. This is negligible compared to the core requirement and does not need to be considered for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be Tp_NTN (5.5*64*Tc) per second.

During 1s, the UE timing change from frequency error could be (1s x (0.1 x 10-6)) = 100ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ±3.1*64*Tc.

Rule 3: The maximum aggregate adjustment rate shall be Tq_NTN (5.5*64*Tc) per 200ms.

During 200ms, the UE timing change from frequency error could be (0.2s x (0.1 x 10-6)) = 20ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ±0.6*64*Tc.

### 3.3.3 UE time adjustment quantisation

Although not explicitly stated in TS 38.133, the UE timing adjustment is assumed to be quantised, and a quantisation step size up to Tq_NTN should be considered.

The effect of quantised UE timing adjustment on the timing adjustment rules is calculated as follows:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be Tq_NTN seconds.

As the magnitude of the timing change in one adjustment is being directly measured, no additional Tq_NTN allowance is needed for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be Tp_NTN per second. This effect needs to be evaluated considering the additional UE timing change caused by frequency error of 3.1*64*Tc per second.

If a time window of exactly 1s is used, and the UE makes the largest allowed timing steps with the longest allowed time between changes as shown in the diagram, the SS could catch one or two UE timing changes depending on the synchronisation between the SS measurement period of 1s and the UE timing change. As this part of the test aims to check the UE minimum adjustment rate, the sliding time window is increased by the minimum amount of one sample (1ms, FDD duplex configuration with SCS 15kHz). This ensures that the SS will not miss any timing changes occurring during the 1s time period specified in the core requirements.

Rule 3: The maximum aggregate adjustment rate shall be Tq_NTN per 200ms.

If a time window of exactly 200ms is used, and the UE makes the largest allowed timing steps with the longest allowed time between changes as shown in the diagram, the SS could catch one or two UE timing changes depending on the synchronisation between the SS measurement period of 200ms and the UE timing change. As this part of the test aims to check the UE maximum adjustment rate, the sliding time window is decreased by the minimum amount of one sample (1ms, FDD duplex configuration with SCS 15kHz). This ensures that the SS will not include any extra timing changes occurring during the 200ms time period specified in the core requirements.

Putting all three rules together into a diagram:

Combined effect of allowed UE frequency error and UE time adjustment quantisation

The diagram above shows the combined effect of allowed UE frequency error, UE time adjustment quantisation and the use of a sliding measurement window (for simplicity the test system timing measurement uncertainty is not included in the diagram).

The dashed green and blue lines show the specified maximum and minimum aggregate adjustment rates respectively. The stepped green and blue lines show possible behaviour if the UE uses the largest possible time quantisation step, and changes the timing infrequently, which gives the worst case for measurement. Smaller or more frequent steps are also allowed.

As long as the UE timing remains within the region delimited by the green and blue dashed lines, all rules will be satisfied. We can observe, from the two worst case examples represented in the diagram, that such behaviour would not meet both rules all the time, due to the effect of the UE frequency error and UE timing adjustment quantisation uncertainties.

The effect of frequency error is shown by dashed black lines and can make the UE Tx timing earlier or later than expected. The combined effect is shown by the green and blue lines with sloping steps.

For the maximum adjustment rate measurement, the 198ms window “Position A” shows a typical measurement, where the test system would expect to see one timing quantisation step, together with a frequency error which may increase the timing change. In window “Position B” the test system would see no timing quantisation steps, but as the Test requirement is a one-sided limit this is unimportant.

For the minimum adjustment rate measurement, the 1.002s window “Position C” shows a typical measurement, where the test system would expect to see two timing quantisation steps, together with a frequency error which may decrease the timing change. In window “Position D” the test system could see three timing quantisation steps, but as the Test requirement is a one-sided limit this is unimportant.

## 3.4	Timing test requirement calculation

When calculating the Test requirements, the test system uncertainty, the effect of allowed UE frequency error where relevant, and the effect of UE time adjustment quantisation where relevant, are included.

All test configurations use FDD with SCS of 15KHz. The test requirement calculations are as follows:

- The Maximum timing change in one adjustment Tq_NTN calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +5.5 * 64 * Tc |
| Test system uncertainty | Not applicable | +1.375 * 64 * Tc |
| UE frequency error allowance | Not applicable | Not applicable |
| Test requirement | Not applicable | +6.875 * 64 * Tc |

- The Minimum aggregate adjustment rate per second Tp_NTN calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | +5.5 * 64 * Tc | Not applicable |
| Test system uncertainty | -1.375 * 64 * Tc | Not applicable |
| UE frequency error allowance | -3.1 * 64 * Tc | Not applicable |
| Test requirement | +1.025 * 64 * Tc | Not applicable |

- The Maximum aggregate adjustment rate per 200 ms calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +5.5 * 64 * Tc |
| Test system uncertainty | Not applicable | +1.375 * 64 * Tc |
| UE frequency error allowance | Not applicable | +0.6 * 64 * Tc |
| Test requirement | Not applicable | +7.475 * 64 * Tc |

## 3.5	Downlink Signal Level Uncertainties

The SS provides 1 cell with AWGN to the UE. We propose to control the following parameters:

- AWGN absolute power, Noc1 ±1.5 dB averaged over BWConfig

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 ±0.3 dB averaged over BWConfig

The downlink uncertainty values are chosen from the 38.533 Annex F.1.1.2.

## 3.6	Key Signal level parameters in test case

To ensure that the SNR does not fall below the core requirement specified in Annex A of TS 38.133, the Es/Noc values are increased by the amount of the SNR uncertainty, +0.3dB, keeping the noise floor constant. Therefore, the test requirements are:

- = 3+0.3 dB = +3.3dB

- = -98 dBm/15 kHz

- = 3+0.3 dB = +3.3dB

- SS-RSRP = -98 + 3.3 = -94.7dBm/15kHz

Therefore:

Io=10*log10-98+10*log52*1210+10-94.7+10*log52*1210=-65.08dBm/9.36MHz

When considering 52RBs and 12 subcarriers per RB.

The calculations are sufficiently simple, and sufficiently good that there is no need for a separate analysis spreadsheet. Therefore, they can be directly implemented in the test case.

# 4. Treatment of other configurations within the same test case

TS 38.533 test case 14.3.1.1 configuration 1 refers to the GSO satellite access configuration. In addition, test 14.3.1.1 also contains another test configuration with identical power levels, bandwidth, and timing accuracy requirements, for NGSO satellite access. There is no test tolerance or measurement uncertainty difference between both cases, so the above analysis can also be leveraged to the NGSO test configuration. There are no other BW/SCS configurations defined.