3GPP TSG-RAN5 Meeting #107	R5-253015

# St. Julian's, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	5.5.1.13

Source: 	EchoStar

Title: 	Support of NB-IoT in-band operation in NR NTN bands

Document for:	Discussion and Endorsement

## 1.	Introduction

During RAN plenary meeting TSGR#107, CRs [1] related to support for NB-IoT in-band operation in NTN NR for NB-IoT NTN bands were approved and subsequently incorporated in the core specifications [2] and [3].

This paper summarizes updates made to the core specifications related to NB-IoT in-band operation in NTN NR requirements, assesses the impact to various conformance specifications and proposes updates to the affected clauses of the affected specifications.

## 2.	Overview of NB-IoT in-band operation in NTN NR related updates to core specifications

A new sub-clause 5.4B.2.3 with a new formula has been introduced to support in-band/guard band operation of NB-IoT in NTN NR in [2].

### 5.4B.2.3 In-band operation In Rel-18, NTN NB-IoT UEs may optionally support in-band operation with NR NTN. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1) The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) For the carrier including NPSS/NSSS, MDL is selected from {-2,-1,0,1}. For in-band operations, MDL = -0.5 is not applicable. In Rel-18, UE is only required to support the same operation mode for anchor and non-anchor carriers. NOTE 1: For in-band operation, RRC signalling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests. NOTE 2: In Rel-18, for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB. NOTE 3: UE is not expected to be aware of where the configured NB-IoT UL carrier is within the NTN NR carrier. It is presumed that operators will ensure, through network configuration during deployment, that there is sufficient guardband between NB-IoT carrier and operating band and/or spectrum block edges to meet emission requirements of clause 6.5B outside its allocated spectrum block. ### 5.4B.3 TX–RX frequency separation For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4B.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1. For in-band operation, the category NB1 and NB2 TX-RX frequency separation is flexible within the assigned channel bandwidth of NR NTN carrier with the TX-RX frequency separation of the NR NTN carriers as specified in TS 38.101-5 [16].

Furthermore, the following note was specified in Table F.1-1 of [3]:

“NOTE:	Rel-18 UEs supporting only standalone operation do not need to meet requirements specified for in-band operation with NR over NTN in clauses 5.4B.2.3 and 5.4B.3. In-band operation with NR is not supported in Rel-17.”

The following observations can be made from review of above updates made to the core specifications:

Observation 1: The NTN UE support for in-band operation of NB-IoT is optional in Rel-18 but mandatory from Rel-19 [2], [3].

Observation 2: The formulae for FDL and FUL are different between stand-alone and in-band operations.

Observation 3:  Since the in-band/guard band operation is within an NR NTN channel, the operationModeInfo IE in MIB-NB is expected to be set to “guardband-r13” to ensure that the UE is assuming that REs are not reserved for E-UTRA PDCCH/CRS and is not expecting reception of E-UTRA CRS.

Observation 4: Because the operationModeInfo IE is always set to indicate guard band operation, assignment of FUL in the guard band is not precluded by the specs as long as the lower/upper channel edges are farther away from the lower/upper spectrum block edges.

Observation 5: The demodulation and RRM requirements have not been updated in the core specifications.

## 3.	RF Conformance Specifications Impact Assessment and Proposals

| TS & Clause | Assessment | Way Forward |
| --- | --- | --- |
| TS 36.508 |  |  |
| Clause 8.1.3.1.1.254 - 256 | NB-IoT in-band/guard band test frequencies for NTN bands need to be specified | Add new in-band/guardband test frequencies for NB-IoT NTN bands as new tables for each NTN band |
| Clauses 8.1.3.3.3 | A new entry for NTN in-band operation may to be updated | Update Table 8.1.3.3.3-1 as appropriate. |
| TS 36.521-4 |  |  |
| Common Clauses 5.4B | Update channel spacing, channel raster, carrier frequency, EARFCN and Tx-Rx frequency separation clauses per updates in the core specifications for in-band operation. | Update Clause 5.4B |
| Clause 6 | For terrestrial NB-IoT Tx test cases (TS 35.521-1), operation mode for all tests is stand-alone Similar approach can be adopted for NB-IoT NTN specifications as well given Tx performance is not expected to be impacted irrespective of where it is operated with the spectrum band. | No further updates are required unless a specific need is expressed via contribution in a future meeting |
| Clause 7 | For terrestrial operation, operation mode for all tests is stand-alone Similar approach can be adopted for NB-IoT NTN specifications as well. | No further updates are required unless a specific need is expressed via contribution in a future meeting |
| Clause 8 | The core demodulation requirements have not been updated | No further updates are required |
| Annex A | Updates are required to reflect applicability of the FRC for inband/guardband | At the minimum, title of clause A.3.12.1 of needs to be updated. Further assess if additional updates in Annex A needs to be made |
| Annex C | Various updates related to in-band aspect need to be updated | Update Table C.1-2 Further assess if additional updates in Annex C needs to be made |
| TS 36.521-2 |  |  |
| Clauses 4.3.1, A.4.3 | It is unclear whether new RF test cases need to be specified for in-band operation in NTN NR. | If new RF test cases are introduced then clauses 4.3.1 and A.4.3 may need to be updated. |
| TS 36.521-3 | Core RRM specifications were not updated and therefore no further updates are necessary to 36.521-3 at this time |  |
| TR 36.903 | No additional RRM tests have been specified |  |
| TR 36.904 | No impact to TT is impacted given that RF tests do not seem to be impacted. Further review may be necessary if RF/Demod test cases are envisioned for in-band/guard-band operation. |  |
| TR 36.905 | No updates are anticipated as RF test cases are unimpacted |  |

Proposal 1: Endorse the way forward proposed as part of the above assessment and invite contributions for future meetings to progress the work.

## 4.	Conclusion

Observation 1: The NTN UE support for in-band operation of NB-IoT is optional in Rel-18 but mandatory from Rel-19 [2], [3].

Observation 2: The formulae for FDL and FUL are different between stand-alone and in-band operations.

Observation 3:  Since the in-band/guard band operation is within an NR NTN channel, the operationModeInfo IE in MIB-NB is expected to be set to “guardband-r13” to ensure that the UE is assuming that REs are not reserved for E-UTRA PDCCH/CRS and is not expecting reception of E-UTRA CRS.

Observation 4: Because the operationModeInfo IE is always set to indicate guard band operation, assignment of FUL in the guard band is not precluded by the specs as long as the lower/upper channel edges are farther away from the lower/upper spectrum block edges.

Observation 5: The demodulation and RRM requirements have not been updated in the core specifications.

Proposal 1: Endorse the way forward proposed as part of the above assessment and invite contribution for future meetings.

## 5.		References

[1]	RP-250611, RAN4 CRs to Maintenance for Closed WIs/SIs for REL-18 and earlier – Batch 23

[2]	TS 36.102 v18.8.0

[3]	TS 36.307 v18.8.0