3GPP TSG-RAN5 Meeting #107	R5-253017

# St. Julian's, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	6.5.6

Source: 	EchoStar

Title: 	Support of NB-IoT in-band operation in NR NTN bands

Document for:	Discussion and Endorsement

## 1.	Introduction

During RAN plenary meeting TSGR#107, CRs [1] related to support for NB-IoT in-band operation in NTN NR for NB-IoT NTN bands were approved and subsequently incorporated in the core specifications [2] and [3].

This paper summarizes updates made to the core specifications related to NB-IoT in-band operation in NTN NR requirements, assesses the impact to various conformance specifications and proposes updates to the affected clauses of the affected specifications.

## 2.	Overview of NB-IoT in-band operation in NTN NR related updates to core specifications

A new sub-clause 5.4B.2.3 with a new formula has been introduced to support in-band/guard band operation of NB-IoT in NTN NR in [2].

### 5.4B.2.3 In-band operation In Rel-18, NTN NB-IoT UEs may optionally support in-band operation with NR NTN. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1) The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) For the carrier including NPSS/NSSS, MDL is selected from {-2,-1,0,1}. For in-band operations, MDL = -0.5 is not applicable. In Rel-18, UE is only required to support the same operation mode for anchor and non-anchor carriers. NOTE 1: For in-band operation, RRC signalling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests. NOTE 2: In Rel-18, for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB. NOTE 3: UE is not expected to be aware of where the configured NB-IoT UL carrier is within the NTN NR carrier. It is presumed that operators will ensure, through network configuration during deployment, that there is sufficient guardband between NB-IoT carrier and operating band and/or spectrum block edges to meet emission requirements of clause 6.5B outside its allocated spectrum block. ### 5.4B.3 TX–RX frequency separation For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4B.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1. For in-band operation, the category NB1 and NB2 TX-RX frequency separation is flexible within the assigned channel bandwidth of NR NTN carrier with the TX-RX frequency separation of the NR NTN carriers as specified in TS 38.101-5 [16].

Furthermore, the following note was specified in Table F.1-1 of [3]:

“NOTE:	Rel-18 UEs supporting only standalone operation do not need to meet requirements specified for in-band operation with NR over NTN in clauses 5.4B.2.3 and 5.4B.3. In-band operation with NR is not supported in Rel-17.”

The following observations can be made from review of above updates made to the core specifications:

Observation 1: The NTN UE support for in-band operation of NB-IoT is optional in Rel-18 but mandatory from Rel-19 [2], [3].

Observation 2: The formulae for FDL and FUL are different between stand-alone and in-band operations.

Observation 3:  Since the in-band/guard band operation is within an NR NTN channel, the operationModeInfo IE in MIB-NB is expected to be set to “guardband-r13” to ensure that the UE is assuming that REs are not reserved for E-UTRA PDCCH/CRS and is not expecting reception of E-UTRA CRS.

Observation 4: Because the operationModeInfo IE is always set to indicate guard band operation, assignment of FUL in the guard band is not precluded by the specs as long as the lower/upper channel edges are farther away from the lower/upper spectrum block edges.

Observation 5: The demodulation and RRM requirements have not been updated in the core specifications.

## 3.	SIG Conformance Specifications Impact Assessment and Proposals

| TS & Clause | Assessment | Way Forward |
| --- | --- | --- |
| TS 36.508 |  |  |
| Clause 8.1.4.3 | MIB-NB, SIB1-NB, SIB2-NB need to be updated with a new condition for in-band/guard-band NTN operation | Update Tables 8.1.4.3.2-1, 8.1.4.3.2-3, 8.1.4.3.3-1 |
| 8.3.1 | Need to specify deployment scenario of in-band/guard-band operation with NR | Updates to Clause 8.3.1 and Table 8.3.1-1 |
| 8.3.2.3 | Reference to lack of support of in-band/guard-band for NTN bands needs to be modified | Update clause 8.3.2.3 to remove reference to lack of support of in-band/guard-band operation |
| TS 36.523-2 |  |  |
| Clauses 4, A4.4 | A new PICS is needed for protocol implementation conformance of in-band/guard-band. A subset of test cases may be applicable for conformance of in-band/guard-band protocol implementation. | Add a new PICS for inband/guardband operation Further assess which of the test cases for which this PICS and Note should be added. |
| TS 36.523-3 |  |  |
| Clauses 9.3 | Similar to variable Tx-Rx spacing, further discuss if a new PIXIT is required to execute the test twice | Discuss and reach consensus on whether a new PIXIT needs to be specified or not. |

Proposal 1: Endorse the way forward proposed as part of the above assessment and invite contributions for future meetings to progress the work.

Proposal 2: Reach consensus on whether PIXIT needs to be specified or not to enable protocol conformance testing for NB-IoT in-band operation in NTN NR.

## 4.	Conclusion

Observation 1: The NTN UE support for in-band operation of NB-IoT is optional in Rel-18 but mandatory from Rel-19 [2], [3].

Observation 2: The formulae for FDL and FUL are different between stand-alone and in-band operations.

Observation 3:  Since the in-band/guard band operation is within an NR NTN channel, the operationModeInfo IE in MIB-NB is expected to be set to “guardband-r13” to ensure that the UE is assuming that REs are not reserved for E-UTRA PDCCH/CRS and is not expecting reception of E-UTRA CRS.

Observation 4: Because the operationModeInfo IE is always set to indicate guard band operation, assignment of FUL in the guard band is not precluded by the specs as long as the lower/upper channel edges are farther away from the lower/upper spectrum block edges.

Observation 5: The demodulation and RRM requirements have not been updated in the core specifications.

Proposal 1: Endorse the way forward proposed as part of the above assessment and invite contribution for future meetings.

Proposal 2: Reach consensus on whether PIXIT needs to be specified or not to enable protocol conformance testing for NB-IoT in-band operation in NTN NR.

## 5.		References

[1]	RP-250611, RAN4 CRs to Maintenance for Closed WIs/SIs for REL-18 and earlier – Batch 23

[2]	TS 36.102 v18.8.0

[3]	TS 36.307 v18.8.0