3GPP TSG-RAN5 Meeting #107	R5-253196

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1592 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 19.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - As explained in clause 2.1.2 of the RAN5-endorsed discussion document R5-250355 in general all 1xx or 2xx responses sent to the UE within an emergency call shall have a P-Asserted-Identity header field i.e. even the 200 OK for PRACK in a non-UE detectable emergency call. - When the dedicated EPS bearer gets established already before the UE has sent the PRACK then the test case would fail when the UE sends the PRACK with the 2nd SDP offer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specific message contents added for 200 OK for PRACK - Clarification added that the dedicated EPS bearer gets established not before the UE has sent the PRACK. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification is not according to the core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is outcome of 1 revision of R5-252211. |  |  |  |  |  |  |  |  |

### 19.3.1	Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not

#### 19.3.1.1	Definition

Test to verify that the UE acts correctly when it receives a 1xx response to an initial request for a dialog from the IM CN, the response containing a P-Asserted-Identity header field set to an emergency number that is recognisable by the UE and the UE sends an UPDATE request with:

- Geolocation header and information if the UE is capable of obtaining location information; and

- Contact header set appropriately

#### 19.3.1.2	Conformance requirement

If the UE receives a 1xx or 200 (OK) response to an initial request for a dialog, the response containing a P-Asserted-Identity header field set to an emergency number as specified in 3GPP TS 22.101 [1A], and:

-	if a public GRUU value (pub-gruu) has been saved associated with the public user identity, the public GRUU value has not been included in the Contact header field of the initial request for a dialog as specified in RFC 5627 [93];

-	if a public GRUU value (pub-gruu) has not been saved and a protected server port was not included in the address in the Contact header field of the the initial request for a dialog; or

-	if the UE has its geographical location information available and the geographical location information has not been included in the initial request for a dialog; then the UE shall send an UPDATE request according to RFC 3311 [29]; and

1)	if available to the UE, and if defined for the access type as specified in subclause 7.2A.4, the UE shall include in the UPDATE request a P-Access-Network-Info header field and it shall contain a location identifier such as the cell id or the identity of the I-WLAN access node;

2)	if the UE has its geographical location information available, or a URI that points to the location information, then the UE shall include it in the UPDATE request in the following way:

I)	if the UE is aware of the URI that points to where the UE's location is stored, include the URI as the Geolocation header field value, as described in RFC 6442 [89]; or

II)	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described  RFC 6442 [89];

3)	if the UE includes a Geolocation header field, the UE shall also include a Geolocation-Routing header field with a "yes" header field value, which indicates that the location of the UE can be used by other entities to make routing decisions, as described in RFC 6442 [89];

4)	if the UE has neither geographical location information available, nor a URI that points to the location information, the UE shall not insert a Geolocation header field in the INVITE request; and

5)	if a public GRUU value ("pub-gruu" header field parameter) has been saved associated with the public user identity, then the UE shall insert the public GRUU ("pub-gruu" header field parameter) value in the Contact header field of the UPDATE request as specified in RFC 5627 [93]; otherwise the UE shall include the address in the Contact header field set in accordance with subclause 5.1.6.8.4, item 8.

NOTE 1:	The IMS emergency specification in 3GPP TS 23.167 [4B] describes several methods how the UE can get its location information from the access network or from a server. Such methods are not in the scope of this specification.

NOTE 2:	It is suggested that UEs only use the option of providing a URI when the domain part belongs to the current P-CSCF or S-CSCF provider. This is an issue on which the network operator needs to provide guidance to the end user. A URI that is only resolvable to the UE which is making the emergency call is not desirable.

NOTE 3:	During the dialog, the points of attachment to the IP-CAN of the UE can change (e.g. UE connects to different cells). The UE will populate the P-Access-Network-Info header field in any request (except ACK requests and CANCEL requests) or response (except CANCEL responses) within a dialog with the current point of attachment to the IP-CAN (e.g. the current cell information).

NOTE 4:	In this version of the specification, only requests creating a dialog can request emergency services.

If the UE receives a 1xx or 200 (OK) response to an initial request for a dialog the response containing a P-Asserted-Identity header field set to an emergency number as specified in 3GPP TS 22.101 [1A], then the UE may indicate the nature of the session to the user.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.10

#### 19.3.1.3	Test purpose

To verify that if the UE is not able to detect that an emergency number has been dialled:

-	in the event the UE receives a 1xx response to an INVITE request the response containing a P-Asserted-Identity header field set to an emergency number, the UE:

-	If the UE is able to obtain its geolocation and the geographical location information has not been included in the initial request for a dialog; then the UE shall include its geolocation information in the UPDATE message

-	If the UE is not able to obtain its geolocation the UE does not include it in the UPDATE message

-	includes a Contact header in the UPDATE message with the correct contents

#### 19.3.1.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC.

Test environment shall be set up to provide the needed input to the UE, in order for the UE to derive its location, if the UE is capable of obtaining location information. This shall be done by use of the test function Update UE Location Information defined in TS 34.109 [117] or in TS 36.509 [118] depending on the RAT being used in the test case, if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

- A non-emergency MO call is initiated up following the generic procedure in Annex C.21 with the following clarification:
The dedicated EPS bearer is established not before the UE has sent the PRACK at step 5 so that the UE shall send the second SDP offer in the UPDATE at step 7.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  |  | Steps 1-13 of Annex C.21. The UE initiates a non-emergency call. |

Specific Message Contents

183 Session Progress (Step 4)

Use the “183 Session Progress (Step 4)” of annex C.21 applying condition A5 of A.2.3 in addition.

200 OK for PRACK (Step 5, 10)

Use the default message "200 OK" in annex A.3.1 with the same P-Asserted-Identity as in the 183 Session Progress at step 4.

UPDATE (Step 7)

Use the "UPDATE (Step 7)" of annex C.21 with the following exceptions:

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Geolocation |  | A1 |  |  |  | Rel-9 |  | RFC 6442 [98] |  |
| locationURI |  |  |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |  |  |  |
| Geolocation-Routing |  | A1 |  | “yes” |  | Rel-9 |  | RFC 6442 |  |
| Contact |  |  |  |  |  |  |  | RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec |  | A2 |  | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |  |  |  |
| addr-spec |  | A3 |  | SIP URI with IP address or FQDN and protected server port of UE |  |  |  |  |  |
| addr-spec |  | A4 |  | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |  |  |  |
| Content-Type |  |  |  |  |  |  |  |  |  |
| media-type |  | A1 not A1 |  | multipart/mixed application/sdp |  |  |  | TS 24.229 [10] RFC 3261 [15] |  |
| Message-body |  | A1 |  | The "application/pidf+xml" multipart-mime body shall contain a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more ‘geopriv’ elements (Note), each containing: - At least one ‘location-info’ element describing the location of the UE. Note: PIDF-LO shall contain at least one <geopriv> element in a <tuple> element according to RFC 4119 or in a <device> or <person> element according to RFC 4119 and RFC 5491. The "application/sdp" multipart-mime body shall contain the SDP message as specified for the "UPDATE (Step 7)" of annex C.21. |  |  |  | RFC 6442 [98] |  |
|  |  | not A1 |  | SDP message as specified for the "UPDATE (Step 7)" of annex C.21. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | UE is capable of obtaining location information |
| A2 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A3 | Not A2 and (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A4 | Not A2 and (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |

200 OK for UPDATE (Step 8)

Use the default message "200 OK" in annex A.3.1 with condition A6.

180 Ringing (Step 9)

Use the default message “180 Ringing” in annex A.2.6 with conditions A4 and A8.

200 OK for INVITE (Step 12)

Use the default message "200 OK" in annex A.3.1 with condition A6.

#### 19.3.1.5	Test requirements

SS must check that in:

- Step 2 the UE sends a non-emergency INVITE with the correct contents

- Step 7 the UE sends the UPDATE message with:

  - the Geolocation header set appropriately, if the UE is capable of obtaining location information

  - Contact header set appropriately