3GPP TSG-RAN5 Meeting #107	R5-253201

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3473 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Qos flow configuration for IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A new Qos flow(QFI=16, 5QI=9) and related DRB are needed to be configured for IMS data channel(IMS DC). |  |  |  |  |  |  |  |  |
| Summary of change: |  | “Reference QoS rule #7” is updated,“Packet filter list #12” and “Reference QoS flow #12” are added to include new qos flow for IMS DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS data channel test case cannot be operated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-252452r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.8.2	5GC configurations

4.8.2.1	Reference QoS rules

Table 4.8.2.1-1: Reference QoS rule #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filters |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-2: Reference QoS rule #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-3: Reference QoS rule #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-2 | Packet filter list #2 |  |
| QoS rule precedence | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-4: Reference QoS rule #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3 | Packet filter list #3 |  |
| QoS rule precedence | ‘0000 00100’B | 4 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-4a: Reference QoS rule #4a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3a | Packet filter list #3a |  |
| QoS rule precedence | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0100’B | QFI 4 (Table 4.8.2.3-2a) |  |

Table 4.8.2.1-5: Reference QoS rule #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-4 | Packet filter list #4 |  |
| QoS rule precedence | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0101’B | QFI 5 (Table 4.8.2.3-3) |  |

Table 4.8.2.1-6: Reference QoS rule #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-5 | Packet filter list #5 |  |
| QoS rule precedence | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0110’B | QFI 6 (Table 4.8.2.3-4) |  |

Table 4.8.2.1-7: Reference QoS rule #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  | IMS_VOICE |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-6 | Packet filter list #6 |  |
| QoS rule precedence | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0111’B | QFI 7 (Table 4.8.2.3-5) |  |
| QoS rule |  |  | IMS_VIDEO |
| QoS rule identifier | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-7 | Packet filter list #7 |  |
| QoS rule precedence | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1000’B | QFI 8 (Table 4.8.2.3-6) |  |
| QoS rule |  |  | IMS_BDC |
| QoS rule identifier | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-12 | Packet filter list #12 |  |
| QoS rule precedence | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’01 0000’B | QFI 16 (Table 4.8.2.3-12) |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |
| IMS_BDC | If this QoS rule is used to setup an IMS bootstrap data channel session |

Table 4.8.2.1-8: Reference QoS rule #8

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0111’B | 7 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1001’B | QFI 9 (Table 4.8.2.3-7) |  |

Table 4.8.2.1-9: Reference QoS rule #9

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1000’B | 8 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1100’B | QFI 10 (Table 4.8.2.3-8) |  |

Table 4.8.2.1-10: Reference QoS rule #10 (MCX signalling)

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1001’B | 9 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1101’B | QFI 11 (Table 4.8.2.3-9) |  |

Table 4.8.2.1-11: Reference QoS rule #11 (MCX user plane data)

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1010’B | 10 (unique per PDU session) | MCPTT |
|  | ‘0000 1011’B | 11 (unique per PDU session) | MCVIDEO |
|  | ‘0000 1100’B | 12 (unique per PDU session) | MCDATA |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0010’B | 2 packet filters | MCPTT |
|  | ‘0011’B | 3 packet filters | MCVIDEO |
|  | ‘0001’B | 1 packet filter | MCDATA |
| Packet filter list | See table 4.8.2.2-8 | Packet filter list #8 | MCPTT |
|  | See table 4.8.2.2-9 | Packet filter list #9 | MCVIDEO |
|  | See table 4.8.2.2-10 | Packet filter list #10 | MCDATA |
| QoS rule precedence | ‘0000 0001’B | 1 (Note 1) | MCPTT |
|  | ‘0000 0010’B | 2 (Note 1) | MCVIDEO |
|  | ‘0000 0011’B | 3 (Note 1) | MCDATA |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1100’B | QFI 12 (Table 4.8.2.3-10) | MCPTT |
|  | ’00 1101’B | QFI 13 (Table 4.8.2.3-10) | MCVIDEO |
|  | ’00 1110’B | QFI 14 (Table 4.8.2.3-10) | MCDATA |
| Note 1: In case of parallel MCX sessions the precedences would need to be unique within the PDU session. |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCPTT | QoS rule for MCPTT user plane data |
| MCVIDEO | QoS rule for MCVideo user plane data |
| MCDATA | QoS rule for MCData user plane data |

4.8.2.2	Reference packet filters

Table 4.8.2.2-1: Packet filter list #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0001’B | Id 1 |  |
| Component type 1 ID | ‘0000 0001’B | Match-all type |  |

Table 4.8.2.2-2: Packet filter list #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0010’B | Id 2 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.2 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C002 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-3: Packet filter list #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0011’B | Id 3 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.3 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C003 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-3a: Packet filter list #3a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘1111’B | Id 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.30 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C030 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-4: Packet filter list #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0100’B | Id 4 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.4 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C004 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-5: Packet filter list #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0101’B | Id 5 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.5 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C005 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-6: Packet filter list #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | 0110’B | Id 6 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 4.8.2.2-7: Packet filter list #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | 0111’B | Id 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 4.8.2.2-8: Packet filter list #8 (MCPTT)

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter [1] |  | Speech |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1000’B | Id 8 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port) |  |
|  | media port + 1 | RTCP remote port |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Packet filter [2] |  | Floor control |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1001’B | Id 9 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port |  |
| Component type 1 Value | media port | SS floor control port as used in the SDP negotiation |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |

Table 4.8.2.2-9: Packet filter list #9 (MCVideo)

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter [1] |  | Speech |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1000’B | Id 8 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port) |  |
|  | media port + 1 | RTCP remote port |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Packet filter [3] |  | Video |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1001’B | Id 9 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port) |  |
|  | media port + 1 | RTCP remote port |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Packet filter [3] |  | Transmission control |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1010’B | Id 10 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port |  |
| Component type 1 Value | media port | SS transmission control port as used in the SDP negotiation |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |

Table 4.8.2.2-10: Packet filter list #10 (MCData)

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter [1] |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1000’B | Id 8 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port |  |
| Component type 1 Value | media port | Port used for the data stream (e.g. MSRP) |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 6 | TCP |  |

Table 4.8.2.2-12: Packet filter list #12

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | '1001’B | Id 9 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port |  |
| Component type 1 Value | media port | SS application media port as used in the SDP negotiation. (Note 1) |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

4.8.2.3	Reference QoS flow descriptions

Table 4.8.2.3-1: Reference QoS flow #1

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' |  | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2: Reference QoS flow #2

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0101’B | 5QI 5 |  |
| EPS bearer identity | ‘0101 0000’B | EBI 5 | Interworking_with_EPS |
| EPS bearer identity | Any not yet assigned value different from ‘5’ |  | EmergencySession AND Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |
| EmergencySession | If this flow is used in the Authorized QoS flow descriptions IE of a PDU Session Establishment Accept message for an emergency PDU session |

Table 4.8.2.3-2a: Reference QoS flow #2a

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0100’B | QFI 4 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-3: Reference QoS flow #3

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0101’B | QFI 5 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-4: Reference QoS flow #4

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0110’B | QFI 6 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-5: Reference QoS flow #5

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0111’B | QFI 7 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameter |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0001’B | 5QI 1 |  |
| GFBR uplink | ‘040002’H | 128 Kbps |  |
| GFBR downlink | ‘040002’H | 128 Kbps |  |
| MFBR uplink | ‘040005’H | 320 Kbps |  |
| MFBR downlink | ‘040005’H | 320 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 6 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-6: Reference QoS flow #6

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1000’B | QFI 8 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameter |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0010’B | 5QI 2 |  |
| GFBR uplink | ‘020012’H | 72 Kbps |  |
| GFBR downlink | ‘020012’H | 72 Kbps |  |
| MFBR uplink | ‘030033’H | 816 Kbps |  |
| MFBR downlink | ‘030033’H | 816 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 7 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-7: Reference QoS flow #7

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 9 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0101 0010’B | 5QI 82 |  |

Table 4.8.2.3-8: Reference QoS flow #8

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 10 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0101 0010’B | 5QI 9 |  |

Table 4.8.2.3-9: Reference QoS flow #9 (MCX signalling)

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1011’B | QFI 11 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0100 0101’B | 5QI 69 MCX signalling according to TS 23.501 [68] Table 5.7.4-1 |  |

Table 4.8.2.3-10: Reference QoS flow #10 (MCX user plane data)

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1100’B | QFI 12 | MCPTT |
|  | ‘00 1101’B | QFI 13 | MCVIDEO |
|  | ‘00 1110’B | QFI 14 | MCDATA |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0100 0001’B | 5QI 65 MCPTT user plane according to TS 23.501 [68] Table 5.7.4-1 | MCPTT |
|  | ‘0100 0011’B | 5QI 67 MCVideo user plane according to TS 23.501 [68] Table 5.7.4-1 | MCVIDEO |
|  | ‘0100 0110’B | 5QI 70 MCData user plane according to TS 23.501 [68] Table 5.7.4-1 | MCDATA |

Table 4.8.2.3-12: Reference QoS flow #12

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘01 0000’B | QFI 16 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 8 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

<End of modified section 1>