3GPP TSG-RAN5 Meeting #107	R5-253407

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3479 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Message exception for 4Tx UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New UL MIMO with 4Tx test cases (e.g 6.3D.1_2) refer the condtion 4TX_UL_MIMO in the test procedure and the message contents, but this condition hasn’t been added in the 38.508-1. In addition, ULFPTx_Mode1 and ULFPTx_Mode2 condition in DCI format 0-1 are only apply to 2Tx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Update default physical layer parameters for DCI format 0_1 to include 4TX_UL_MIMO, ULFPTx_Mode1_4Tx and ULFPTx_Mode2_4Tx condition. 2.Update default PUSCH-Config and SRS-Config to include 4TX_UL_MIMO condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL MIMO with 4Tx test cases cannot be operated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.1.2, 4.6.3, 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-252482r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.1.1.2	Physical layer parameters for DCI format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_1 are specified in table 4.3.6.1.1.2-1.

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 |
|  | 11 | “1011” | 4TX_UL_MIMO or (ULFPTx_Mode1_4Tx and 2L_UL_MIMO) |
|  | 4 | “100” | ULFPTx_Mode1_4Tx and 1L_UL_MIMO |
|  | Dependent on UE reported full power PMI and higher layer parameter codebookSubset | 4 or 5 bit as per Table 7.3.1.1.2-3 in TS 38.212 | ULFPTx_Mode2_4Tx and 1L_UL_MIMO |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | Dependent on UE reported full power PMI | 4 bit as per Table 7.3.1.1.2-2 in TS 38.212 | ULFPTx_Mode2_4Tx and 2L_UL_MIMO |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| ChannelAccess-CPext-CAPC | Not present (0 bit) | - |  |
|  | 2 as per Table 7.3.1.1.1-4A in TS 38.212 | "10" | SharedSpectrum AND pc_ul_Semi_StaticChAccess_r16 |
| Open-loop power control parameter set indication | Not present | - |  |
| Priority indicator | Not present | - |  |
| Invalid symbol pattern indicator | Not present | - |  |
| Minimum applicable scheduling offset indicator | Not present | - |  |
| SCell dormancy indication | Not present | - |  |
| Sidelink assignment index | Dependent on test parameters 2 bits |  | SL_Mode1 |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| 4TX_UL_MIMO | UL-MIMO test cases with 4 Tx antenna ports |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 and 2 Tx antenna ports |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 and 2 Tx antenna ports |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power and 2 Tx antenna ports |
| SL_Mode1 | For SL mode 1 transmission test cases. |
| SharedSpectrum | Operation with shared spectrum channel access |
| 2L_UL_MIMO | UL-MIMO test cases with 2 layers |
| 1L_UL_MIMO | UL-MIMO test cases with 1 layer |
| ULFPTx_Mode1_4Tx | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 and 4 Tx antenna ports |
| ULFPTx_Mode2_4Tx | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 and 4 Tx antenna ports |

<End of modified section 1>

<Start of modified section 2>

–	PUSCH-Config

Table 4.6.3-118: PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPUSCH | Not present |  |  |
| txConfig | Not Present |  | Short_DCI |
|  | codebook |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB | Not present |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| frequencyHopping | Not present |  |  |
| frequencyHoppingOffsetLists | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| pusch-AggregationFactor | Not present |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| transformPrecoder | enabled |  | TRANSFORM_PRECODER_ENABLED |
|  | Not present | TRANSFORM_PRECODER_DISABLED |  |
| codebookSubset | Not present |  | Short_DCI |
|  | nonCoherent |  |  |
| maxRank | Not present |  | Short_DCI |
|  | 1 |  |  |
|  | 2 |  | 2TX_UL_MIMO,2L_UL_MIMO |
|  | 4 |  | 4TX_UL_MIMO |
| rbg-Size | Not present |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| tp-pi2BPSK | Not present |  |  |
| maxRank-v1810 | Not present |  |  |
| sTx-2Panel-r18 | Not present |  |  |
| multipanelSchemeSDM-r18 | Not present |  |  |
| multipanelSchemeSFN-r18 | Not present |  |  |
| codebookTypeUL-r18 | Not present |  |  |
| applyIndicatedTCI-State-r18 | Not present |  |  |
| dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 | Not present |  |  |
| dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 | Not present |  |  |
| pusch-ConfigDCI-0-3-r18 | Not present |  |  |
| pusch-ConfigDCI-0-3-r18 CHOICE { |  |  | DCI_0_3 |
| setup SEQUENCE { |  |  |  |
| resourceAllocationDCI-0-3-r18 | resourceAllocationType1 |  |  |
| rbg-SizeDCI-0-3-r18 | Not present |  |  |
| resourceAllocationType1GranularityDCI-0-3-r18 | Not present | granularity is 1 PRB |  |
| numberOfBitsForRV-DCI-0-3-r18 | 2 |  |  |
| harq-ProcessNumberSizeDCI-0-3-r18 | 4 |  |  |
| uci-OnPUSCH-ListDCI-0-3-r18 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH { |  |  |  |
| UCI-OnPUSCH[1] SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-AccessConfigListDCI-0-1-r16 | Not present |  |  |
| ul-AccessConfigListDCI-0-1-r16 CHOICE { |  |  | SharedSpectrum |
| setup SEQUENCE { |  |  |  |
| ul-AccessConfigListDCI-0-1-r16 SEQUENCE (SIZE (1..64)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 28 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (DFT-s-OFDM UL waveform is configured) |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| 4TX_UL_MIMO | UL-MIMO test cases with 4 Tx antenna ports |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| DCI_0_3 | Transmission of DCI_0_3 is required |
| SharedSpectrum | Operation with shared spectrum channel access |
| 2L_UL_MIMO | UL-MIMO test cases with 2 layers |

<End of modified section 2>

<Start of modified section 3>

–	SRS-Config

Table 4.6.3-182: SRS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | 2TX_UL_MIMO |
|  | ports4 |  | 4TX_UL_MIMO |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
|  | 0 |  | FR2_MBWP |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { | SRS-SpatialRelationInfo |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| 4TX_UL_MIMO | UL-MIMO test cases with 4 Tx antenna ports |
| FR1_100MHz | FR1 is used under the test. CBW is set to 100MHz. |
| FR2_100MHz | FR2 is used under the test. CBW is set to 100MHz. |
| FR2_MBWP | FR2 is used under the test. Test case has multiple dedicated BWP. |

<End of modified section 3>

<Start of modified section 4>

Table 5.4.1-4: PUSCH-Config

| Derivation Path: Clause 4.6.3 Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| maxRank | 2 |  | 2TX_UL_MIMO or 2L_UL_MIMO |
|  | 4 |  | 4TX_UL_MIMO |
|  | 1 |  | ULFPTx_Mode1 or ULFPTx_Mode2 or ULFPTx_ModeFull |
| ul-FullPowerTransmission-r16 | Not present |  | 2TX_UL_MIMO or 4TX_UL_MIMO |
|  | fullpowerMode1 |  | ULFPTx_Mode1 |
|  | fullpowerMode2 |  | ULFPTx_Mode2 |
|  | fullpower |  | ULFPTx_ModeFull |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| 4TX_UL_MIMO | UL-MIMO test cases with 4 Tx antenna ports |
| 2L_UL_MIMO | UL-MIMO test cases with 2 layers |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

<End of modified section 4>