3GPP TSG-RAN5 Meeting #107	R5-253496

# Malta, May 19 - 23 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3958 | rev | 1 | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ATG SA event triggered reporting test cases 19.5.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA event triggered reporting test cases test cases are missing in TS 38.533 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initial versions of the test cases were added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain missing and the WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.5.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Final revision of R5-252789 to add Editor’s Notes regarding AT commants and antenna array. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

## 19.5	Measurement procedure for ATG

### 19.5.1	Intra-frequency measurements

#### 19.5.1.0	Minimum conformance requirements

TBD

#### 19.5.1.1	SA FR1 event triggered reporting tests without gap without SSB index reading under non-DRX

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- Message contents are FFS

- Test procedure is missing

- AT Command to set the UE speed and position is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.1.1.1	Test purpose

The purpose of this test is to TBD

19.5.1.1.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause TBD.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.1.1.

19.5.1.1.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.1.1.4.1-1

Table 19.5.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.5.1.1-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.1-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.1-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Test environment parameters are given in Table 19.5.1.1.4.1-2.

Table 19.5.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.1.1.4.1-3 and 19.5.1.1.4.1-4.

2.	Message contents are defined in clause 19.5.1.1.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

Table 19.5.1.1.4.1-3: General test parameters for TBD

TBD

19.5.1.1.4.2	Test procedure

TBD

19.5.1.1.4.3	Message contents

TBD

19.5.1.1.5	Test requirements

Table 19.5.1.1.4.1-3, 19.5.1.1.5-1, define the primary level settings for all tests.

Table 19.5.1.1.5-1: Cell specific test parameters for TBD

TBD

TBD

#### 19.5.1.2	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX

#### 19.5.1.3	SA FR1 event triggered reporting tests without gap under non-DRX with SSB index reading

#### 19.5.1.4	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading

### 19.5.2	Inter-frequency measurements

#### 19.5.2.0	Minimum conformance requirements

TBD

#### 19.5.2.1	SA FR1-FR1 event triggered reporting tests without SSB time index detection when DRX is used

#### 19.5.2.2	SA FR1-FR1 event triggered reporting tests with SSB time index detection when DRX is not used

#### 19.5.2.3	SA FR1-FR1 event triggered reporting tests without gap when DRX is not used

## 19.6	Measurement Performance requirements for ATG