3GPP TSG-RAN5 Meeting #107	R5-253603

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0093 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR NTN - Removing 256QAM - Core specs alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Thales, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In NR NTN core specs, since Release 17, 256QAM modulation has been removed arguing “256QAM is a very unrealistic scenario for NTN never agreed previously in any RAN4 meetings in the context of NTN (in Rel-17, Rel-18 or even on-going Rel-19)”. See R4-2502947 and R4-2502343. RAN5 specs 38.521-5 should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed any reference to 256QAM modulation in 38.521-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specifcation will remain misaligned with core specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.3, 6.4.2.1, 6.4.2.1a, 6.5.2.2, A.2.2.5, A.2.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note for Editors: please remove the empty rows in Table 6.2.2.3-1, Table 6.2.3.3.2-2, Table 6.2.3.3.3-2, Table 6.2.3.3.4-2, Tables 6.2.3.4.1-1/2/4/5/6, Table 6.5.2.2.4.1-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -Added ZTE as co-sourcing company to resolve overlap with R5-252866 that will be withdrawn. -Added missing change in section 6.2.3.1. -Corrected indices in Table 6.2.3.4.1-2. -Updated Table 6.2.3.5-3 accordingly. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 6.2.2	UE maximum output power reduction

#### 6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

#### 6.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

NOTE:	Test execution is not necessary if TS 38.521-5 clause 6.5.2.4.1 is executed.

#### 6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-1 for channel bandwidths ≤ 100 MHz. The ∆MPR is set to zero.

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 |  | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 |  |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |

RB allocation ranges for Outer and Inner RB allocations are specified in TS 38.521-1 [2] 6.2.2.3.

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.2.

#### 6.2.2.4	Test description

##### 6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | N/A for Maximum Power | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Default |  | CP-OFDM QPSK | Inner Full |
| 17 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 18 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 19 | Default |  | CP-OFDM QPSK | Outer Full |
| 20 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 21 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 22 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 23 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 24 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 273 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 283 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 293 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.2.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

##### 6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1 ms over consecutive active uplink slots.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

##### 6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [12] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: DMRS-UplinkConfig Test ID 33 - 35 in Table 6.2.2.4.1-1

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.4.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 [12] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1.

Table 6.2.2.5-1: UE MPR test requirements for power class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 2 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 3 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 4 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 6 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 17 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 21 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 23 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 27 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 28 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 29 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 30MHz | 0.7 dB |

## <<< Skip unchanged sections >>>

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1 except for 256QAM. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2 except for 256QAM. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

#### 6.2.3.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

#### 6.2.3.3	Minimum conformance requirements

##### 6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.3.1 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.3.1-3, and Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3.2 | n254 | 5, 10, 15 |  | Clause 6.2.3.3.2 |
| NS_04N | 6.5.3.3.3.3 | n254 | 5 |  | Clause 6.2.3.3.3 |
| NS_05N | 6.5.3.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.3.4 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. |  |  |  |  |  |

Table 6.2.3.3.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-3: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |

#### 6.2.3.3.2	A-MPR for NS_03N

Table 6.2.3.3.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.3.2-2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

#### 6.2.3.3.3	A-MPR for NS_04N

Table 6.2.3.3.3-1: A-MPR regions for NS_04N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

Table 6.2.3.3.3-2: A-MPR for NS_04N

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |

#### 6.2.3.3.4	A-MPR for NS_05N

Table 6.2.3.3.4-1: A-MPR regions for NS_05N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | <= 3.6 | > 0.36 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.3.4-2: A-MPR for NS_05N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |

##### 6.2.3.3.5	A-MPR for NS_24

Table 6.2.3.3.5-1: A-MPR regions for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | RBend*12*SCS MHz | LCRB*12*SCS MHz | RBend*12*SCS MHz | LCRB*12*SCS MHz |
| 5MHz | Fc=1,992.5 |  | >3.24 |  |  |  |  |
| 5MHz | Fc=1,997.5 |  | >3.24 |  |  |  |  |
| 5MHz | Fc=2,002.5 |  | >2.16 | >3.78 | ≤1.98 | ≤3.6 | ≤1.98 |
| 10MHz | Fc=1,985 | >5.4 |  |  |  |  |  |
| 10MHz | Fc=1,995 |  | >4.5 | >7.56 | ≤4.32 | ≤7.38 | ≤4.32 |
| 10MHz | Fc=2,000 | >6.84 |  | <2.88 |  | ≥3.06 ≤6.66 | >1.44 |
| 15MHz | Fc=1,987.5 |  | >7.02 | >11.52 | ≤6.84 | ≤11.34 | ≤6.84 |
| 15MHz | Fc=1,997.5 | >9.36 |  | <3.6 |  | ≥3.78 ≤9.18 | >1.44 |
| 20MHz | Fc=1,990 | >13.5 |  | <4.5 |  | ≥4.68 ≤13.32 | >2.16 |
| 20MHz | Fc=1,995 | >12.6 |  | <5.4 |  | ≥5.58 ≤12.42 | >1.44 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.2-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |

Table 6.2.3.3.5-2: A-MPR for NS_24

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.3.1.

#### 6.2.3.4	Test description

##### 6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in clause A.2. Configurations of PDSCH and PDCCH before measurement are specified in clause C.2.

Table 6.2.3.4.1-1: Test Configuration table for NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s OFDM | Pi/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | Pi/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 16 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 19 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_24

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Refer to test parameters (5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 1-4 | 1,992.5 |  | 5 |  |  | Default |  |  | N/A for A-MPR testing |  |  | DFT-s OFDM |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 5-8 | 1,997.5 |  | 5 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 9-20 | 2,002.5 |  | 5 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | Edge_1RB_Right | Edge_1RB_Left |
| 21-24 | 1,985 |  | 10 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Edge_1RB_Right |  | N/A | N/A |
| 25-28 | 1,985 |  | 10 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 29-40 | 1,995 |  | 10 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | Edge_1RB_Right | Edge_1RB_Left |
| 41-52 | 2,000 |  | 10 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Edge_1RB_Right |  | Edge_1RB_Left | 36@0 |
| 53-56 | 2,000 |  | 10 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 57-68 | 1,987.5 |  | 15 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | Edge_1RB_Right | Edge_1RB_Left |
| 69-80 | 1,997.5 |  | 15 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Edge_1RB_Right |  | Edge_1RB_Left | 50@0 |
| 81-84 | 1,997.5 |  | 15 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 85-96 | 1,990 |  | 20 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Edge_1RB_Right |  | Edge_1RB_Left | 74@0 |
| 97-100 | 1,990 |  | 20 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 101-112 | 1,995 |  | 20 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Edge_1RB_Right |  | Edge_1RB_Left | 69@0 |
| 113-116 | 1,995 |  | 20 |  |  | Default |  |  |  |  |  |  |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM |  | Outer_Full |  | N/A | N/A |
| 117-119 |  | 1,992.5 |  |  | 5 |  |  | Default |  |  |  |  |  | CP-OFDM |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 120-122 |  | 1,997.5 |  |  | 5 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 123-131 |  | 2,002.5 |  |  | 5 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 132-134 |  | 1,985 |  |  | 10 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Edge_1RB_Right | N/A | N/A |
| 135-137 |  | 1,985 |  |  | 10 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 138-146 |  | 1,995 |  |  | 10 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 147-155 |  | 2,000 |  |  | 10 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 156-158 |  | 2,000 |  |  | 10 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 159-167 |  | 1,987.5 |  |  | 15 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 168-176 |  | 1,997.5 |  |  | 15 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 177-179 |  | 1,997.5 |  |  | 15 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 180-188 |  | 1,990 |  |  | 20 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 189-191 |  | 1,990 |  |  | 20 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 192-200 |  | 1,995 |  |  | 20 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 201-203 |  | 1,995 |  |  | 20 |  |  | Default |  |  |  |  |  |  |  | QPSK 16 QAM 64 QAM |  | Outer_Full | N/A | N/A |
| 204 | 1,992.5 | 5 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 205 | 1,997.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 206 | 1,985 | 10 | Default |  |  | QPSK | Edge_1RB_Left | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: In test IDs with multiple modulations, each UL Modulation shall be tested separately against Range A, B, and C. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-3: FFS

Table 6.2.3.4.1-4: Test Configuration table for NS_03N

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Ch BW column) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 2 | Low | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 3 | Low | 5 | Default |  |  | QPSK | Outer_Full (A3) | N/A | N/A |
| 4 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 5 | Low | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 6 | Low | 10 | Default |  |  | QPSK | 10@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 7 | Low | 10 | Default |  |  | QPSK | 40@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 8 | Low | 10 | Default |  |  | QPSK | 36@0 (A6) | 18@0 (A6) | 9@0 (A6) |
| 9 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 10 | 1621.5 | 10 | Default |  |  | QPSK | Outer_Full(A1) |  |  |
| 11 | Low | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 12 | Low | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 13 | Low | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 14 | Low | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| 15 | Low | 5 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 16 | Low | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 17 | Low | 5 | Default |  |  | QPSK | Outer_Full (A3) | N/A | N/A |
| 18 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 19 | Low | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 20 | Low | 10 | Default |  |  | QPSK | 11@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 21 | Low | 10 | Default |  |  | QPSK | 41@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 22 | Low | 10 | Default |  |  | QPSK | 36@0 (A6) | 18@0 (A6) | 9@0 (A6) |
| 23 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 24 | 1621.5 | 10 | Default |  |  | QPSK | Outer_Full(A1) |  |  |
| 25 | Low | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 26 | Low | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 27 | Low | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 28 | Low | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-5: Test Configuration table for NS_04N

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 5 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 2 | Default | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 3 | Default | 5 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 4 | Default | 5 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 5 | Default | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 6 | Default | 5 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6: Test Configuration table for NS_05N

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10MHz, 15MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 5 | Default | N/A | DFT-s-OFDM | QPSK | 5@20 (A3) | N/A | N/A |
| 2 | Default | 5 | Default |  |  | QPSK | Outer_Full (A1) | N/A | N/A |
| 3 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 4 | Default | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 5 | Default | 10 | Default |  |  | QPSK | 10@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 6 | Default | 10 | Default |  |  | QPSK | 40@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 7 | Default | 10 | Default |  |  | QPSK | 40@0 (A6) | 20@0 (A6) | 10@0 (A6) |
| 8 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 9 | Default | 10 | Default |  |  | QPSK | Edge_1RB_right(A6) |  |  |
| 10 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 11 | Default | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 12 | Default | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 13 | Default | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| 14 | Default | 5 | Default | N/A | CP-OFDM | QPSK | 5@20 (A3) | N/A | N/A |
| 15 | Default | 5 | Default |  |  | QPSK | Outer_Full (A1) | N/A | N/A |
| 16 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 17 | Default | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 18 | Default | 10 | Default |  |  | QPSK | 11@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 19 | Default | 10 | Default |  |  | QPSK | 41@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 20 | Default | 10 | Default |  |  | QPSK | 40@0 (A6) | 20@0 (A6) | 10@0 (A6) |
| 21 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 22 | Default | 10 | Default |  |  | QPSK | Edge_1RB_right(A6) |  |  |
| 23 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 24 | Default | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 25 | Default | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 26 | Default | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.1.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

##### 6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1 ms).

NOTE:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

##### 6.2.3.4.3	Message contents

6.2.3.4.3.1	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_100) |  |  |

6.2.3.4.3.2	Message contents exceptions for network signalling value "NS_24"

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_24) |  |  |

6.2.3.4.3.3	Message contents exceptions for network signalling value "NS_03N"

1.	Information element additionalSpectrumEmission is set to NS_03N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03N) |  |  |

6.2.3.4.3.4	Message contents exceptions for network signalling value "NS_04N"

1.	Information element additionalSpectrumEmission is set to NS_04N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_04N) |  |  |

6.2.3.4.3.5	Message contents exceptions for network signalling value "NS_05N"

1.	Information element additionalSpectrumEmission is set to NS_05N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05N) |  |  |

#### 6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-3. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-1: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 30MHz | 0.7 dB |

Table 6.2.3.5-2: UE Power Class 3 test requirements (NS_100) for n256

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_24) for n256

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 5, 9-11, 21, 25, 29-31, 41-43, 53, 57-59, 69-71, 81, 85-87, 97, 101-103, 113 | 23 | N/A | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2, 6, 12-13, 22, 26, 32-33, 44-45, 54, 60-61, 72-73, 82, 88-89, 98, 104-105, 114 | 23 | N/A | 2.0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3, 7, 14-16, 23, 27, 34-36, 46-48, 55, 62-64, 74-76, 83, 90-92, 99, 106-108, 115 | 23 | N/A | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4, 8, 17-18, 24, 28, 37-38, 49-50, 56, 65-66, 77-78, 84, 93-94, 100, 109-110, 116 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 19-20, 39-40, 51-52, 67-68, 79-80, 95-96, 111-112 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 117-118, 120-121, 123-127, 132-133, 135-136, 138-142, 147-151, 156-157, 159-163, 168-172, 177-178, 180-184, 189-190, 192-196, 201-202, 204-206 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 119, 122, 128-129, 134, 137, 143-144, 152-153, 158, 164-165, 173-174, 179, 185-186, 191, 197-198, 203 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 130-131, 145-146, 154-155, 166-167, 175-176, 187-188, 199-200 | 23 | N/A | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03N) for n254

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 2 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 3 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 4 | 23 | 1 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 5 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11.0 - TT |
| 6 | 23 | 1 | 2.0 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 7 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 8 | 23 | 1 | 2.0 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 9 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 10 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 11 | 23 | 1 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 12 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11.0 - TT |
| 13 | 23 | 1 | 2.0 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 15 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 16 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 17 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 18 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10.0 - TT |
| 19 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8.0 - TT |
| 20 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 21 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 22 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 23 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 24 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 25 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10.0 - TT |
| 26 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8.0 - TT |
| 27 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 28 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-5: UE Power Class 3 test requirements (NS_04N) for n254

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25 +TT | 18.0 - TT |
| 2 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25 +TT | 15.5 - TT |
| 3 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25 +TT | 18.0 - TT |
| 4 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25 +TT | 16.0 - TT |
| 5 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25 +TT | 12.0 - TT |
| 6 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25 +TT | 15.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6: UE Power Class 3 test requirements (NS_05N) for n254

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 2 | 23 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 3 | 23 | 1 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 4 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11 - TT |
| 5 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 6 | 23 | 1 | 5.0 | 0 | 18 | 4.0 | 2 | 25.0 + TT | 14 - TT |
| 7 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 8 | 23 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 10 | 23 | 1 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 11 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11 - TT |
| 12 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 13 | 23 | 1 | 5.0 | 0 | 18 | 4.0 | 2 | 25.0 + TT | 14 - TT |
| 14 | 23 | 3 | 0.0 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 15 | 23 | 3 | 3.0 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 16 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10 - TT |
| 17 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8 - TT |
| 18 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 3 | 6.5 | 0 | 16.5 | 4.0 | 2 | 25.0 + TT | 12.5 - TT |
| 20 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 3 | 3.0 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10 - TT |
| 24 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8 - TT |
| 25 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 3 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-2. |  |  |  |  |  |  |  |  |  |

## <<< Skip unchanged sections >>>

#### 6.4.2.1	Error Vector Magnitude

6.4.2.1.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in sub-clause 6.4.2.4.3. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and the duration of PUCCH/PUSCH channel, or one hop, if frequency hopping is enabled for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient as defined in subclause 6.3.3.3.

6.4.2.1.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

6.4.2.1.3	Minimum conformance requirements

The RMS average of the basic EVM measurements for 10 sub-frames excluding any transient period for the average EVM case, and 60 sub-frames excluding any transient period for the reference signal EVM case, for the different modulation schemes shall not exceed the values specified in Table 6.4.2.1.3-1 for the parameters defined in Table 6.4.2.1.3-2. For EVM evaluation purposes, all PRACH preamble formats 0-4 and all 5 PUCCH formats are considered to have the same EVM requirement as QPSK modulated.

Table 6.4.2.1.3-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |

Table 6.4.2.1.3-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | ³ Table 6.3.1.3-1 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 38.101-1 [5] clause 6.4.2.1, as indicated in TS 38.101-5 [11] clause 6.4.2.

6.4.2.1.4	Test description

6.4.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.2.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.2.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | All |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 3) | RB allocation (NOTE 1) |
| 13 |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 23 |  | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 |  | DFT-s-OFDM QPSK | Inner Full |
| 4 |  | DFT-s-OFDM QPSK | Outer Full |
| 5 |  | DFT-s-OFDM 16 QAM | Inner Full |
| 6 |  | DFT-s-OFDM 16 QAM | Outer Full |
| 7 |  | DFT-s-OFDM 64 QAM | Outer Full |
| 8 |  | CP-OFDM QPSK | Inner Full |
| 9 |  | CP-OFDM QPSK | Outer Full |
| 10 |  | CP-OFDM 16 QAM | Inner Full |
| 11 |  | CP-OFDM 16 QAM | Outer Full |
| 12 |  | CP-OFDM 64 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |

Table 6.4.2.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | See Table 6.4.2.1.4.1-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | See Table 6.4.2.1.4.1-1 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | See Table 6.4.2.1.4.1-1 |  |  |
| Test Parameters |  |  |  |  |  |
| ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | RB allocation | Waveform | PUCCH format | RB index |
| 1 | CP-OFDM QPSK | Full RB (Note 1) | CP-OFDM | PUCCH format = Format 1 Length in OFDM symbols = 14 | 0 |
| 2 | CP-OFDM QPSK | Full RB (Note 1) | CP-OFDM | PUCCH format = Format 1 Length in OFDM symbols = 14 | NRB-1 |
| 3 | CP-OFDM QPSK | Full RB (Note 1) | DFT-s-OFDM | PUCCH format = Format 3 (Note 3) Length in OFDM symbols = 14 | 0 |
| 4 | CP-OFDM QPSK | Full RB (Note 1) | DFT-s-OFDM | PUCCH format = Format 3 (Note 3) Length in OFDM symbols = 14 | NRB-1 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: For FDD, set K1 value (PDSCH-to-HARQ-timing-indicator) as follows: K1 = 2 if mod(i,5) = 0 K1 = 2 if mod(i,5) = 1 K1 = 4 if mod(i,5) = 2 K1 = 3 if mod(i,5) = 3 K1 = 2 if mod(i,5) = 4 where i is slot index per frame |  |  |  |  |  |

Table 6.4.2.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 | Normal |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 | See Table 6.4.2.1.4.1-1 |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 | See Table 6.4.2.1.4.1-1 |
| Test SCS as specified in Table 5.3.5-1 | SCS defined in TS 38.211 [17] subclause 6.3.3.2 determined by PRACH Configuration Index |
| PRACH preamble format |  |
|  | FDD |
| PRACH Configuration Index | 17 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, in Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6. 	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.4.2.1.4.3.

6.4.2.1.4.2	Test procedure

Test procedure for PUSCH:

1.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.3.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

1.4.	Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin, where:

-	Pmin is the minimum output power according to Table 6.3.1.3-1.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [5], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

1.5.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

NOTE1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.4.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

Table 6.4.2.1.4.2-1: Void

Test procedure for PUCCH:

2.1.	PUCCH is set according to Table 6.4.2.1.4.1-2.

2.2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 6.4.2.1.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. There is no PUSCH transmission.

2.3.	SS send appropriate TPC commands for PUCCH to the UE until the UE transmit PUCCH at PUMAX level. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.4.	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

2.5. Send uplink power control commands for PUCCH to the UE using 1dB power step size to ensure that the UE PUCCH output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin, where:

-	Pmin is the minimum output power according to Table 6.3.1.3-1.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 2.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where the UE power step tolerance is specified in TS 38.101-1 [5], Table 6.3.4.3-1 and is 2.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

2.6.	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

NOTE1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.4.2.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

Test procedure for PRACH:

3.1.	The SS shall set RS EPRE according to Table 6.4.2.1.4.1-3.

3.2.	PRACH is set according to Table 6.4.2.1.4.1-3.

3.3.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure.

3.4.	The UE shall send the signalled preamble to the SS.

3.5.	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

3.6.	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated PRACH transmission power.

3.7.	Repeat step 5 and 6 until the SS collect enough PRACH preambles. Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E).

6.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 with the following exceptions:

Table 6.4.2.1.4.3-1: RACH-ConfigCommon: PRACH measurement

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | Not present |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| one | n8 |  | FR1 |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf64 |  |  |
| rsrp-ThresholdSSB | RSRP-Range |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
|  | RSRP-Range |  | SUL |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  | PRACH Format A3 |
| l839 | 0 | NR Cell 1 | PRACH Format 0 |
|  | TBD | Other than NR Cell 1 | PRACH Format 0 |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| } |  |  |  |

Table 6.4.2.1.4.3-2: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 17 | Paired Spectrum | PRACH Format 0 |
|  | 7 | Unpaired Spectrum | PRACH Format 0 |
| msg1-FDM | four |  | FR1 |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 15 |  |  |
| preambleReceivedTargetPower | -92 |  | Test point 1 |
|  | -74 |  | Test point 2 |
| preambleTransMax | n7 |  |  |
| powerRampingStep | dB0 |  |  |
| ra-ResponseWindow | sl20 |  |  |
| } |  |  |  |

Table 6.4.2.1.4.3-3: PUSCH-TimeDomainResourceAllocationList: PRACH measurement

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [7] Table 8.2-1. |  |
| k2 | 6 | K2+ Δ=8 acc. to TS 38.214 [16] Table 6.1.2.1.1-5 | Unpaired Spectrum for SCS15kHz and PRACH Format 0 |
| } |  |  |  |
| } |  |  |  |

Table 6.4.2.1.4.3-4: PDSCH-ServingCellConfig: PUCCH format3 measurement

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| xOverhead | Not present |  |  |
| nrofHARQ-ProcessesForPDSCH | n6 |  | FDD |
| pucch-Cell | Not present |  |  |
| maxMIMO-Layers | Not present |  |  |
| processingType2Enabled | Not present |  |  |
| pdsch-CodeBlockGroupTransmissionList-r16 | Not present |  |  |
| } |  |  |  |

6.4.2.1.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, shall not exceed the values in Table 6.4.2.1.5-1.

The PUSCH, derived in Annex E.4.6.2, shall not exceed the values in Table 6.4.2.1.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4.2.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 + TT |
| QPSK | % | 17.5 + TT |
| 16 QAM | % | 12.5 + TT |
| 64 QAM | % | 8 + TT |
| Note 1: TT is defined in Table 6.4.2.1.5-2. |  |  |

Table 6.4.2.1.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 0 |
| QPSK | % | 0 |
| 16 QAM | % | 0 |
| 64 QAM | % | 0 |

The PUCCH EVM derived in Annex E.5.9.2 shall not exceed 17.5%.

The PRACH EVM derived in Annex E.6.9.2 shall not exceed 17.5%.

## <<< Skip unchanged sections >>>

#### 6.4.2.1a	Error Vector Magnitude including symbols with transient period

6.4.2.1a.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

6.4.2.1a.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation and support short transient period capability.

6.4.2.1a.3	Minimum conformance requirements

In 6.4.2.1, EVM has been defined by excluding the symbols which have a transient period. In this section, measurement interval is defined for the symbols with a transient period to include these symbols in the RMS average EVM computation when the UE reports a transient period capability other than the default. Before calculating the EVM, the measured waveform is corrected for sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM. The symbols with transient period should not be used for equalization. Only CP-OFDM waveform is used for conformance testing.

In the case of PUSCH or PUCCH transmissions when the mean power, modulation or RB allocation across slot or subslot boundaries is expected to change the EVM result over the symbols where the transient occurs is calculated according to Table 6.4.2.1a.3-1.

Table 6.4.2.1a.3-1: EVM definition for reported transient period

| Reported transient capability (us) | EVM definition | tpstart (µs) | SCS4 |
| --- | --- | --- | --- |
| 2 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | -0.5 | 15kHz or 30kHz5 |
| 4 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | -1 | 15kHz |
| 7 | EVMafter=min⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | -2.7 | 15kHz |
| NOTE 1: EVMl , EVMh, EVMl_tp,and EVMh_tp are defined in Annex E.4.7 NOTE 2: EVMafter is the EVM for a symbol right after a transition; EVMbefore is the EVM for a symbol right before a transition NOTE 3: tpstart denotes the start position of the EVM exclusion window as shown in Annex E.4.7 NOTE 4: SCS denotes the SCS that can be used in the conformance test NOTE 5: 30kHz shall be used in the conformance test unless the UE signals in supportedSubCarrierSpacingUL in FeatureSetPerCC that it only supports 15kHz in the corresponding band |  |  |  |

The RMS average of the basic EVM measurements over 108 subframes calculated only on the symbols where the transient occurs for the different modulation schemes shall not exceed the values specified in Table 6.4.2.1a.3-2 for the parameters defined in Table 6.4.2.1a.3-3.  This requirement can be verified with 64 QAM modulation.

Table 6.4.2.1a.3-2: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 10 |

Table 6.4.2.1a.3-3: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | ³ Table 6.3.1-1 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 38.101-1 [5] clause 6.4.2.1a, as indicated in TS 38.101-5 [11] clause 6.4.2.

6.4.2.1a.4	Test description

6.4.2.1a.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.2.1a.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.2.1a.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 15 kHz (Note 3) |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM 64 QAM | Outer Full24@04 and 1@05 |
| NOTE 1: This RB allocation is used for all UL slots when UL power is fixed within the UL power control window. Refer to Table 6.4.2.1a.4.2-1 for the UL RB allocation to be used during the EVM measurement. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: For UE supporting 2 us transient period, 30kHz shall be used in the conformance test unless the UE signals in supportedSubCarrierSpacingUL in FeatureSetPerCC that it only supports 15kHz in the corresponding band. NOTE 4: Applicable to slots 3 and 9 for SCS 15 kHz, and slots 8 and 19 for SCS 30 kHz. NOTE 5: Applicable to slots 4 and 8 for SCS 15 kHz, and slots 9 and 18 for SCS 30 kHz. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, in Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4.2.1a.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.4.2.1a.4.3.

6.4.2.1a.4.2	Test procedure

Figure 6.4.2.1a.4.2-1: Error Vector Magnitude including symbols with transient period

Figure 6.4.2.1a.4.2-2: Test power patterns for Error Vector Magnitude including symbols with transient period

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.2.1a.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slots 3, 4, and 8, 9 for 15 kHz SCS, on slots 8, 9, and 18, 19 for 30 kHz SCS.

2.	Set Uplink RB allocation as 24@0 as defined in Table 6.4.2.1a.4.1-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level +11.5 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

3.	Schedule the UE's PUSCH data transmission as described in Figure 6.4.2.1a.4.2-2 and Uplink RB allocation pattern as defined in Table 6.4.2.1a.4.1-1 while transmitting 0 dB TPC command for PUSCH via the PDCCH.

4.	Measure the EVMbefore and EVMafter using Global In-Channel Tx-Test (Annex E) applying tpstart of Table 6.4.2.1a.3-1 according to the declared enhanced transient capability.

5.	Repeat test steps 3~4 with UL RMC according to Table 6.4.2.1a.4.1-1 until the SS collect enough EVMbefore and EVMafter.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

6.4.2.1a.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.4.2.1a.5	Test requirement

The EVMbefore and EVMafter derived in Table 6.4.2.1a.3-1 shall not exceed the values in Table 6.4.2.1a.5-1. Both rising and falling edges of transient occurring at the respective slot boundaries are considered to have the same EVM requirements.

Table 6.4.2.1a.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 10 + TT |
| Note 1: TT is defined in Table 6.4.2.1a.5-2. |  |  |

Table 6.4.2.1a.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 0 |

## <<< Skip unchanged sections >>>

#### 6.5.2.2	Spectrum emission mask

##### 6.5.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth

##### 6.5.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

##### 6.5.2.2.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the ± edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth.

Table 6.5.2.2.3-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20 |  |
| ± 0-1 | -13 | -13 | 1 % of channel BW |
| ± 1-5 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 |  |  |
| ± 6-10 | -25 |  |  |
| ± 5-BWChannel |  | -13 |  |
| ± BWChannel-(BWChannel+5) |  | -25 |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.5.2.2.

##### 6.5.2.2.4	Test description

6.5.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.5.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

Table 6.5.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 24 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 34 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Edger_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 15 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 225,6 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 235,6 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 245,6 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: VOID NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 6: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508 [12] Table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and Table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State [to be updated] with generic procedure parameters [to be updated], Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.5.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2.4.1-1, Table 6.5.2.2.4.1-2, Table 6.5.2.2.4.1-2a and Table 6.5.2.2.4.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level. Allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.2.5-1 to 6.2.2.5-9. The period of the measurement shall be at least the continuous duration of 1 ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5.2.2.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [12] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents are according to TS 38.508-1 [12] subclause 4.6 with the following exceptions: SIB19 message contents according to TS 38.508-1 [12] clause 5.6.2.1. In addition the below PUSCH-Config IE needs to be configured.

Table 6.5.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [12] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5.2.2.4.3-2: DMRS-UplinkConfig (Test ID 28 – 30 in Table 6.5.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 6.5.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1, and the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1.

Table 6.5.2.2.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20 |  |
| ± 0-1 | -13+TT | -13+TT | 1 % of channel BW |
| ± 1-5 | -10+TT | -10+TT | 1 MHz |
| ± 5-6 | -13+TT |  |  |
| ± 6-10 | -25+TT |  |  |
| ± 5-BWChannel |  | -13+TT |  |
| ± BWChannel-(BWChannel+5) |  | -25+TT |  |

Table 6.5.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 100MHz | 1.5 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

## <<< Skip unchanged sections >>>

### A.2.2.5	Void

## <<< Skip unchanged sections >>>

### A.2.2.9	Void

## <<< END OF CHANGES >>>